3GPP TSG RAN WG1 #101			R1-2004036

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.5

Source: 	LG Electronics

Title: 	Discussion on UE features for URLLC/IIoT

Document for:	Discussion and decision

- Introduction

In this contribution, we provide our view on UE feature/capability for URLLC/IIoT.

NR_L1enh_URLLC

| LGE | 11-2 - Fine to remove component 2) 11-3 - No need of component 3), should be removed 11-4 - Need further discussion on component 4) 11-4a - Update FG description as “Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting PUCCH transmission associated with HARQ-ACK codebooks with different priorities at a UE” - Need further discussion on component 4) 11-4b - Share the similar view with Intel. Update FG description as “Dynamic indication of 2-level PHY priority for HARQ-ACK feedback in DCI scheduling PDSCH when configured to monitor both sets of DCI formats 0_1/1_1 and 0_2/1_2 in a BWP” - FG 11-1 and 11-4 should be the prerequisites. 11-6 - Similar to 5-17, 5-16 also needs to be considered as prerequisite. At least, it could be necessary to add note, “A UE supporting this feature and 5-20 shall also support 5-16” 11-7/7a - In Component 3, “cancelled symbol” needs to change to “cancelled resource” - Statements with bracket can be removed. It is not necessary to specify UE behavior in UE feature list. 11-7b - Fine to add this FG 11-9a - The text in ‘Note’ column should be removed |
| --- | --- |

NR_IIOT

| LGE | 12-1 - Update the component 1) as: “Configuration of 2-level PHY priority for CG PUSCH and SR, dynamic indication of 2-level PHY priority for dynamic scheduled PUSCH when configured to monitor either set of DCI formats 0_1/1_1 or 0_2/1_2 in a BWP ” - Fine to remove the bracket on prerequisite “[11-4]” - No need of xDD/FRx differentiation 12-1a - Fine to keep this FG but need of similar update for consistency to FG description: “dynamic indication of 2-level PHY priority for dynamic scheduled PUSCH when configured to monitor both sets of DCI formats 0_1/1_1 and 0_2/1_2 in a BWP ” - No need of xDD/FRx differentiation 12-3 - The text in ‘Note’ column should be removed |
| --- | --- |