GPP TSG RAN WG1 #101		R1-2004053

e-Meeting, May 25th – June 5th, 2020

Source:	OPPO

Title:	Remaining Issues on UL Positioning Reference Signal

Agenda Item:	7.2.8.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our views on various remaining issues of uplink positioning reference signals and provides text proposals for specification update.

- Discussion

  - TS 38.211

In Section 6.4.1.4.4 of TS 38.211, the periodicity and slot offset is specified for SRS-ResourceSet and SRS-PosResourceSet-r16. There is a redundancy description on SRS-PosResourceSet-r16. Thus we have the following proposal:

Proposal 1: Remove the redundant description on SRS-PosResourceSet-r16 from Section 6.4.1.4.4 of TS 38.211

The following give the corresponding text proposals for Proposal 1.

In TS 38.211 Section 6.4.1.4.4 CR R1-2003163 <omitted text> ##### 6.4.1.4.4 Sounding reference signal slot configuration For an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, a periodicity (in slots) and slot offset are configured according to the higher-layer parameter periodicityAndOffset-p or periodicityAndOffset-sp in the SRS-Resource IE or the SRS-PosResource-r16 IE, or periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 in the SRS-PosResource-r16 IE. Candidate slots in which the configured SRS resource may be used for SRS transmission are the slots satisfying SRS is transmitted as described in clause 11.1 of [5, TS 38.213]. <omitted text>

  - TS 38.213

Some RRC parameter in current version of TS 38.213 are not aligned with the one in TS 38.331. Thus we have the following proposal

Proposal 2: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- SRS-Positioning-Config   ->  SRS-PosResourceSet-r16

In TS 38.211, the differentiation of traditional SRS and SRS for positioning is based on whether or not the SRS resources are configured by SRS-Config. However, RAN2 has decided to configure both traditional SRS and SRS for positioning in the same SRS-Config. We need to differentiate these two types of SRS by using SRS-ResourceSet and SRS-PosResourceSet-r16.

Proposal 3: Use SRS-ResourceSet and SRS-PosResourceSet-r16 to differentiate the traditional SRS and SRS for positioning

The following gives the corresponding text proposal for proposal 2 and proposal 3.

In TS 38.213 Section 7 # 7 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by IE SRS-Positioning-Config SRS-PosResourceSet-r16 as described in Clause 7.3.1. A PUSCH/PUCCH/SRS/PRACH transmission occasion is defined by a slot index within a frame with system frame number , a first symbol within the slot, and a number of consecutive symbols . <omitted text>

In TS 38.213 Section 7 - 7.3.1 UE behaviour If a UE transmits SRS based on a configuration by IE SRS-Config SRS-ResourceSet on active UL BWP of carrier of serving cell using SRS power control adjustment state with index , the UE determines the SRS transmission power in SRS transmission occasion as [dBm] <omitted text>

In TS 38.213 Section 7 ##### Section 7.3.1 UE behaviour <omitted text> If a UE transmits SRS based on a configuration by IE SRS-Positioning-Config SRS-PosResourceSet-r16 on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO,SRS,b,f,cqs and αO,SRS,b,f,cqs are provided by p0-r16 and alpha-r16 respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-PosResourceSetId-r16 from SRS-PosResourceSet-r16, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in Clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos-r16 - if a ssb-IndexNcell-r16 is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS-ResourceId-r16 is provided, referenceSignalPower is provided by dl-PRS-ResourcePower-r16 <omitted text>

There are two typos in description of SRS for positioning in TS 38.213 and suggest to make the following changes to correct typos

Proposal 4: Correct the variables for the formula of power control for SRS for positioning

In TS 38.213 ##### Section 7.3.1 UE behaviour <omitted text> If a UE transmits SRS based on a configuration by IE SRS-Positioning-Config on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO_SRS,b,f,cqsPO,SRS,b,f,cqs and αSRS,b,f,cqs αO,SRS,b,f,cqs are provided by p0-r16 and alpha-r16 respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-PosResourceSetId-r16 from SRS-PosResourceSet-r16, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in Clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS-Pos-r16 - if a ssb-IndexNcell-r16 is provided, referenceSignalPower is provided by ss-PBCH-BlockPower-r16 - if a dl-PRS-ResourceId-r16 is provided, referenceSignalPower is provided by dl-PRS-ResourcePower-r16 If the UE determines that the UE is not able to accurately measure PLb,f,cqd, or the UE is not provided with pathlossReferenceRS-Pos-r16, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain for all SRS resource sets provided by SRS-PosResourceSet-r16 in addition to the up to four pathloss estimates that the UE maintains per serving cell for PUSCH/PUCCH/SRS transmissions. <omitted text>

- Conclusion

In this contribution, we presented our views on remaining issues of SRS for NR positioning and corresponding text proposals are provided. Based on the discussion, the following proposals are provided:

Proposal 1: Remove the redundant description on SRS-PosResourceSet-r16 from Section 6.4.1.4.4 of TS 38.211

Proposal 2: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- SRS-Positioning-Config   ->  SRS-PosResourceSet-r16

Proposal 3: Use SRS-ResourceSet and SRS-PosResourceSet-r16 to differentiate the traditional SRS and SRS for positioning

Proposal 4: Correct the variables for the formula of power control for SRS for positioning

- Reference

- TS 38.211, NR; Physical channels and modulation, v16.1.0

R1-2003163, Corrections to NR positioning support (CR for TS 38.211)

- TS 38.213, NR; Physical layer procedures for control, V16.1.0

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification, v16.0.0

- R1-2003179,  Corrections on Positioning (CR for TS 38.213)