3GPP TSG RAN WG1 Meeting #101-e	R1-2004163

E-meeting, May 25 – June 5, 2020

Agenda Item:	6.2.4.3

Source:	Huawei, HiSilicon

Title:	Corrections to 36.211 for the 2.5 kHz subcarrier spacing MBSFN

Document for:	Discussion and Decision

# Introduction

The work item, i.e., LTE-based 5G terrestrial broadcast [1] is complete and specification is in maintenance. The new numerology with 100us CP and 400us core symbol duration is specified for PMCH to support of mobility of up to 250km/h.

However, the current version of TS 36.211 [2] does not correctly reflect the 2.5 kHz numerology for PMCH in terms of the time duration for PMCH.

This contribution provides a Text Proposal to TS 36.211 accordingly.

# Discussion

In Table 6.7-1 of TS 36.211 V16.1.0 clause 6.7 as shown below, the number of OFDM symbol for PDCCH of MBSFN subframes/slots with Δf=2.5 kHz and Δf≈0.37 kHz are missing. In clause 6.1, for subframes/slots using , Δf=2.5 kHz, , or Δf=182944Ts≈0.37 kHz, the non-MBSFN region is of zero size. Therefore, the number of OFDM symbols for PDCCH of MBSFN subframes/slots withΔf=2.5 kHz and Δf≈0.37 kHz can be incorporated into the row for MBSFN subframes with or as highlighted in yellow as follows. The corresponding text proposal is provided in Appendix.

## 6.7 Physical control format indicator channel The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe. The set of OFDM symbols possible to use for PDCCH in a subframe is given by Table 6.7-1. Table 6.7-1: Number of OFDM symbols used for PDCCH Subframe; Number of OFDM symbols for PDCCH when; Number of OFDM symbols for PDCCH when. Subframe 1 and 6 for frame structure type 2 or a subframe for frame structure type 3 with the same duration as the DwPTS duration of a special subframe configuration; 1, 2; 2. MBSFN subframes with and configured with 1 or 2 cell-specific antenna ports; 1, 2; 2. MBSFN subframes with and configured with 4 cell-specific antenna ports; 2; 2. MBSFN subframes with or; 0; 0. Non-MBSFN subframes (except subframe 6 for frame structure type 2) configured with positioning reference signals; 1, 2, 3; 2, 3. All other cases; 1, 2, 3; 2, 3, 4. The UE may assume the PCFICH is transmitted when the number of OFDM symbols for PDCCH is greater than zero unless stated otherwise in [4, clause 12].

Proposal : Agree the text proposal for TS 36.211 as Appendix.

# References

RP-190732, “LTE-based 5G terrestrial broadcast,” Qualcomm Incorporated, RAN#83, Shenzhen, China, March 18-21, 2019.

TS 36.211, “Physical channels and modulation,” V16.1.0, March, 2020.

# Appendix: Text proposal for TS 36.211

### < Unchanged parts are omitted >

## 6.7	Physical control format indicator channel

The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe. The set of OFDM symbols possible to use for PDCCH in a subframe is given by Table 6.7-1.

Table 6.7-1: Number of OFDM symbols used for PDCCH

| Subframe | Number of OFDM symbols for PDCCH when | Number of OFDM symbols for PDCCH when |
| --- | --- | --- |
| Subframe 1 and 6 for frame structure type 2 or a subframe for frame structure type 3 with the same duration as the DwPTS duration of a special subframe configuration | 1, 2 | 2 |
| MBSFN subframes with and configured with 1 or 2 cell-specific antenna ports | 1, 2 | 2 |
| MBSFN subframes with and configured with 4 cell-specific antenna ports | 2 | 2 |
| MBSFN subframes with , Δf=2.5 kHz , or MBSFN slots with Δf≈0.37 kHz | 0 | 0 |
| Non-MBSFN subframes (except subframe 6 for frame structure type 2) configured with positioning reference signals | 1, 2, 3 | 2, 3 |
| All other cases | 1, 2, 3 | 2, 3, 4 |

The UE may assume the PCFICH is transmitted when the number of OFDM symbols for PDCCH is greater than zero unless stated otherwise in [4, clause 12].

## < Unchanged parts are omitted >