3GPP TSG RAN WG1 Meeting #101-e	R1-2004166

E-meeting, May 25 – June 5, 2020

Agenda Item:	6.2.5.3

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for LTE MIMO

Document for:	Discussion and Decision

# Introduction

In this contribution, our views on remaining issues of UE features for LTE DL MIMO [1] are provided.

# Discussion

In [1], there is a remaining issue of FG 3-1:

| 3-1 | Additional SRS symbols within normal UL subframes without frequency hopping | 1. Support of additional 1~13 SRS symbols within normal UL subframes with repetitions, |  | Yes | N/A | Network cannot utilize additional SRS symbols within normal UL subframes | Per UE | TDD only | N/A | FFS: How to capture the limitation that a UE may support additional SRS in cells with PUSCH, but not in PUSCH-less SCells. This may be realized by additional capability signaling (including new FG), or change the “additional SRS” (3-1) to per BoBC | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

On support of additional SRS in PUSCH-less SCells, there are two options considered:

- Option 1: An additional capability signaling (including new FG).

- Option 2: Change the FG 3-1 to per BoBC.

With option 1, a UE reports whether it can support additional SRS for carrier based switching. With option 2, UE report FG 3-1 per BoBC so that if there is carrier based switching in that band combination and the UE does not support combination with additional SRS, then UE just report no support of FG 3-1. However, with option 2, if a UE does not support the combination, then the UE will not be able to support additional SRS alone. Therefore, option 1 seems to be more flexible than option 2. In addition, with option 2, per BoBC will require more bits than option 1 if the new FG is per UE.

Based on the above discussion, the following is proposed:

Proposal 1: A new FG is introduced to indicate the support of additional SRS for carrier based switching, which is per UE and optional with capability signaling.

# Conclusions

In this contribution, our views on UE features for LTE-MIMO is provided. The following proposal is made.

Proposal 1: A new FG is introduced to indicate the support of additional SRS for carrier based switching, which is per UE and optional with capability signaling.

# References

R1-2003196, Summary on email discussion [100b-e-LTE-UEFeatures-Remaining], Moderators (AT&T, NTT DOCOMO), E-Meeting, 20th-30th April, 2020.