3GPP TSG RAN WG1 Meeting #101-e	R1-2004167

E-meeting, May 25 – June 5, 2020

Agenda Item:	6.2.1.1

Source:	Huawei, HiSilicon

Title:	Corrections on UE-group wake-up signal

Document for:	Discussion and Decision

# Introduction

In eMTC Rel-16, the agreements related to UE-group wake-up signal are summarized in [1]. This paper presents text proposals for TS 36.211.

# Discussion

Agreement If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

As shown in the agreements above, the common WUS can be configured to be legacy WUS or a non-legacy WUS.

The corresponding RRC signaling in the running CR of TS 36.331 [2] is as follows:

… (Copied from the running CR of TS 36.331 [2]) … GWUS-Config information element … GWUS-Config-r16 ::= SEQUENCE { … commonSequence-r16 ENUMERATED {g0, g126} OPTIONAL, -- Need OR … GWUS-Config field descriptions. commonSequence Presence of the field indicates common WUS sequence is configured. Value g0 indicates common WUS sequence for the shared WUS resource corresponds to g = 0, and value g126 indicates common WUS sequence for the shared WUS resource corresponds to g = 126, see TS 36.211 [21].. …. …

The related description in the latest TS 36.211 is as follows:

… (Copied from TS 36.211) … ### 6.11B.1 Sequence generation … The common MWUS sequence shall be determined by g=126 unless the resource is shared with non-group MWUS and common MWUS is configured to be non-group MWUS in which case g=0. …

As shown above, the terminology is not aligned, i.e., TS 36.211 uses “non-group MWUS”, and TS 36.331 uses “g0”.

And it seems the field description in TS 36.331, i.e., “value g126 indicates common WUS sequence for the shared WUS resource is g=126” is duplicate with TS 36.211. Because according to current TS 36.211, if the configured value is not “g0”, the common WUS sequence will be g=126.

To solve the issues above, and to avoid referencing HL parameter name directly, the following proposal and TP are given.

Proposal 1: Agree on the TP in Section 2 and capture its contents in TS 36.211.

------------------------------------- Start of Text Proposal for TS 36.211------------------------------- ------------------------------------------- Unchanged parts omitted -------------------------------------- ### 6.11B.1 Sequence generation … where is the actual duration of MWUS as defined in [4]. For a UE not configured with group MWUS, g=0. For a UE configured with group MWUS, g=14NgroupWUS+1 for 0≤NgroupWUS≤7, where NgroupWUS is determined by the UE group to which the UE is associated as determined by higher layers [10]. In resource that is not shared with non-group MWUS, the common MWUS sequence shall be determined by g=126. In resource that is shared with non-group MWUS, the common MWUS sequence is determined by higher layers [9].g=0 -------------------------------------------- Unchanged parts omitted ------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

# Conclusion

The contribution discusses the corrections on UE-group wake-up signal. The proposal based on the analysis in the contribution is as following:

Proposal 1: Agree on the TP in Section 2 and capture its contents in TS 36.211.

# References

- R1-1913594, “RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE”, Ericsson, RAN1#99, Reno, USA, 18th – 22nd November 2019.

- R2-2003923, “Miscellaneous Rel-16 eMTC corrections”, Qualcomm Incorporated, RAN2#109bis-e, Online, 20-30 April, 2020