3GPP TSG RAN WG1 #101		R1-2004185

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.5.5

Source: 	Sony

Title:	Remaining issues on inter-UE multiplexing for eURLLC

Document for:	Discussion / decision

# Introduction

In RAN1#100bis-e, we made the following agreement:

Agreements:

  UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions is not affected by UL CI.

Agreeement:

- If both UL CI and intra-UE priority indicator are configured for a given UE, support a new RRC parameter to configure Behavior #1

  - Behaviour #1: UL CI is only applicable to the UL transmissions indicated/configured as low priority level

- When the RRC parameter is not provided to the UE, behaviour #2 is used

  - Behaviour #2: UL CI is applicable to UL transmission irrespective of its priority level

- Note: the RRC signaling details will be decided by RAN2

This contribution discusses some remaining issues on inter-UE multiplexing.

# Discussions

The two L1 priority levels (i.e. High and Low) for PUSCH are used to resolve intra-UE collisions at the physical layer.  Since there are 16 priorities at the Logical level and only two L1 priority levels, the L1 intra-UE priorities are not suitable for use at the inter-UE level.  An example to illustrate this using 2 UEs is shown in Figure 1 [1], where the two UEs are configured with L1 priority indicators and:

- UE1 transmits PUSCH#1 for a real time game, therefore PUSCH#1 L1 priority = High

- UE2 transmits PUSCH#2 for an emergency, which is triggered when the user falls down, thereby PUSCH#2 is also indicated as High L1 priority

Although PUSCH#1 and PUSCH#2 are indicated with High L1 priority at the intra-UE level, at the inter-UE level PUSCH#2 has higher priority than PUSCH#1 since PUSCH#2 is for an emergency and PUSCH#1 is for entertainment purposes.  However, the intra-UE L1 priority cannot distinguish whether PUSCH#1 or PUSCH#2 has higher priority, if the resources for these two PUSCHs overlap.  Hence, the intra-UE L1 priority cannot be used at the inter-UE level.

Figure 1: Two UEs with high priority UL transmissions

Observation 1: The intra-UE L1 priority indicator cannot be used to resolve inter-UE prioritisation.

An agreement was made in RAN1#100bis-e to address the issue described in Figure 1, where the UE supporting multi-priority traffic (e.g. eMBB and URLLC) can be configured to either obey one of the following behaviours:

- Behaviour 1: Cancel only low L1 priority UL transmissions that are indicated in the UL CI

- Behaviour 2: Cancel any UL transmissions that are indicated in the UL CI regardless of L1 priority

Using the example in Figure 1, UE1 can be configured with Behaviour 2 whilst UE2 is configured with Behaviour 1.  When PUSCH#1 & PUSCH#2 collide and the colliding resources are indicated in the UL CI, UE1 will cancel PUSCH#1 whilst UE2 will transmit PUSCH#2.  However, this configuration can only address very limited cases and is unable to solve the case where two UEs are configured with Behaviour 2 or the case where a Behaviour 1 UE transmits a low priority PUSCH and a Behaviour 2 UE transmits a high priority PUSCH.  For example, in Figure 2 we have:

- UE1 transmits PUSCH#1 for a real time game, therefore PUSCH#1 L1 priority = High

- UE3 transmits PUSCH#3 for uploading a photo to the cloud, therefore PUSCH#3 L1 priority = Low

Here UE1 is once again configured with Behaviour 2.  Regardless whether UE3 is configured with Behaviour 1 or Behaviour 2, if PUSCH#1 and PUSCH#3 collide, then both PUSCHs will be cancelled by the UL CI and UE1 suffers from self-pre-emption, which is not the desired outcome since we would want PUSCH#1 to be transmitted and PUSCH#3 to be cancelled.  That is, a UE transmitting a high priority PUSCH and is configured with Behaviour 2 will suffer from self-pre-emption.  It should also be noted that UE1 cannot be RRC reconfigured to Behaviour 1 quickly enough to react to such collisions.

Figure 2: UE2 configured with Behaviour 2 suffers from self-pre-emption

Observation 2: UE configured with Behaviour 2 (i.e. cancel all UL transmissions indicated by the UL CI) can cause self-pre-emption where the UE’s URLLC PUSCH is the pre-emptor and is being cancelled by the UL CI.  This can occur in a situation where the UE with Behaviour 2 is transmitting a high priority UL transmission and it collides with:

- Another UE configured with Behaviour 1 transmitting a lower priority UL transmission

- Another UE configured with Behaviour 2 transmitting a lower priority UL transmission

Observation 3: RRC Reconfiguration of a UE between Behaviour 1 and Behaviour 2 is impractical and too slow to meet the low latency requirement of eURLLC.

UL CI is group common and addresses multiple UEs since it was argued that this is an efficient use of resource.  Hence, we expect a mixture of UEs configured with Behaviour 1 and Behaviour 2 to monitor the same UL CI and so we expect self-pre-emption to occur in a typical deployment.

Observation 4: Self-pre-emption is likely to occur in a typical deployment.

The self-pre-emption issue was discussed in RAN1#100bis-e and a solution that was almost agreed was proposed in an email discussion [2] as follows:

- A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4.

- UE does not expect to receive a PDCCH carrying UL grant with its ending symbol no earlier than the first symbol of a PDCCH carrying DCI format 2_4 scheduling a transmission in resource colliding with the ones indicated by DCI formant 2_4 or in the cancelled symbols that do not overlap with the resource indicated by the DCI format 2_4, if the UE has to cancel another transmission in resource indicated by the DCI format 2_4

  - FFS under what condition (if there is such a condition), that UE can be scheduled by an UL grant to transmit in any resource if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4, and if the UL grant is received after the DCI format 2_4

The rationale for this proposal is:

- UL CI is intended to cancel PUSCH (UL transmissions) that have already been scheduled rather than PUSCH that are yet to be scheduled.

- From UE implementation point of view, it is not desirable for a UE to be cancelling resources indicated by the UL CI and processing a transmission on resources that are being indicated as cancelled by the same UL CI.

- For intra-UE prioritization, when a high priority PUSCH overlaps with a low priority PUSCH, the UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols of the low priority PUSCH.  We expect this behaviour to be applicable for PUSCH cancelled by UL CI, that is the UE is not expected to be scheduled to transmit in symbols (overlapping or non-overlapping in frequency) of a PUSCH from the same UE that has been cancelled by UL CI.

Figure 3 shows 4 cases where a UE receives two UL grants, UG1 & UG2 such that UG1 is received before the UL CI and UG2 is received at or after the UL CI (NOTE: UG1 and UG2 belong to the SAME UE):

- In Case 1 and Case 2, the UL CI does not indicate any resource in the RUR that would lead to the UE to cancel its PUSCH scheduled by the earlier UL grant, UG1.  Since the UE is not cancelling any UL transmission in the resources indicated by the UL CI, the UE is allowed to transmit the PUSCH in any resources scheduled by UG2 and in Case 2, it is allowed to transmit in resources that are indicated as being pre-empted by the UL CI.

- In Case 3, some of the resources indicated by the UL CI overlaps with the PUSCH scheduled by UG1 and therefore the UE needs to process a PUSCH cancellation.  Since the PUSCH scheduled by UG2 does not overlap with any of the resources indicated by the UL CI, it is allowed to be transmitted.

- In Case 4, some of the resources indicated by the UL CI overlaps with the PUSCH scheduled by UG1 and therefore the UE needs to process a PUSCH cancellation.  Since the PUSCH scheduled by UG2 also overlaps with resources indicated by the UL CI, it is not allowed to transmit.  This is an error case.

Figure 3: Cases where the PUSCH scheduled by the 2nd UL grant (UG2) can or cannot be transmitted

The proposal can therefore prevent self-pre-emption in a UE especially for UEs configured with Behaviour 2.  Using the example in Figure 2, where two UEs, UE1 and UE3 are configured with Behaviour 2, then it is possible to use the proposal above to enable UE1’s URLLC PUSCH to pre-empt UE3’s eMBB PUSCH.  Referring to Figure 4, at time t0, UL grant UG3 schedules UE3 with an eMBB PUSCH between time t4 to t7.  UE1 has an urgent URLLC PUSCH and due to congestion, the gNB has to use some of UE3’s PUSCH resource for UE1’s PUSCH.  Here, the UL CI indicates the resources occupied by UE1’s PUSCH.  Since UE3’s UL grant UG3 is received before the UL CI, it obeys the UL CI and cancels its eMBB PUSCH.  In order to avoid UE1 from self-pre-empting itself, the UL grant UG1 for UE1 is transmitted at the same time (or after) the UL CI.  Since UE1 is not cancelling any UL transmission, it can therefore be scheduled on resources indicated by the UL CI.

Figure 4: Avoiding self-pre-emption for UEs configured with Behaviour 2

We expect that a majority of UEs supporting multiple traffic types (e.g. eMBB & URLLC) will be configured with Behaviour 2 and only a few would have extreme high priority and be configured with Behaviour 1.  Hence, the proposal above is essential and therefore should be agreed.

Proposal 1: Agree on the proposal made in RAN1#100bis-e [2], i.e.:

- A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4.

- UE does not expect to receive a PDCCH carrying UL grant with its ending symbol no earlier than the first symbol of a PDCCH carrying DCI format 2_4 scheduling a transmission in resource colliding with the ones indicated by DCI formant 2_4 or in the cancelled symbols that do not overlap with the resource indicated by the DCI format 2_4, if the UE has to cancel another transmission in resource indicated by the DCI format 2_4

The proposal made in RAN1#100bis-e [2] has the following FFS:

- FFS under what condition (if there is such a condition), that UE can be scheduled by an UL grant to transmit in any resource if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4, and if the UL grant is received after the DCI format 2_4

The scenario where it may be beneficial to allow the UL grant to schedule a PUSCH in any resources even if the UE has to cancel an UL transmission, is for UL CI with a long monitoring periodicity, e.g. 5 to10 slots long.  For such scenario, it is possible that the scheduler may want to schedule a URLLC PUSCH for a UE even though it had previously cancelled an eMBB transmission of that UE.  For example, in Figure 5 we have an UL CI with a monitoring periodicity of 5 slots and a RUR with 2×10 partition.  At time t0, UG1 schedules UE1 with an eMBB PUSCH to transmit at time t5 to t7 in Slot n+1.   At time t2, UL CI (CI1) indicates that the upper portion between time t6 to t8 is pre-empted by another UE and consequently UE1 cancels its eMBB PUSCH, since UG1 is received before the UL CI (CI1).  The UL CI also indicates the lower portion between time t11 to t14 as being occupied because it is pre-empted by UE2’s URLLC PUSCH and since UE3’s eMBB PUSCH overlaps some of the indicated resources, UE3 cancels its PUSCH so that UE2’s URLLC can be transmitted.  At time t9, UL Grant UG2 is received by UE1.  Although Proposal 1 does not allow UE1’s PUSCH to be scheduled on resources indicated by the UL CI (CI1) since UE1 had cancelled an eMBB PUSCH in Slot n+1, it may be beneficial to allow UG2 to schedule UE1’s URLLC PUSCH in any resources in Slot n+3, including those indicated by UL CI (CI1) since the previous PUSCH cancellation occurred a long time ago.  That is, we expect the UE to have already executed the cancellation of UE1’s eMBB PUSCH X symbols after reading the UL CI (CI1) and so it should not be an issue to schedule the UE in different resources that are indicated by the same UL CI.  This provides flexibility for the gNB scheduler.

Observation 5: If there is sufficient time for the UE to process the cancellation of an UL transmission after reading the UL CI, it is feasible for the UE to be scheduled with another PUSCH that can occupy other resources that are indicated by that UL CI.

Figure 5: UL CI with long monitoring periodicity

Since the scenario in Figure 5 is targeted at UL CI with long monitoring periodicity, we do not expect the cancelled PUSCH (e.g. UE1’s eMBB PUSCH in Slot n+1) and the PUSCH (e.g. UE1’s URLLC PUSCH in Slot n+3) scheduled by a later UL Grant that is received after the UL CI, to overlap in time or frequency.  We expect X to be at least 1 slot.

Proposal 2: The UE can transmit in any resources if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4 and if the UE receives the corresponding UL Grant at least X symbols after the last symbol of the PDCCH carrying the DCI format 2_4, where the cancelled transmission does not overlap in time or frequency with the UE’s transmission corresponding to that UL Grant.

- X = 1 slot.

# Conclusion

In this contribution, we discuss some remaining issues for inter-UE multiplexing.  We observe the following:

Observation 1: The intra-UE L1 priority indicator cannot be used to resolve inter-UE prioritisation.

Observation 2: UE configured with Behaviour 2 (i.e. cancel all UL transmissions indicated by the UL CI) can cause self-pre-emption where the UE’s URLLC PUSCH is the pre-emptor and is being cancelled by the UL CI.  This can occur in a situation where the UE with Behaviour 2 is transmitting a high priority UL transmission and it collides with:

- Another UE configured with Behaviour 1 transmitting a lower priority UL transmission

- Another UE configured with Behaviour 2 transmitting a lower priority UL transmission

Observation 3: RRC Reconfiguration of a UE between Behaviour 1 and Behaviour 2 is impractical and too slow to meet the low latency requirement of eURLLC.

Observation 4: Self-pre-emption is likely to occur in a typical deployment.

Observation 5: If there is sufficient time for the UE to process the cancellation of an UL transmission after reading the UL CI, it is feasible for the UE to be scheduled with another PUSCH that can occupy other resources that are indicated by that UL CI.

We therefore propose the following:

Proposal 1: Agree on the proposal made in RAN1#100bis-e [2], i.e.:

- A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4.

- UE does not expect to receive a PDCCH carrying UL grant with its ending symbol no earlier than the first symbol of a PDCCH carrying DCI format 2_4 scheduling a transmission in resource colliding with the ones indicated by DCI formant 2_4 or in the cancelled symbols that do not overlap with the resource indicated by the DCI format 2_4, if the UE has to cancel another transmission in resource indicated by the DCI format 2_4

Proposal 2: The UE can transmit in any resources if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4 and if the UE receives the corresponding UL Grant at least X symbols after the last symbol of the PDCCH carrying the DCI format 2_4, where the cancelled transmission does not overlap in time or frequency with the UE’s transmission corresponding to that UL Grant.

- X = 1 slot.

# References

[1] R1-2001817, “Remaining issues in inter-UE multiplexing,” Sony, RAN1#100bis-e

[2] R1-2002937, “Summary of [100b-e-NR-L1enh-URLLC-InterUE-01],” vivo, RAN1#100bis-e