3GPP TSG RAN WG1 #101			R1-2004226

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.5.6

Source:	Apple Inc.

Title:	Remaining Issues in Enhanced Configured Grant Transmission

Document for:	Discussion/Decision

# 1. Introduction

In Rel-16, up to 12 configured grants can be configured and active at a BWP, some of them may be configured at UL, some of them may be configured at SUL. In this contribution, we clarify UE’s behavior concerning CG transmission.

# 2. Discussion on configured grant transmission

Figure 1 CG transmission's impact to other channels/signals

As shown in the illustration above, when the UE transmits over CG, it may lead to the inhibition/dropping of other channels/signals’ transmission:

- In a first example, PUSCH transmission over CG would overlap with a PUCCH carrying BFRQ;

- In a second example, PUSCH transmission over CG would overlap with a PUCCH carrying HARQ-ACK in response to a DL PDSCH transmission;

- In a third example, PUSCH transmission over CG would overlap with a periodic/semi-persistent PUCCH transmission for CSI reporting;

- In a fourth example, PUSCH transmission over CG would overlap with a SRS transmission which may be periodic, semi-persistent or aperiodic;

….

When there are many CG configurations configured for a UE – some of them may have short periodicities, the occasions for such inhibition/dropping are also increased. Then it becomes necessary to clarify UE’s behavior in terms of CG transmission. First when a UE selects a CG for uplink transmission, it needs to ensure the transmission over CG does not lead to drop a higher priority transmission. Second, when a UE performs uplink transmission with CG (configured grant), depending which carrier between UL and SUL is used for the transmission prior to the CG transmission, the CG transmission itself, and the transmission after the CG transmission, the UE may take zero, one or two switching times, which should be included in the assessment of impacted channels due to CG transmission.

In the following, we consider 3 cases for CG transmission.

Case 1

Consideration on choosing SUL or UL for configured grant PUSCH transmission:

If the transmission before the CG transmission, the CG transmission itself, the transmission after the CG transmission are on the same carrier, then the interruption time is just the CG transmission occasion as shown in Figure 2.

Figure 2 CG transmission without UL/SUL switching

Case 2

Consideration on choosing SUL or UL for configured grant PUSCH transmission:

If the transmission before the CG transmission, the CG transmission itself are on the same carrier, but the transmission after the CG transmission are on a different carrier, then the interruption time  =  {the CG transmission occasion} +  {a switching time} as shown in Figure 3.

Figure 2 CG transmission with a single switching

Case 3

Consideration on choosing SUL or UL for configured grant PUSCH transmission:

If the transmission before the CG transmission, and the transmission after the CG transmission are on a  carrier, but the CG transmission itself is on another carrier, then

the interruption time = {switching time 1} + {the CG transmission occasion} + {switching time 2} as shown in Figure 3.

Figure 3 CG transmission with 2 switching times

When configured grants are configured for both SUL and UL, for prioritization purpose, to simplify UE processing, then the interruption time can be used to capture potential transition times, which includes the CG transmission duration and two switching times.

Then for the case with DG overlapping with a CG’s interruption time, when the PDCCH scheduling the DG transmission comes at least N2 symbols before the start of the CG interruption time:

- for a high priority DG overlapping with the CG transmission occasion’s interruption time, the DG transmission is prioritized;

- for a low priority DG overlapping with a low priority CG transmission occasion’s interruption time, the low priority DG is prioritized;

- for a low priority DG overlapping with a high priority CG transmission occasion’s interruption time, one of two behaviors can be selected:

  - the low priority DG is dropped which provides a consistent prior treatment or

  - the low priority DG is prioritized which defers to network’s scheduling choice;

  - As consistent treatment of priority is preferred, the first behavior should be selected.

In summary, we have

Proposal:

- If configured grants are configured on both SUL/UL, an interruption time includes a switching time prior to the CG transmission occasion, the CG transmission occasion itself, and a switching time after the CG transmission occasion, otherwise the interruption time includes the CG transmission occasion;

- The UE does not expect a DG PUSCH overlapping with a CG’s interruption time if the PDCCH scheduling the DG PUSCH is less than N2 symbols away from start of the interruption time of the CG;

- When the PDCCH scheduling a DG transmission comes at least N2 symbols before the start of the CG interruption time:

  - for a high priority DG overlapping with the CG transmission occasion’s interruption time, the DG transmission is prioritized;

  - for a low priority DG overlapping with a low priority CG transmission occasion’s interruption time, the low priority DG is prioritized;

  - for a low priority DG overlapping with a high priority CG transmission occasion’s interruption time, the low priority DG is dropped.

# 3. Conclusions

In this contribution we share our views on enhanced configured grant transmission. We have

Proposal:

- If configured grants are configured on both SUL/UL, an interruption time includes a switching time prior to the CG transmission occasion, the CG transmission occasion itself, and a switching time after the CG transmission occasion, otherwise the interruption time includes the CG transmission occasion;

- The UE does not expect a DG PUSCH overlapping with a CG’s interruption time if the PDCCH scheduling the DG PUSCH is less than N2 symbols away from start of the interruption time of the CG;

- When the PDCCH scheduling a DG transmission comes at least N2 symbols before the start of the CG interruption time:

  - for a high priority DG overlapping with the CG transmission occasion’s interruption time, the DG transmission is prioritized;

  - for a low priority DG overlapping with a low priority CG transmission occasion’s interruption time, the low priority DG is prioritized;

  - for a low priority DG overlapping with a high priority CG transmission occasion’s interruption time, the low priority DG is dropped.