3GPP TSG RAN WG1 #101                                                      	R1-2004244

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Apple Inc.

Title:	Views on Rel-16 eMIMO UE feature list

Document for:	Discussion/Decision

# Introduction

Based on the email discussion on RAN1 reflector, the latest UE feature list for Rel-16 eMIMO is quoted in the Appendix based on the agreement captured in the Chairman notes [1] and LS to RAN2 [2]. In this contribution, we provide our views based on the latest UE feature list for Rel-16 eMIMO.

# Multi-beam Related Rel-16 eMIMO UE FGs

FG 16-1a-1: SSB/CSI-RS for L1-SINR measurement

We have the following comments

- Similar as FG2-24, this FG can be

  - Per band, FFS UE under-reporting issues if UE can share processing power across bands

  - No TDD/FDD differentiation

  - Yes FR1/FR2 differentiation

- We support the current components description. The reason is that, in our view, this FG needs to have components to capture the following issues

  - The independent support/not-support of ZP-IMR and NZP-IMR

  - Capture the UE capability related to

    - Computational complexity, i.e. in terms of active RS within a slot

    - Memory, i.e. in terms of configured RS

  - The CMR related capability

    - 2 ports CMR, similar as FG2-24

    - Density, similar as FG2-24

  - Aperiodic CSI capability since AP-CSI is the most time critical processing

  - The UE under-reporting issue, i.e. processing power or memory can be potentially shared, partially or fully, between (1) CMR/ZP-IMR/NZP-IMR (2) CCs (3) Bands

  - We also prefer to separate SSB and CSI-RS for CMR in component 1 at least

    - When SSB is configured as CMR together with IMR, it is not easy to align the time domain pattern of SSB and IMR, therefore, it complicates the UE processing to find one to one QCL mapping of SSB and IMR as required by the current specification.

FG 16-1b: TCI state activation and spatial relation update

FG 16-1b-2: Spatial relation update across multiple CCs

For the second component, we prefer to remove the FFS, i.e. to confirm the component 2 in FG 16-1b and FG 16-1b-2. As we explained previously, it was agreed that for intra-band CA, UE can only use common beam management, i.e. use one beam for Tx or Rx at any given time in all CCs in the same band. This is due to the UE hardware limitation, i.e. only one RF phase shifter. The same hardware limitation also exists for inter-band CA, especially when two bands are very close in frequency. However, the current specification does not handle this issue adequately, therefore we believe the two components are very crucial to address this issue.

Therefore, we propose to change the component 2 for the 2 FG as follows:

- Component 2 for FG 16-1b: group of bands that share the same spatial receiving filter

- Component 2 for FG 16-1b-2: group of bands that share the same spatial transmitting filter

Both FGs should be reported per UE and only applies to FR2.

FG 16-1e: Pathloss reference RS activation via MAC CE

- For component 2, we suggest to remove FFS and define it at least “within a slot across all CCs in a band”. We are fine that for each CC, UE is required to maintain/track at most 4 activated PL RS for PUSCH/PUCCH/SRS. However, we need to consider the total number of PL RS that UE has to maintain/track for CA operation, similar as PDCCH BD and non-overlapping CCE, it is more preferred not to assume that the number of activated PL RS always linearly scaled as a function of the number of CCs

- For component 3, given the latest agreement in RAN1#100bis, it should be removed.

FG 16-1f: SCell beam failure recovery

- Component 1: Some clarification is needed about the maximum number. Is this about the BFD processing, CBD processing, or at any given slot, the maximum number of SCells that can be triggered with BFR?

- Component 2: We prefer to remove FFS, it is not necessary for UE to support PUCCH-BFR for SCell BFR

- Component 3: We prefer to remove the FFS, define it at least “within a slot across all CCs in a band”

FG 16-1g: Resources for beam management, [pathloss measurement, BFD, and BFR]

- We prefer to include at least RS for CBD in this feature group, processing of CBD is similar as processing of CSI-RS used for L1-RSRP reporting, and it could contribute significantly to the UE peak envelope processing complexity

- We also prefer to cover both cases, similar as FG2-24 in Rel-15

  - The UE processing complexity limitation, in terms of the active RS with a slot

  - The UE memory limitation, in terms of the configured RS

- As results, we prefer to change the components as

  - The max number of SSB/CSI-RS/[CSI-IM] resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured/activated within a slot to perform L1-RSRP measurement, L1-SINR measurement, new beam identification, [pathloss measurement and BFD]

  - The max number of SSB/CSI-RS/[CSI-IM] resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to perform L1-RSRP measurement, L1-SINR measurement, new beam identification, [pathloss measurement and BFD]

# Multi-TRP Related Rel-16 eMIMO UE FGs

FG 16-2a: 16-2a	Multi-DCI based multi-TRP

- Component 3: This can be removed since it is overlapping with FG16-2a-10

- We need to discuss the limitation on MIMO layers, i.e. whether to reuse per FSPC maxNumberMIMO-LayersPDSCH

- Component 4: for overlapping type of PDSCH, it should be

  - Non-overlapping RE and Non-overlapping in time

FG 16-2a-0: 16-2a Overlapping PDSCHs in time and non-overlapping in frequency

FG 16-2a-1: 16-2b Overlapping PDSCHs in time and non-overlapping in frequency

- We are fine to have “the maximal number of PDSCH scrambling sequences per serving cell”

- For the other 2 overlapping types of PDSCH, we prefer to have

  - Fully-overlapping PDSCH in RE

    - All REs in PDSCH 1 overlap with PDSCH 2 in RE and All REs in PDSCH 2 overlap with PDSCH 1 in RE

  - Partially-overlapping PDSCH in RE

    - At least one RE in PDSCH 1 does not overlap with PDSCH 2 in RE or at least one RE in PDSCH 2 does not overlap with PDSCH 1 in RE

FG 16-2a-4: HARQ-ACK for multi-DCI based multi-TRP - separate

- Component 2: We prefer to split into the following 3 cases. Note that Rel-15 does not support more than 1 HARQ-ACK in a slot

  - Two TDMed long PUCCHs within a slot for HARQ-ACK

  - TDMed short PUCCH and long PUCCH within a slot for HARQ-ACK

  - TDMed short PUCCH and short PUCCH within a slot for HARQ-ACK

FG 16-2a-1: Single-DCI based SDM scheme

- Component 1: If DCI indicating 2 TCI states in a TCI codepoint is captured as basic component, it should be consistently captured for the other schemes including FDMSchemeA, FDMSchemeB and TDMSchemeA, i.e., FG 16-2b-2/3/4

- Component 2:  Based on previous agreement, support of 2 port PTRS should be optional, we need to find a way to correct it such as use bitmap, i.e. {1 port PTRS, 2 port PTRS}

FG 16-2a-3: Single-DCI based FDMSchemeB

- Component 2:  Based on previous agreement, CW soft combining should be optional, we need to find a way to correct it such as use bitmap, i.e. {CW hard combining, CW soft combining}

FG 16-2a-5: Single-DCI based inter-slot TDM

- Component 1:  Inter-slot TDM has single TCI fall back mode to operate as DPS, therefore, strictly speaking, it is not a basic component for UE to support MAC CE to activate two TCI states for a TCI codepoint or DCI to indicate two TCI states

# MU CSI Related Rel-16 eMIMO UE FGs

FG 16-3a: Regular eType-II

- We need to discuss the dependency of the parameter setting on the triplets UE reported

  - For example, similar as Rel-15, UE can report the supported maximum L for each number of ports of CSI-RS for CMR.

FG 16-3a: Support of PMI sub-bands with R=2

If triplet is reported, we need to consider the ways to reduce the overhead. For example, for the triplets that UE reports UE support R=2, UE can also support R=1 so that the triplets do not need to be reported twice.

# UL Full Power Related Rel-16 eMIMO UE FGs

16-5b: UL full power transmission mode 1

- Component 2: we need to cover the possibilities of downgrade configuration including {4 port non-coherent, 4 port partial-coherent, 2 port non-coherent}, therefore, component 2 seems to be needed

- Component 3: This can be removed

16-5b: UL full power transmission mode 2

- Component 2: we need to cover the possibilities of downgrade configuration including {4 port non-coherent, 4 port partial-coherent, 4 port full-coherent, 2 port non-coherent, 2 port coherent}

- For each configuration, UE should be able to flexibly indicates the following 3 components independently

  - The TPMI bitmap or list that UE supports full power transmission, including the possibility that UE does not support any

  - The maximum number of SRS resources per SRS resource set, prefer to have value {1, 2, 4}

  - Whether different number of ports can be configured per SRS resource set

    - At least the case when 4 ports SRS resource is configured in an SRS resource set, whether 2 ports SRS resource can be configured in the same SRS resource set.

# Reference

- Chairman notes, 3GPP TSG RAN WG1 Meeting #100bis

- R1-2003073, “Rel16_RAN1_UE features NR_afterR1#100bisE”, 3GPP TSG-RAN WG1 Meeting #100bis-e, e-Meeting, April 20 – 30, 2020

# Appendix

Latest UE feature list for Rel-16 eMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per band |  |  |  |  | Optional with capability signalling Component-1 to 4, candidate value set for MB_1 and MB_1-x is {0, 8, 16, 32, 64} Component-5 to 8, candidate value set for MC_1 and MC_1-x is {0, 4, 8, 16, 32, 64} Component-9, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-10: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} Component-11, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} |
|  | 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling Candidate value set is {1, 2, 4} |
|  | 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
|  | 16-1b | TCI state activation and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state | Component 1: 2-1, 2-4 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1b-3 | Spatial relation update for PUCCH group | - Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N | Y |  |  | TBD |
|  | 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | TBD [Per band] | N | Y |  |  | TBD |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | TBD [Per UE] | N | N |  |  | TBD |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-1g | Resources for beam management, [pathloss measurement, BFD, and BFR] | - The maximum number of SSB/CSI-RS/[CSI-IM] resources [within a slot] across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, and new beam identification] - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - [Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency] [for FR1] FFS whether default value of R is needed | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
|  | 16-2a-0 | Overlapping PDSCHs in time and non-overlapping in frequency | - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency [merge with 16-2a? Merge with 16-2a-8?] [FR2 only?] |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-1 | Simultaneous reception of Multiple PDSCHs overlapping in frequency | - Support fully overlapping PDSCHs at frequency and partially/fully overlapping at time - Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default values of component 3/4 to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK |  |  |  |  |  |  |  |  | Note: this FG is a WA in RAN1 |  |
|  | 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - [Support of two TDMed long PUCCHs in a slot] |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-5 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-7 | Maximum number of activated TCI states | - Support of maximum per CORESETPoolIndex - Support of total maximum FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-8 | Simultaneous reception with different Type-D | whether UE supports receiving time-overlapping [PDSCHs]/PDCCHs with different Type-D FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  |  |  |  |  |  |  |  | Note: If the type of 16-2a is agreed to be FSPC or Fs this FG will be removed |  |
|  | 16-2a-10 | Value of BD factor | R= [(1,2] for BD/CCE |  |  |  |  |  |  |  |  |  |  |
|  | 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | TBD |
|  | 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
|  | 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
|  | 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | [per FSPC] | N | TBD |  |  | TBD |
|  | 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 2 candidate values {10 CBs, TBD} | TBD |
|  | 16-2b-5 | Single-DCI based inter-slot TDM | - Support of MAC CE to activate two TCI states for a TCI codepoint - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | TBD |
|  | 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - FFS: CBSR with hard amplitude restriction | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-4 | CBSR | ALT 1) CBSR with soft amplitude restriction (capture consequence if not supported hard amplitude restriction is supported) ALT 2) CBSR |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
|  | 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] |
|  | 16-5a | UL full power transmission [mode 0] | - Supported UL full power transmission [mode 0] | 2-13, 2-14 | Y | N/A | Y | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | TBD |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} - FFS: New UL codebook set(s) per supported Tx | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-5c | [UL full power transmission mode 2] | - [Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N | N |  |  | TBD |
|  | 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |
|  | 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types |  |  |  |  | [Per band] |  |  |  |  |  |
|  | 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | FFS: the detailed component design |  |  |  |  |  |  |  |  |  |  |