3GPP TSG RAN WG1 #101 		R1-2004250
e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.4.1.1

Source:	Apple Inc.

Title:	On potential techniques for coverage enhancement

Document for:	Decision

# Introduction

In RAN#86 meeting, new study item on NR coverage enhancement [1] was agreed. Some of objectives of this study item are showing below,

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

In this contribution, we discuss the various potential techniques for NR coverage enhancement.

# Potential techniques for coverage enhancement

Some of the techniques for coverage enhancement were already captured in Rel.15 NR spec, these techniques include the transmission/reception repetition in time domain and frequency hopping.

- Time domain repetition

Time domain repetition technique can increase the received signal power density via the energy accumulation and improve probability of successful channel decoding. It is extensively applied to various NR channels, i.e., PDSCH, PUSCH, PUCCH, CG PUSCH. The maximum repetition number defined in Rel.15 is  8. The PUSCH repetition number is extended to 16 in Rel.16.

Repetition is effective and simple solution to enhance the coverage for specific channel, if the evaluation identifies the coverage bottleneck, the repetition number can be extended further to improve the coverage.

- Frequency hopping

Frequency hopping was proved to be an effective way to improve the coverage, it is already applied to NR PUSCH and PUCCH. To extend the frequency hopping application, it can extend the hopping between two frequency locations to multiple frequency locations if the system bandwidth is larger enough. Thus, the additional frequency diversity gain can be explored.

- Time domain repetition plus frequency domain hopping

If it is identified the required repetition number is larger to fulfill the coverage requirement, then repetition plus the frequency hopping is the better choice, the channel is repeated several times in each hop,  the gain of repetition and frequency diversity can be achieved in this way, this technique was already standardized in LTE eMTC and NB-IoT.

- DMRS enhancements

Channel estimation plays an important role for data decoding. The repetition gain would be reduced with the non-ideal channel estimation in low SNR range, which results in larger repetition number. To improve the channel estimation performance in coverage limited scenario, the following techniques can be considered, cross-slot channel estimation, increasing the DMRS density, etc.

Proposal 1: Time domain repetition, frequency hopping and DMRS enhancement can be considered for coverage enhancement.

Generally, the coverage bottleneck is on the UL channel, such as PUSCH/ PUCCH, due to the UE power limitation. To support the larger cell radius in rural area, the msg3 performance could be the UL coverage bottleneck, if the msg3 reception is failure, it would impact the whole PRACH process, it will trigger the UE restart the random access procedure, which cause the access delay and lower the resource usage efficiency. In this sense, msg3 coverage enhancement can be considered.

Proposal 2: msg3 PUSCH coverage enhancement can be considered.

# Summary

In this contribution, we discuss the potential coverage enhancement techniques, and have the following proposals:

Proposal 1: time domain repetition, frequency hopping and DMRS enhancement can be considered for coverage enhancement.

Proposal 2: msg3 PUSCH coverage enhancement can be considered.

# References

- RP-193240, “New SID on NR coverage enhancement”, China Telecom