3GPP TSG RAN WG1 #101		                                                                               R1-2004305

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.4.1.2

Source:	InterDigital Inc.

Title:	Simulation assumptions for FR2 UL

Document for:	Discussion

# Introduction

RAN approved an SI on NR coverage enhancements in RAN#86 [1]. The objective of the SI is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The services that benefit from coverage enhancement include eMBB and VoIP. Simulation parameters for UL transmission are proposed in this contribution. Our contribution [2] summarizes potential techniques that can be applied for coverage enhancement.

# Simulation assumptions for FR2 UL

For eMBB and industrial use cases, coverage enhancement is essential feature for improvement. Simulation parameters for UL transmission, targeting eMBB for FR2, are presented in this section.

Waveform

As discussed in our contribution for FR1 evaluation assumptions [3], both OFDM and DFTsOFDM should be included in the evaluation. Motivations for considering both waveforms are explained in [3].

Proposal 1: Include both OFDM and DFTsOFDM for FR2 evaluation for coverage enhancement

Channel

In addition, in FR2, in long range communication, line-of-sight will be less likely as direct path of communication will most likely blocked by obstacles such as buildings or moving objects. For these reasons, CDL channel models which do not have LOS, namely, CDL-A and CDL-C are recommend for indoor and urban-macro scenarios, respectively. Considering long-distance communication, using the examples from TR 38.901, 43ns and 616ns are chosen for indoor and urban-macro scenarios.

Proposal 2: For channel models used in evaluation for coverage enhancement, CDL-A with DS=43ns and CDL-C with DS=616ns are recommended as indoor and urban-macro scenario, respectively

As discussed in our contribution for FR1 evaluation assumptions [3], number of DMRS symbols should be optimized and minimum number of DMRS symbols should be used as baseline in performance evaluation.

Proposal 3: Number of DMRS symbols and their positions in the time domain should be decided as benchmark

Amplifier model

For realistic computation of MCL, nonlinearity should be incorporated in evaluation. Inclusion of nonlinearity will also provide insights into difference in performance between OFDM and DFTsOFDM as OFDM requires larger backoff than OFDM.

Proposal 4: Nonlinear amplification model should be included for calculation of MCL

Slot hopping

Both intra-slot and inter-slot hopping, combined with slot bundling is recommended to be included in the evaluation for coverage expansion. Benefits of repetition is shown in the initial results in the contribution for FR1 [3].

Proposal 5: Include repetition as baseline technique for coverage enhancement

Finally, evaluation parameters, focusing on DFTsOFDM, are summarized in Table 1 and locations of DMRS are shown in Table 2. Similar set of parameters can be considered for OFDM. The number of PRBs are computed to meet the target data rate given the spectral efficiency. When repetition is used, the number of PRBs is scaled by the number of repetitions to meet the target data rate.

Evaluation of MCL for eMBB

Similar to FR1, parameters required to compute MCL are recommended in this subection. The definition of MCL can be found in [7]. For eMBB, as there are no latency requirements, SINR required to meet the target throughput should be used. Thus, the following proposal are made.

Proposal 6: For eMBB in FR2, MCL is evaluated for the target throughput

Finally, following the parameters in TR 38.802 [4] for FR2, maximum UE transmit power of 23 dBm and gNB noise figure of 7 dB can be assumed for calculation of MCL.

Proposal 7: For calculation of MCL for FR2, UE transmit power of 23dBm and gNB noise figure of 7dB should be used

# Conclusion

Simulation parameters for FR2 are shown in this contribution. Potential techniques for coverage expansion are introduced in the accompanying contribution [2].

Proposal 1: Include both OFDM and DFTsOFDM for FR2 evaluation for coverage enhancement

Proposal 2: For channel models used in evaluation for coverage enhancement, CDL-A with DS=43ns and CDL-C with DS=616ns are recommended as indoor and urban-macro scenario, respectively

Proposal 3: Number of DMRS symbols and their positions in the time domain should be decided as benchmark

Proposal 4: Nonlinear amplification model should be included for calculation of MCL

Proposal 5: Include repetition as baseline technique for coverage enhancement

Proposal 6: For eMBB in FR2, MCL is evaluated for the target throughput

Proposal 7: For calculation of MCL for FR2, UE transmit power of 23dBm and gNB noise figure of 7dB should be used

# References

RP-193240, “New SID on NR coverage enhancements”, China Telecom.

R1-2004273, “Potential techniques for coverage enhancements,” InterDigital, RAN1#101e, May, 2020.

R1-2004304, “Simulation assumptions and throughput performance for FR1 UL,” InterDigital, RAN1#101e, May, 2020.

TS 38.214, “Physical layer procedures for data,” ver. 16.0.1, Dec. 2019.

TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,” ver. 16.0.1, Dec. 2019.

TS 38.211, “Physical channels and modulation,” ver. 16.1.0, March, 2020.

TR 36.824, “LTE coverage enhancements,” ver. 11.0.0, June, 2016.

Table 1 FR2 UL simulation parameters

| Simulation Assumptions (to maximize MCL) | eMBB Values |
| --- | --- |
| Frequency Region | FR2 |
| Duplexing mode | TDD |
| Carrier Frequency (GHz) | 30 |
| Subcarrier Spacing (kHz) | 60 |
| Tx Bandwidth Configuration (MHz) | 100 |
| Waveform | W CP DFT-s-OFDM |
| CP type | Normal |
| PHY channel | PUSCH |
| Allocation (# of PRB) | {7,7,8,8} PRBs for MCS index=9, {27,29,31,34} PRBs for MCS index=1 to achieve 5Mbps. If repetition is turned on, the number of PRBs is scaled up by factor of 4 |
| Allocation (# of OFDM symbols) | 14 |
| DM-RS Type | Configuration Type 1 (according to Table 6.4.1.1.3-1 in TS 38.211 [6]) |
| DM-RS (# of OFDM symbols) | Type B, {1, 2, 3, 4} (Please see DMRS layout in Table 2) |
| MCS index, table | - MCS index=9 from Table 6.1.4.1-1 in TS 38.214 [4], MCS index 9 is chosen since it offers maximum spectral efficiency for QPSK - MCS index=1 from Table 6.1.4.1-1 in TS 38.214, MCS index 1 is chosen since it offers maximum spectral efficiency for pi/2 BPSK |
| HARQ sequence | 0,2,3,1 |
| Repetition/HARQ scheme | No Repetition: 12 HARQ processes Repetition: 4 repetitions per bundle, 3 HARQ processes Max number of HARQ retransmissions: Unlimited |
| Hopping | Inter-slot |
| Antenna Configuration | UE Antenna configuration - Number of TXRU : 1 - # of transmission layer : 1 (DFTsOFDM) - (M, N, P, Mg, Ng) = (2, 4, 2, 1, 1) ; (dV, dH) = (0.5, 0.5)λ, companies describe antenna configurations assumed in simulations BS antenna configuration - (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1); (dV, dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65°, directivity 8 dBi) |
| PA impairment | ideal |
| Channel model (TDL type, DS) | CDL-A DS=43ns (Indoor long delay profile from TR 38.901 [5]), DS chosen as the worst case scenario CDL-C DS=616ns (Urban Macro O2I long delay profile from TR 38.901) , DS chosen as the worst case scenario {Comment from FH : Descriptions below are taken from TR 38.802, link level sim for multi-antenna (DMRS). They have incorporated random AoD, ZoD, AoA, ZoA angles to randomize orientation of UE and BS. Frequency of the change can be up to implementation.} ASA, ASD, ZSA, ZSD follow the values in sec 7.7.1 in TR 38.901 The angles of UE, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of BS, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in subclause 7.7.5.1 in TR 38.901 [15] accordingly. |
| Speed | 3 km/hr |
| Receiver: CHEST | Realistic |
| TX pattern per frame | For TDD, referring to one of the patterns from LTE, use the pattern XXUUXXXUUX (10 slots=2.5ms @ SCS=60kHz) where U is a uplink slot and X is downlink/flexible; 2 out of 5 slots are reserved for uplink transmission. For inter-slot frequency hopping pattern, refer to 6.3 in TS 38.214. Below is the portion of the spec that describes frequency hopping for inter-slot. [TS 38.214, 6.3] In case of inter-slot frequency hopping, the starting RB during slot is given by: , where is the current slot number within a radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Subclause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops. |
| Data rate | 5 Mbps for indoor [5] Mbps for urban [5] Mbps for suburban |

Table 2 DMRS placement for PUSCH (type B) Table 6.4.1.1.3-3 from TS 38.211

| 1 symbol DMRS configuration (# symbols used for PUSCH = 13) | 2 symbol DMRS configuration (# symbols used for PUSCH = 12) |
| --- | --- |
| 3 symbol DMRS configuration (# symbols used for PUSCH = 11) | 4 symbol DMRS configuration (# symbols used for PUSCH = 10) |