3GPP TSG RAN WG1 #101		R1-2004313

e-Meeting, May 25th – June 5th, 2020

Source:	NEC

Title:	Study on the channel access mechanism

Agenda Item:	8.1.2

Document for:	Discussion and Decision

Introduction

In RAN#86, a study item [1] and a working item [2] to support NR from 52.6 GHz to 71 GHz were agreed. The motivation is to support NR operation above 52.6 GHz to 71 GHz considering, both, licensed and unlicensed operation by leveraging FR2 design to the extent possible.

According to the SID and WID, the following items need to be studied

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

  - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this contribution, we discuss the channel access mechanism for 52.6 GHz to 71 GHz.

Discussion

TR 38.807 [3] has studied the requirements for NR beyond 52.6 GHz. One of the features in the frequency ranges above 52.6 GHz potentially is larger spectrum allocations and larger bandwidths that are not available for bands lower than 52.6 GHz which can support widely ranging use cases, such as V2X, IAB, NR licensed and unlicensed, fixed wireless access. According to the TR, a larger bandwidth is for unlicensed usage. So the study the required channel access mechanism is necessary.

There are probably two factors will impact the channel access mechanism for NR for 52.6 GHz to 71 GHz. One is the new numerology to be introduced to support larger bandwidth. The related discussion is being discussed in [4]. The other is regulatory requirement which is the focus of this contribution.

Table 4.2.1-1: Current Licensing situation for various countries between frequency 52.6 GHz and 71 GHz [3]

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-711) |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U (Mobile) |  |  |  | U (Mobile) |  |  |
| ITU Region 2 | USA |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  |  |  | U (Mobile) |  |  |  |  |
|  | Japan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U (Mobile) |  |  |  |  |  |  |
| NOTE 1: Access regime currently under discussion in CEPT |  |  |  |  |  |  |  |  |  |  |  |  |

ETSI has studied Multiple-Gigabit/s radio equipment operating in the 60 GHz band in [5]. The channel access has been specified. So the channel access specified in NR-U [6] should be updated accordingly for working around 60GHz.

Proposal 1: Change the Channel access mechanism of TS 37.123 based on the regulation of ETSI EN 302 567

Conclusion

In this contribution, we discussed the channel access mechanism for 52.6 GHz to 71 GHz, and propose,

Proposal 1: Change the Channel access mechanism of TS 37.123 based on the regulation of ETSI EN 302 567

References

- RP-193259, Study on supporting NR from 52.6 GHz to 71 GHz

- RP-193229, Extending current NR operation to 71 GHz

- TR38.807, Study on requirements for NR beyond 52.6 GHz

- R1-2004312    Study on the required changes to NR using existing DL/UL NR waveform

- ETSI EN 302 567 V2.1.1 (2017-07)

- TS37.213