3GPP TSG RAN WG1 #101			                             R1-2004342

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.4.2.1

Source: 	ITL

Title: 	Mode-1 resource allocation for NR V2X

Document for:	Discussion and decision

Introduction

In RAN1#100b-e-Meeting, the following agreements were made [1]:

[100bis-e-NR-5G_V2X_NRSL-RA_Mode1-01]: Agreements: - The slots for sidelink transmission for CG type-1 are determined using the UL formula in 38.321 with the following changes: - Using slot-level granularity instead of symbol-level granularity (i.e., remove numberOfSymbolsPerSlot, “symbol number in the slot”, S in the formula) - periodicity is in number of slots - timeDomainOffset is expressed in number of slots - FFS: frame indexing (e.g., SFN, or virtual frame number) - FFS: whether logical or physical slots are used Agreements: - For LTE Uu scheduling NR, the slots for sidelink transmission are determined in the same way as for SL configured grant type-1 when NR Uu schedules NR SL. - SFN=0 refers to the LTE DL carrier. Agreements: - For CG, the periodicities supported are the same as for periodic resource reservation in Mode-2 (i.e., the list given by SL-ResourceReservePeriod-r16) [100e-NR-5G_V2X_NRSL-RA_Mode1-02]: Agreements: - In the determination of the UL slot used for SL HARQ-ACK reporting to the gNB: - k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion assuming that the starting time of the frame for sidelink transmission is given by the starting time of the frame for the downlink reception. - FFS: corrections to cover the asynchronous case Agreements: - A UE does not expect to be scheduled to transmit the UL report corresponding to a PSFCH reception earlier than Tprep after the end of the PSFCH. - This includes the effect of time advance. - Tprep = (N+X) ∙ (2048+144) ∙ k ∙ 2 –μ ∙ T_c where: - Working assumption: N is 14, 18, 28 and 32 corresponds to the SCS configuration μ of 0, 1, 2 and 3, μ = min(μ_SL, μ_UL) - k = T_s / T_c (parameters as defined in 38.211) - FFS X (including the possibility of value 0) Agreements: - The working assumption (as in proposal 3 in the summary) from RAN1#100-e is confirmed. Agreements: - The working assumption (as in proposal 4 in the summary) from RAN1#100-e is confirmed. - If the SL transmission does not use SL HARQ feedback (if supported by RAN2), the UE reports NACK to request further resources for blind retransmission and ACK otherwise. [100e-NR-5G_V2X_NRSL-RA_Mode1-03]: Agreements: - Higher layer signaling is used to configure the values of the PSFCH to PUCCH gap (NOTE: this is referred to as sl-FeedbackToUL-ACK in the following) - The field PSFCH-to-HARQ_feedback timing indicator: - Selects one of the configured values of the PSFCH to PUCCH gap, except in the case that, together with PUCCH resource indicator, it indicates that no PUCCH resource is provided. - FFS Presence in DCI format 3_0 and size (0-3 bits). Conclusion: - In preparing the TP for SL HARQ-ACK reporting to the gNB using type-1 codebook: - The following parameters and the corresponding parts of the Rel-15 specification are not used: - Parameters related to transmission of more than 1 TB - harq-ACK-SpatialBundlingPUCCH - maxNrofCodeWordsScheduledByDCI - Parameters related to CBG transmission - PDSCH-CodeBlockGroupTransmission - maxCodeBlockGroupsPerTransportBlock - Parameters related to slot aggregation - pdsch-AggregatoinFactor - Set of row indices R and pdsch-TimeDomainAllocationList - firstActiveDownlinkBWP-Id - The following functionality from the Rel-15 specification is not supported - HARQ-ACK for SPS PDSCH release - Capability to receive more than one unicast PDSCH per slot - Reporting of UCI (e.g., SR, CSI) NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way. [100e-NR-5G_V2X_NRSL-RA_Mode1-04]: Conclusion: - In preparing the TP for SL HARQ-ACK reporting to the gNB using type-1 codebook, at least the following changes are made to the Rel-15 specification: - A single cell is considered - DCI format 3_0 is used instead of formats 1_0 and 1_1. - SL- RNTI and SL-CS-RNTI replace C-RNTI and CS-RNTI, respectively. - sl-FeedbackToUL-ACK replaces dl-DataToUL-ACK - “PSFCH-to-HARQ_feedback timing indicator” replaces “PDSCH-to-HARQ_feedback timing indicator” - sl-PSFCH-ToPUCCH is used to determine PSFCH-to-PUCCH gap for SL CG type-1 - SL BWP replaces DL BWP - The ratio 2^(u_SL-u_UL) replaces the ratio 2^(u_DL-u_UL) - SL resource pool configuration is used to determine whether a slot belong to the SL pool or not. - The period of PSFCH resources is used: - to determine the number of PSFCH reception occasions for one PUCCH transmission - to determine whether a given SL slot has PSFCH resources or not - to determine the number of HARQ-ACK bits associated with each PSFCH reception occasion. NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way Agreements: - Counter sidelink index assignment (SAI) is supported. - 2 bits are used for type-2 codebook. - FFS size for type-1 codebook (1 or 2 bits). - The field is always present in DCI format 3_0. Conclusion: - In preparing the TP for SL HARQ-ACK reporting to the gNB using type-2 codebook, at least the following changes are made to the Rel-15 specification: - A single cell is considered - DCI format 3_0 is used instead of formats 1_0 and 1_1. - SL- RNTI and SL-CS-RNTI replace C-RNTI and CS-RNTI, respectively. - DAI is replaced by SAI - “PSFCH-to-HARQ_feedback timing indicator” replaces “PDSCH-to-HARQ_feedback timing indicator” - Slot offsets K0 is replaced by “Time Gap” and “Time Resource Assignment” fields in DCI. - SL resource pool bitmap is used to determine whether a slot belongs to the SL pool or not. - The period of PSFCH resources is used to determine the monitoring occasions. - MinTimeGapPSFCH is used to determine the monitoring occasions. - U_DAI is replaced by U_SAI, which denotes the total number of a DCI format 3_0 that the UE detects within M PDCCH monitoring occasions. - NOTE: This is not intended to change the reporting of DL HARQ-ACK in any way.

In this contribution, we discuss about remaining details on Mode-1 resource allocation for NR V2X sidelink (SL) and provide our proposals and views on each of topics.

Discussion

  - Determination of SL slot for CG type 1

For configured grant type-1, it has been agreed that the UL formula in 38.321 with some changes as seen agreement in section 1 is used to determine the slots for SL for CG type-1. One of remaining issues is how to handle the async SL and UL case different from sync case that DL SFN=0 is reference timing. Because SL and UL are asynchronous that is, at least slot boundary is not aligned. It can be ambiguous which slots are used for the configured grants according to the relevant CG configuration. To solve it, as discussed in previous meetings, the virtual DFN can be considered instead of SFN in sync case. The virtual DFN can be defined by having value 0 starting with the first SL slot that starts no earlier than TSFN,0 – TTA/2, where TSFN,0 is the time corresponding to the start of SFN=0 in the DL carrier at the UE and TTA is the timing advance value at the time of configuring the grant. It seems this approach is simple and resolve the timing ambiguity issue for the CG resource configuration.

Proposal 1: It is preferred that virtual DFN is used to determine the granted slot for CG type 1

- Where the virtual DFN is defined by having value 0 starting with the first SL slot that starts no earlier than TSFN,0 – TTA/2, where TSFN,0 is the time corresponding to the start of SFN=0 in the DL carrier at the UE and TTA is the timing advance value at the time of configuring the grant

For the discussion of logical slot Vs physical slot, our understanding is that this issue has been already addressed in LTE V2X and thus, its relationship between physical slot (i.e. ms unit) and logical slot (based on resource pool configuration) would be also handled with similar manner in LTE V2X. So, if we follow the same manner for NR V2X, there won’t be no problem related to any inconsistency between them.

Proposal 2: It is proposed to consider the logical slots for the SL slot configuration for CG type-1

  - Details on SL DCI format

As agreed in RAN1#98bis and 99, two different SL DCIs including both SL RNTI for dynamic grant and SL CS RNTI for CG type 2 grant should have the same size. The two different SL DCIs can have same DCI format size by zero padding and distinguish it by using different SL RNTIs. For alignment of the size to one of the existing DCI formats in NR, it is necessary to align the size possibly with padding bits, in order to avoid further blind decoding and higher complexity.

If all the NR Uu DCI format sizes are smaller than SL DCI format size, for this case, it could be assumed that UE is always expected to have NR Uu DCI format having the size equal to or greater than the size of SL DCI format.

Proposal 3: When both DCI 0_1 and DCI 3_0 are configured, DCI 3_0 is zero-padded to align the size of DCI 0_1

For the DCI format 3_0 with CRC scrambled with SL RNTI and SL-CS-RNTI, if a UE is configured with both DCI 3_0 with different SL RNTIs, the DCI format 3_0 with CRC scrambled with SL RNTI is zero padded until it has same size with that of SL-CS-RNTI.

Proposal 4: When both DCI 3_0 with SL RNTI and DCI 3_0 with SL-CS-RNTI are configured, DCI 3_0 with SL RNTI is zero-padded to align the size of DCI 3_0 with SL-CS-RNTI

Regarding SL DCI contents, some SL DCI contents highlighted by red colour additionally needs to be considered on top of currently agreed SL DCI contents as shown in following table:

DCI format 3_0: - Time gap – [x] bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause x.x.x of [6, TS 38.214] - HARQ process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213] - New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213] - Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause x.x.x of [6, TS 38.214] - SCI format 0-1 fields according to clause 8.3.1.1: - Frequency resource assignment. - Time resource assignment. - PSFCH-to-HARQ feedback timing indicator – 3 bits as defined in clause x.x.x of [6, TS 38.214]. - PUCCH resource indicator – 3 bits as defined in clause x.x.x of [6, TS 38.214]. - Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise [x] bits as defined in clause x.x.x of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI. - Counter SAI – 2bits if dynamic SL HARQ-ACK codebook is configured when reporting the SL HARQ-ACK to gNB; otherwise 0 bit. - Activation/release – 1bit if DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 0 bit.

Proposal 5: It is supported to introduce the activation/release (if scrambled by SL-CS-RNTI) in SL DCI format content

Summary and conclusions

In this contribution, we provided our proposals on Mode-1 resource allocation. The suggestions of this contribution are summarized as follows:

Proposal 1: It is preferred that virtual DFN is used to determine the granted slot for CG type 1

- Where the virtual DFN is defined by having value 0 starting with the first SL slot that starts no earlier than TSFN,0 – TTA/2, where TSFN,0 is the time corresponding to the start of SFN=0 in the DL carrier at the UE and TTA is the timing advance value at the time of configuring the grant

Proposal 2: It is proposed to consider the logical slots for the SL slot configuration for CG type-1

Proposal 3: When both DCI 0_1 and DCI 3_0 are configured, DCI 3_0 is zero-padded to align the size of DCI 0_1

Proposal 4: When both DCI 3_0 with SL RNTI and DCI 3_0 with SL-CS-RNTI are configured, DCI 3_0 with SL RNTI is zero-padded to align the size of DCI 3_0 with SL-CS-RNTI

Proposal 5: It is supported to introduce the activation/release (if scrambled by SL-CS-RNTI) in SL DCI format content

- Reference

RAN1#100bis-e-Meeting Chairman’s Note