3GPP TSG-RAN WG1 Meeting #101	Tdoc R1-2004351

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.6

Source:	Ericsson

Title:	UE features for eMIMO

Document for:	Decision

# 1	Introduction

In this contribution, we discuss the UE features for eMIMO.

# 2	Discussion

In [1], the outcome of the email discussion for the UE features for Release-16 are captured. Here we provide input to the discussion on the UE features related to eMIMO. The input is provided per sub-agenda item.

## 2.1	Enhancements to multi-beam operation

The currently proposed features for enhanced multi-beam operation are listed below. Our proposed changes are introduced directly in Table 1, and commented below.

Table 1: Proposed updates for UE features related to multi-beam enhancements.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know | V2X | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR or IMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR or IMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per UE |  |  |  |  | Optional with capability signalling |
| 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling Candidate value set is {1, 2, 4} |
| 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting | 16-1a-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 16-1b | TCI state activation across multiple CCs and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state | Component 1: 2-1, 2-4 |  | N/A |  | [Per BC or per band] Per UE | N | Y |  |  | TBD Optional with capability signalling |
| 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-59, 2-60 |  | N/A |  | [Per BC or per band] Per UE | N | Y |  |  | TBD Optional with capability signalling |
| 16-1b-3 | Spatial relation update for PUCCH group | Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | [Per BC or per band] Per UE | N | Y |  |  | TBD Optional with capability signalling |
| 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | [Per band] | N | Y |  |  | TBD Optional with capability signalling |
| 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | [Per UE] | N | N |  |  | TBD Optional with capability signalling |
| 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | [Per UE] | N | N |  |  | TBD Optional with capability signalling |
| 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | [Per UE] | N |  |  |  | TBD Optional with capability signalling |
| 16-1g | Resources for beam management, [pathloss measurement, BFD, and BFR] | - The maximum number of unique SSB/CSI-RS/[CSI-IM] resources [within a slot] across all CCs configured for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, and new beam identification]. - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | [Per UE] | N |  |  |  | TBD Optional with capability signalling |

Comments:

FG 16-1a-1:

The RS-related feature group should have two types of sub components: i) components related to the max number of RS resources per slot across CCs, and ii) components related to the max number of configured RS resources. The first type is related to how many measurements the UE would have to perform in a certain period of time, whereas the second is more related to the memory in the UE.

For the “per slot” capability, we note that the interference calculation on CSI-IM and NZP-CSI-RS for IMR are identical. A UE capable of interference measurements on N CSI-IM resources should also be capable of interference measurements on N NZP-CSI-RS resources, or any combination of k CSI-IM resources and N-k NZP-CSI-RS resources. There does not seem to be any reason to distinguish between the two. Also, the RSRP part of the measurement is quite similar. Based on this we propose to only have a capability that states the sum of configured resources for CMR and IMR. (component 4)

A similar argument applies to the “configured” capability. As this is related to the memory consumption in the UE, the capability needs to capture the memory consumption of the various resources. Here we see that the memory required to store an SSB, a single-port CSI-RS or CSI-IM resource would be similar. We thus propose to combine those into one component (component 5). Then, the memory required to store a 2-port CSI-RS resource could be larger, whereas its purpose is less important. We thus propose to add the total number of 2-port CSI-RS resources as one component (component 9).

No need for FDD/TDD differentiation. No need for FR1/FR2 differentiation.

The feature should be optional with capability signalling.

FG 16-1:

The TCI state activation across CCs is a feature where one MAC CE is used to signal an update of the TCI state in multiple CCs. In R15, one MAC CE was used to indicate that the UE shall activate a TCI state with a certain Id on one CC. In R16, one MAC CE can be used to indicate that the UE shall activate a TCI state with a certain Id on one CC and the TCI state with the same Id is activated on another CC. Note that the TCI states are still different: it is only the Id that is the same. So the feature is not about sharing TCI states across CCs. Hence, the FFS point should be deleted. There is no RF impact of this feature, the UE only needs to support to a new type of MAC CE signalling. As this is a baseband feature, it should be per UE.

The feature should be optional with capability signalling.

FG 16-1b-2:

The spatial relation activation across CCs is a feature where one MAC CE is used to signal an update of the spatial relation in multiple CCs. In R15, one MAC CE was used to indicate that the UE shall activate a spatial relation with a certain Id on one CC. In R16, one MAC CE can be used to indicate that the UE shall activate a spatial relation with a certain Id on one CC and the spatial relation with the same Id is activated on another CC. Note that the spatial relations may still be different: it is only the Id that is the same. So the feature is not about sharing spatial relations across CCs. Hence, the FFS point should be deleted. As this is a baseband feature, it should be per UE.

The feature should be optional with capability signalling.

FG 16-1b-3:

As this is a baseband feature, it should be per UE.

The feature should be optional with capability signalling.

FG 16-1c:

The feature type is per band.

The feature should be optional with capability signalling.

FG 16-1d:

The feature only indicates support/no support – similar to the signalling for SP-SRS.

The feature type is per UE.

The feature should be optional with capability signalling.

FG 16-1e:

The functionality for bullet 2 is unclear: should it really be “maximum number of activated pathloss reference RS update”? Propose to remove.

The feature type is per UE.

The feature should be optional with capability signalling.

FG 16-1f:

PUCCH-BFR should be a mandatory part of SCell BFR – if the UE reports that is supports BFR on one (or more) SCells, it should also support PUCCH-BFR. The additional UE complexity should be small, since the normal SR structure is reused. For component 3, the different options in the brackets have different implications – is it a limitation on processing or memory? We note that a “per slot” limitation can be included in 16-1g. For component 4, we should align with SpCell beam failure detection.

The feature type is per UE.

The feature should be optional with capability signalling.

FG 16-1g:

We are positive to introducing the feature. OK to include configured RS resources for L1-RSRP, L1-SINR and candidate beam identification. We would like to stress that the RS resources should be unique: an SSB that is configured for L1-RSRP measurements and candidate beam identification is counted as one resource.

There is no need for the FFS points: 2 and 3 are only RSRP measurements, and there is no need to distinguish them. For bullet 4, there is an identical FG in R15 – however the maximum value of the feature is 128.

The feature type is per UE.

No need for FR1/FR2 differentiation.

The feature should be optional with capability signalling.

For UE features related to multi-beam enhancements, agree on the updates captured in Table 1.

## 2.2	Enhancements to multi-TRP/panel transmission

The currently proposed features for enhanced multi-TRP operation are listed below. Our proposed changes are introduced directly in Table 2 and commented below.

Table 2: Proposed updates for UE features related to enhancements to multi-TRP/panel transmission.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know | V2X | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - [Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency] [for FR1] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default value of R is needed | FFS |  | N/A |  | TBD [per band / per FSPC] | No | TBD |  |  | FFS Optional with capability signalling |
| 16-2a-0 | PDSCHs with full Overlapping in frequency and full overlapping in time (fully overlapping REs) in frequency and PDSCHs in time and non-overlapping in frequency | - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency [merge with 16-2a? Merge with 16-2a-8?] [FR2 only?] - Support fully overlapping PDSCHs in frequency and fully overlapping in time (fully overlapping REs). - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] FFS whether default values of component 2 to be included in 16-2a | 16-2a |  |  |  | per band |  |  |  |  | Optional with capability signalling |
| 16-2a-1 | PDSCHs with partial overlapping frequency and full/partial overlapping in time (partially overlapping REs) Simultaneous reception of Multiple PDSCHs overlapping in frequency | - Support fully overlapping PDSCHs at frequency and partially/fully overlapping at time - Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time (partially overlapping REs) - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default values of component 23/4 to be included in 16-2a | 16-2a |  |  |  | per band |  |  |  |  | Optional with capability signalling |
| 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK | 16-2a |  |  |  | per UE |  |  |  | Note: this FG is a WA in RAN1 | Optional with capability signalling |
| 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH | 16-2a |  |  |  | per UE |  |  |  |  | Optional with capability signalling |
| 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - [Support of two TDMed long PUCCHs in a slot] | 16-2a |  |  |  | per UE |  |  |  |  | Optional with capability signalling |
| 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK | 16-2a |  |  |  | per UE |  |  |  |  | Optional with capability signalling |
| 16-2a-5 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2a |  |  |  | per UE |  |  |  |  | Optional with capability signalling |
| 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex | 16-2a |  |  |  | per band |  | FR2 only |  |  | Optional with capability signalling |
| 16-2a-7 | Maximum number of activated TCI states | - Support of maximum number of TCI states per CORESETPoolIndex - Support of total maximum number of TCI states FFS whether default values to be included in 16-2a | 16-2a |  |  |  | per UE |  |  |  |  | Optional with capability signalling |
| 16-2a-8 | Simultaneous reception with different Type-D | - whether UE supports receiving time-overlapping [PDSCHs]/PDCCHs with different Type-D - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency for FR2 FFS whether default values to be included in 16-2a | 16-2a |  |  |  | per Band |  | FR2 only |  |  | Optional with capability signalling |
| 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  |  |  |  |  |  |  |  | Note: If the type of 16-2a is agreed to be FSPC or Fs this FG will be removed | Optional with capability signalling |
| 16-2a-10 | Value of BD factor | R=[(1,2] for BD/CCE | 16-2a |  |  |  | Per UE |  |  |  |  | Optional with capability signalling |
| 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | TBD Optional with capability signalling |
| 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD Optional with capability signalling |
| 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD Optional with capability signalling |
| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | [per FSPC] | N | TBD |  |  | TBD Optional with capability signalling |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 2 candidate values {10 CBs, TBD} | TBD Optional with capability signalling |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of MAC CE to activate two TCI states for a TCI codepoint - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | TBD Optional with capability signalling |

Comments:

Regarding Mandatory/Optional column:

All features related to multi-TRP/multi-panel transmission should be optional with capability signalling.

FG 16-2a:

- The component ‘3.  The value of R=[1,2] for BD/CCE’ can be removed as it is now part of 16-2a-10.

- Remove the brackets around ‘Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency’ as this is one of the basic features for multi-dci based multi-TRP scheduling.  Note that if this feature is removed, then only TDM based scheduling can be done which will result in throughput degradation.  We can support this feature for FR1 as part of 16-2a.

- In our view, “The maximal number of PDSCH scrambling sequences per serving cell” currently captured under 16-2a-1 is a better fit as part of 16-2a.  Hence, we suggest to move “The maximal number of PDSCH scrambling sequences per serving cell” from 16-2a-1 to 16-2a.

- The feature type is ‘per band’.

FG 16-2a-0:

- This feature group name needs to be changed to ‘PDSCHs with full Overlapping in frequency and full overlapping in time (fully overlapping REs)’.

- A first component for this FG is ‘Support fully overlapping PDSCHs in frequency and fully overlapping in time (fully overlapping REs)’.

- A second component for this FG is ‘The maximum number of MIMO layers of each scheduled PDSCH’.  The text with ‘[if  PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain]’ can be removed.

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per band’.

FG 16-2a-1:

- This feature group name needs to be changed to ‘PDSCHs with partial overlapping frequency and full/partial overlapping in time (partially overlapping REs)’.

- A first component for this FG is ‘Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time (partially overlapping REs)’.

- A second component for this FG is ‘The maximum number of MIMO layers of each scheduled PDSCH’.  The text with ‘[if  PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain]’ can be removed.

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per band’.

FG 16-2a-2:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per UE’.

FG 16-2a-3:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per UE’.

FG 16-2a-4:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per UE’.

FG 16-2a-4a:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per UE’.

FG 16-2a-5:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per UE’.

FG 16-2a-6:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per band’.

- The FR1/FR2 differentiation column can be set to ‘FR2 only’

FG 16-2a-7:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per UE’.

FG 16-2a-8:

- Add a second component ‘Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency for FR2’

- Remove ‘[PDSCHs]’ from the first component.

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per band’.

FG 16-2a-10:

- 16-2a is a prerequisite for this FG.

- The feature type is ‘per UE’.

FG 16-2b-0:

- Remove 16-2b from prerequisite as 16-2b does not exist.

FG 16-2b-1:

- Remove 16-2b from prerequisite as 16-2b does not exist.

FG 16-2b-2:

- Remove 16-2b from prerequisite as 16-2b does not exist.

FG 16-2b-3:

- Remove 16-2b from prerequisite as 16-2b does not exist.

FG 16-2b-4:

- Remove 16-2b from prerequisite as 16-2b does not exist.

FG 16-2b-5:

- Remove ‘Support of MAC CE to activate two TCI states for a TCI codepoint’ as this feature can also work for single TRP.

- Remove last FFS ‘TCI state mapping to PDSCH transmission occasions (Cyclical mapping  or Sequential mapping)’ as this does not need to be a UE capability.

- Remove 16-2b from prerequisite as 16-2b does not exist.

For UE features related to multi-TRP/panel transmission enhancements, agree on the updates captured in Table 2.

## 2.3	CSI Enhancements for MU-MIMO support

Table 3: Proposed updates for UE features related to CSI enhancements for MU-MIMO support.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know | V2X | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-3a | Regular eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 - Support of parameter combinations 1-6 - Support of rank 1,2 - FFS: CBSR with hard amplitude restriction | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
| 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 |  |  |  |  |  |  |  |  |  |  |
| 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
| 16-3a-4 | CBSR | ALT 1) CBSR with soft amplitude restriction (capture consequence if not supported hard amplitude restriction is supported) ALT 2) CBSR |  |  |  |  |  |  |  |  |  |  |
| 16-3b | Port selection eType-II | Basic components: - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 | TBD |  | N/A |  | FFS: Per band or Per band per BC | N | N |  |  | Optional |
| 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
| 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
| 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types |  |  |  |  | [Per band] |  |  |  |  |  |
| 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | FFS: the detailed component design |  |  |  |  |  |  |  |  |  |  |

Comment:

For a detailed discussion on 16-7 and the FFS in 16-8, see our tdoc R1-2004527.

For UE features related to MU-CSI, agree on the updates captured in Table 3.

## 2.4	Low PAPR RS

Table 4: Proposed updates for UE features related to low PAPR RS.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know | V2X | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | N | N |  |  | [Optional] Mandatory with capability signalling |
| 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
| 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N | N |  |  | FFS: Optional with capability signalling |
| 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | TBD | Y | N/A | Y | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |

Comment:

FG 16-4: This is not an enhancement. This is a correction of a critical flaw in Rel-15 DMRS design. Hence, it needs to be mandatory with capability signalling.

For UE features related to low PAPR RS, agree on the updates captured in Table 4.

## 2.5		Full TX Power UL transmission

Table 5: Proposed updates for UE features related to full TX Power UL transmission.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-5a | UL full power transmission [mode 0] with Ul-FullPowerTransmission set to ’fullpower’ | - Supported UL full power transmission [mode 0] Support for UL full power transmission with factor ‘s’ from 38.213 section 7.1 fixed to ‘s’ = 1 | 2-13, 2-14 | Yes | N/A | Yes | FFS: Per FS or Per band or Per band per BC Per band | N/A | N/A |  |  | FFS |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - [Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx}] - [New UL codebook set(s) per supported Tx] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC Per band | No | No |  |  | FFS |
|  | 16-5c | [UL full power transmission mode 2] | - [Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | No | No |  |  | FFS |
|  | 16-5c-1 | UL full power transmission mode 2 with multiple SRS resources | - Number of ports to support mode 2: {2_1, 4_1, 4_2_1} - The maximum number of SRS resources in set with ‘usage’ = codebook Note: UE indicating mode 2 shall support full power transmission for 1 antenna port | 2-13, 2-14 |  | N/A |  | Per band | No | No |  |  | Optional |
|  | 16-5c-2 | UL full power transmission mode 2 with full power TPMI | - Number of ports to support mode 2: {2, 4} - TPMI(s) or TPMI group which delivers full power: - For 2 Tx: {one of TPMI=0 and TPMI=1 is supported}, or - For 4 Tx non-coherent: {one of Group 0 – Group 3}, or - For 4 Tx partial coherent: { one of Group 0 – Group 6} Note: UE indicating mode 2 shall support full power transmission for 1 antenna port | 2-13, 2-14 |  | N/A |  | Per band | No | No |  |  | Optional |

Comments:

In general:

Our changes are marked in blue, and the updates from the outcome of the eMIMO UE capability discussion are in red.

Our preference is to have full power be per-band capability, which is in line with RAN2’s request to limit per-band-per-band combination signalling.  The more band combinations there are, the harder it is to schedule the UEs, and the less likely the features are to be implanted in CA.  So unless there is some strong motivation, we prefer the simpler capability.

FG 16-5a:

Regarding the term ‘mode 0’, the following alternative got some traction in RAN1#100bis-e email discussions.

| 16-5a | UL full power transmission with Ul-FullPowerTransmission set to ’fullpower’ | Support for UL full power transmission with factor ‘s’ from 38.213 section 7.1 fixed to ‘s’ = 1 |
| --- | --- | --- |

The alternative below was supported by a couple of companies, but in our view seems less informative, since the two columns are identical.  The reference to the RAN1 spec in the alternative above is more precise and might be useful, although probably not essential, to help RAN2 with an RRC field definition for the capability.

| 16-5a | UL full power transmission with Ul-FullPowerTransmission set to ’fullpower’ | UL full power transmission with Ul-FullPowerTransmission set to ’fullpower’ |
| --- | --- | --- |

FG 16-5b:

The changes proposed by the moderator marked in red are needed.  This was discussed during the work item, and no need was identified for additional Mode 1 TPMIs.

FG 16-5c:

Whether the maximum number of SRS resources and the supported full power TPMIs are in separate feature groups for Mode 2 was debated at length in RAN1#100bis-e, and no decision was reached.  According to the RAN1#97 agreement on Mode 2:

UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI

Therefore, a UE can use one of SRI or TPMI or both in Mode 2 operation.  Independent SRS port UE and TPMI capabilities are consistent with, but not required for, this since a UE can be configured for less than what it is capable of.

According to the agreement in RAN#98, ‘a maximum of 4 SRS resources are supported in Mode 2’ and ‘either up to 2 or 4 SRS resources are supported’.

Agreement

- For 4 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

  - Depending on UE capability, either up to 2 or 4 SRS resources are supported

- For 2 TX UEs, a maximum of 4 SRS resources are supported in Mode 2 for usage set to ‘codebook’ in a set

  - Depending on UE capability, either up to 2 or 4 SRS resources are supported

- For mode 2 UEs, up to 2 different spatial relation info can be configured for all SRS resources with usage set to ‘codebook’

Note: it does not mean to support simultaneous transmission of multiple SRS resources usage is set to ‘codebook’

Two different interpretations have been put forward in RAN1#100bis-e discussion [100b-e-NR-UEFeatures-LS]:

      - A Mode 2 UE must support at least 2 SRS resources.

The rationale is that up to only 2 or 4 resources, not 1 or 3 resources are identified as supported depending on UE capability.  If a UE could support 1 SRS resource in Mode 2, that would have been listed in the agreement.  If one argues that 1 SRS resource can be supported, then 3 SRS resources should also be supported.

      - A Mode 2 UE can support a maximum of 2 or 4 resources, as we did not agree to a minimum number of SRS resources in Mode 2

The interpretation of either ‘up to 2 or 4 SRS resources are supported’ cannot in this interpretation mean that a Mode 2 UE that supports only 1 SRS resource indicates capability up to 2 or 4.  So ‘supported’ must mean that the specification supports operation with up to 2 or 4 resources.  But if the specification supports up to 4 it also supports up to 2.  Also, ‘a maximum of 4 SRS resources are supported in Mode 2’ seems to address the maximum number of SRS resources that is allowed for 4 Tx in specification, otherwise the sub-bullet would not have been needed at all.  Lastly, the sub bullet specifically mentions UE capability, so we fail to see how this interpretation is possible.

So we would conclude that the existing agreements require that a Mode 2 UE support at least 2 SRS resources.  On the other hand, it may be questioned why a UE should be required to support multiple SRS resources if it can support full power via TPMI indication.  There pro’s and con’s to SRI operation and TPMI operation in mode 2.  SRI indication allows the network to know what one port transmission looks like, especially for DCI format 0_0, and SRI indication can indicate increased, but not full power, transmission for rank 1 operation with 4 Tx, whereas TPMI indication alone can’t do this.  On the other hand, multiple SRS resources have the potential to increase SRS resource overhead and battery drain.  While our expectation is that SRS overhead should quite small if e.g. aperiodic SRS transmission is used, it is certainly straightforward to say that if a UE transmits multiple SRS resources at the same rate, the overhead does increase.

Observations:

Current agreements require Mode 2 UEs to support at least 2 SRS resources

A UE can be independently configured in Mode 2 to use full power TPMI with one SRS resource or to use multiple SRS resources without capability for full power TPMI

There are pro’s and con’s to either of these configurations, although from a performance perspective SRI indication may be advantageous if only one of the two is supported.

For the sake of progress, we are OK to have a new agreement allowing a UE capable of only 1 SRS resource in codebook based operation to be included in full power Mode 2, provided that capability for multiple SRS resources with different numbers of ports and for the support of full power TPMIs are independent.

Proposal:

Mode 2 UEs support one of at most 1, 2, or 4 SRS resources

One feature group is defined that indicates if the UE supports 2 or 4 SRS resources in Mode 2

Another feature group is defined that indicates support for full power TPMIs in Mode 2 with at least one SRS resource

The two feature groups are independent, but the UE supports at least one for Mode 2

Regarding the detailed components:

For the number of Tx in 16-5c-1 (on multiple SRS resources for Mode 2)

  - All UEs support full power with single port, and UEs may/may not be able to virtualize from 4 ports to 2, so the port combinations are 2_1, 4_1, 4_2_1.

‘FFS: Number of ports per SRS resource’ should be removed

  - We see no reason to restrict the port combinations

‘FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set’ should be removed, since the maximum of 2 was agreed already as shown above.

For UE features related to full Tx power UL transmission, agree on the updates captured in Table 5, such that
		• Mode 2 UEs support either at most 1, 2, or 4 SRS resources
		• One feature group is defined that indicates if the UE supports 2 or 4 SRS resources in Mode 2
		• Another feature group is defined that indicates full power TPMIs in Mode 2 with at least one SRS resource
		• The two feature groups are independent, but the UE supports at least one for Mode 2

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE features related to multi-beam enhancements, agree on the updates captured in Table 1.

Proposal 2	For UE features related to multi-TRP/panel transmission enhancements, agree on the updates captured in Table 2.

Proposal 3	For UE features related to MU-CSI, agree on the updates captured in Table 3.

Proposal 4	For UE features related to low PAPR RS, agree on the updates captured in Table 4.

Proposal 5	For UE features related to full Tx power UL transmission, agree on the updates captured in Table 5, such that   
• Mode 2 UEs support one of at most 1, 2, or 4 SRS resources   
• One feature group is defined that indicates if the UE supports 2 or 4 SRS resources in Mode 2   
• Another feature group is defined that indicates support for full power TPMIs in Mode 2 with at least one SRS resource   
• The two feature groups are independent, but the UE supports at least one for Mode 2

# References

R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T, NTT DOCOMO INC., RAN1 #100bis-E, April 2020