3GPP TSG-RAN WG1 #101-e		R1-2004361

e-Meeting, May 25- Jun 05, 2020

Source:	Ericsson

Title:	Remaining issues for NR-DC UL Power Control

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

# Introduction

In this document we discuss remaining issues related to specification of uplink power control for NR-DC and propose some corrections to TS 38.213.

# Discussion

## T_offset WA for dynamic power sharing

A working assumption related to UE capability signaling for T_offset was made in RAN1#100-e. According to the WA, MN is required to understand SCG configuration to properly use T_offset and we raised concern about this aspect in email discussion [100e-NR-LTE_NR_DC_CA_enh-ULPC-02]. As a conclusion of the email discussion, a LS was sent to RAN2 to check this issue [1].

RAN2 discussed this issue in and sent a reply LS in [2].

Excerpt from report of RAN2#109bis-e

“Chair summary on Toffset: In the current standard, there is no explicit support for the R1 WA (MN is not required to read/understand SN configuration). There is some support among R2 companies to make an effort to specify new functions that might resolve this, but several companies don’t want to standardize anything. Several companies think that non-standard network implementation can support/enable the R1 WA. For now thre is not sufficient support among companies to take action in R2. Chair is not sure how mature this attitude is and suggest to wait. Possibly we send a Reply LS to R1 from Next meeting.”

Excerpt from RAN2 reply LS [2]

“RAN2 would like to thank RAN1 for the LS on uplink power control for NR-NR Dual-Connectivity.

RAN2 is still discussing the reply to RAN1 but has no consensus yet on introducing new inter-node signalling for T_offset.

However, RAN2 would like to remind that it was agreed in Rel-15 that MN and SN are not required to comprehend each other’s UE configuration for MR-DC. Therefore, RAN1 making assumption that such comprehension is possible is not correct in RAN2 view.”

From RAN2 discussion and the LS (excerpts above), it is now clear that the RAN1 WA on Toffset is not aligned with RAN2 agreement -- “MN and SN are not required to comprehend each other’s UE configuration for MR-DC”.

Observation 1

- RAN1 WA on Toffset is not aligned with RAN2 agreement that “MN and SN are not required to comprehend each other’s UE configuration for MR-DC” as indicated in the RAN2 LS

RAN2 is now discussing new functions i.e., introduction of inter-gNB signaling in an effort to fix the WA made by RAN1. However, RAN1 made the WA with the understanding that it did not require any inter-gNB signaling and following text was added in the LS [1] to reflect this – “Furthermore, it was agreed that RAN1 has no intention to ask RAN2 to work on inter-gNBs signaling in Rel-16 MR-DC work item for this working assumption”

Given the above status, we see below two options to resolve the T_offset issue from RAN1 perspective

- Fix the WA made by RAN1 by revising the definition of T_offset to align with the RAN2 agreement that “MN and SN are not required to comprehend each other’s UE configuration for MR-DC”;

- No further action to fix the WA in RAN1#101-e. Rely on RAN2 to find a mechanism to fix the RAN1 WA and if they are unable to do so, fix it later in RAN1 by also considering any further LS replies received from RAN2.

Observation 2

- From RAN1 perspective, there are below two options to resolve the T_offset issue

  - Fix the WA made by RAN1 by revising the definition of T_offset to align with the RAN2 agreement that “MN and SN are not required to comprehend each other’s UE configuration for MR-DC”;

  - No further action to fix the WA in RAN1#101-e. Rely on RAN2 to find a mechanism to fix the RAN1 WA and if they are unable to do so, fix it later in RAN1 by also considering any further LS replies received from RAN2.

Considering the late stage of the WI discussions (RAN4 is already waiting for RAN1 specs to stabilize to specify performance requirements), our preference is to fix this issue in RAN1 in this meeting rather than wait and rely on other WGs to find a solution. As proposed in email discussion [100e-NR-LTE_NR_DC_CA_enh-ULPC-02], the issue can be fixed by defining T_offset capability signaling to indicate one of two fixed time offset values (large=4ms, small=1.5ms). This removes the linking of T_offset with RRC configurations of MCG and SCG and is fully aligned with RAN2 MR-DC framework and also current UE capability framework for dynamic power sharing.

Proposal 1

- Fix the WA made by RAN1 by revising the definition of T_offset as follows

  - T_offset is a fixed time offset indicated by UE capability signaling, and the UE can indicate one of below two values

    - Long: 4ms

    - Short: 1.5ms

## Other issues for dynamic power sharing

### Issue 1: Removal of earlier text on dynamic power sharing

It was clarified in the WA sent to RAN2 that first value of T_offset UE capability corresponds to – “DPS without look-ahead” and second value corresponds to -- “DPS with look-ahead”. Given this clarification, below text in 38.213 should be removed as there is no agreement for supporting the corresponding behavior and below text was present in the specs before details of dynamic power sharing UE behavior were finalized.

“

-	if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

“

Corresponding example TP is shown below in TP1

Proposal 2

- Align 38.213 text to reflect the current WA that first value of T_offset UE capability corresponds to “DPS without look-ahead” and second value corresponds to “DPS with look-ahead”, and there is no additional UE behavior for with/without ‘look ahead’ cases.

---------------------------- start TP1 to sub clause 7.6.2 of 38.213v16.1.0 ---------------------------------

<.. unchanged text omitted >

i1i2PMCGactuali1+PSCGactuali2>PTotalNR-DCi2i2PMCGactuali1+PSCGactuali2≤PTotalNR-DCi2PMCGactuali1PSCGactuali2i1i2PMCGPSCGPTotalNR-DC

i1i2i1i2PMCGPSCG

If a UE indicates a capability for dynamic power sharing between the MCG and the SCG and

-	is provided NR-DC-PC-mode = Dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

where

-	Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax},

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a first value for the capability, and

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a second value for the capability.

and the UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-    overlap with the transmission occasion on the SCG

---------------------------- end TP1 ---------------------------------

### Issue 2: Handling of TPC commands in DCI format 2-2, 2-3

The UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-  overlap with the transmission occasion on the SCG

Above text was added to 38.213 so that when UE computes the transmit power for an SCG transmission starting at T0, it need not consider MCG grants assignments received within time T0-T-offset to T0.

However, the text does not cover the possibility of UE receiving a TPC command on MCG within T0-T-offset that changes power of an overlapping MCG transmission overlapping with the SCG transmission starting at T0.

This can happen e.g. when the overlapping MCG transmission is triggered by a configured grant, and a DCI format 2_2 based TPC command is received by the UE between time T0-T-offset and T0.

We propose the following to address the above issue

Proposal 3

- When UE has an SCG UL transmission and an overlapping MCG UL transmission, then for adjusting the power of the MCG UL transmission

  - the UE shall only consider TPC commands that are provided by DCI format 2-2, 2-3 in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG

Corresponding example TP is shown below

---------------------------- start TP2 to sub clause 7.6.2 of 38.213v16.1.0 ---------------------------------

<.. unchanged text omitted >

If a UE

-	is provided NR-DC-PC-mode = Dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

where

-	Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax},

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a first value for the capability, and

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a second value for the capability.

The UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-  overlap with the transmission occasion on the SCG

and if the UE has a transmission on MCG that overlaps with the transmission occasion on the SCG, for adjusting the transmission power of the MCG transmission, the UE shall only consider TPC commands that are provided by DCI format 2-2, 2-3 in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG.

---------------------------- end TP2 -----------------------------------------------------------------------------------

### Issue 3: Removing Tproc,CSI for second T-offset capability

Note: This issue needs to be resolved only if RAN1 does not fix the T_offset working assumption to align with RAN2 agreement that “MN and SN are not required to comprehend each other’s UE configuration for MR-DC” (e.g. as in Proposal 1)

The two values of T_offset capability are currently defined such that only difference between them is lack of Tproc,CSImux in T-offset computation for 2nd value compared to that of 1st value. However, the values for Tproc,CSI and Tmux,CSI seem to be not very different, with latter being just slightly bigger, e.g. by 2 symbols in 15/30 kHz numerology.

Tproc,CSI :

, with  for ,  for  and  for

|  | Tproc,CSI | T^mux_proc,CSI |
| --- | --- | --- |
| 15 kHz | 10 to 40 symbols ( 0.7 to 2.85 ms) | 12 to 42 symbols ( 0.85 to 3 ms) |
| 30 kHz | 13 to 72 symbols ( 0.46 to 2.57 ms) | 15 to 74 symbols ( 0.5 to 2.57 ms) |

Given the above, there is not sufficient distinction warranting two values for the capabilities – therefore, the working assumption should be updated to revise the second value for the capability. Two options exist for the second value of capability - 1) UE reports a numerical value as the second capability (e.g in absolute ms), or 2) update the expression for the second value for the UE capability. If waiting for RAN2 feedback regarding the overall working assumption, it is proposed to revise the working assumption as per latter option.

Proposal 4

- Tproc,CSI  is not included in Toffset definition when UE indicates second value of UE capability.

Corresponding example TP is shown below

---------------------------- start TP3 to sub clause 7.6.2 of 38.213v16.1.0 ---------------------------------

<.. unchanged text omitted >

If a UE

-	is provided NR-DC-PC-mode = Dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

where

-	Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax},

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a first value for the capability, and

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a second value for the capability.

The UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

---------------------------- end TP3 -----------------------------------------------------------------------------------

# Conclusions

In this document, we discuss remaining issues for NR-NR DC power control and make the following observations and proposals

Note: Proposal 4 is not needed if RAN1 revises the Toffset definition to align with RAN2 agreements according to Proposal 1.

Observation 1

- RAN1 WA on Toffset is not aligned with RAN2 agreement that “MN and SN are not required to comprehend each other’s UE configuration for MR-DC” as indicated in the RAN2 LS.

Observation 2

- From RAN1 perspective, there are below two options to resolve the T_offset issue

  - Fix the WA made by RAN1 by revising the definition of T_offset to align with the RAN2 agreement that “MN and SN are not required to comprehend each other’s UE configuration for MR-DC”;

  - No further action to fix the WA in RAN1#101-e. Rely on RAN2 to find a mechanism to fix the RAN1 WA and if they are unable to do so, fix it later in RAN1 by also considering any further LS replies received from RAN2.

Proposal 1

- Fix the WA made by RAN1 by revising the definition of T_offset as follows

  - T_offset is a fixed time offset indicated by UE capability signaling, and the UE can indicate one of below two values

    - Long: 4ms

    - Short: 1.5ms

Proposal 2:

- Align 38.213 text to reflect the current WA that first value of T_offset UE capability corresponds to “DPS without look-ahead” and second value corresponds to “DPS with look-ahead”, and there is no separate UE behavior for with/without ‘look ahead’ cases.

Proposal 3:

- When UE has an SCG UL transmission and an overlapping MCG UL transmission, then for adjusting the power of the MCG UL transmission

  - the UE shall only consider TPC commands that are provided by DCI format 2-2, 2-3 in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG

Proposal 4

- Tproc,CSI  is not included in Toffset definition when UE indicates second value of UE capability.

A consolidated example TP reflecting above proposals is shown below (separate TPs are shown in section 2)

---------------------------- start consolidated TP1-TP3 to sub clause 7.6.2 of 38.213v16.1.0 -----------------------------

<.. unchanged text omitted >

i1i2PMCGactuali1+PSCGactuali2>PTotalNR-DCi2i2PMCGactuali1+PSCGactuali2≤PTotalNR-DCi2PMCGactuali1PSCGactuali2i1i2PMCGPSCGPTotalNR-DC

i1i2i1i2PMCGPSCG

If a UE indicates a capability for dynamic power sharing between the MCG and the SCG and

-	is provided NR-DC-PC-mode = Dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as

-	minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power

-	PTotalNR-DC, if the UE does not determine any transmissions on the MCG

where

-	Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax},

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a first value for the capability, and

-	Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a second value for the capability.

The UE does not expect to have transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and

-    overlap with the transmission occasion on the SCG

and if the UE has a transmission on MCG that overlaps with the transmission occasion on the SCG, for adjusting the transmission power of the MCG transmission, the UE shall only consider TPC commands that are provided by DCI format 2-2, 2-3 in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG.

---------------------------- end consolidated TP1-TP3 ---------------------------------

# References

[1] R1-2001421, LS on uplink power control for NR-NR Dual-Connectivity, RAN1, RAN1#100-e, Feb 24-Mar 5 2020.

[2] R1-2003262, LS reply on uplink power control for NR-NR Dual-Connectivity, RAN2, RAN1#101-e, May 25-Jun 5, 2020.

# Annex A

Below is the WA assumption that RAN1 sent RAN2 for review in R1-2001421

RAN1 would like to inform RAN2 the following working assumption made in the RAN1 #100-e meeting:

Update the previous agreement as follows (changes in red): Agreements: · For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset =<= T_proc,2 max⁡{Tproc,MCGmax,Tproc,SCGmax,}, where: - Tproc,MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - Tproc,SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the SCG. - This is the “DPS without look-ahead”. - Alt.2: T_offset =<= 2*T_proc,2max⁡{Tproc,MCGmax,Tproc,SCGmax,}, where: - Tproc,MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, and/or Tproc,2mux, as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - Tproc,SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux and/or Tproc,2mux, as specified in TS38.213 and TS38.214 based on the configurations for the SCG. - This is the “DPS with look-ahead”. - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms - To be addressed in the CR stage - A UE reports the UE capability of Alt.1 and/or Alt.2. - Details up to UE feature list discussion