3GPP TSG-RAN WG1 #101-e		R1-2004362

e-Meeting, May 25-June 5, 2020

Source:	Ericsson

Title:	Remaining issues for single Tx UL enhancements

Agenda Item:	7.2.10.2

Document for:	Discussion and Decision

# Introduction

In this document we discuss remaining issues related specification of single-tx operation for EN-DC and propose some corrections to TS 38.213.

# Discussion and Proposal

In RAN1#100bis-e following was agreed

Excerpt from RAN1 LS to RAN2 [1]

Given the above agreement, UE behavior associated with tdm-PatternConfig-r16 (which can only be configured for Rel16 or later) is not applicable for “Type 2 UEs” (i.e., UEs not supporting dynamic power sharing).

Observation 1

- UE behavior associated with tdm-PatternConfig-r16 is not applicable for UEs not supporting dynamic power sharing (i.e., “Type 2 UEs”) due to RAN1#100bis-e agreements.

From RAN1 spec perspective, at least the part shown in TP1 below is not clear considering the above agreement and should be clarified as shown in the TP.

Proposal 1: Agree to TP1 below

---------------------------- start TP1 to sub clause 7.6.1 of 38.213v16.1.0 --------------------------------------

7.6.1		EN-DC

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by p-MaxEUTRA and a maximum power  for transmissions in FR1 on the SCG by p-NR-FR1.

The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power for the SCG in FR1 as described Clauses 7.1 through 7.5 using  as the maximum transmission power. The UE determines transmission power for the SCG in FR2 as described Clauses 7.1 through 7.5.

A UE does not expect to be configured for operation with shortened TTI and/or processing time [13, TS 36.213] on a cell that is included in an EN-DC configuration.

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows.

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and

-	if UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG in FR1, and

-	if  in any portion of slot  of the SCG,

the UE reduces transmission power in any portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG in FR1, respectively. The UE is not required to transmit in any portion of slot  of the SCG if  would need to be reduced by more than the value provided by XSCALE in order for  in any portion of slot  of the SCG. The UE is required to transmit in slot  of the SCG if  would not need to be reduced by more than the value provided by XSCALE in order for  in all portions of slot .

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]).

---------------------------- end TP1 to sub clause 7.6.1 of 38.213v16.1.0 --------------------------------------

# References

[1] R1-2003072 – “LS on Rel-16 RAN1 UE features lists for NR”, RAN1, 3GPP RAN1#100bis-eMeeting, April 2020.