3GPP TSG-RAN WG1 Meeting #101-e	R1-2004365

e-Meeting, May 25- June 5, 2020

Agenda Item:	7.2.10.5

Source:	Ericsson

Title:	Remaining issues for inter-band CA with unaligned frame boundary

Document for:	Discussion and Decision

# 1		Introduction

In this paper, we discuss some remaining aspects for CA with unaligned frame boundaries.

# 2	Discussion and proposal

The specification for CA with unaligned frame boundary were updated for 38.211, 38.213 and 38.214 in last RAN1 meeting. For 38.213 endorsed CR (R1-2003183), the duplication of HARQ codebook pseudo-code for the case without CA-slot-offset and the case with CA-slot-offset seems a bit cumbersome due to lot of spec replication (up to 4 pages long now). Especially considering potential maintenance issues that may come up, it may be preferable to maintain a single pseudo-code with appropriate condition defined therein and this is feasible since most of the pseudo-code is repeated.

So, we propose the following. The proposed change does not make any change in the UE behavior but is only intended to make the spec a bit more readable and easily maintainable.

Proposal 1: Consider use of a single pseudo-code in subclause 9.1.2.1 in 38.213 with suitable conditions defined to reflect the corresponding behaviors with and without ca-slot-Offset configuration.

Example TP based on 38.213-g10, subclause 9.1.2.1 is shown below.

<begin TP>

<omit unchanged text>

Set  - index of occasion for candidate PDSCH reception or SPS PDSCH release

Set

Set

Set  to the cardinality of set

Set k =0 – index of slot timing values , in descending order of the slot timing values, in set  for serving cell

If a UE is not provided CA-slot-offset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information

D1=D2=0 ;

else

D1= Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL ; D2= (Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL

end if

while

if modnU-K1,k+1+D2,max⁡(2μUL-μDL,1)=0

Set  – index of a DL slot within an UL slot

while

Set  to the set of rows

Set  to the cardinality of

Set  – index of row in set

if slot  starts at a same time as or after a slot for an active DL BWP change on serving cell  or an active UL BWP change on the PCell and slot  nU-K1,k∙2μDL-μUL+nD+D1 is before the slot for the active DL BWP change on serving cell  or the active UL BWP change on the PCell

nD=nD+1;

else

while

if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot  nU-K1,k∙2μDL-μUL+nD-NPDSCHrepeat+1+D1 to slot nU-K1,k∙2μDL-μUL+nD+D1, at least one symbol of the PDSCH time resource derived by row  is configured as UL where  is the k-th slot timing value in set ,

;

else

;

end if

end while

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

nD=nD+1;

end if

end while

end if

;

end while

modnU-K1,k+(Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c)∙2μUL+1,max⁡(2μUL-μDL,1)=0

nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD

nD=nD+1

nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD-NPDSCHrepeat+1nU-K1,k∙2μDL-μUL+Nslot,offsetUL2μoffset,UL-Nslot,offset,cDL2μoffset,DL,c∙2μDL+nD

nD=nD+1

<omit unchanged text>

<end TP>