3GPP TSG RAN WG1 #101-e	Tdoc R1-2004368

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.7

Source:	Ericsson

Title:	Discussion on UE features for UEPS

Document for:	Discussion and Decision

# 1	Introduction

We provide our input for UE PS related features based on the latest RAN1 agreed UE feature list in R1-2003073.

# 2	Discussion

We provide our input on the remaining issues for the features related to UE power savings (in R1-2003073) with following suggested updates.

- For all FGs (19-1/2/3/4a):

- Regarding the “FFS: whether or how to handle licensed/unlicensed differentiation”, our view is that the power saving features can be supported all cases including licensed and unlicensed spectrum since latter is just another spectrum/band. See R1-2004586 for our view on this topic in general. With respect to the UE power savings features, we do not see a need to handle licensed vs unlicensed differentiation specially. Even if licensed and unlicensed bands are not deployed at the same time, implementations will be reused between the two and new testing should not be needed.

  - For features like 19-3 and 19-4a, such differentiation is not required at all as the features correspond to RRC parameter configuration or UE assistance based on RRC signaling which should be transparent to the band/spectrum where such signaling occurs.

- 19-1

- Regarding FR1/FR2 differentiation bit meaning, our view is that the bit allows a UE to indicate  whether the UE supports DCI 2-6 monitoring on primary cell in FR1 or on primary cell is in FR2.

  - Whether the UE is also configured with Scells in same or different frequency range should not impact the meaning of this bit – note DCI 2-6 triggers the DRX ON duration timer which is applied for DRX which is common across all serving cells, and moreover support of DRX itself is a per-UE capability without any dependencies on FR1/FR2 differentiation, etc.

- Also, since the UE indicates the minimum time gap value that it supports for a given SCS of primary cell. The absence of the minimum time gap value for a SCS can imply that UE does not support the DCI 2-6 monitoring with the corresponding numerology.

- 19-2

- From our perspective, having FR1/FR2 differentiation would still be useful, especially given supporting the feature in FR1 does not necessarily mean it should automatically work for FR2.  However, we do acknowledge that this has been agreed but hope there could be consensus to add FR1/FR2 differentiation.

19-3:

- The per-BWP MIMO layer parameter under discussion is just an indication of the UE’s ability to tell the NW that NW can configure different max MIMO layers for different BWPs and hence we do not see why using that parameter as part of configuration should be precluded. It is of course up to the UE if it can or cannot use this for power savings in some or all bands. Signaling per UE is enough with FR1/FR2 differentiation

- We propose to have per-UE indication, and allow indicating differentiation between FRx. (Same as current proposal, removing brackets.)

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A | Yes | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH | FFS | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - 1. Support of maximum number of MIMO layer configuration per DL BWP | See Note | Yes | N/A |  | [Per UE ] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots | 19-2 | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |