3GPP TSG RAN WG1 #101			R1-2004379

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary on UE features for additional enhancements for NB-IoT

Agenda Item:	6.2.5.2

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the discussions and proposals in AI 6.2.5.2 regarding UE features for additional enhancements for NB-IoT.

Based on the discussions summarized in Section 2, following is the suggested list of email discussions/approvals for AI 6.2.5.2.

FL proposal of list of email discussion/approval:

[101-e-LTE-UEFeatures-NBIoT-01] Email discussion/approval on remaining issues on UE features for NB-IoT (25th – 29th May)

- Discuss and decide any necessary update for the UE features list for NB-IoT based on identified issues/proposals in R1-2004379

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Ericsson | It’s fine to have a single thread per WI, but perhaps we can then agree to discuss all the LTE-MTC/NB-IoT alignment issues in just one of the two threads (either one is fine for me). |
| Moderator (NTT DOCOMO) | Then, the email thread for NB-IoT can be used to discuss all the LTE-MTC/NB-IoT alignment issues, and agreed change(s) for LTE-MTC UE features list will be just confirmed in the email thread for LTE-MTC. |

1.1	Updated FL proposal

Based on the preparation phase email discussion, the moderator recommends following email discussion/approval for UE features for additional enhancements for NB-IoT in AI 6.2.5.2.

[101-e-LTE-UEFeatures-NBIoT-01] Email discussion/approval on remaining issues on UE features for NB-IoT (25th – 29th May)

- Discuss and decide any necessary update for the UE features list for NB-IoT based on identified issues/proposals in R1-2004379

- Note that all the LTE-MTC/NB-IoT alignment issues will be discussed in this email discussion

- Discussion on UE features for additional enhancements for NB-IoT

2.1	FG2-1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-1 | UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | Rel-15 NWUS | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-1 with those for 1-1: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | It is noted that many of the same features exist in both LTE_eMTC5 and NB_IOTenh3 WI, therefore it is suggested to align the naming and the corresponding description of these FGs in the feature file. 1-1; Group WUS without group resource alternation; 1. Group WUS without group resource alternation. 1-2; Group WUS with group resource alternation; 1. Group WUS with group resource alternation. 2-1; UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation); - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation). 2-2; UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation). - Either add UE-group wake-up signal in 1-1/2 or remove this term and use Group WUS in 2-1/2. |
| --- | --- |

2.2	FG2-2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-2 | UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | 2-1 | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured. | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-2 with those for 1-2: [3]

- Consequence if the feature is not supported by the UE

  - Add “and group resource alternation”: [2]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | 2-2; UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); 2-1; Yes; N/A; The network cannot wake-up a group of users with one wake-up signal and group resource alternation.; Per UE; FDD only; N/A; If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured.; Optional with capability signalling. |
| --- | --- |
| [3] | It is noted that many of the same features exist in both LTE_eMTC5 and NB_IOTenh3 WI, therefore it is suggested to align the naming and the corresponding description of these FGs in the feature file. 1-1; Group WUS without group resource alternation; 1. Group WUS without group resource alternation. 1-2; Group WUS with group resource alternation; 1. Group WUS with group resource alternation. 2-1; UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation); - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation). 2-2; UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation). Either add UE-group wake-up signal in 1-1/2 or remove this term and use Group WUS in 2-1/2. |

2.3	FG2-3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-3 | Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) | - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) |  | Yes | N/A | UE cannot transmit without an UL grant | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-3 with those for 1-3: [3]

- Consequence if the feature is not supported by the UE

  - Change to “UL data transmission will use EDT or connected mode instead of PUR”: [2]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | 2-3; Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS); - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS); Yes; N/A; UL data transmission will use EDT or connected mode instead of PUR; Per UE; FDD only; N/A; RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list.; Optional with capability signalling. |
| --- | --- |
| [3] | 1-3; PUR for full-PRB in CEmodeA. 2-3; Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS). - Align the description for PUR. Either change the name of FG 1-3 to 'Transmission in preconfigured UL resources (PUR) for full-PRB in CEmodeA' , or change the name of FG 2-3 to ' PUR (with potential UE-specific cyclic shift for DMRS ) |

2.4	FG2-4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-4 | PUR with serving cell RSRP for TA validation | - PUR with serving cell RSRP for TA validation | 2-3 | Yes | N/A | PUR will not use serving cell RSRP for TA validation | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. TA validation mechanisms based on ‘Serving cell changes’, ‘TA timer for idle mode’ and ‘TA always valid’ are mandatory for PUR UEs | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-4 with those for 1-7: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-7; PUR serving cell RSRP TA validation. 2-4; PUR with serving cell RSRP for TA validation. - For 1-7 change to 'PUR with serving cell RSRP for TA validation' |
| --- | --- |

2.5	FG2-6

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-6 | Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-6 with those for 1-10: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-10; Multi-TB unicast for DL in CEmodeA. 2-6; Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission). - Change the name 'Multi-TB' to 'Multi-TB scheduling' in LTE_eMTC5 |
| --- | --- |

2.6	FG2-12

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-12 | DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-12 with those for 1-23: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-23; Subframe level resource reservation for DL in CEmodeA. 2-12; DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers.. - Align the description for resource reservation. |
| --- | --- |

2.7	FG2-14/15

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-14 | Quality report in Msg3 for non-anchor access in idle | - Quality report in Msg3 for non-anchor access in IDLE mode | multicarrier-NPRACH-r14 | No | N/A | The network cannot receive the quality report for non-anchor carriers in IDLE mode | Per-UE | FDD only | N/A |  | Optional without capability signalling |
|  | 2-15 | Quality report in connected mode | 1. Quality report in connected mode other than Msg3 for anchor and non-anchor carriers |  | Yes | N/A | The network cannot receive the quality report other than in Msg3 | Per-UE | FDD only | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-14/15 with those for 1-29/30: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-29; DL quality report in Msg3 in Idle. 1-30; DL quality report in Connected. 2-14; Quality report in Msg3 for non-anchor access. 2-15; Quality report in connected mode. - Align the description for quality report. |
| --- | --- |

# Reference

[1]	R1-2003196	Summary on email discussion [100b-e-LTE-UEFeatures-Remaining]	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2003788	UE features for NB-IoT	Qualcomm Incorporated

[3]	R1-2003800	Discussion on UE features for additional enhancements for NB-IoT	ZTE

# Appendix: latest version of UE features list for additional enhancements for NB-IoT [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-1 | UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | Rel-15 NWUS | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-2 | UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | 2-1 | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured. | Optional with capability signalling |
|  | 2-3 | Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) | - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) |  | Yes | N/A | UE cannot transmit without an UL grant | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 2-4 | PUR with serving cell RSRP for TA validation | - PUR with serving cell RSRP for TA validation | 2-3 | Yes | N/A | PUR will not use serving cell RSRP for TA validation | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. TA validation mechanisms based on ‘Serving cell changes’, ‘TA timer for idle mode’ and ‘TA always valid’ are mandatory for PUR UEs | Optional with capability signalling |
|  | 2-5 | PUR with L1 ACK | - PUR with L1 ACK | 2-3 | Yes | N/A | PUR will not use L1 ACK | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 2-6 | Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-7 | Multi-TB scheduling for unicast in DL with a single DCI (Non-interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Non-interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (non-interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-8 | Multi-TB scheduling for unicast in UL with a single DCI (Interleaved transmission) | - Multi-TB scheduling for unicast in UL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in UL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-9 | Multi-TB scheduling for unicast in UL with a single DCI(Non-interleaved transmission) | - Multi-TB scheduling for unicast in UL with a single DCI(Non-interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in UL with a single DCI (non-interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-10 | Multi-TB scheduling for unicast in DL in a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | - Multi-TB scheduling for unicast in DL in a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | 2-6 | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-11 | Multi-TB scheduling for SC-MTCH | 1. Scheduling of multiple transport blocks for SC-MTCH in a single DCI 2. Scheduling of multiple transport blocks for SC-MTCH in a single DCI with scheduling gaps | SC-PTM | No | N/A | The network cannot schedule transmission of multiple TBs with a single DCI | Per UE | FDD only | N/A | The basic multicast (SC-PTM) functionality was introduced for LTE-M/NB-IoT in Rel-14 without capability signaling. | Up to RAN2 |
|  | 2-12 | DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-12a | DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | 2-12 | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-13 | UL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - UL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-13a | UL resource reservation with slot-level and symbol(s)-level granularity of NB-IoT non-anchor carriers. | - UL resource reservation with slot-level and symbol(s)-level granularity of NB-IoT non-anchor carriers. | 2-13 | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-14 | Quality report in Msg3 for non-anchor access in idle | - Quality report in Msg3 for non-anchor access in IDLE mode | multicarrier-NPRACH-r14 | No | N/A | The network cannot receive the quality report for non-anchor carriers in IDLE mode | Per-UE | FDD only | N/A |  | Optional without capability signalling |
|  | 2-15 | Quality report in connected mode | 1. Quality report in connected mode other than Msg3 for anchor and non-anchor carriers |  | Yes | N/A | The network cannot receive the quality report other than in Msg3 | Per-UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-16 | NRS on a non-anchor carrier for paging | 1. Presence of NRS on a set of subframes on a non-anchor carrier when no paging NPDCCH is transmitted | Paging non-anchor carrier | No | N/A | UE does not know whether NRS is transmitted on a non-anchor carrier when paging is not present | Per UE | FDD only | N/A |  | Optional without capability signalling |