3GPP TSG RAN WG1 #101			R1-2004386

e-Meeting, May 25th – June 5th, 2020

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on sidelink synchronization mechanism

Agenda Item:	7.2.4.3

Document for: 	Discussion and Decision

- Introduction

At RAN1#100bis-e meeting [1], we had many discussions via email and many agreements were reached for 5G V2X with NR-SL WID. There are a few remaining issues to make RAN1 specifications completed. In this contribution, we share our views on SL synchronization mechanism for 5G V2X with NR-SL.

- Discussions

  - PSBCH contents – TDD configuration indication

Agreements: - For indication of TDD configuration: - X=1 bit indicates the number of patterns - Value 0 indicates one pattern is used. - Value 1 indicates two patterns are used. - Y=4 bits indicate the periodicity information - When one pattern is used, Y indicates the periodicity of the pattern. - When two patterns are used, Y jointly indicates the periodicities of the two patterns. - Z=7 bits indicate the UL slots - UL slots are jointly indicated by 7 bits when two patterns are configured. - FFS other details.

At the last e-meeting, the above agreements were reached for TDD configuration indication. The remaining issue is details of how to use 7 bits to indicate UL slots. The following is reconfirmation of NR-Uu mechanism.

- Patterns

In NR-Uu, two patterns can be configured by pattern1 and pattern2 in TDD-UL-DL-ConfigCommon. Each parameter provides TDD-UL-DL-Pattern, where periodicity and the number of UL slots are included. The values can be different between pattern1 and pattern2.

- Periodicity

There are two parameters for periodicity in TDD-UL-DL-Pattern: dl-UL-TransmissionPeriodicity and dl-UL-TransmissionPeriodicity-v1530. If dl-UL-TransmissionPeriodicity-v1530 is provided, the periodicity is the value provided by dl-UL-TransmissionPeriodicity-v1530. Otherwise, the periodicity is the value provided by dl-UL-TransmissionPeriodicity.

Value set of dl-UL-TransmissionPeriodicity is {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}. Value set of dl-UL-TransmissionPeriodicity-v1530 is {ms3, ms4}. When two patterns are configured, a multiple of the sum of two periodicities shall be 20 msec. For example, it is not expected that one is ms0p5 and the other one is ms3.

- UL slots

nrofUplinkSlots in TDD-UL-DL-Pattern provides the number of consecutive full UL slots at the end of each pattern. Value set of nrofUplinkSlots is integer of [0, 320], while only 0 to 80 slots can be configured for UL slots in NR-Uu Rel-15.

Now we can use 7 bits to indicate the number of UL slots, i.e. maximum 0 to 127 can be indicated. Table 1 shows periodicities and candidate number of UL slots. We can see from the table that in some periodicity combinations (with red color) more than 128 candidates are possible and 7 bits are insufficient. For example, two patterns are configured with 10 ms periodicities, one of 20×20 = 400 combinations of UL slots could be configured. How to indicate this case should be decided.

Table 1: Periodicities and candidate number of UL slots.

Table 2: Periodicities and candidate number of UL slots with less granularity.

As the solution, we believe that lower granularity per pattern is practical one. Even when one of M to N slots is configured for nrofUplinkSlots with one slot granularity, one of from M to N slots can be indicated with multiple slots granularity. Examples are provided in Table 2. Values with blue color are updates from table 1.

- For SCS = 30 kHz, let us consider two patterns are configured with 10 ms periodicities. The original candidate number of UL slot combinations is 20∙20=400. Lower granularities are used to reduce the number as follows. 10∙10=100 candidates are existing; hence it can be indicated with the 7 bits.

  - For the 1st pattern, one of 1 to 20 slots can be indicated with 2 slots granularity (here defined as U). That is, U is one of {2, 4, 6, 8, 10, 12, 14, 16, 18, 20}.

    - Note: 0 is unnecessary for U. U = 0 can be indicated by one pattern case.

  - Similarly, for the 2nd pattern, one of 0 to 19 slots can be indicated with 2 slots granularity (here defined as V). V is one of {0, 2, 4, 6, 8, 10, 12, 14, 16, 18}.

    - Note: Full slots (i.e. 20) is unnecessary for V. V = 20 can be indicated by one pattern case.

- For SCS = 120 kHz, let us consider two patterns are configured with 1 ms and 3 ms periodicities. The original candidate number of UL slot combinations is 8∙24=196. Larger granularity is used for the 2nd pattern. 8∙12=96 candidates are existing; hence it can be indicated with the 7 bits.

  - For the 1st pattern, granularity is not changed. Candidate list of U is {1, 2, 3, 4, 5, 6, 7, 8}.

  - For the 2nd pattern, one of 0 to 23 slots can be indicated with 2 slots granularity. Candidate list of V is {0, 2, 4, 6, 8, 12, 14, 16, 18, 20, 22}

Proposal 1:

- For TDD configuration indication in PSBCH, in case of two patterns,

  - if the number of UL slot combinations of the two patterns is more than 128, UL slots are indicated with granularity of more than one slot.

- Conclusion

In this contribution, we discussed the remaining issues on SL synchronization mechanism. Observations/Proposals are summarized as following:

Proposal 1:

- For TDD configuration indication in PSBCH, in case of two patterns,

  - if the number of UL slot combinations of the two patterns is more than 128, UL slots are indicated with granularity of more than one slot.

References

- 3GPP RAN1#100bis e-meeting,	chairman’s notes