3GPP TSG RAN WG1 #101e			R1-2004424

e-Meeting, May 25th – June 5th, 2020

Source:	NTT DOCOMO, INC.

Title:	Baseline coverage performance for FR1

Agenda Item:	8.4.1.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#86 meeting, the new SI on NR coverage enhancement was approved [1]. The objectives of the SI are captured as follows.

The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows. - The target scenarios and services include - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1 - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2. - TDD and FDD for FR1. - VoIP and eMBB service for FR1. - eMBB service as first priority and VoIP as second priority for FR2. - LPWA services and scenarios are not included. - Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation - UL channels (including PUSCH and PUCCH) are prioritized for FR1. - Both DL and UL channels for FR2. - Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services - The target channels include at least PUSCH/PUCCH - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions) - Study the additional enhanced solutions for FR2 if any - Evaluate the performance of the potential solutions based on link level simulation.

In this contribution, we discuss on the simulation assumption for baseline coverage performance for both DL and UL based on link-level simulation for FR1.

- Discussion on simulation assumption for baseline coverage performance for FR1

2.1 Maximum coupling loss (MCL) methodology

The maximum coupling loss (MCL) is the limit value of the coupling loss between the UE antenna ports and gNB antenna ports over channel link, and the MCL methodology is widely used for the study on the coverage of the service [2-3]. Therefore, MCL methodology is used for the study of NR coverage enhancement, and proposed MCL calculation template is described in Table 1.

Table 1: MCL calculation template

| Physical channel name | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) |  |
| (2) Tx antenna gain (dBi) |  |
| Receiver |  |
| (3) Rx antenna gain (dBi) |  |
| (4) Thermal noise density (dBm/Hz) |  |
| (5) Receiver noise figure (dB) |  |
| (6) Interference margin (dB) |  |
| (7) Occupied channel bandwidth (Hz) |  |
| (8) Effective noise power |  |
| = (4) + (5) + (6) + 10 log(7) (dBm) |  |
| (9) Required SINR (dB) |  |
| (10) Receiver sensitivity |  |
| = (8) + (9) (dBm) |  |
| (11) Implementation mergin (including cable/body loss) (dB) |  |
| (12) Shadow fading margin |  |
| (13) Penetration mergin (dB) |  |
| (14) MCL |  |
| = (1) + (2) + (3) - (10) - (11) - (12) - (13) (dB) |  |

Proposal 1: Maximum coupling loss (MCL) methodology is used for the study of NR coverage enhancement. The proposed MCL calculation template is given in Table 1.

2.2 General parameters

General parameters for MCL evaluation should be defined as the simulation assumption, and the candidate parameters are listed as follows. Several parameters are already captured in [1], and values used in [2-5] can be the baseline for the simulation assumption. Our proposed parameters are summarized in Table 2.

- Service and scenario: captured in [1]

- Target data rate: captured in [1] for eMBB, and referring to [2] for VoIP

- Center frequency: middle frequency among B77, B78, and B79 is considered

- System bandwidth: maximum bandwidth is considered

- SCS: 30 kHz may be considered for FR1.

- gNB and UE Tx power: referring to [3]

- gNB and UE antenna configurations: 4 UE Rx antenna port is considered as in [4]

- gNB and UE noise figures: referring to [3]

- Thermal noise PSD: referring to [2]

- Radio channel: referring to [5]

- Delay spread: referring to [5]

- Mobile speed: consider mobile speed of pedestrian

Table 2: General parameters for MCL evaluation

|  | FR1 |  |  |  |
| --- | --- | --- | --- | --- |
| Service | VoIP |  | eMBB |  |
| Scenario | OtoI | Rural | OtoI | Rural |
| Target data rate | 12.2 kbps | 12.2 kbps | 10 Mbps | 1 Mbps |
| Center frequency | 4.1 GHz |  |  |  |
| System bandwidth | 100 MHz |  |  |  |
| SCS | 30 kHz |  |  |  |
| gNB Tx power | 44 dBm |  |  |  |
| UE Tx power | 23 dBm |  |  |  |
| gNB antenna configuration | 4 Tx, 4 Rx |  |  |  |
| UE antenna configuration | 2 Tx, 4 Rx |  |  |  |
| gNB receiver noise figure | 5 dB |  |  |  |
| UE receiver noise figure | 7 dB |  |  |  |
| Thermal noise PSD | -174 dBm/Hz |  |  |  |
| Radio channel | TDL-C | TDL-C | TDL-C | TDL-C |
| Delay spread | 240 | 363 | 240 | 363 |
| Mobile speed | 3 km/h | 3 km/h | 3 km/h | 3 km/h |

Proposal 2: General parameters listed in table 2 should be defined for MCL evaluation. Our proposed values are also listed in the table.

2.3 Channel-specific parameters

Channel specific parameters for MCL evaluation should be defined as the simulation assumption, and the candidate parameters are listed as follows. Several parameters can be selected by each company based on their considerations, and the selected values need to be indicated by each company with their evaluation results. Our proposed parameters are summarized in Table 3.

- QoS target: referring to [2], and residual BLER after retransmission (rBLER) may be sufficient for the evaluation to consider HARQ process at cell edge.

- Max number of HARQ retransmissions: to be indicated by each company

- Number of RBs: to be indicated by each company

- MCS number: to be indicated by each company

- Aggregation level: to be indicated by each company

- Frequency hopping: to be indicated by each company

- Number of repetition: to be indicated by each company

- PUSCH format: considering beam management for FR1, long PUSCH format may be selected

- Payload size: to be indicated by each company

Table 3: UL and DL channels parameters for MCL evaluation

| Channel Assumptions |  | PDCCH | PDSCH | PUCCH | PUSCH |
| --- | --- | --- | --- | --- | --- |
| QoS Target | VoIP | 1% BLER | 2% rBLER | 1% BLER | 2% rBLER |
|  | eMBB |  | 2% rBLER |  | 2% rBLER |
| Max number of HARQ retransmissions |  | - | TBI | - | TBI |
| Number of RBs |  | TBI | TBI | TBI | TBI |
| MCS number |  | - | TBI | - | TBI |
| Aggregation level |  | TBI | - | - | - |
| Frequency hopping |  | - | - | TBI | TBI |
| Number of repetition |  | - | - | TBI | TBI |
| PUCCH format |  | - | - | long format | - |
| Payload size |  | TBI | - | TBI | - |

TBI : to be indicated by each company

Proposal 3: Channel-specific parameters listed in table 3 should be defined for MCL evaluation. Our proposed values are also listed in the table. Parameters with TBI (to be indicated) are not defined, so that each company has to indicate its values when presenting the results.

- Initial MCL evaluation for FR1

In this section, our initial MCL results are introduced as an initial MCL evaluation for FR1. The general parameters listed in Table 2 are used and channel specific parameters for the evaluation are listed in Table 4.

Table 4: Channel-specific parameters for initial MCL evaluation

| Channel Assumptions |  | PDCCH | PDSCH | PUCCH | PUSCH |
| --- | --- | --- | --- | --- | --- |
| QoS Target | VoIP | 1% BLER | 2% rBLER | 1% BLER | 2% rBLER |
|  | eMBB |  | 2% rBLER |  | 2% rBLER |
| Max number of HARQ retransmissions |  | - | 4 | - | 4 |
| Number of RBs |  | 48 | 1 for VoIP 40 for eMBB (OtoI) 4 for eMBB (rural) | 1 for VoIP 8 for eMBB | 4 for VoIP 15 for eMBB (OtoI) 2 for eMBB (rural) |
| MCS number |  | - | 9 | - | 9 |
| Aggregation level |  | 16 | - | - | - |
| Frequency hopping |  | - | - | no | no |
| Number of repetition |  | - | - | 4 | - |
| PUCCH format |  | - | - | Format 1 for VoIP Format 3 for eMBB | - |
| Payload size |  | 24 bits | - | 1 bit for VoIP 8 bits for eMBB | - |

Figure 1 shows the link level simulation results for DL/UL channels for outdoor to indoor (O to I) and rural scenarios with VoIP and eMBB. As shown in Fig.1, different scenarios (O to I and rural scenarios) don’t provide remarkable difference for BLER performance. Based on the simulated BLER results, MCL are derived using the calculation template (Table 1).

Figure 1: Link level simulation results for DL/UL channels

Figure 2 shows the MCL evaluation results for FR1 for O to I and rural scenarios with VoIP and eMBB. The MCL performance target is considered based on the MCL evaluation results, and set at the value of 2nd best channel for each scenario, so that improvement of worst and 2nd worst channels need to be considered. As shown in Fig.2, we can find that the improvement of PUSCH (approximately 10 dB for VoIP and 15 dB for eMBB) is the most essential point for the coverage enhancement, because of 21 dB of Tx power difference between UE and gNB as in Table 2. In addition, we can also find that the improvement of PDCCH (approximately 5 dB) for VoIP is the next essential point, because of 24 bits of payload size with limited time and frequency domain resource (e.g. 48 PRBs and 2 OFDM symbols). Finally, there are approximately 15 - 20 dB difference between O to I and rural scenarios because of the penetration loss.

Figure 2: Initial MCL evaluation results for FR1.

Observation 1: Improvement of PUSCH may be considered as the first priority, and improvement of PDCCH may be considered as the second priority for FR1 coverage enhancement.

- Conclusion

In this contribution, we discussed on the simulation assumption for baseline coverage performance for both DL and UL based on link-level simulation. Based on the discussion, we made following proposals.

Proposal 1: Maximum coupling loss (MCL) methodology is used for the study of NR coverage enhancement. The proposed MCL calculation template is given in table 1.

Proposal 2: General parameters listed in table 2 should be defined for MCL evaluation. Our proposed values are also listed in the table.

Proposal 3: Channel-specific parameters listed in table 3 should be defined for MCL evaluation. Our proposed values are also listed in the table. Parameters with TBI (to be indicated) are not defined, so that each company has to indicate its values when presenting the results.

Observation 1: Improvement of PUSCH may be considered as the first priority, and improvement of PDCCH may be considered as the second priority for FR1 coverage enhancement.

References

- 3GPP, RP-193240, China Telecom, “New SID on NR coverage enhancement,” Dec. 2019.

- 3GPP, TR36.824 v11.0.0, Jun. 2012.

- 3GPP, RP-191531, Huawei, Ericsson, Telecom Italia, CATT, VIVO, ZTE, “IMT-2020 Link budget template towards final submission to ITU-R (submission 2),” Jun. 2019.

- 3GPP, TS38.101-1 v16.3.0, Apr. 2020.

- 3GPP, TR38.901 v16.1.0, Jan. 2020.