3GPP TSG RAN WG1 #101	R1-2004440

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.1.1

Source: 	Qualcomm Incorporated

Title: 	Remaining Issues and Clarification on Channel Structure for Two-Step RACH

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss remaining issues on the channel structure for two-step RACH that require clarification and correction.

# Discussion

## Clarification of UE Procedures for msgA PRACH Mapping

When UE maps msgA preambles to PUSCH occasions after validation, it is unclear from [1] what is the criteria for UE to select one out of multiple preambles when the mapping ratio Npreamble >1.

Therefore, we have the following proposal to clarify the msgA PRACH mapping procedures in [1]:

Proposal 1: Correct Section 8.1A of TS 38.213 to clarify the PRACH mapping procedures for Type-2 random access, according to the text proposal TP1.

---------------------------------------------------   Start of TP1 to Section 8.1A of TS 38.213   -----------------------------------------

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

Each consecutive number of Npreamble preamble indexes from valid PRACH occasions in a PRACH slot

-	first, in increasing order of preamble indexes within a single PRACH occasion

-	second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions

-	third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot

are mapped to a valid PUSCH occasion and the associated DMRS resource

-	first, in increasing order of frequency resource indexes fid for frequency multiplexed PUSCH occasions

-	second, in increasing order of DMRS resource indexes within a PUSCH occasion, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index [4, TS 38.211]

-	third, in increasing order of time resource indexes tid for time multiplexed PUSCH occasions within a PUSCH slot

-	fourth, in increasing order of indexes for Ns PUSCH slots

where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by msgA-PUSCH-PreambleGroup, and TPUSCH is a total number of valid PUSCH occasions per PUSCH configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by msgA-DMRS-Configuration.

When a UE has selected Type-2 random access procedure and needs to choose one out of multiple preamble indexes (Npreamble>1)  to transmit, the criterion of preamble selection can be configured by the network and signaled in SI or dedicated RRC signaling. If not configured, UE randomly chooses a preamble index.

--------------------------------------------------   End of TP1 to Section 8.1A of TS 38.213   -------------------------------------------

# Conclusion

In this contribution, we have discussed the remaining issues on channel structure of two-step RACH that require clarification and correction. To summarize, we have the following proposals:

Proposal 1: Correct Section 8.1A of TS 38.213 to clarify the PRACH mapping procedures for Type-2 random access, according to the text proposal TP1.

# References

- TS 38.213, V16.1.0.