3GPP TSG RAN WG1 #101e 	R1- 2004458

May 25th – June 5th, 2020

Agenda item:	7.2.5.2

Source: 	Qualcomm

Title: 	Remaining Issues on UCI Enhancements for URLLC

Document for:	Discussion and Decision

# 1	Introduction

In this document, the text proposals for the following topics are presented. Each corresponding section also includes a reason for the proposed addition or modification.

| Section 2 | Timeline for cancellation due to intra-UE prioritization |
| --- | --- |
| Section 3 | Timeline extension for the HP channel |
| Section 4 | Multiplexing versus cancellation order |
| Section 5 | PUCCH resource selection for PRI < 3 bits |
| Section 6 | Collision handling amongst multiple HARQ-ACK PUCCHs and PUSCH |
| Section 7 | Type-2 HARQ-ACK codebook determination with different DAI bitwidths |
| Section 8 | CPU release with uplink interruption |
| Section 9 | Type-1 codebook with two DCI formats |
| Section 10 | Impact of intra-UE cancellation to other physical layer procedures |
| Section 11 | UE procedure to cancel configured uplink transmission |
| Section 12 | Priority determination for DCI format 0_1/1_1 and 0_2/1_2 |

# 2	      Timeline for Cancellation due to Intra-UE Prioritization

During the last meeting, the following agreement was reached:

Agreement:

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is expected to cancel the low-priority UL transmission starting from Tproc,2 +d1 after the end of PDCCH scheduling the high-priority transmission, where

  - Tproc,2 is correponding to UE processing time capability for the carrier.

  - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability

  - Note: d_2,1=0 is for cancellation

- The minimum processing time of the high priority channel is extended by d2 symbols

  - Value d2 is the time duration corresponding to 0,1,2 symbols reported by UE capability

The overlapping condition is per repetition of the uplink transmission

This agreement applies to the scenarios where the high-priority channel is scheduled by DCI. For this case, and from the agreement, it is not clear how the Tproc,2 should be defined. In the last meeting, there was some discussion on how to relax the cancellation timeline for the UE as the exact cancellation timeline is not implementation friendly. Two options are more likely:

Option 1: The agreement is kept as is, but the time pointed by Tproc,2+d1 will be defined as the latest time at which the UE should cancel the low priority channel.

Option 2: The latest time for cancellation is the start of the first symbol of the high priority channel; in addition, the gap between the end of the CORESET in which a high priority DCI is detected and the beginning of the high priority channel is Tproc,2+d1.

Overall, if the relaxation for cancellation is to be accommodated, Option 2 is a better approach in terms of overall system performance. Hence, we propose to update the previous agreement as follows:

Proposal #1:  Update the agreement on cancellation timeline for intra-UE prioritization as follows:

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is expected the gap between the end of CORESET in which the high priority grant is detected an the starting symbol of the high priority channel to be smaller than Tproc,2 +d1, where

  - Tproc,2 is corresponding to UE processing time capability for the carrier.

  - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability

  - Note: d_2,1=0 is for cancellation

- The minimum processing time of the high priority channel is extended by d2 symbols

- The UE is expected to cancel the overlapping low priority channel at the first overlapping symbol at the latest.

In addition, it should be noted that since handling collision involves cancelling a transmission and replacing it with another transmission, this case is more similar to PUCCH overriding (although cancellation of an ongoing transmission is in fact could be more challenging) as captured in Section 9.2.3 of TS 38.213 than cancellation due to SFI. This is explained in more details as follows:

Section 9.2.3 of 38.213:

If a UE detects a first DCI format indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than  from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in clause 4.1 of [4, TS 38.211] and  corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, for ,  for ,  for ; otherwise ,  for ,  for ,  for ,  for .

- This is equivalent to detecting a LP DCI

- This is equivalent to detecting a HP DCI

- This is equivalent to Tproc,2+d1 gap starting from the end of the HP PDCCH

Based on the clause copied above, the SCS and N3 for deriving the PUCCH overriding timeline are determined as follows:

- The SCS used to derive Tproc,2 is the minimum SCS amongst the SCS of all DCI formats (scheduling the first PUCCH and the second PUCCH) and the SCS of PUCCH.

- N3 is based on minimum processing timing capability #2 if the carrier on which the PDCCH of the second PUCCH is detected and all the carriers with their HARQ-ACK multiplexed on a PUCCH resource are configured with the processing capability #2. Otherwise, N3 is based on minimum processing capability #1.

Based on the above discussion and the potential similarities between PUCCH resource overriding and intra-UE prioritization, we propose the following:

Proposal #2: If a high-priority channel is scheduled by a DCI and if it overlaps with a low priority channel, the SCS for Tproc,2 calculation is determined as the smallest SCS configuration of the PDCCH providing the DCI for the low priority channels, the SCS configuration for the PDCCH providing the DCI for the high priority channel, the SCS of the low priority channels to be cancelled and the SCS of the high priority channel.

- For any low priority configured channel (i.e., transmitted without a DCI), only the SCS of the low priority channel itself is accounted for.

Proposal #3: If a high-priority channel is scheduled by a DCI and if it overlaps with low priority channels, the value of N2 for Tproc,2 calculation is determined as follows:

- If the overlapping group consists of a high priority PUCCH carrying HARQ-ACK and low priority PUCCHs and/or PUSCHs and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the high priority DCI format and for all serving cells corresponding to the low priority HARQ-ACK information transmission in the overlapping group and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for all serving cells with corresponding low priority PUSCHs in the overlapping group, N2 is 5 for μ=0,  5.5 for μ=1 and 11 for μ=2; otherwise, N2 is 10 for μ=0,  12 for μ=1, 23 for μ=2 and 36 for μ=3.

- If the overlapping group consists of a high priority PUSCH and low priority PUCCHs and/or PUSCHs and if processingType2Enabled of PUSCH-ServingCellConfig is set to enable for the serving cell with the high priority DCI format and for all serving cells with corresponding low priority PUSCHs in the overlapping group and if processingType2Enabled of PDSCH-ServingCellConfig is set to enable for all serving cells corresponding to the low priority HARQ-ACK information transmission in the overlapping group, N2 is 5 for μ=0,  5.5 for μ=1 and 11 for μ=2; otherwise, N2 is 10 for μ=0,  12 for μ=1, 23 for μ=2 and 36 for μ=3.

In addition, if the high priority channel is for SR or CG-PUSCH transmission, the cancellation time should be left to the UE implementation. If the high priority PUCCH is for carrying SPS HARQ-ACK, then, the gNB should not schedule any low priority dynamically granted transmission that overlaps with it.

Proposal #4: If the high priority channel is for SR or CG-PUSCH transmission, the cancellation time should be left to the UE implementation.

Proposal #5: A UE is not expected to be scheduled to transmit a low priority channel overlapping with a high priority PUCCH carrying HARQ-ACK for SPS.

Proposal #6: In case a high priority channel overlaps with SRS, the following TP can be adopted.

Modified clause (Section 6.2.1 of TS 38.214)

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent symbols within the last 6 symbols of the slot, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter [SRS-for-positioning] the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

If a UE is not configured with [intra-UE Prioritization] and PUSCH and SRS are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

If a UE is configured with [intra-UE Prioritization] and  the transmission of a PUSCH/PUCCH with a priority index 1  would overlap in time with an SRS transmission on a serving cell, the UE does not transmit SRS in the overlapping symbol(s).

End

# 3	Timeline Extension for High-Priority Channels

In RAN1 #99, it was agreed that the minimum processing time of the high priority channel can be extended by d2 symbols based on the UE capability if the high priority channel overlaps with a low priority channel according to the following agreement:

Agreement:

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is expected to cancel the low-priority UL transmission starting from Tproc,2 +d1 after the end of PDCCH scheduling the high-priority transmission, where

  - Tproc,2 is correponding to UE processing time capability for the carrier.

  - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability

  - Note: d_2,1=0 is for cancellation

- The minimum processing time of the high priority channel is extended by d2 symbols

  - Value d2 is the time duration corresponding to 0,1,2 symbols reported by UE capability

The overlapping condition is per repetition of the uplink transmission

However, the timeline extension is not captured in the specification. Hence, we propose the following additions:

Modified clause (Section 5.3 of TS 38.214)

If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after Tproc,1=(N1+d1,1+d2)(2048+144)⋅κ2-μ⋅TC after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message.

-	N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	If the PDSCH DM-RS position l1 for the additional DM-RS in Table 7.4.1.1.2-3 in clause 7.4.1.1.2 of [4, TS 38.211] is l1=12 then N1,0=14 in Table 5.3-1, otherwise N1,0=13.

-	If the UE is configured with multiple active component carriers, the first uplink symbol which carries the HARQ-ACK information further includes the effect of timing difference between the component carriers as given in [11, TS 38.133].

-	For the PDSCH mapping type A as given in clause 7.4.1.1 of [4, TS 38.211]: if the last symbol of PDSCH is on the i-th symbol of the slot where i < 7, then d1,1 = 7 - i, otherwise d1,1 = 0.

- If the UE reports the capability of [intra-UE prioritization], and if the PUCCH of a larger priority index is overlapping with PUCCH/PUSCH of a smaller priority index, d2 is determined by the reported UE capability [XXXXX].

End

Modified clause (Section 6.4 of TS 38.214)

If the first uplink symbol in the PUSCH allocation for a transport block, including the DM-RS, as defined by the slot offset K2 and the start and length indicator SLIV of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting Tproc,2=max(N2+d2,1+d2)(2048+144)⋅κ2-μ⋅TC,d2,2after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the transport block.

-	N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1.

-	If the UE is configured with multiple active component carriers, the first uplink symbol in the PUSCH allocation further includes the effect of timing difference between component carriers as given in [11, TS 38.133].

-	If the scheduling DCI triggered a switch of BWP, d2,2 equals to the switching time as defined in [11, TS 38.133], otherwise d2,2=0.

- If the UE reports the capability of [intra-UE prioritization], and if the PUSCH of a larger priority index is overlapping with a PUCCH of a smaller priority index, d2 is determined by the reported UE capability [XXXXX].

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PUSCH-ServingCellConfig is configured for the cell and set to enable,

-	If the PUSCH indicated by the DCI is overlapping with one or more PUCCH channels, then the transport block is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the transport block is transmitted on the PUSCH indicated by the DCI.

End

# 4	Multiplexing and Cancelation order

During the last RAN 1 meeting, the following agreement was reached:

Agreement:

- To resolve collision between UL transmissions, a UE performs the following:

  - Step 1: Resolve collision between UL transmissions with same priority.

  - Step 2: Resolve collision between UL transmissions with different priorities.

While this 2-step principle may work in many scenarios, there are cases in which it will bring significant impact to UE implementation. In Fig. 1 below, we list two scenarios for comparison. In both scenarios, UE will need to resolve the collision between one LP PUSCH, one HP PUCCH, and one HP PUSCH, and the HP PUSCH overlaps with the HP PUCCH.

In the first scenario (i.e., Fig. 1a), the HP PUSCH overlaps with the LP PUSCH, but the HP PUCCH does not overlap with the LP PUSCH. In this case, the 2-step principle in the agreement works fine. Namely, the UE may first determine to multiplex the HP PUCCH onto the HP PUSCH, and then determine to cancel the LP PUSCH based on LP PUSCH overlapping with the HP PUSCH.

However, the 2-step principle becomes problematic (for UE implementation) in the second scenario (i.e., Fig. 1b). According to this agreement, the UE would first resolve the collision between high priority channels, and then resolve the collision between the remaining high priority channel (in this case, it is the PUSCH) and the low priority channel. In the scenario shown in Fig. 1b, since the HP PUSCH does not overlap with the LP PUSCH, the UE must transmit both LP PUSCH and HP PUSCH.  However, such a rule cannot be implemented by the UE. Indeed, the problem is that after decoding the HP DL grant that schedules the HP PUCCH (the time at which this event happens is denoted by T1), the UE may identify that the HP PUCCH overlaps with the LP PUSCH. Without any knowledge on whether there will be a future HP PUSCH or not at T1, the UE needs to make its intra-UE prioritization decision, i.e., it will determine to cancel the LP PUSCH. In other words, UE cannot wait to check whether the HP PUCCH is to be cancelled by another future high priority PUSCH/PUCCH transmission or not.

(a)

(b)

Fig. 1 Scenarios for Intra-UE multiplexing and cancellation across different priorities

To solve this issue, we propose to reuse the same “no look-ahead” rule to determine intra-UE cancelation/multiplexing as in the case of uplink power control in NR Rel-15. For example, if the UE receives a high priority grant that triggers a cancelation of low priority channels, the UE does not need to take into account any future high priority grants (regardless of whether the future high priority grant would change the cancelation decision or not).

Based on the discussion above, we propose the following.

Proposal #7: When resolving collision between a high priority uplink transmission and one or more low priority uplink transmissions, the UE is not expected to take into account other high priority uplink transmissions scheduled after the uplink transmission.

- In other words, no look ahead for intra-UE cancelation determination.

# 5	Collision handling amongst multiple HARQ-ACK PUCCHs and PUSCH

In NR Rel-16, sub-slot baesd HARQ-ACK feedback and CSI feedback is supported. In this case, it may be possible that multiple HARQ-ACK feedbacks or CSI feedbacks in respective sub-slots overlap with a PUSCH transmission in the slot of the subslots (see Fig. 1). In this case, the existing rule for piggybacking UCI on PUSCH in 38.213 does not work. To solve this issue, we propose to treat this as an error case.

Fig. 2. Sub-slot based UCI transmissions of the same type overlapping with a slot-based PUSCH

Modified clause (Section 9.1.3 of 38.213)

A UE does not expect to multiplex in a PUSCH transmission in one slot with SCS configuration  UCI of same type that the UE would transmit in PUCCHs in different slots with SCS configuration  if .

If a UE is configured with subslotLength-ForPUCCH, the UE does not expect to multiplex in a PUSCH transmission in one slot UCI of the same type that the UE would transmit in PUCCHs in different sub-slots of the slot.

End

# 6	PUCCH resource selection for PRI < 3 bits

In previous RAN1 meetings, it was agreed that the size of the PUCCH resource indicator (PRI) in DCI format 1_2 can be configurable between 0 to 3 bits.

Agreements:

Support configurable size for “PUCCH resource indicator (0 or 1 or 2 or 3 bits)” for the new DCI format for DL scheduling.

• New RRC parameter is introduced to for the configuration

However, how to determine the PUCCH resource when the number of PUCCH resources in a PUCCH resource set is greater than 2NPRI has not been discussed in the meetings, where NPRI denotes the number of bits for the PRI. In our view, the same mechanism used to indicate the PUCCH resource from the first PUCCH resource set in NR Rel-15 can be reused. Namely, the UE may determine the PUCCH resource based on both the PRI and the index of the starting CCE of the PDCCH.

Modified clause (Section 9.1.3 of 38.213)

For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource after determining a set of PUCCH resources for  HARQ-ACK information bits, as described in Clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or a value of dl-DataToUL-ACKForDCIFormat1_2 for DCI format 1_2, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH where, for PUCCH resource determination, detected DCI formats are first indexed in an ascending order across serving cells indexes for a same PDCCH monitoring occasion and are then indexed in an ascending order across PDCCH monitoring occasion indexes. For indexing DCI formats within a serving cell for a same PDCCH monitoring occasion, if the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs on an active DL BWP of a serving cell, and with ACKNACKFeedbackMode = JointFeedback for the active UL BWP, detected DCI formats from PDCCH receptions in the first CORESETs are indexed prior to detected DCI formats from PDCCH receptions in the second CORESETs.

The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, provided by resourceList for PUCCH resources from a set of PUCCH resources provided by PUCCH-ResourceSet with a maximum of eight PUCCH resources.

Denote by NPRI the number of bits for the PUCCH resource indicator and set nPRI=2NPRI. When the size  of resourceList is larger than nPRI, when a UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a last DCI format in a PDCCH reception, among DCI formats with a value of the PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or a value of dl-DataToUL-ACKForDCIFormat1_2 for DCI format 1_2, indicating a same slot for the PUCCH transmission, the UE determines a PUCCH resource with index , , as

rPUCCH=nCCE,p⋅⌈RPUCCH/nPRI⌉NCCE,p+ΔPRI⋅RPUCCHnPRI                                          if  ΔPRI<RPUCCH mod nPRInCCE,p⋅⌊RPUCCH/nPRI⌋NCCE,p+ΔPRI⋅RPUCCHnPRI+RPUCCH mod nPRI     if ΔPRI≥ RPUCCH mod nPRI

where  is a number of CCEs in CORESET  of the PDCCH reception for the DCI format as described in Clause 10.1,  is the index of a first CCE for the PDCCH reception, and  is a value of the PUCCH resource indicator field in the DCI format. When NPRI=0, then UE determines that ΔPRI=0.

Table 9.2.3-2: Mapping of PUCCH resource indication field values to a PUCCH resource in a PUCCH resource set with maximum 8 PUCCH resources for a PUCCH resource indicator field of 3 bits

| PUCCH resource indicator | PUCCH resource |
| --- | --- |
| 3 bits |  |
| '000' | 1st PUCCH resource provided by pucch-ResourceId obtained from the 1st value of resourceList |
| '001' | 2nd PUCCH resource provided by pucch-ResourceId obtained from the 2nd value of resourceList |
| '010' | 3rd PUCCH resource provided by pucch-ResourceId obtained from the 3rd value of resourceList |
| '011' | 4th PUCCH resource provided by pucch-ResourceId obtained from the 4th value of resourceList |
| '100' | 5th PUCCH resource provided by pucch-ResourceId obtained from the 5th value of resourceList |
| '101' | 6th PUCCH resource provided by pucch-ResourceId obtained from the 6th value of resourceList |
| '110' | 7th PUCCH resource provided by pucch-ResourceId obtained from the 7th value of resourceList |
| '111' | 8th PUCCH resource provided by pucch-ResourceId obtained from the 8th value of resourceList |

End

# 7	Type-2 HARQ-ACK codebook determination with different DAI bitwidths

In NR Rel-16, it was agreed that both DCI format 1_1 and 1_2 can be used to schedule high and low priority HARQ-ACK codebook. On the other hand, the size of the DAI in DCI format 1_2 may be different in DCI format 1_1. In this case, the pseudo code in Section 9.1.3 in 38.213 for Type 2 codebook determination may not work if the UE needs to multiplex HARQ-ACKs corresponding to DCI format 1_1 and 1_2 in the same codebook. Another issue for HARQ-ACK codebook determination is that, when the size of the counter DAI in DCI format 1_2 is 0 bit, the UE can not detect missing DCI(s) corresponding to DCI format 1_2. To solve the above issues, we propose the following TP.

Modified clause (Section 9.1.3 of 38.213)

Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by  the value of the counter DAI in a DCI format scheduling PDSCH reception or SPS PDSCH release on serving cell  in PDCCH monitoring occasion  according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by  the value of the total DAI in a DCI format in PDCCH monitoring occasion  according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion .

A UE does not expect to multiplex in a HARQ-ACK codebook first HARQ-ACK information scheduled by DCI format 1_2 and second HARQ-ACK information bit scheduled by DCI format 1_0 or 1_1 if the bitwidth of the counter DAI  in DCI format 2_1 is different from that of the DCI format 1_0 or 1_1. If the bitwidth of counter DAI field in DCI format 1_2 is configured to be 0 bit, a UE does not expect to multiplex in a HARQ-ACK codebook the first HARQ-ACK information scheduled by a DCI format 1_2 and second HARQ-ACK information scheduled by DCI format 1_0 or 1_1 or 1_2.

End

# 8	CPU Release with Uplink Interruption

In the current specification, the occupied CPUs are assumed to remain occupied until the last symbol of PUCCH/PUSCH that carries the CSI report. However, in Rel. 16, and for a UE that supports the intra-UE prioritization capability or uplink cancellation indication, the channels that carry CSI may need to be cancelled. Hence, in order to not impose any additional burden on the UE for supporting these features, we propose to keep this behaviour unchanged even in cases when the uplink channels are interrupted. Hence, the following clarification is proposed to be added:

Modified clause (Section 5.2.1.6 of 38.214)

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the PUSCH carrying the report.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the PUSCH carrying the report.

If a UE reports the capability of [intra-UE prioritization] and if the PUCCH/PUSCH carrying the report is overlapping with a PUCCH/PUSCH of a larger priority index in time, the occupied CPUs are assumed to remain occupied until the last symbol of PUCCH/PUSCH carrying the report before cancellation.

End

# 9	Type-1 Codebook with Two DCI Formats

In previous RAN1 meetings, it was agreed that both DCI format 1_1 and 1_2 can be used to schedule HARQ-ACK belong to the same codebook. Furthermore, DCI format 1_1 and 1_2 may use different sets of K1 values and TDRA tables. In this scenario, the pseudo code in 38.213 may need to be changed. To address the abovementioned issues, we propose the following additions and modifications.

Modified clause (Section 9.1.2 of TS 38.213)

A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1 or DCI format 1_2. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1 or DCI format 1_2.

……

#### 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell , an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of  occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . If serving cell  is deactivated, the UE uses as the active DL BWP for determining the set of  occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values  associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for DCI format 1_1 on serving cell ,  is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for DCI format 1_0

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 for serving cell  and the DCI format 1_1 can be used to schedule HARQ-ACK in the Type-1 HARQ-ACK codebook, and the UE is either not configured to monitor PDCCH for DCI format 1_2 on the serving cell c or the DCI format 1_2 cannot schedule HARQ-ACK in the Type-1 HARQ-ACK codebook,   is provided by dl-DataToUL-ACK for DCI format 1_1

c)  If the UE is configured to monitor PDCCH for DCI format 1_2 on serving cell c and the DCI format 1_2 can be used to schedule HARQ-ACK in the Type-1 HARQ-ACK codebook, and the UE is either not configured to monitor PDCCH for DCI format 1_1 on the serving cell c or the DCI format 1_1 cannot schedule HARQ-ACK in the Type-1 HARQ-ACK codebook,  K1 is provided by dl-DataToUL-ACK-ForDCIFormat1_2-r16 for DCI format 1_2

d) If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 on serving cell c, and both DCI format 1_1 and DCI format 1_2 can schedule HARQ-ACK in the Type-1 HARQ-ACK codebook, K1 is provided by the union set of dl-DataToUL-ACK for DCI format 1_1 and dl-DataToUL-ACK-ForDCIFormat1_2-r16 for DCI format 1_2

b)	on a set of row indexes  of a table that is provided either by

- a first set of row indexes of a table that is provided by pdsch-TimeDomainAllocationList in pdsch-ConfigCommon or

- by Default PDSCH time domain resource allocation A [6, TS 38.214], or

- by the union of the first set of row indexes and a second set of row indexes, if provided by pdsch-TimeDomainAllocationList in pdsch-Config and if the following condition hold

i) the UE is configured to monitor PDCCH for DCI format 1_1 for serving cell  and the DCI format 1_1 can be used to schedule HARQ-ACK belonging to the Type-1 HARQ-ACK codebook

ii) the UE is not configured to monitor PDCCH for DCI format 1_2 on the serving cell c or the DCI format 1_2 cannot schedule HARQ-ACK belonging to the Type-1 HARQ-ACK codebook

or

- by the union of the first set of row indexes and a third set of row indexes, if provided by pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 in pdsch-Config, and if the following condition holds

i) the UE is configured to monitor PDCCH for DCI format 1_2 on serving cell c and the DCI format 1_2 can schedule HARQ-ACK belonging to the Type-1 HARQ-ACK codebook

ii) the UE is not configured to monitor PDCCH for DCI format 1_1 on the serving cell c or the DCI format 1_1 cannot schedule HARQ-ACK belonging to the Type-1 HARQ-ACK codebook,

or

- by the union of the first set of row indexes and the second set of row indexes and the third set of row indexes if the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 on serving cell c, and both DCI format 1_1 and DCI format 1_2 can schedule HARQ-ACK in the Type-1 HARQ-ACK codebook,

associated with the active DL BWP and defining respective sets of slot offsets , start and length indicators SLIV,and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214].

End

# 10        Impact of intra-UE cancellation on other physical layer procedures

# For a UE that supports intra-UE cancellation, it may cancel a PUSCH or SRS or PUCCH transmission upon receiving another high-priority grant. However, it is not clear how these canceled PUSCH/SRS transmissions will affect other physical-layer procedures.

# As an example, consider the following scenario. The gNB may indicates a first transmit power control (TPC) command δ1 on the first UL grant, and indicate a second TPC command δ2 on the second grant. Furthermore, the first PUCSH may be cancelled due to conflict with another high priority channel. However, when determining the transmit power for the second PUSCH, the UE may accumulate both δ1 and  δ2. In other words, although the first PUSCH is cancelled, the control information in the corresponding scheduling DCI shall still be honored by the UE, and may be applicable to future transmissions from the UE. Similarly, the UE may determine the NDI toggle state for the second PUSCH based on the NDI indicated in the scheduling DCI of the first PUSCH (provided that they belong to the same HARQ process).

Fig. 3. Impact of intra-UE cancellation on TPC accumulation

Based on the above observation, we make the following proposal.

Proposal 8: If a UE would cancel a PUSCH or PUCCH or SRS due to collision with another high priority transmission from the same UE, the UE behavior at least for the following procedure shall not be affected by the cancellation of the PUSCH, PUCCH or SRS

- TPC accumulation

- NDI determination

- FFS other physical and MAC layer procedures

# 11	UE Procedure to cancel configured uplink transmission

In RAN1 #98b it was agreed that:

Agreement:

For both DG and CG with “Rel-16 PUSCH transmission scheme”, if dynamic SFI is not configured, semi-static flexible symbols are used for PUSCH. Segmentation occurs at least around semi-static DL symbols.

- FFS segmentation also around dynamically indicated invalid symbols for UL transmissions in the UL grant (if supported for DG and/or Type 2 CG) and/or semi-statically configured invalid symbols for UL transmissions (if supported)

- FFS how to handle the conflict with dynamic DL transmission for CG

In this section we discuss the second FFS. We expand the discussion here into a more general case where the uplink transmission is a configured PUSCH transmission, or a configured PUCCH carrying HARQ-ACK for SPS PDSCH reception. In Rel-15, dynamic SFI or a dynamic indication of PDSCH or CSI-RS reception may cancel configured UL transmission, when Tproc,2 timeline with is met, as specified in 38.214. First, it should be noted that, this part of spec is not supported anywhere else in Rel-15 specification. More precisely, a Rel-15 UE was not supposed to cancel an ongoing uplink transmission through indication by a DCI. Second, the procedure needs to be revisited when priority of configured PUSCH or PUCCH is higher than DCI scheduling PDSCH and/or triggering CSI-RS.

To address the second issue above, we propose the following additions and modifications:

Modified clause (Section 11.1.1 of TS 38.213)

If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1, or DCI format 0_2 or DCI format 1_2, indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the UE does not expect to receive a PDSCH scheduled or a CSI-RS triggered by a PDCCH where the priority indicator field in the corresponding DCI is either not configured or set to 0, that overlaps with the transmission of a PUSCH corresponding to a configured grant with a priority set to 1 in configuredGrantConfig, or a PUCCH configured to carry HARQ-ACK for SPS PDSCH receptions with a priority set to 1 in SPS-Config

-	the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time  for the corresponding PUSCH processing capability [6, TS 38.214] assuming  and  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0, DCI format 1_0, DCI format 1_1 or DCI format 0_1 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0

-	the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols.

# 12        Priority indication for DCI format 0_1/0_2/1_1/1_2

In NR Rel-16, it was agreed that both DCI format 0_1 and DCI format 0_2 can be used to schedule high and low priority HARQ-ACK and PUSCH, and this feature is UE optional.

Agreement:

When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities.

- This feature is UE optional

For a UE that is not capable of supporting this feature (i.e., dynamic priority indication with mixed DCI formats), a DCI format (from the formats 0_1/1_1/0_2/1__2) may either schedule a high priority transmission or a low priority transmission, but not both. Therefore, these UEs need to determine the priority of transmissions associated with the DCI formats 0_1/1_1/0_2/1_2.

One naïve approach to determine the priority of HARQ-ACK or PUSCH scheduled by DCI format 1_1/1_2 or 0_1/0_2 is to configure a priority indication field in at least one of the DCI formats, and let the DCI format always indicate high priority. Note that, as agreed in previous RAN1 meetings, if the priority indication field is not configured in a DCI format, the transmissions scheduled by the DCI format is always treated as low priority. Thus, as long as the priority indication field is configured in a DCI format, the priority indication may be used to indicate high priority. The problem with this approach is that, the priority indication bit configured in a DCI format will be wasted, since its value cannot be dynamically changed. As such, this contradicts the motivation to introduce the new DCI format, and will inevitably impact the reliability of the new DCI format.

A better approach is to let the associate the DCI formats 0_1/1_1 and 0_2/1_2 with fixed priorities. In principle, if only one priority is configured to the UE, then both DCI format 0_1/1_2 and 0_2/1_2 are associated with low priority. If two priorities are configured to the UE, then it is reasonable to have 0_1/1_1 to be associated with low priority, and to have 0_2/1_2 to be associated with high priority.

For DCI format 1_1/1_2, this approach is straightforward. Indeed, the UE may determine whether one or two HARQ-ACK codebooks are configured by checking the number of PUCCH-configs configured to the UE. However, for DCI format 0_1/0_2, this approach is less straightforward, since there is no direct mechanism to let the UE know whether one or two PUSCH priorities are configured. The only proxy available for this purpose is the RRC parameter PriorityIndicator-ForDCIFormat0_2, which was designed to indicate whether the priority indicator field is present in the DCI format 0_2 or not. If this RRC parameter is set to enabled, then it means that there’re two priority levels configured for PUSCH. In our view, the easiest approach to enable the priority determination for DCI format 0_2 is to reuse this parameter but change the possible value of this parameter to be {dynamic, semi-static}. This way, if PriorityIndicator-ForDCIFormat0_2  is set to dynamic, then the UE determines that there’re two PUSCH priorities configured, and the UE will have 1 bit priority indication field in the DCI to dynamically indicate the priority of the scheduled PUSCH using the DCI. If PriorityIndicator-ForDCIFormat0_2 is set to semi-static, then the UE determines that there’re two PUSCH priorities configured, and DCI format 0_2 will always indicate high priority. If PriorityIndicator-ForDCIFormat0_2  is not configured, then there is only one PUSCH priority, and the PUSCH scheduled by DCI format 0_2 is associated with the low priority.

Based on the discussions above, we have the following proposal:

Proposal 9: If a UE is not capable of supporting dynamic switching of HARQ-ACK/PUSCH priority via both of the pairs 0_1/1_1 or 0_2/1_2 when the UE is configured with DCI format 0_1 / 1_1 and 0_2/1_2, the UE is expected to assume a fixed priority by DCI format

- DCI format 0_1/1_1 is always low priority

- If two HARQ-ACK priorities are configured, then DCI 1_2 is high priority; otherwise it is of low priority

- For DCI format 0_2, change the value range of the RRC parameter PriorityIndicator-ForDCIFormat0_2 from {enabled} to {dynamic, semi-static};

  - If this parameter is configured to be semi-static, the DCI format 0_2 is of high priority; if this parameter is not configured, then DCI format 0_2 is of low priority;

  - UE is not expected to be configured with PriorityIndicator-ForDCIFormat0_2 to be dynamic.

The proposed TP can be found below. In the TP below, the capability [XXXX] is aimed to define the capability that both DCI format 0_1/1_1 and 0_2/1_2 can be used to schedule PDSCH/PUSCH with different priorities.

Modified clause (Section 9 of 38.212)

A PUSCH or a PUCCH, including repetitions if any, can be of priority index 0 or of priority index 1. If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0. If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, and the UE indicates the capability of [XXXX], a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. Otherwise, if the UE does not indicate the capability of [XXXX],

- A priority index associated with a PUSCH scheduled by DCI format 0_2 can be provided by PriorityIndicator-ForDCIFormat0_2. If PriorityIndicator-ForDCIFormat0_2 is configured to be semi-static, a PUSCH transmission scheduled by DCI format 0_2 is of priority index 1. If by PriorityIndicator-ForDCIFormat0_2 is not configured, then a PUSCH transmission scheduled by DCI format 0_2 is of priority index 0. UE is not expected to be configured with PriorityIndicator-ForDCIFormat0_2 equal to dynamic.

- If two HARQ-ACK codebooks are configured, then a PUCCH scheduled by DCI format 1_2 is of priority index 1; otherwise, a PUCCH scheduled by DCI format 1_2 is of priority index 0.

If, after resolving overlapping for PUCCH and/or PUSCH transmissions of a same priority index, a UE determines to transmit

-	a first PUCCH of larger priority index, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE does not transmit the PUSCH or the second PUCCH

-	a PUSCH of larger priority index, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE does not transmit the PUCCH

-	a first PUSCH of larger priority index on a serving cell, a second PUSCH of smaller priority index on the serving cell, and a transmission of the first PUSCH would overlap in time with a transmission of the second PUSCH, the UE does not transmit the second PUSCH, where at least one of the two PUSCH is not scheduled by a DCI format

End