| 3GPP TSG-RAN WG1 Meeting #101 e-Meeting, May 25th – June 5th, 2020 | R1-2004498 |
| --- | --- |

Agenda item:	8.4.1.2

Source: 	Qualcomm Incorporated

Title: 	Baseline FR2 coverage performance

Document for:	Discussion/Decision

# Introduction

In this document we present our results on coverage of 5G NR using link level simulations, followed by a link budget analysis. For this study, basic parameters necessary for link level evaluations are partly chosen to be in line with the parameters specified in the 37.910 technical report titled ‘Study on self-evaluation towards IMT-2020. The difference is that effect of shadowing and body and head loss is also incorporated in the link budget analysis. Parameters such as UE EIRP is selected based on 3GPP TS 38.101-2 while for gNB EIRP is based on existing and upcoming deployments

# LLS Assumption for Baseline Coverage Performance

A list of parameters common to all link-level simulations for each test environment is given below. These parameters are applied to all uplink/downlink channels.

Common parameters

Table 1. General simulation assumptions

| Parameter | Urban |
| --- | --- |
| Carrier Frequency | 28GHz |
| Multiplexing | TDD (DDDU) |
| System BW | 100MHz |
| gNB EIRP | 60 dBm |
| UE EIRP | 22.4 dBm |
| Numerology | 120kHz SCS |
| BS antenna gain (dB) | 5 dB |
| UE antenna gain (dB) | 4 dB |
| Thermal noise level | -174 dBm/Hz |
| Channel | NLOS (CDL-B 100ns), |
| UE speed | 3km/h |
| # of gNB antennas | 256 per polarization (32x8) |
| # of UE antennas | 4 per polarization |
| gNB array gain | 17dB for broadcast, 22.5dB for unicast |
| # of gNB beams | 33 for broadcast, 132 for unicast |
| # of cells | 1 |
| UL data rate target | 50kbps, 5Mbps |
| DL data rate target | 1Mbps, 25Mbps |

Channel-specific parameters

With these common parameters as the basis, we then proceed to pick channel-specific parameters that are used for the link budget analysis. In general, the parameters are chosen such that they are representative of the basic requirements to sustain a 5G NR link in uplink and downlink. In specific, with a cell-edge UE in mind, when reasonable, a combination of minimal payload and maximum allowable resources for a control channel are chosen so as to enable sustaining the control channel in low SNR regimes. Our choice of parameters for each of the control and data channels are presented below.

The following table lists the parameters chosen for PUCCH. PUCCH payload sizes are chosen to be either 1 bit or 11 bits.   For PUCCH with 11 bits, it is chosen to ensure that UE can report at least one beam (7 bits for RSRP and 4 bits for using 16 SSBs) for beam switching purpose. If two beams are reported in PUCCH, the payload will increase to 19 bits considering up to 16 SSBs are used.  For a 1-bit payload, PUCCH Format 1 is chosen. For the 11-bit payload, PUCCH Format 3 spanning 14 OFDM symbols are chosen. In all instances a single RB allocation is chosen.

Table 2 PUCCH Parameters for link-level simulations and link budget analysis

| Parameter |  |  |
| --- | --- | --- |
| PUCCH format | PUCCH format 1 | PUCCH format 3 |
| UCI payload (bit) | 1 bit | 11 bits |
| PUCCH duration (symbols) | 14 | 14 |
| Number of UEs | 1 UE | 1 UE |
| # RBs for long PUCCH | 1 RB | 1 RB |
| Frequency hopping | Enabled | Enabled |

The next table presents the basic parameters for PUSCH. An entire uplink slot is assumed to be dedicated to the transmission of PUSCH, with 11 data symbols and 3 DMRS symbols. Special attention is also given to msg3, which is also a PUSCH transmission with a payload size of either 56 bits or 72 bits: we use 56 bits for link budget analysis

Table 3 PUSCH Parameters for link-level simulations and link budget analysis

| PUSCH Parameter | Value |
| --- | --- |
| Slot structure | 11 data symbols and 3 DMRS symbols |
| Waveform | CP-OFDM |
| PUSCH rank | 1 |
| PUSCH RBs | Variable 1-64 RBs |
| PUSCH DMRS | Type 1 with 3 symbols with no data on DMRS symbols |
| Precoding | [1 1] precoder with CDD across ports. |
| Msg3 payload size | 56 bits |
| Performance metric | 10% BLER after 1st tx |

PRACH format B4 is used for our link level simulations.

Table 4 PRACH Parameters for link-level simulations and link budget analysis

| Parameter | Value |
| --- | --- |
| Format | B4 (12 symbols, sequence length of 139) |
| Bandwidth | 12 RBs |
| Numerology | 120kHz SCS |
| Frequency offset | +/- 0.05 ppm at TRP, +/-0.1 ppm at UE |
| Performance metric | 0.1% false alarm, 1% miss-detection |

# Baseline FR2 Coverage

Link budget analysis for UL and DL channels is presented in Table 5. Note that the effects of shadowing, body and head loss are also incorporated in the link budget analysis

Table 5. Link budget for different channels based on the link-level simulation results

|  | Channel | Bandwidth (MHz) | Power Boost | Link budget (dB) |
| --- | --- | --- | --- | --- |
| DL | Unicast PDSCH, 25Mbps | 100 | -- | 132.6 |
|  | Unicast PDSCH, 1Mbps | 100 |  | 147.4 |
|  | PSS/SSS | 28.8 | Yes | 128.8 |
|  |  |  | No | 123.4 |
|  | PBCH | 28.8 | Yes | 127.8 |
|  |  |  | No | 122.4 |
|  | Msg2 PDCCH (also RMSI PDCCH) | 69.12 | Yes | 124.7 |
|  |  |  | No | 123.1 |
|  | Unicast PDCCH | 69.12 |  | 130.2 |
|  |  |  | No | 128.6 |
| UL | Unicast PUSCH, 5Mbps | 100 |  | 119.7 |
|  | Unicast PUSCH, 50kbps | 5.76 |  | 138.6 |
|  | Msg3 PUSCH, 56 bits payload | 5.76 |  | 128.6 |
|  | PRACH | 16.68 |  | 127.8 |
|  | Msg4 PUCCH (1 bit) Based on 0.01 error rate of ACK to DTX, as Msg4 PUCCH is ACK-only | 5.76 |  | 131 |
|  | PUCCH (1 bit) for unicast PDSCH ACK/NACK | 5.76 |  | 140.5 |
|  | Unicast PUCCH (11 bits) L1-report for one beam | 5.76 |  | 131.5 |
|  | Unicast PUCCH (19 bits, total 25 bits with CRC) L1-report for two beams | 5.76 | --- | 128.5 |

A summary of potential bottlenecks (and their link budget different compared to PBCH with power boost) is shown in Table 6:

Table 6. Summary of the main link budget bottlenecks and vulnerable channels

| Link budget (in dB) | RMSI PDCCH | Msg2 PDCCH | Msg3 | PRACH | L1-report for two beams | 5Mbps PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
| With power boost Without power boost | 124.7 123.1 | 124.7 123.1 | 128.6 | 127.8 | 128.5 | 119.7 |
| Target of 127.8 dB link budget (PBCH with pow boost) | -3.1 -4.7 | -3.1 -4.7 | +0.8 | 0 | +0.7 | -8.1 |

As shown in the simulation results, downlink broadcast channels and Msg1/2/3 during RACH procedure have a relative weakness coverage in FR2, because of lower gain of wide broadcast beams (i.e. SSB beams). Among broadcast channels, RMSI PDCCH is the main bottleneck.

For RACH, the whole procedure can be benefited from coverage enhancement. In particular, Msg2 PDCCH is the clear bottlenecks, and PRACH and Msg3 PUSCH are also vulnerable.

Among the unicast channels, 5Mbps PUSCH is the clear bottleneck. However, it should be noted that the link budget for PUSCH is very much dependent on the rate and targeting a lower UL rate for coverage may be more reasonable for FR2.

Observation 1:  Unicast PUSCH with high targeted data rate, e.g., 5Mbps, and broadcast PDCCH are the main bottlenecks in FR2.

Proposal 1: Methods for coverage enhancement of broadcast PDCCH, e.g., RMSI PDCCH, should be studied.

Proposal 2: Methods for coverage enhancement of RACH procedure, especially Msg2 PDCCH and Msg3 PUSCH and PRACH, should be studied.

It should be noted that the link budget analysis that is presented for unicast channels assumes reliable assignment of the best beam. In case that the current serving beam is becoming weaker, the gNB needs to rely on L1 report (L1-RSRP) transmitted over PUCCH (which is at least 11 bits for one beam), while the beam gain is much lower than the nominal value. Therefore, in practice, PUCCH carrying L1-RSRP report may need coverage enhancement, even though the link budget analysis does not show it. It should be also noted that when two or more beams are reported, the PUCCH payload becomes larger. For example, for the L1-report of two beams, assuming 4bits for the index of each beam, the payload becomes 4+4+7+4=19 bits which can have more coverage vulnerability (as shown in the link budget analysis).

Observation 2: L1-report can have coverage vulnerability, especially for the case of reporting more than one beam.

Proposal 3: Methods for coverage enhancement of L1 report should be studied.

# Conclusion

In this document, a preliminary link-level evaluation of FR2 baseline coverage was presented. Based on the simulation results, the following are the observation and proposals:

Observation 1: 5Mbps unicast PUSCH, Msg3 PUSCH, and broadcast PDCCH are the main bottlenecks in FR2.

Proposal 1: Methods for coverage enhancement of RMSI PDCCH should be studied.

Proposal 2: Methods for coverage enhancement of RACH procedure, especially Msg2 PDCCH and Msg3 PUSCH, should be studied.

Observation 2: L1-report can have coverage vulnerability, especially for the case of reporting more than one beam.

Proposal 3: Methods for coverage enhancement of L1 report should be studied.

# Reference

- TR37.910

- TS38.101-2

- R1-2004499, “Potential techniques for coverage enhancement”, Qualcomm