3GPP TSG-RAN WG1 #101-e	R1-2004500

e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.1.1

Source: 	Qualcomm Incorporated

Title: 	NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

Document for:	Discussion/Decision

# Introduction

In RAN #86, a Rel-17 study item for NR operation in a frequency regime between 52.6GHz and 71GHz has been approved [1]. As a part of the study item, the study on the waveform is focused on the feasibility of using existing DL/UL waveforms and required changes for the specific frequency regime. Additionally, scaling of numerologies and modification of parameters (e.g., channelization, bandwidth, etc.) of existing FR2 can be studied, considering any practical RF impairments in the frequency regime over 52.6GHz, including higher phase noise, larger propagation loss, and lower power amplifier efficiency, to name a few. In particular, attention should be paid to identify any potential critical issues with physical signal and channels in the high frequency regime, so that they could be addressed during the work item phase [2].

In this contribution, we discuss some key aspects of numerology selection and waveform design, and figure out potential issues for the high frequency regime. In the discussion, we use current FR2 design as baseline. On the other hand, since a main use case of 52.6GHz to 71GHz is unlicensed operation, some design considerations for Rel-16 NR-U can be applicable as well. For example, since the channel access can be under LBT requirement, the enhancements to HARQ and Configured Grants can directly apply. Similarly, considering that we also have PSD limitation for unlicensed band operation in 60GHz band, Rel-16 NR-U design for PUSCH, PUCCH, and PRACH can be revisited for applicability. Most of the work can be done in the work item phase, but in the study item phase, we need to identify issues.

# Discussion

## Numerologies for high frequency regime

In order to minimize specification burden and maximize the leverage of FR2 based implementations, it would be desirable to extend the FR2 operation up to 71GHz with minimal changes, possibly with the adoption of one or a few new numerologies (i.e., larger subcarrier spacings than 120kHz). In this section, we discuss the feasibility of different numerologies in the high frequency regime, with the support of evaluation results provided in Section 2.3.

### Reusing existing FR2 numerology

As the first step of the feasibility study, direct application the existing FR2 numerologies to the high frequency regime can be considered. In other words, 60kHz and 120kHz SCS may be used for control and data channels, and 120kHz and 240kHz may be used for SSBs in the high frequency regime. Also, for the PRACH, different preamble formats of different lengths with the SCS matched to the control and date channels can be used. Note that, to minimize the specification burden, the strategy should be focused on reusing the FR2 design without any changes or with minor modifications. Besides the reduced specification burden, the strategy of reusing the FR2 numerology has additional benefits:

- Expedited provision of imminent commercial opportunities for high data rate communication

- Low non-recurring expense in addition to FR2 networks

- Relatively large coverage, comparable to FR2 (i.e., with up to 400MHz per-CC bandwidth)

Although the spectral proximity between FR2 and the high frequency regime may hint the feasibility of reusing the FR2 numerology, the factors that are specific to the high frequency regime, such as practical deployment scenarios or RF impairments, should carefully be investigated. For example, due to excessive phase noise and Doppler effect in high frequency regime, the study on the robustness of physical channels should be revisited. In Section 2.3.2, block error rate (BLER) curves of PDSCH in the high frequency regime are plotted for different SCSs. In the performance evaluation, an existing PTRS pattern is assumed for phase noise compensation. Additionally, for further reference, PDSCH performances without PTRS are also provided.

The evaluation results in Section 2.3.2 provide important insights related to the use of legacy FR2 numerologies (i.e., 60kHz and 120kHz) for the high frequency regime. For example, when PTRS-based phase noise correction is not applicable, the following observations are made from Figure 2:

Observation 1: For low MCSs (e.g., MCS 7) with QPSK modulation, no or little performance degradation is observed for SCSs 60kHz and 120kHz in the high frequency regime, even if PTRS-based phase noise correction is not applied.

Observation 2: For mid-range MCSs with 16QAM modulation, SCS 120kHz sustains the performance while SCS 60kHz starts showing an error floor.

From Observation 1, it can be concluded that the performance of broadcast messages without PTRS, such as SIB1, paging, and Msg 2/4 transmissions with legacy FR2 numerologies would not be impacted in the high frequency regime. Furthermore, if we translate Observation 2 to the uplink channels without PTRS, it follows that Msg A PUSCH or Msg 3 transmission up to 16QAM modulation would not be impacted, at least for SCS 120kHz.

When PTRS-based phase noise correction is applicable, e.g., for unicast channels, the followings are observed from Figure 3:

Observation 3: SCS 60kHz in the high frequency regime suffers from severe performance degradation at very high MCSs (e.g., MCS 28), even with PTRS-based phase noise correction.

Observation 4: SCS 120kHz in the high frequency regime starts showing an error floor at very high MCSs (e.g., MCS 28), even with PTRS-based phase noise correction. However, the error floor is sustained below 1%.

Based on the observations above, it would be concluded that 60kHz SCS does not suit the high frequency regime due to the phase noise susceptibility. On the other hand, provided that the phase noise is corrected with PTRS, 120kHz SCS retain reasonable performance below 1% BLER throughout all MCSs.

### New numerologies higher than 120kHz

As discussed in the previous section, applying 120kHz SCS in the high frequency regime would be beneficial in various aspects, in particular for applications and scenarios requiring a relatively large coverage with low to medium peak rates, such as eMBB with outdoor deployment. However, during the Rel-16 study on NR beyond 52.6GHz (TR 38.807), more diverse potential use cases and deployment scenarios have been identified for the high frequency regime. A prominent case among them is the very high peak rate, short-range communication, which may not be efficiently served by the legacy 120kHz SCS. Furthermore, in the indoor hotspot scenario and on 60GHz unlicensed spectrum, NR would have to coexist with IEEE 802.11ad/ay, which currently supports a very high peak date rate with a large bandwidth, e.g., multiple of 2.16GHz. Therefore, at least for efficient spectrum sharing and for competitiveness, NR in the high frequency regime should also support a very large CC bandwidth (e.g., ~2GHz).

With a low SCS, without relying on CA, supporting a very large bandwidth is rather challenging and requires some modification of existing specification. That is to say, with up to 275 PRBs and 4k FFT, current NR specification can support up to 400MHz CC bandwidth with 120kHz SCS. Thus, to support 2GHz bandwidth, the number of maximum PRBs per CC and the FFT size should be increased. This may impose additional burden from the specification perspective, as well as from the implementation perspective.

As an alternative approach, a higher SCS than 120kHz may be introduced for the high frequency regime. The values of maximum supported CC bandwidth for different SCSs, with up to 275 PRB allocation (4k FFT), are listed in Table 1. From the table, with 960kHz SCS, over 2GHz CC bandwidth can readily be supported without changing the current RRC specification. Additionally, 960kHz SCS offers enhanced robustness to the phase noise, which is verified by the performance evaluation in Section 2.3.2.

Observation 5: SCS 960kHz in the high frequency regime retain high performance throughout all MCSs, if the PTRS-based phase noise correction is used.

As a fair concern, the larger the SCS is, the more susceptible to the inter symbol interference the performance is, because the CP length decrease with the SCS. However, in the expected deployment scenarios for the high SCS, such as indoor hotspot or personal area network, the channel delay spread is not as severe as other scenarios. Also, in the high frequency regime, as well as in FR2, spatial beamforming with antenna arrays is typical mode of operation; with a highly directional beam, only a portion of the multi-path components of the channel would be captured and the effective delay spread is reduced. To justify the argument, the performance of SCS 960kHz has been tested with different values of the RMS channel delay spread, from 10ns to 200ns, in Section 2.3.3. From the results in Section 2.3.3, the following is observed:

Observation 6: SCS 960kHz with NCP (73ns) shows reasonable performance to the channel delay spread up to 200ns.

Along with the introduction of a higher SCS, such as 960kHz, it would be natural to consider the same SCS for SSB and PRACH, at least for the ease of multiplexing among different channels. Therefore, based on Observation 1 through Observation 6, the following is proposed:

Proposal 1: For physical control, data, and random access channels and for SSB in the high frequency regime from 52.6GHz to 71GHz, SCSs of 120kHz and 960kHz should be considered.

Table 1: Maximum supported CC bandwidth for different SCSs, with 4k FFT and 275 PRBs.

| SCS | 60kHz | 120kHz | 240kHz | 480kHz | 960kHz |
| --- | --- | --- | --- | --- | --- |
| Max. Bandwidth | 198MHz | 396MHz | 792MHz | 1584MHz | 3168MHz |

## Potential issues with operation in high frequency regime

### RF impairment

Utilizing the high frequency regime above 52.6GHz, there are some challenges that have not been explored enough in Rel-15 and Rel-16, since they are either less significant or not problematic in FR1 and FR2. The challenges include higher phase noise, lager propagation loss (e.g., due to high atmospheric absorption), low power amplifier efficiency, etc. In this subsection, some potential RF-related issues that may impact the study on the high frequency regime are discussed.

#### Phase noise

Phase noise has been casually regarded as a source of nonideality during FR2 waveform and signal/channel design. For the high frequency regime between 52.6GHz and 71GHz, however, the impact of phase noise would be exacerbated, and further investigation is required in relation to the study on the candidate numerologies of the high frequency regime. Therefore, as the first step, a study on the realistic phase noise model for the high frequency regime is necessary.

In 3GPP, a preceding study on the phase noise model for FR2 has been conducted and the results are captured in TR 38.803 (Subclause 6.1.10 or 6.1.11). For example, the example 1 model in TR 38.803 is based on measurements of a prototype system at 29.55GHz frequency and fitted by a generalized multi-pole/zero model. Having the 29.55GHz model as a baseline, in TR 38.803, projected phase noise models for some higher carrier frequencies, such as 45GHz and 70GHz, have also been provided. The projected high frequency models may offer some insight on the phase noise in the high frequency regime, but it is still questionable whether one or two discrete models are sufficient to represent the broad spectrum of the high frequency regime. It would desirable to consult RAN4 about a realistic phase noise model, which can characterize the broad spectrum of the high frequency regime, but it does not look viable considering the limited SI time units in RAN1. Therefore, RAN1 may stick to the existing phase noise models, e.g., those in TR 38.803, during the SI phase. Additionally, some modifications can be considered to the existing models, to reflect the difference between FR2 and the high frequency regime. For example, a scaling factor depending on the frequency difference may be applied, as shown in Appendix (Section 5.1). Once a baseline PN model or any modification is determined, RAN1 may further send a LS to RAN4 to confirm whether it is feasible.

Proposal 2: Existing FR2 phase noise models, possibly with some modifications, are reused for the study on the high frequency regime. RAN1 may send a LS to RAN4 asking for further confirmation.

#### Power amplifier nonlinearity

During the studies in Rel-15 and Rel-16, the PA nonlinearity has not collected much attention, because it only has noticeable impact on small and power-limited devices and thus is more important for UL transmission. However, in the high frequency regime, since the efficiency of power amplifier typically decreases with the operating frequency, it is more likely that the PAs of both UE and gNB are driven near the nonlinear (saturation) region [3]. The PA nonlinearity corrupts the envelope and constellation of the transmit signal and causes frequency dependent distortion, which would impact the DL and UL performance with respect to:

- Transmit spectrum characteristics and mask

- Multiuser interference between FDM separated UEs

- Average transmit power for a given modulation and waveform

Although PAPR and cubic metric provide a rough guideline to determine a proper operating point for nearly linear PA operation, they do not comprehensively describe the waveform performance and their unqualified use can lead to unrealistic expectations about the actual performance of certain waveform. Therefore, it is important to take a realistic PA model into account during the studies.

It is RAN4’s area to set up RF requirements combining diverse factors, such as band, spectrum utilization, PA nonlinearity, etc. For example, abstracting RF nonidealities from different sources, RAN4 provides Tx EVM requirements in TS 38.104. During the earlier study on NR waveform in RAN1, there was an effort to capture a realistic PA model for link-level evaluation of sub 6GHz and above. As an outcome of the effort, a modified Rapp model was recommended by RAN4 [4]. More specifically, the modified Rapp model had been used in studies for radio systems at 60GHz carrier frequencies with wideband, e.g., ~2GHz bandwidth [3]. Therefore, if needed, the modified Rapp model may be considered as the nonlinear PA model for the study on the high frequency regime.

Proposal 3: If needed, the impact of power amplifier nonlinearity may also be investigated during the study on the high frequency regime.

### Physical signal/channel design issues

Reusing a legacy FR2 numerology, such as 120kHz SCS, for the high frequency regime is beneficial from the standardization perspective, since the mature designs of FR2 may largely be applied with a minimum effort. On the other hand, in order to introduce a new higher numerology for the high frequency regime, such as 960kHz SCS, all the issues related to the waveform and signal/channel designs need to be carefully revisited, particularly with the RF characteristics, propagation environment, and practical UE’s capabilities into consideration. In this subsection, the design issues are categorized and discussed.

#### Physical control and data channel design

At the first glance, the short slot duration is an apparent source of many problems, in the design of physical channels with a higher numerology. Although the slot duration contracts with the increasing numerology, device’s processing capability does not necessarily scale with the numerology in practice. Thus, although the same structure of physical channels and associated procedures are maintained by the principle of scalable numerology in NR, the frequency-range-specific or numerology-specific configuration and parameters may be determined separately.

As an example, for PDCCH, the maximum number of blind decoding or non-overlapping CCEs for each numerology does not directly scale with the numerology, but it is rather determined taking a feasible device implementation into account. Also, for PDSCH or PUSCH, NR already supports different processing capabilities in number of symbols, which do not directly scale with the numerology. For the new higher numerology, the same principle should be applied. However, due to the extremely short slot duration, some timeline-related problems may emerge. For example, due to limited PDCCH processing capability, either the maximum number of BD/CCE per slot may be too small, or the PDCCH monitoring periodicity as small as one slot may not be supported for some devices, which detrimentally results in limited scheduling flexibility. Likewise, the PDSCH or PUSCH processing timeline would excessively be relaxed due to limited capability, which may result in reducing the scheduling duty cycle or increasing the maximum number of HARQ processes.

#### Physical signal design

The extremely short slot and symbol duration of the new higher numerology in the high frequency regime can also impact the physical signal design.

RS patterns with beam sweeping

For the multi-beam operation in FR1 and FR2, any RF interruption time caused by a change of the Tx/Rx beams is invisible to the Tx/Rx processes, because it is tacitly assumed to be absorbed in the CP duration. However, the assumption would be infeasible with the new numerology. For a quick reference, with 120kHz SCS, the CP duration amounts to 586ns; this may be long enough to contain the beam switching interruption time and further provide enough margin to avoid the inter-symbol interference in the dispersive channels. However, with 960kHz SCS, the CP duration shrinks to 73ns, which may impose stringent design challenges to the RF frontend or incur performance degradation in the multi-beam operation. Therefore, the short CP duration may affect any RS patterns requiring beam sweeping, such as SSB or CSI-RS for beam management.

To avoid the design challenges or performance degradation, a beam switching gap of one or more whole symbols may be explicitly defined, so that the UE is not required to transmit or receive during the gap on the cells that share the same RF frontend. For example, the SSB pattern with the higher numerology may take the beam switching gap into account between two adjacent bursts. In Figure 1, some candidate SSB patterns with the beam switching gap are shown in comparison with the existing 120kHz SSB pattern.

(a)

(b)

(c)

Figure 1: SSB patterns: (a) SSB pattern for 120kHz SCS, (b) and (c) new SSB pattern with beam switching gap

Time and frequency acquisition and tracking

For time and frequency acquisition and tracking with SSB and TRS, the acquisition range and tracking resolution depend on the selected numerology, since the spacing between the SSB or TRS REs in absolute time and frequency is determined by the numerology. Thus, when the numerologies of the SSB and TRS are scaled, the characteristics of time-frequency estimation may also change. If the existing 1 or 2-slot TRS pattern is applied to the higher numerology, the TRS symbols are more densely placed in the time domain due to the short slot duration. Although this may increase the range of the frequency and Doppler spread estimation, the resolution of the estimation would be decreased. On the other hand, since the TRS tones are more sparsely located due to the larger SCS, the range of timing and delay spread estimation may also be impacted.

Therefore, for the application of existing SSB and TRS patterns to the high frequency regime, a thorough study on the time and frequency acquisition and tracking performance should be conducted. If any performance issue is revealed from the study, new designs of the time-frequency patterns of SSB or TRS should be pursued during the work item phase.

Channel estimation

NR supports various DMRS patterns for different physical channels. Particularly for shared data channels, some parameters associated with the DMRS (e.g., time/frequency density, CDM pattern, etc.) are configurable, in order to adopt to the operating conditions. Even with the flexibility of existing DMRS patterns, the shrink of the slot duration may affect the channel estimation performance.

For example, due to the large SCS, the distance between two adjacent DMRS REs in the frequency domain is large. In the highly frequency-selective channels with MIMO transmission, this may result in the loss of orthogonality between DMRS ports in the same CDM group. Consequently, either the channel estimation performance may be degraded, or the complexity would be increased. Therefore, new designs for the DMRS patterns and multiplexing schemes may need to be investigated for improved channel estimation performance in the high frequency regime.

#### Timing relationship

The extremely short slot and symbol duration of the higher numerology may impact the timing relationship of physical processes. This is mainly because of the fact that the device’s processing capability does not directly scale with the slot and symbol duration. As a result, a feasible set of numbers for the PDSCH/PUSCH processing capability (i.e., N1 and N2) cannot simply be extrapolated from the existing values for other numerologies. Furthermore, to avoid the excessive Rx buffering during PDCCH processing, scheduling restriction by means of minimum scheduling offset values (i.e., K0min/K2min) may be applied. The maximum number of HARQ processes would also be affected by the processing timeline, as discussed in Section 2.2.2.1. Further aspects of processing timelines that need further study are listed below:

- PDSCH/PUSCH processing capability (i.e., N1 and N2)

- Minimum scheduling offsets (e.g., K0min/K2min) and aperiodic CSI-RS/SRS triggering offsets

- Maximum number of HARQ processes

- Dynamic SFI and SPS/CG cancellation timing

- Minimum time gap for wake-up and SCell dormancy indication (DCI format 2_6)

- BWP switch delay

- Multi-beam operation timing (timeDurationForQCL, beamSwitchTiming, beam switch gap, etc.)

- Aperiodic CSI computation delay (i.e., Z and Z’)

Therefore, based on discussion above, the following is proposed in relation to the physical signal and channel design for the high frequency regime:

Proposal 4: When a new numerology (e.g., 960kHz) is introduced for the high frequency regime, the impact of the extremely short slot duration to the existing physical signals and channels should be investigated.

## Performance evaluation

### Evaluation settings

In the evaluation, we particularly focused on two types of RF nonidealities: phase noise and power amplifier (PA) nonlinearity. For the phase noise model, a projected model can be derived from a baseline model, which is defined in TR 38.803, based on the scaling rule given in Appendix (Section 5.1). For the nonlinear PA model, we assume a modified Rapp model, which is given in Appendix (Section 5.2).

The parameter settings for the evaluation are given in Table 2.

Table 2: Simulation parameters for high frequency regime.

| Parameters | Values |
| --- | --- |
| Carrier frequency | 60 GHz |
| Bandwidth | 400MHz |
| Antenna config | (M,N,P,Mg,Ng)=(4,8,2,2,2) @gNB, (M,N,P,Mg,Ng)=(2,4,2,1,2) @UE |
| Doppler | 167Hz (3km/h at 60GHz) |
| Channel Model and pre-beamforming DS | CDL-B with 10ns delay spread |
| Phase noise mask | Scaled version of 29.55GHz model (See Appendix) |
| Nonlinear PA model | Modified Rapp model (See Appendix) |
| SCS | 60/120/240/480/960KHz |
| CP config | Normal CP |
| PDCCH symbol index | 0, 1 |
| PDSCH symbol index | 2-11 |
| Number of allocated RBs | 512/256/128/64/32 (for 60/120/240/480/960kHz SCS, respectively) |
| DMRS symbol index | 3, front-loaded |
| PTRS | symbol index 4-12, every 4 RB |
| MCS table | 64QAM |

### DL performance in the presence of phase noise

This section is devoted to the PDSCH performance evaluation in the high frequency regime in the presence of phase noise.

- Performance without PTRS-based phase noise correction

A set of PDSCH performance curves are plotted in Figure 2 for different MCSs and SCSs. In the figure, the horizontal axis represents the pre-beamforming CINR at each Rx antenna element and the vertical axis represents the PDSCH BLER for the initial transmission (i.e., w/o HARQ combining). For each MCS and SCS pair, a pair of curves is provided; the first one of the pair corresponds to the PDSCH BLER in the absence of phase noise, which can be used as a benchmark, and the other corresponds to the PDSCH BLER in the presence of phase noise. For both cases, PTRS-based phase noise correction is not applied.

In Figure 2 (a), it is observed that the phase noise has minor impact on the performance for low MCS with QPSK modulation. However, as the MCS increases, the BLER performance of the lower SCS starts to deteriorate. In Figure 2 (b), with MCS 16 (16QAM), SCS 120kHz still sustains reasonable performance, while SCS 60kHz shows significant performance degradation due to its susceptibility to the Doppler and phase noise. At a very high MCS, in Figure 2 (d), the performance degradation due to phase noise is noticeable for all SCSs. However, the performance for SCSs 480kHz and 960kHz is still acceptable, with only 3.5dB degradation from the benchmark “PN OFF” performance at 10% BLER.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 2: PDSCH performance without PTRS: (a) MCS 7 (QPSK), (b) MCS 16 (16QAM), (c) MCS 22 (64QAM), and (d) MCS 28 (64QAM).

- Performance with PTRS-based phase noise correction

A set of PDSCH performance curves are plotted in Figure 3 for different MCS and SCSs. For comparison with the performance without PTRS-based phase noise correction in Figure 2, we apply PTRS-based phase noise correction for all curves in Figure 3.

In Figure 3 (a)-(c), it is observed that the PTRS-based phase noise correction can effectively recover the benchmark performance without phase noise. At a very high MCS, in Figure 3 (d), the performance degradation due to phase noise is noticeable for all SCSs. However, except SCS 60kHz, the performance delta from the benchmark is about 4dB, which is still in the acceptable range. For SCS 60kHz, the degradation is rather signification even with PTRS-based phase noise correction. From this observation, it could be concluded that SCS 60kHz is not suitable for the high frequency regime.

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 3: PDSCH performance with PTRS-based phase noise correction: (a) MCS 7 (QPSK), (b) MCS 16 (16QAM), (c) MCS 22 (64QAM), and (d) MCS 28 (64QAM).

### Impact of channel delay spread

In this section, the sensitivity to the channel delay spread is analysed for SCS 960kHz. The same parameter settings as in Table 2 are assumed except the channel model. For the first set of evaluation, the CINR values corresponding to 10% BLER are listed in Table 3 for CDL-B (NLOS) and CDL-D (LOS) channels. The RMS delay spread for each channel is swept from 10ns to 200ns, to represent different propagation environments, e.g., indoor hotspot and UMi (TR 38.901, Section 7.7.3). Note that, as the NCP for 960kHz is 73ns, some values of the delay spared in Table 3 are longer than the CP length.

From Table 3, the performance tends to deteriorate as the channel delay spread increases. The degradation is prominent for higher MCSs in the NLOS channel – for LOS channel, the performance degradation is marginal. Specifically, 90% of the post-beamforming RMS delay spread of CDL-B/CDL-D is shorter than a half/quarter of the pre-beamforming RMS delay spread.

Although the performance degradation with increasing delay spread is noticeable for CDL-B channel, it is not necessarily caused by the inter-symbol interference. It is rather due to the limitation channel estimation performance, as discussed in Section 2.2.2.2. It is further justified in Table 4, which lists the CINR values corresponding to 10% BLER for CDL-B channel, with normal and extended CPs. From the table, it is observed that the use of ECP does not help much improving the performance, implying that ISI is not the dominant factor of the performance degradation.

Table 3: CINR (in dB) corresponding to 10% BLER in CDL-B and CDL-D channels with different delay spreads.

| MCS | RMS delay spread |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CDL-B channel |  |  |  |  | CDL-D channel |  |  |  |  |
|  | 10ns | 20ns | 40ns | 100ns | 200ns | 10ns | 20ns | 40ns | 100ns | 200ns |
| 7 | -21.3 | -21.1 | -20.4 | -17.8 | -7.8 | -28.3 | -28.2 | -28.2 | -28.2 | -28.1 |
| 16 | -13.8 | -13.6 | -13.0 | -9.6 | - | -20.6 | -20.6 | -20.6 | -20.8 | -20.9 |
| 22 | -8.6 | -8.3 | -7.7 | -1.3 | - | -15.4 | -15.4 | -15.4 | -15.4 | -15.6 |
| 24 | -6.6 | -6.0 | -5.3 | 8.9 | - | -13.1 | -12.9 | -12.9 | -12.9 | -13.1 |
| 26 | -3.5 | -3.2 | -2.4 | - | - | -10.2 | -10.1 | -10.1 | -10.1 | -10.4 |
| 28 | 0.6 | 1.0 | 2.3 | - | - | -5.9 | -5.7 | -5.6 | -5.5 | -5.6 |

Table 4: CINR (in dB) corresponding to 10% BLER in CDL-B channel with NCP and ECP.

| MCS | Delay spread |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NCP |  |  |  |  | ECP |  |  |  |  |
|  | 10ns | 20ns | 0ns | 100ns | 200ns | 10ns | 20ns | 40ns | 100ns | 200ns |
| 7 | -21.3 | -21.1 | -20.4 | -17.8 | -7.8 | -21.2 | -21.1 | -20.6 | -17.9 | -8.3 |
| 16 | -13.8 | -13.6 | -13.0 | -9.6 | - | -14.1 | -13.9 | -13.3 | -10.2 | - |
| 22 | -8.6 | -8.3 | -7.7 | -1.3 | - | -8.6 | -8.4 | -7.7 | -2.4 | - |
| 24 | -6.6 | -6.0 | -5.3 | 8.9 | - | -6.3 | -6.1 | -5.5 | 1.4 | - |
| 26 | -3.5 | -3.2 | -2.4 | - | - | -3.1 | -2.8 | -2.0 | - | - |
| 28 | 0.6 | 1.0 | 2.3 | - | - | 0.7 | 1.0 | 2.3 | - | - |

### Impact of power amplifier nonlinearity

In this section, the impact of PA nonlinearity is investigated. The modified Rapp model (see Appendix, Section 5.2) is assumed and, for the other simulation parameters, the values in Table 2 are reused for the evaluation. However, to separate the effect of nonlinearity from the other RF nonidealities, phase noise is not applied.

As a performance metric for measuring the impact of PA nonlinearity, Tx EVM (TS 38.104, Appendix B) is calculated at different operating points of the PA through numerical simulation. The operating point of the PA is determined by the average input power of the PA relative to the 3dB saturation point and given by an offset value from the 3dB saturation point. For example, if the offset if 20dB, the average input power of the PA is determined as Pavg=P3dB,sat-20dB, where P3dB,sat is the 3dB saturation point of the PA.

In Table 5, the Tx EVM performance is provided. In the table, the EVM performance of ideal PA (i.e., linear PA) is also included for reference. Note that, even with the ideal PA, the EVM is not zero due to the numerical error of simulation. With the nonlinear PA model, a noticeable EVM degradation is observed as the nonlinearity increases (i.e., reduced offset). However, with up to 10dB offset, the Tx EVM requirements for different modulation schemes in TS 38.104 (Section 6.5.2) are still satisfied. Since the increased Tx EVM limits SINR at the receiver, the performance of higher MCSs may be impacted. The detailed analysis of the actual impact is for further study.

Table 5. Tx EVM with nonlinear PA.

| Modulation | Ideal PA | Modified Rapp, 20dB offset | Modified Rapp, 10dB offset |
| --- | --- | --- | --- |
| QPSK | 0.8% | 1.9% | 6.5% |
| 16QAM | 1.0% | 2.0% | 7.4% |
| 64QAM | 0.5% | 1.9% | 7.7% |

# Conclusion

Observation 1: For low MCSs (e.g., MCS 7) with QPSK modulation, no or little performance degradation is observed for SCSs 60kHz and 120kHz in the high frequency regime, even if PTRS-based phase noise correction is not applied.

Observation 2: For mid-range MCSs with 16QAM modulation, SCS 120kHz sustains the performance while SCS 60kHz starts showing an error floor.

Observation 3: SCS 60kHz in the high frequency regime suffers from severe performance degradation at very high MCSs (e.g., MCS 28), even with PTRS-based phase noise correction.

Observation 4: SCS 120kHz in the high frequency regime starts showing an error floor at very high MCSs (e.g., MCS 28), even with PTRS-based phase noise correction. However, the error floor is sustained below 1%.

Observation 5: SCS 960kHz in the high frequency regime retain high performance throughout all MCSs, if the PTRS-based phase noise correction is used.

Observation 6: SCS 960kHz with NCP (73ns) shows reasonable performance to the channel delay spread up to 200ns.

Proposal 1: For physical control, data, and random access channels and for SSB in the high frequency regime from 52.6GHz to 71GHz, SCSs of 120kHz and 960kHz should be considered.

Proposal 2: Existing FR2 phase noise models, possibly with some modifications, are reused for the study on the high frequency regime. RAN1 may send a LS to RAN4 asking for further confirmation.

Proposal 3: If needed, the impact of power amplifier nonlinearity may also be investigated during the study on the high frequency regime.

Proposal 4: When a new numerology (e.g., 960kHz) is introduced for the high frequency regime, the impact of the extremely short slot duration to the existing physical signals and channels should be investigated.

# References

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71GHz”, RAN #86, Intel Corporation

- RP-193229, “New WID on Extending current NR operation to 71GHz”, RAN #86, Qualcomm

- R4-163314, “Realistic power amplifier model for the New Radio evaluation”, RAN4 #79, Nokia

- R4-163344, “Response LS on realistic power amplifier model for NR waveform evaluation”, RAN4 #79

# Appendix: Evaluation modeling and methodology

## Phase noise model

For the baseline phase noise model, we assume the generalized multi-pole/zero model as described in Subclause 6.1.10 in TR 38.803 (Example 1), which is particularly based on the measurement at 29.55 GHz carrier frequency. Having the 29.55GHz model as a baseline, in TR 38.803, projected phase noise models for different carrier frequencies, such as 45GHz and 70GHz, have also been provided. Although a general formula for deriving a projected phase noise model for arbitrary carrier frequency between 52.6GHz and 71GHz would require detailed study and verification, a simple scaling rule is assumed as the starting point in our study: to derive a phase noise model for  fRF = 60 GHz carrier frequency, a scaling factor is applied to the baseline model (i.e., 29.55GHz), assuming fIF = 10 GHz for the IF frequency:

SF=20*log10fRF-fIF29.55-fIF.

The power spectral density of the derived model is shown in Figure 4, in comparison with the baseline model.

Figure 4. Phase noise model for 60GHz.

## Nonlinear power amplifier model

In our study, we assume the modified Rapp model [3], [4], which is recommended by RAN4 for evaluations of above 6GHz and especially at the carrier frequencies between 30GHz and 70GHz, and for ~2GHz bandwidth. The AM-AM (amplitude to amplitude modulation) and AM-PM (amplitude to phase modulation) responses of the modified Rapp model are given in the following table:

| AM-AM response | AM-PM response |
| --- | --- |
| FAM-AMx=Gx1+GxVSAT2p12p | FAM-PMx=Axq1+2Bq |
| - G=16 - VSAT=1.9 - p=1.1 | - A=-345 - B=0.17 - q=4 |

The given AM-AM and AM-PM responses of the modified Rapp model is shown in Figure 5.

Figure 5: AM-AM and AM-PM responses of modified Rapp model.

# Appendix: System-level performance evaluation

System-level performance is conducted to compare the performance of FR2 and the new high frequency regime (52.6GHz-71GHz). The assumed evaluation parameters and deployment scenarios are shown in Table 6 and Table 7, respectively.

Table 6: System-level evaluation parameters.

| Parameters | Values |
| --- | --- |
| Carrier freq. | 28 / 60 GHz |
| gNB ant array | 128/256 antenna elements per polarization |
| UE ant array | 16 antenna elements per polarization |
| Traffic | DL only/ UL only, Full buffer traffic |
| Codebook | DFT codebook at gNB and UE |

Table 7: Deployment scenarios for system-level evaluation.

| InH deployment | UMi deployment |
| --- | --- |
| - Single sector pointing down - 12 gNB over 120 m x 50 m - 10 UE per gNB | - Three sector pointing vertical - 19 gNB, with three sectors each - 10 UE per sector - ISD: 20/50/100m |

## InH, open office

Different configurations of system bandwidth and DL/UL EIRP are assumed for the InH system-level evaluation. The detailed configurations are compared in Table 8. For each configuration in Table 8, the DL and UL spectral efficiencies are evaluated and tabularized in Table 9 and Table 10, respectively. From the results, it is observed that the 60GHz band can support comparable spectral efficiencies to FR2 for the InH scenario.

Table 8: InH evaluation configurations.

|  |  | 28 GHz |  |  |  |  |  | 60 GHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cfg 1 | Cfg 2 | Cfg 3 | Cfg 4 | Cfg 5 | Cfg 6 | Cfg 1 | Cfg 2 | Cfg 3 | Cfg 4 | Cfg 5 | Cfg 6 |
| BW (MHz) |  | 100 | 100 | 400 | 400 | 800 | 800 | 50 | 50 | 500 | 500 | 2160 | 2160 |
| EIRP (dBm) | DL | 52 | 40 | 52 | 40 | 52 | 40 | 52 | 40 | 52 | 40 | 52 | 40 |
|  | UL | 31 | 25 | 31 | 25 | 31 | 25 | 31 | 25 | 31 | 25 | 31 | 25 |
| gNB (M,N,P) |  | 128 per pol (16,8,2) |  |  |  |  |  |  |  |  |  |  |  |
| UE ant (M,N,P) |  | Two panels at UE: 16 per pol (4,4,2) 1 (max. RSRP) panel is chosen at association |  |  |  |  |  |  |  |  |  |  |  |
| Note |  | Cfg 1, 3, 5 are typical for DL Cfg 2, 4, 6 are more practical for UL |  |  |  |  |  | Cfg 2, 4, 6 comply with regulatory limit for DL Cfg 2, 4, 6 are more practical for UL |  |  |  |  |  |

Table 9: DL spectral efficiency.

| spectral efficiency (bps/Hz) | 28 GHz |  |  | 60 GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5th Percentile, per UE | Median, per UE | Mean, per cell | 5th Percentile, per UE | Median, per UE | Mean, per cell |
| Cfg 1 | 0.45 | 0.95 | 8.83 | 0.49 | 0.92 | 8.51 |
| Cfg 2 | 0.46 | 0.95 | 8.75 | 0.44 | 0.89 | 8.24 |
| Cfg 3 | 0.47 | 0.95 | 8.82 | 0.46 | 0.93 | 8.58 |
| Cfg 4 | 0.39 | 0.92 | 8.4 | 0.28 | 0.79 | 7.14 |
| Cfg 5 | 0.44 | 0.96 | 8.8 | 0.4 | 0.91 | 8.15 |
| Cfg 6 | 0.35 | 0.88 | 8.06 | 0.15 | 0.54 | 5.59 |

Table 10: UL spectral efficiency.

| spectral efficiency (bps/Hz) | 28 GHz |  |  | 60 GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5th Percentile, per UE | Median, per UE | Mean, per cell | 5th Percentile, per UE | Median, per UE | Mean, per cell |
| Cfg 1 | 0.38 | 0.66 | 6.65 | 0.51 | 0.68 | 6.77 |
| Cfg 2 | 0.38 | 0.67 | 6.56 | 0.35 | 0.66 | 6.34 |
| Cfg 3 | 0.4 | 0.67 | 6.61 | 0.37 | 0.65 | 6.43 |
| Cfg 4 | 0.33 | 0.66 | 6.31 | 0.2 | 0.6 | 5.57 |
| Cfg 5 | 0.4 | 0.68 | 6.62 | 0.22 | 0.58 | 5.52 |
| Cfg 6 | 0.27 | 0.64 | 6.03 | 0.1 | 0.43 | 4.39 |

## UMi, 60GHz

Different configurations of system bandwidth and DL/UL EIRP are assumed for the 60GHz UMi system-level evaluation. The detailed configurations are compared in Table 11. For each configuration in Table 11, the DL and UL spectral efficiencies are evaluated and tabularized in Table 12. From the results, it is observed that the 60GHz band can support the UMi scenario of a few 10s of meters of ISD with reasonably high cell spectral efficiency.

Table 11: UMi evaluation configurations.

|  |  | 60 GHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| BW (MHz) |  | 50 | 50 | 500 | 500 | 2160 | 2160 |
| EIRP (dBm) | DL | 40 | 60 | 40 | 60 | 40 | 60 |
|  | UL | 25 | 31 | 25 | 31 | 25 | 31 |
| gNB (M,N,P) |  | 256 per pol (32,8,2) |  |  |  |  |  |
| UE ant (M,N,P) |  | Two panels at UE: 16 per pol (4,4,2) 1 (max. RSRP) panel is chosen at association |  |  |  |  |  |
| Note |  | Cfg 1, 3, 5 comply with regulatory limit for DL |  |  |  |  |  |

Table 12: DL/UL Spectral efficiency.

| ISD (m) | spectral efficiency (bps/Hz) | DL |  |  | UL |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5th Percentile, per UE | Median, per UE | Mean, per cell | 5th Percentile, per UE | Median, per UE | Mean, per cell |
| 100 | Config 1 | 0.02 | 0.23 | 2.78 | 0.05 | 0.31 | 3.40 |
|  | Config 2 | 0.27 | 0.70 | 7.76 | 0.10 | 0.44 | 4.63 |
|  | Config 3 | 0.002 | 0.05 | 0.86 | 0.005 | 0.10 | 1.36 |
|  | Config 4 | 0.12 | 0.56 | 6.00 | 0.02 | 0.22 | 2.59 |
|  | Config 5 | 0 | 0.01 | 0.27 | 0.001 | 0.03 | 0.51 |
|  | Config 6 | 0.06 | 0.36 | 4.17 | 0.004 | 0.10 | 1.32 |
| 50 | Config 1 | 0.06 | 0.33 | 3.93 | 0.09 | 0.40 | 4.10 |
|  | Config 2 | 0.30 | 0.80 | 8.22 | 0.14 | 0.50 | 4.88 |
|  | Config 3 | 0.006 | 0.10 | 1.48 | 0.01 | 0.16 | 1.95 |
|  | Config 4 | 0.21 | 0.67 | 6.96 | 0.05 | 0.30 | 3.29 |
|  | Config 5 | 0.001 | 0.03 | 0.55 | 0.003 | 0.05 | 0.87 |
|  | Config 6 | 0.11 | 0.48 | 5.38 | 0.01 | 0.16 | 1.91 |
| 20 | Config 1 | 0.07 | 0.31 | 3.80 | 0.05 | 0.25 | 2.92 |
|  | Config 2 | 0.19 | 0.59 | 7.08 | 0.06 | 0.25 | 2.79 |
|  | Config 3 | 0.01 | 0.11 | 1.49 | 0.02 | 0.14 | 1.88 |
|  | Config 4 | 0.16 | 0.53 | 6.34 | 0.04 | 0.20 | 2.48 |
|  | Config 5 | 0.002 | 0.03 | 0.58 | 0.004 | 0.06 | 0.98 |
|  | Config 6 | 0.11 | 0.42 | 5.03 | 0.02 | 0.14 | 1.80 |