3GPP TSG-RAN WG1 Meeting #101-e	R1-2004549

e-Meeting, May 25th - June 5th, 2020

Agenda Item:	7.2.4.6

Source:	Ericsson

Title:	QoS management for NR sidelink

Document for:	Discussion, Decision

# 1	Introduction

This contribution addresses the remaining issues related to QoS management and congestion control for NR SL Rel-16.

# 2	Discussion

A remaining issue in congestion control for NR SL is how resources which have been reserved but have not been used or will not be used due to HARQ feedback and/or pre-emption should be counted in channel occupancy ratio (CR).

Currently, the CR is defined in TS 38.215 as follows:

Sidelink Channel Occupancy Ratio (SL CR) evaluated at slot n is defined as the total number of sub-channels used for its transmissions in slots [n-a, n-1] and granted in slots [n, n+b] divided by the total number of configured sub-channels in the transmission pool over [n-a, n+b].

In our view, the CR should be as close as possible to the true resource occupation of the UE. Therefore, we believe the resources which have been reserved but have not been used or will not be used due to HARQ feedback and/or pre-emption should not be counted in the CR evaluation. Note also that the pre-empted resources are counted in the CR evaluation of the pre-empting UEs already.

Do not count reserved-but-not-used resources in CR evaluation.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not count reserved-but-not-used resources in CR evaluation.