3GPP TSG RAN WG1 meeting #101e          	  R1-2004577

e-Meeting, May 25th – June 5th, 2020

Agenda item:		7.2.7.1

Source:	Nokia, Nokia Shanghai Bell

Title:		On open issues related to DCI format 2_6

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we discuss about the open items related to monitoring of DCI format 2_6.

# 2	Discussion

## 2.1	Monitoring of DCI 2_6 scrambled by PS-RNTI and other procedures outside active time

RAN2 send a LS to RAN1 [2] regarding the UE behaviour when UE monitors DCI format 2_6 outside active time and BFR procedure is activated. RAN2 asked from RAN1 as follows:

- To confirm RAN2 understanding that if DCP and RAR search spaces are not quasi-collocated, a collision between DCP and RAR addressed to C-RNTI will impact legacy RAR handling. - From RAN2 point of view, the understanding is that RAR addressed all RNTIs should be prioritized over DCP by the UE. - RAN2 would like to ask if RAN1 has any concerns with the understanding above? If RAN1 doesn’t have any concerns, what is RAN1 preference on where to capture this behavior e.g. TS 38.213 or in TS 38.321 via a DCP monitoring exception rule similar to overlap with DRX Active time?

In CFRA based BFR procedure when UE is monitoring for response, it will go to ActiveTime after it receives the response (PDCCH scrambled by C-RNTI). Hence, it is possible that monitoring occasions for PDCCH scrambled by PS-RNTI would collide for example monitoring of PDCCH with C-RNTI when UE is outside the ActiveTime. If the UE is not able to monitor both simultaneously PDCCH scrambled by PS-RNTI and e.g. PDCCH with RA-RNTI due to different RX spatial filter, it would be simplest to assume that in case of such collision the UE will wake-up for the onDuration (i.e. start the drx-onDurationTimer). From RAN1 perspective the monitoring occasions of PS-RNTI that collide with for example the monitoring of RA-RNTI could be considered invalid.

The section 10.2 of 38.213 defines that in the case when UE is configured to monitor PDCCH candidates in overlapping monitoring occasions, in multiple CORESETs, UE monitors PDCCHs only in a CORESET associated with the highest priority search-space set and also other CORESETs having same QCL-TypeD properties. So in case there are multiple overlapping CORESETs, the monitoring is based on QCL-TypeD properties of the CORESET associated with the highest priority search-space set. In R15 UE prioritizes CSS over USS, and then, within given SS type, if multiple, based on the lowest SS set id.

In Section 6 of 38.213 it is determined that UE will assume same antenna port quasi-collocation parameters as the ones associated with index qnew, where qnew corresponds the RS index provided to the higher layers from the beam candidate set q1. UE will continue monitoring PDCCH candidates in the search space set provided by recoverySearchSpaceId until the UE receives a MAC CE activation command for a TCI state or tci-StatesPDCCH-ToAddList and/or tci-StatesPDCCH-ToReleaseList. It is also defined that UE does not expect to be provided another search space set for monitoring PDCCH in the CORESET associated with the search space set provided by recoverySearchSpaceId. I.e. only single SS set can be associated to the CORESET.

If BFR monitoring and DCI format 2_6 are in different SS sets and recoverySearchSpaceId points to a SS set that has higher priority than SS set, namely CSS with lower index than that used for DCI format 2_6, it should be clear that BFR monitoring is prioritized over DCI format 2_6 monitoring. However, following the restriction set in Section 6, these SS sets would need to be assigned in different CORESETs. As the Type0-CSS (associated with CORESET#0) is not very well suited for BFR monitoring, different SS set would need to be defined. This would also imply that recovery search space would need to be assigned a separate CORESET from CORESET carrying DCI format 2_6, i.e. e.g. CORESET#1 and CORESET#2.

One alternative would be to configure TYPE-3 CSS with DCI format 1_0, DCI format 2_6 (and possibly other GC-PDCCHs) and associate that also as recoverySearchSpaceId . This would need to be assigned to a separate CORESET from CORESET#0 (where Type0-CSS is assigned). In this case, the UE would monitor the CORESET covering both simultaneously, but it would need to be ensured that UE would follow the QCL relation based on index qnew. Hence, it would be needed to clarify that when UE is not able to monitor DCI format 2_6 from the intended QCL, it should wake-up for the next onDuration. Also, in this case an additional CORESET(s) would likely need to be configured for USS and remaining CSS, in addition to CORESET#0 (Type0-CSS etc) and CORESET#1 (for BFR).

As in practise UE’s might support only two CORESETs, it could be preferable to try to concentrate the SS sets to as few different CORESETs as possible by assigning all common SS sets to CORESET#0, and assigning (single) USS to CORESET#1. In this case, as the common SS set configurations would not be identical (i.e. Type0-CSS vs Type1-CSS) and would need to be set to different SS sets, more than one SS sets would be assigned to CORESET#0, making it impossible to associate recoverySearchSpaceId  SS set to CORESET#0. Hence, separate CORESET would need to be assigned for recoverySearchSpaceId SS set. This could be configured as USS or the USS could be assigned to CORESET#0. In this case it would not be possible to ensure based on prioritization rules that UE would prioritize BFR response monitoring over DCI format 2_6 based on the current rules.

Note the overlapping CORESETs is assumed as a typical case due to UE capability restrictions for span of PDCCH monitoring in a slot.

From above set of rules it follows, that either the configuration needs to be very constrained to achieve prioritisation of BFR over DCI format 2_6 , limiting the network flexibility to define the UE PDCCH monitoring of different DCIs forcing all to use same SS sets and there can be configuration cases when DCI format 2_6 monitoring would be prioritized over RAR monitoring as RAN2 considers. Accounting practical UE capability restrictions, it may not be feasible to circumvent these. Hence to ensure that RAR monitoring is always prioritized over DCI format 2_6 monitoring, UE should be allowed to consider the overlapping monitoring occasions invalid, and start the onDurationTimer if needed. This behaviour could be captured to RAN1 specification or to RAN2 specification.

Observation 1: DCI format 2_6 (DCP) monitoring can impact to the legacy RAR monitoring.

Proposal 1: When monitoring occasions of DCI format 2_6 overlaps with PDCCH monitoring in search space given recoverySearchSpaceId as described in Section 6 of 38.213, UE should follow the legacy DRX operation.

Proposal 2: Send a LS to RAN2 indicating that RAN1 agrees that there could be impact to the legacy RAR monitoring configuration and that the behaviour related to DCP and RAR monitoring could be captured to RAN2 specification.

## 2.2	Minimum processing delay for DCI format 2_6

Related to the configuration of the monitoring occasions for PDCCH with CRC scrambled by PS-RNTI, the following agreements were made in RAN1#100bis-e.

Under agenda item 7.2.7.1 following working assumption was taken:

working assumption: - The value of minimum time gap is decoupled with SCell dormancy indication. - Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap(slots). Value 1; Value 2. 15; 1; 3. 30; 1; 6. 60; 1; 12. 120; 2; 24.

In discussions under MR-DC enhancements in agenda item 7.2.10.3 following conclusion was made:

Conclusion - For at least DCI format 2_6, there is no restriction that the DCI format with SCell dormancy indication is received only in the first 3 symbols of a slot - Note: No TP required - Note: If any restriction is introduced for DCI 2_6 in UE power savings WI, whether/not it applies also for DCI format 2_6 with SCell dormancy indication can be discussed further. - Discuss further whether to introduce restriction for DCI format 0_1,1_1

The working assumption and conclusion made in RAN1#100bis-e appear to be consistent, thus it would appear possible to confirm the working assumption made regarding the minimum time gap in RAN1#100bis-e

Proposal 3: Confirm the working assumption take in the RAN1#100bis-e:

working assumption:

- The value of minimum time gap is decoupled with SCell dormancy indication.

- Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap(slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

# 3	Conclusions

In this contribution we have discussed about PDCCH-based power saving channel design.

In Section 2.1 we looked at the DCI format 2_6 monitoring occasions determination:

Observation 1: DCI format 2_6 (DCP) monitoring can impact to the legacy RAR monitoring.

Proposal 1: When monitoring occasions of DCI format 2_6 overlaps with PDCCH monitoring in search space given recoverySearchSpaceId as described in Section 6 of 38.213, UE should follow the legacy DRX operation.

Proposal 2: Send a LS to RAN2 indicating that RAN1 agrees that there could be impact to the legacy RAR monitoring configuration and that the behaviour related to DCP and RAR monitoring could be captured to RAN2 specification.

On the processing delay, i.e. the minimum gap between onDurationTimer and DCI format 2_6 monitoring and Scell dormancy behaviour we propose to confirm the working assumption in Section 2.2 as follows:-

Proposal 3: Confirm the working assumption take in the RAN1#100bis-e:

working assumption:

- The value of minimum time gap is decoupled with SCell dormancy indication.

- Two values of minimum time gap in terms of slots per SCS are specified based on the assumption that PDCCH carrying DCI format 2_6 can be at any symbol of the slot indicated by monitoringSymbolsWithinSlot of SearchSpace IE as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap(slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

# References

- R1-2001507, LS on DCP, RAN2

- R2-2004175, LS on RAN2 DCP Open Issues, RAN2

- R1-2003177, Corrections on UE power savings, Samsung