3GPP TSG-RAN WG1 Meeting #101	R1-2004631

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.4

Source:	Ericsson

Title:	General Considerations for the Coverage Enhancement SI

Document for:	Decision

# Introduction

This contribution provides our views on the evaluation approaches, the criteria that should be used for coverage study, and the target scenarios according to the SID [1] for coverage enhancement study item and the NR self-evaluation towards IMT-2020 [2].

# Discussion

## Evaluation approaches and coverage criteria

While coverage can be straightforwardly understood as the area over which a cell can provide service, simple definitions of coverage may not be useful to study coverage enhancement. A ‘pure’ coverage scenario can be thought of as where the UE operates in an interference free environment, uses all available resource, it is the only UE in the system, and only one cell exists. Using only link level simulation to study coverage for such a scenario may be considered, since there is only one radio link that can exist, and it will not be affected by others. However, such ‘link level only’ approaches to coverage characterization suffer from a variety of problems:

The antenna pattern of the gNB will generally vary with the distance from the gNB, and may be attenuated toward the cell edge from the downtilt used to improve capacity.

Different physical channels may be beamformed or precoded differently.

Cells do not operate at zero load, and some channels are always-on and/or may degrade more in the presence of interference than others.

Link budgets have historically used approximations such as a fixed antenna gain or interference margin to account for such effects, but these are rather less accurate when used for heavily beamformed systems such as NR. Therefore, as described in more detail in [3] we propose to explicitly simulate antenna patterns and interference experienced by each UE in a cell, where different antenna patterns are applied for different physical channels.

Observation 1:

‘Link level only’ coverage analyses have the following drawbacks, which limits their usefulness for NR:

The antenna pattern of the gNB will generally vary with the distance from the gNB, and may be attenuated toward the cell edge from the downtilt used to improve capacity.

Different physical channels may be beamformed or precoded differently.

Cells do not operate at zero load, and some channels are always-on and/or may degrade more in the presence of interference than others

The notion of ‘pure’ coverage is also challenged by design tradeoffs for capacity. For example, uplink coverage can be enhanced by increasing the ratio of uplink to downlink slots. However, this will degrade downlink capacity, which can be problematic because networks typically serve much more downlink than uplink data. Similarly, networks commonly use downtilt, which will reduce the size of the cell but avoid inter-cell interference. Lastly, coverage can be increased by reducing peak data rate or relaxing latency requirements in the cell. Therefore, constraints on resource allocation, and network configurations should be identified for the features and use cases of interest to operators in order to have a meaningful measure of coverage.

Proposal 1:

Coverage is defined as the fraction of a cell over which a feature operates at the required performance and under the assumed constraints

Coverage evaluations explicitly simulate antenna patterns and interference experienced by each UE in a cell.

  - Different antenna patterns are applied for different physical channels.

In order for a UE to be served with a feature, it needs to receive and transmit a set of messages carried on different physical channels. If any one of these messages is out of coverage, the entire feature is not supported. Therefore, the enhancements that should be focused upon are those that address the messages and physical channels with the worst coverage to support a desired feature.

Proposal 2:

The NR coverage enhancement study focuses on improving coverage bottlenecks.

## Target scenarios

The target scenarios and services included in the NR coverage enhancement study item description are:

- Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long range rural scenario) for FR1

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

- Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - eMBB service as first priority and VoIP as second priority for FR2.

Since the main interest from a study of coverage is which channels are bottlenecks, it is the relative performance of the channels that is of primary interest. Consequently, it is most important to exercise different aspects of the air interface that vary with the scenario, such as the amount of beamforming gain, number of antennas, transmit power, noise figure, expected UE speed, etc. Large scale channel model properties such as path loss, lognormal fading, building penetration, will impact the absolute but not the relative coverage of channels. Consequently, a few representative scenarios can be enough to determine the parameters needed for coverage evaluations.

Proposal 3:

- For FR1, simulation parameters used in the coverage enhancement study are based on the rural (Rural-eMBB & Low Mobility Large Cell) and urban (Dense Urban-eMBB) scenarios of the IMT 2020 evaluation TRs 37.910 and 38.913

  - The rural and urban macro scenarios are used as baselines and the ISDs are extended to stress the coverage.

  - A low mobility large cell (LMLC), a rural macro, and an extreme rural scenario are defined for low band operation

  - Detailed parameters are given in [4]

Proposal 4:

- For FR2, simulation parameters used in the coverage enhancement study are based on the urban (Dense Urban-eMBB) and indoor (Indoor Hotspot-eMBB) scenarios of the IMT 2020 evaluation TRs 37.910 and 38.913

  - An urban macro and an indoor scenario are defined for millimeter wave operation

  - Detailed parameters are given in [5]

# Conclusion

Based on the discussion in the previous sections we make the following observation and propose:

Observation:

‘Link level only’ coverage analyses have the following drawbacks, which limits their usefulness for NR:

The antenna pattern of the gNB will generally vary with the distance from the gNB, and may be attenuated toward the cell edge from the downtilt used to improve capacity.

Different physical channels may be beamformed or precoded differently.

Cells do not operate at zero load, and some channels are always-on and/or may degrade more in the presence of interference than others

Proposals:

Coverage is defined as the fraction of a cell over which a feature operates at the required performance and under the assumed constraints

Coverage evaluations explicitly simulate antenna patterns and interference experienced by each UE in a cell.

  - Different antenna patterns are applied for different physical channels.

The NR coverage enhancement study focuses on improving coverage bottlenecks.

For FR1, simulation parameters used in the coverage enhancement study are based on the rural (Rural-eMBB & Low Mobility Large Cell) and urban (Dense Urban-eMBB) scenarios of the IMT 2020 evaluation TRs 37.910 and 38.913

  - The rural and urban macro scenarios are used as baselines and the ISDs are extended to stress the coverage.

  - A low mobility large cell (LMLC), a rural macro, and an extreme rural scenario are defined for low band operation

  - Detailed parameters are given in [4]

- For FR2, simulation parameters used in the coverage enhancement study are based on the urban (Dense Urban-eMBB) and indoor (Indoor Hotspot-eMBB) scenarios of the IMT 2020 evaluation TRs 37.910 and 38.913

  - An urban macro and an indoor scenario are defined for millimeter wave operation

  - Detailed parameters are given in [5]

# References

RP-193240, “New SID on NR coverage enhancement”, China Telecom, RAN #86, December 2019.

“Study on self evaluation towards IMT-2020 submission”,3GPP technical report 37.910, September 2019.

R1-2004632, “Evaluation methodology for coverage enhancements”, Ericsson, RAN1 #101, May 2020.

R1- 2004352, “Simulation Parameters and Initial Results for FR1”, Ericsson, RAN1 #101, May 2020.

R1- 2004353, “Simulation Parameters and Initial Results for FR2”, Ericsson, RAN1 #101, May 2020.