3GPP TSG-RAN WG1 Meeting #101	R1-2004632

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.4.1

Source:	Ericsson

Title:	Evaluation methodology for coverage enhancements

Document for:	Decision

# Introduction

The coverage enhancements work area should include a verification of that control channels support the data channels down to reasonable data rates. In this paper a methodology for such evaluations is proposed.

In short, it is proposed to determine SINR requirements for different control channels using link level simulation. SINR distributions for the different channels and for desired scenarios are then generated by means of system-level simulations. By combining these results, it can be seen what fraction of users have coverage for the different channels. If a certain channel has significantly worse coverage than others, this channel constitutes a bottleneck for overall coverage and is a candidate for enhancements.

It should be noted that this methodology will identify bottlenecks by answering the question ‘In a certain scenario, what fraction of users have coverage for a certain channel?’. An alternative /complementary way to define coverage bottlenecks is by means of isotropic loss. A channel that supports significantly lower isotropic loss than others forms a bottleneck for overall coverage. In the proposed methodology, the maximum isotropic loss supported by a channel can be assessed from the distribution of isotropic loss in the studied scenario. Moreover, it is expected that the bottlenecks will be the same between the 2 methods discussed above.

# Conventional link budgets

A conventional link budget typically assesses coverage in terms of minimum isotropic loss (also denoted pathloss) as (in dB scale):

Liso=Ptx+Gtx+Grx-Pnoise+Interference-SINRreq

The isotropic loss can be transformed to a distance using a propagation model.

For the purpose of comparing the coverage of different channels, some observations can be made. For channels with the same transmit power Ptx, antenna gains Gtx+Grx, and interference levels Interference, it is sufficient to compare SINR requirements. This can be the case for some channels but is not generally valid (e.g. downlink and uplink channels have different characteristics). For channels with the same antenna gains, it is sufficient to compare the maximum coupling loss (Liso-Gtx-Grx). This is the case for conventional, non-AAS, systems, and often used during the development of LTE. It is however not true for systems with UE-specific beamforming for certain channels and broadcast beams for other channels. In such systems, the antenna gains differ between channels. Not only do the maximum antenna gains differ, but also the actual gains, which depend on how well inside the main-lobe or range of steerability of the antennas, differ. This is quite complex and difficult to accurately capture in a conventional link budget. The same goes for the interference, which is dependent on the antenna scheme used for the interfering channel. This is elaborated on in the next Section. Note that the effect of not experiencing the maximum antenna gain is there also in conventional systems, but then the effect is the same for all channels, and relative comparisons are not affected.

Observations:

- For coverage comparisons of different channels, it is not generally sufficient to compare SINR requirements.

- For coverage comparisons of different channels, it is not generally sufficient to compare maximum supported coupling loss.

- It is difficult to accurately capture the effects of differing antenna gains and interference margins in a conventional link budget.

# More details on antenna gains

In this chapter the impact of the different antenna schemes on the SINR and coverage is further analyzed. The intention is to motivate the proposed methodology.

On system-level, the downlink SINR for user i served by base station j and interfered by base stations k ≠ j can be expressed as:

SINRi=SiIi+N=Pij∙giso ij∙gtx ij∙grx ijk≠jak∙Pik∙giso ik∙gtx ik∙grx ik+N

It is seen that the desired signal (Si) depends on the transmit power Pij, the isotropic gain giso ij, between transmitter j and receiver i, and the transmitter and receiver antenna gains gtx ij and  grx ij. Note that here the isotropic gain is the gain between the transmitter and receiver if they would both have isotropic antennas. The product of the isotropic gain and the antenna gains is referred to as coupling gain (giso ij∙gtx ij∙grx ij). The interference Ii depends on similar components and in addition on the activity level of the interfering signal ak. Note that the interfering signal, and thereby the antenna scheme, can be different from that used by the desired signal.

For coverage purposes, it is of interest to determine the SINR at the ‘cell edge’ or for low percentiles in the distribution over users.  If the antenna gains were fixed, it would be straightforward to calculate a maximum isotropic loss for which the SINR requirement is met. This is however not the case. The antenna gains to the serving and interfering base stations vary between users. This is illustrated for two cases (SSB and PDSCH) in Figure 1, showing samples of antenna gain vs. isotropic gain for serving base station links. It is seen that the antenna gain varies between users (samples) and that the correlation with the isotropic gain is not very strong. It is also seen that the antenna gain is different for the SSB and PDSCH. This makes comparisons of maximum coupling loss (isotropic loss - antenna gain) not suitable for coverage comparisons in cases like this with channels with different antenna solutions.

Figure 1. Example of base station antenna gain for SSB (swept in four directions) and PDSCH (UE-specifically beamformed) vs. isotropic loss for different users. This is for an Urban Macro scenario with 500m ISD, and base station with an 8x8 antenna array.

Figure 2 shows samples of resulting SINR, which also depends on the gain to interfering cells, vs isotropic gain. Here it is seen that the SINR for a given isotropic gain varies greatly (tens of dBs). These aspects and their interdependencies are quite complex, and it is difficult and likely inaccurate to represent antenna gains and interference levels with single values for use in standard link budgets. To address this, it is proposed to use system level simulations to directly generate SINR distributions.

Here it is observed that the antenna gains differ between channels. Some are broadcast over the cell, some are beam-swept, and some to different degrees UE-specifically beamformed. The antenna gains for a certain channel further differ between users. Not all users are within the main-lobe or range of steerability of the antennas, and hence do not experience the maximum antenna gain. It is observed that also these aspects differ between channels and between users.

Figure 2. Example of downlink SINR vs. isotropic loss for different users.

Observation:

- The variation in antenna gains and SINR for a given isotropic loss is large.

# The proposed methodology

The coverage of a certain signal or channel depends on the ‘service quality’ required on that channel, e.g. error rate and/or data rate, the link quality required to reach that acceptable service quality, and the ability of the system to generate the link quality.

The link quality can be measured as Signal-to-Interference-and-Noise Ratio (SINR). The SINR required to reach a certain service quality is effectively determined by means of link simulations.

On system-level, the downlink SINR for user i served by base station j and interfered by base stations k≠j can be expressed as:

SINRi=SiIi+N=Pij∙giso ij∙gtx ij∙grx ijk≠jak∙Pik∙giso ik∙gtx ik∙grx ik+N

Given the complexity of the relation between these parameters affecting the SINR, and the difficulty to accurately capture that in a conventional link budget, it is proposed to generate SINR distributions by means of system-level simulations. A channel with significantly worse coverage than others is considered a candidate for enhancements.

Given the division into link- and system-level simulations, the question arises on how to model the antenna capabilities on link- and system-level. Roughly speaking, the larger subset of the antennas that is modelled on link-level, the more accurately signal-processing-related aspects are modelled, whereas the larger subset is modelled on system level the more accurately the beamforming gains of the desired signal and suppression of interference are modelled. In order to reduce complexity of the link simulations and properly model the beamforming gains on system-level, the subset of antennas simulated on link level is kept small. In the downlink direction, the number of receive antennas is two or four, and the number of transmit antennas is two (hence 2x2 or 2x4 configurations are studied). In the uplink direction, the number of transmit antennas is one, and the number of receive antennas is two in [1] and [2], while it is four in [3] (hence 1x2 and 1x4 configurations are studied). The remaining number of antennas are modelled on system-level.

This approach is similar to the one used for the IMT-2020 mobility evaluations, where link- and system-level evaluations are combined to determine if the link quality required to support a certain speed is supported for 50% of the users.

Proposals:

- Use link level simulations to determine required SINRs for different channels

- Use system level simulations to determine SINR distributions, and the fraction of users that have coverage for different channels

- Consider channels with significantly worse coverage than others as candidates for enhancements

# Conclusion

Based on the discussions in previous sections, we have following observations.

Observations:

- For coverage comparisons of different channels, it is not generally sufficient to compare SINR requirements.

- For coverage comparisons of different channels, it is not generally sufficient to compare maximum supported coupling loss.

- It is difficult to accurately capture the effects of differing antenna gains and interference margins in a conventional link budget.

- The variation in antenna gains and SINR for a given isotropic loss is large.

And based on the observations and discussions, we have following proposals.

Proposals:

- Use link level simulations to determine required SINRs for different channels

- Use system level simulations to determine SINR distributions, and the fraction of users that have coverage for different channels

- Consider channels with significantly worse coverage than others as candidates for enhancements

# References

R1-2004352, “Simulation Parameters and Initial Results for FR1”, Ericsson, RAN1 #101, May 2020.

R1-2004353, “Simulation Parameters and Initial Results for FR2”, Ericsson, RAN1 #101, May 2020.

R1- 2004355, “Coverage Enhancements: Four BS Antennas at Low Band”, Ericsson, RAN1 #101, May 2020.