3GPP TSG-RAN WG1 Meeting #101-e	R1-2004646

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.8.4

Source:	Ericsson

Title:	Maintenance of Physical-layer procedures to support UE/gNB measurements

Document for:	Discussion, Decision

# Introduction

This contribution address maintenance issues pertaining to Physical-layer procedures to support UE/gNB measurements for NR positioning.

# Discussion

### Clarifications on RSTD reference info

In RAN1#100bis-e meeting, some clarifications on RSTD reference info were discussed.  Various text proposals were discussed to clarify the case when the UE may use a different reference for RSTD measurement.  Some proposals tried to clarify the spec with regards to what the UE may use as reference for RSTD measurement which includes the following:

- different DL PRS resources within a single DL PRS resource set;

- a different DL PRS resource set which can be any DL PRS resource set associated with the [ID];

- different DL PRS resources associated with an [ID] other than the [ID] provided;

- different DL PRS resources associated with an [ID] other than the [ID] provided;

However, it was argued by some companies that it is unnecessary to list all these possibilities in 38.214.  Upon further discussion, the following TP was proposed in [1]:

TP for clause 5.1.6.4 (PRS reception procedure) of TS 38.214: […] The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a PRS resource set ID, and optionally a single PRS resource ID or a list of PRS resource IDs. The UE may use a different reference time for the RSTD measurement. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the [ID], the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. […]

We think this proposal is reasonable hence propose to agree on this TP after taking into account the changes already made to this clause by the endorsed CR [2] in RAN1#100bis-e.

Endorse text proposal 1 in Section 5 for inclusion in 38.214.

Text proposal 1 (38.214):

### 5.1.6.5 PRS reception procedure ----unchange text omitted -------- The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo-r16. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-expectedRSTD-r16and nr-DL-PRS-expectedRSTD-uncerainty-r16. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo-r16may include an dl-PRS-ID-r16, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs. The UE may use a different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID-r16, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. ----unchange text omitted --------

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Endorse text proposal 1 in Section 5 for inclusion in 38.214.

- References

R1-2002989, “Summary of Email Discussion [100b-e-NR-Pos-04]”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 Meeting #100b-e, April 20 – 30, 2020.

R1-2003142, “Corrections of NR positioning support”, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG1 Meeting #100b-e, April 20 – 30, 2020.

# Text Proposals for Specification CRs

Text proposal 1 (38.214):

### 5.1.6.5 PRS reception procedure ----unchange text ommitted -------- The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo-r16. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-expectedRSTD-r16and nr-DL-PRS-expectedRSTD-uncerainty-r16. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo-r16may include an dl-PRS-ID-r16, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs. The UE may use a different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID-r16, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference. ----unchange text ommitted --------