3GPP TSG-RAN WG1 Meeting #101-e	R1-2004657

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	6.2.1.3

Source:	Ericsson

Title:	TP for 36.211 on DL DMRS handling for LTE-MTC resource reservation

Document for:	Discussion, Decision

# Introduction

RAN1#100bis-e discussed DL DMRS handling for Rel-16 LTE-MTC resource reservation in the email discussion “[100b-e-LTE-eMTC5-Coex-NR-01]”. For more background information, see “Issue #3” in the email discussion summary in R1-2002797. This contribution presents a 36.211 TP based on the discussion.

| Reason for change: | RAN1#99 made the following agreement for Rel-16 LTE-MTC which needs to be captured in the specification: - In DL frequency-domain and DL time-domain resource reservation, DMRS REs can be reserved if and only if all other non-CRS REs in the same slot and PRB are also reserved. |
| --- | --- |
| Summary of change: | The agreement is implemented in the specification. |
| Consequences if not approved: | Resource reservation may be incorrectly implemented for DL DMRS. |
| Clauses affected: | 6.10.3.2, 6.10.3A.2 |

# Text proposal for 36.211

#### 6.10.3.2	Mapping to resource elements

<Unchanged parts are omitted>

For BL/CE UEs, if higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, and the Resource reservation field in the DCI is set to 1, then in case of PDSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	If and only if all OFDM symbols in a PRB are reserved, the demodulation reference signal transmission in that PRB is dropped.

<Unchanged parts are omitted>

#### 6.10.3A.2	Mapping to resource elements

<Unchanged parts are omitted>

For BL/CE UEs, if higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured, then in case of MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	If and only if all OFDM symbols in a PRB are reserved, the demodulation reference signal transmission in that PRB is dropped.

<Unchanged parts are omitted>