3GPP TSG RAN WG1 #101	R1-2004664

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (Nokia)

Title:	Summary#2 for AI 7.2.4.6 QoS management

Agenda item:	7.2.4.6

Document for:	Discussion and Decision

# Introduction

In this contribution we summarize the outcome of the email discussions for the remaining issues in NR V2X QoS Management (agenda item 7.2.4.6); this includes the following email discussions:

- [101-e-NR-5G_V2X_NRSL-QoS-01] Email discussion/approval regarding  channel occupancy ratio - treatment of resources reserved, but not used due to HARQ feedback and/or pre-emption by 5/28, with potential TP by 6/3 – Torsten (Nokia)

- [101-e-NR-5G_V2X_NRSL-QoS-02] Email discussion/approval regarding the assumption on P_("MAX" ,CBR) when maximumtransmitPower-SL is not provided, by 5/28, with potential TP by 6/3 – Torsten (Nokia)

Details on the candidate issues can be found in Summary#1, R1-2004663.

# [101-e-NR-5G_V2X_NRSL-QoS-01]

## Outcome

Conclusion:

- Future granted resources which have been released due to HARQ feedback are not counted in the evaluation of CR

Agreements:

- Endorse the TP to clarify the meaning of “granted” in TS 38.215

# [101-e-NR-5G_V2X_NRSL-QoS-02]

## Outcome

Agreements:

- Take as a conclusion that no further discussion in RAN1 regarding whether TS 38.213 should state explicitly that CBR-based power control does not apply to a PSSCH transmission using resources allocated using mode 1

- The latest TP to 38.213 (Section 16.2.1) is endorsed.

# TPs

## 38.215 TP on “granted”

101-e-NR-5G_V2X_NRSL-QoS-01] endorsed the following TP:

| Reason for change: | Clarify meaning of “granted” in the definition of channel occupancy ratio (CR) |
| --- | --- |
| Summary of change: | Add a note to explain that “granted” refers to the configured sidelink grant as defined in TS 38.321. |
| Consequences if not approved: | Potential lack of clarity about the meaning of “granted”. |
| Clauses affected: | 5.1.26 |

----------------------------------------------------begin text proposal for 38.215-----------------------------------------------

### 5.1.26	Sidelink channel occupancy ratio (SL CR)

<<<unchanged text omitted>>>

NOTE 6:	A resource is considered granted if it is a member of a configured sidelink grant as defined in TS 38.321 [7].

----------------------------------------------------end text proposal for 38.215-----------------------------------------------

## 38.213 TP on PMAX,CBR

[101-e-NR-5G_V2X_NRSL-QoS-02] endorsed the following TP:

| Reason for change: | The value of PMAX,CBR was set to 0 (dBm) if maximumtransmitPower-SL is not provided. |
| --- | --- |
| Summary of change: | Limit PMAX,CBR does not apply if maximumtransmitPower-SL is not provided. |
| Consequences if not approved: | PSSCH transmission power will be limited to 0 dBm if maximumtransmitPower-SL is not provided. |
| Clauses affected: | 16.2.1 |

----------------------------------------------------begin text proposal for 38.213-----------------------------------------------

### 16.2.1	PSSCH

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i as follows:

If maximumtransmitPower-SL is provided then

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

else

PPSSCH(i)=minPCMAX,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of maximumtransmitPower-SL based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; PMAX,CBR=0

<<<unchanged text omitted>>>

----------------------------------------------------end text proposal for 38.213-----------------------------------------------

# References

- RP-200129, “Revised WID: 5G V2X with NR sidelink”

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR

- TR 38.885, Study on NR Vehicle-to-Everything (V2X)

# Background

## WI Objectives

At RAN#83, a new work item “5G V2X with NR sidelink” (5G_V2X_NRSL) was approved ‎[1]. Two of the objectives are relevant for the present agenda item:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. - … - Congestion control [RAN1, RAN2] 4. Specify support for QoS management [RAN2, RAN3, RAN1]

## Earlier Agreements

The following relevant agreements have been reached in previous meetings:

### QoS

Agreements: From RAN1 perspective, at least the following QoS-related parameters relevant to physical layer studies are considered: - Priority - latency - reliability

Agreements: RAN1 studies further how to use - priority, - latency, - reliability, - minimum required communication range (as defined by higher layers) if agreed to use in the physical layer aspects of at least - resource allocation and - congestion control and - resolution of in-device coexistence issues and - power control

In the Sidelink resource allocation mode 2 agenda item, the following working assumption was reached:

Working assumption: - An indication of a priority of a sidelink transmission is carried by SCI payload - This indication is used for sensing and resource (re)selection procedures - This priority is not necessarily the higher layer priority

Agreements: - For the priority indication in 1st stage SCI: - Up to RAN2 on how to define the mapping between the priority indication and the corresponding QoS - Size is 3 bits (as a working assumption)

### Sidelink Congestion Control

Agreements: - Introduce at least one congestion metric for NR sidelink - FFS details – to be done in WI phase (if included)

Agreements: - Congestion control is supported at least for sidelink mode 2 - Note: details of congestion control can be covered in the work item phase, not in this SI.

Conclusion: - It is deemed beneficial to report Sidelink Congestion Metrics(s) to a gNB - Consequently, it is recommended to specify the corresponding details in the WI phase

Agreements: Support at least NR CBR as congestion metric for NR sidelink congestion control. - LTE CBR is the baseline for defining NR CBR.

Agreements: - LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control.

Agreements: - Higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs.

Agreements: - For PSCCH/PSSCH multiplexing option 3, one CBR measurement over a resource pool is defined. - PSFCH resources, if (pre)configured, are excluded from this CBR measurement.

Agreements: Define NR sidelink Channel Occupancy Ratio (CR) measurement. - LTE CR is the baselines

Agreements: - Congestion control can restrict the values of at least the following PSSCH/PSCCH TX parameters per resource pool: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions – already agreed in mode 2 AI - Upper bound of TX power (including zero TX power) - Congestion control can set an upper bound on channel occupancy ratio (CR), CRlimit. - Ranges/bounds of the transmission parameters and CRlimit are functions of QoS and CBR. - In addition to congestion control (in use or not in use), the above parameters can be restricted by reusing the same mechanism as in LTE - For speed, further discussion on absolute vs. relative speed - FFS other parameter(s) that can be restricted - FFS whether or not to tie the speed with a UE capability

Agreements: Lookup table links CBR range with values of the transmission parameters and CRlimit for each value of the indication of a priority of a sidelink transmission carried by SCI payload (as per WA from RAN1#98), Lookup table is (pre)configured. Details up to RAN2. - Up to 16 (as a working assumption) CBR ranges are supported - The working assumption will be automatically confirmed in RAN1#99 if no further input

Agreements: - Sidelink RSSI (SL-RSSI) measurement is used for CBR estimation

Agreements: A sidelink resource is busy for the purpose of CBR measurement if Sidelink RSSI measured by the UE in that resource exceeds a (pre-)configured threshold.

Agreements: The CBR measurement time window size is 100 ms and 100 slots by (pre-)configuration. CR window size is { 1000 ms, 1000 slots } by (pre)-configuration

Agreement: - The future segment of the CR evaluation window reuses the same behaviour as in the LTE V2X sidelink. - FFS whether additional constraints on UE’s choice of values for a and b are needed

Agreement: For the constraints on past/future window in CR evaluation: - n+b shall not exceed the last transmission opportunity of the grant for the current transmission - b >= 0 - b < (a+b+1)/2 Notes: - in the first bullet point above, LTE’s “should” has been replaced by “shall”

Agreement: - UE evaluates CR and applies CR_limit for every (re)transmission.

Agreement: - The CBR processing time is given by UE capability according to the following table µ; Congestion process time 1 (slots); Congestion processing time 2 (slots). 0; 2; 2. 1; 2; 4. 2; 4; 8. 3; 8; 16. - A UE shall only apply a single CBR/CR processing time capability in SL. - CR processing time is the same as CBR processing time.

Agreement: - The slot index in the definition of CBR is the physical slot index. Agreement: - The slot index in the definition of CR is the physical slot index.

### TX Parameter Restrictions

Agreements: - Only TX parameter restriction based on absolute speed can be (pre)configured in Rel-16.

Agreement: - In addition to congestion control (in use or not in use), the following PSSCH/PSCCH TX parameters per resource pool can be restricted by reusing the same mechanism as in LTE: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions Note: This reverts the agreement made in RAN1#98b, which included “Upper bound of TX power” in the set of TX parameters that can be restricted using this mechanism.