3GPP TSG RAN WG1 Meeting #101-e	R1-2004687

e-Meeting, May 25th – Jun 5th, 2020

Agenda item:	6.2.4

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Summary of issues for LTE-based 5G terrestrial broadcast

Document for:	Discussion and Decision

# Summary of issues

The following corrections have been submitted to this meeting:

- Issue #1: Approval of editor CR for TS 36.201

- Issue #2: Control region length related issues

- Issue #3: Semistatic CFI

- Issue #4: Categories for 0.37kHz SCS

# Issue #1: Approval of editor CR for TS 36.201

In x4200, the editor submitted the CR to capture the TP agreed in RAN1#100b-e. It is the moderator’s view that this does not need to be discussed during the e-meeting and can be handled directly in the editor CR phase.

# Issue #2: Control region length related issues

In x4667 and x4163 it is proposed to modify Table 6.7-1 to specify that the new numerologies have 0 symbols for control. The TPs are slightly different but have the same intention.

<TP 2.1 Alt1, TS36.211>

  - 6.7	Physical control format indicator channel

The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe. The set of OFDM symbols possible to use for PDCCH in a subframe is given by Table 6.7-1.

Table 6.7-1: Number of OFDM symbols used for PDCCH

| Subframe | Number of OFDM symbols for PDCCH when | Number of OFDM symbols for PDCCH when |
| --- | --- | --- |
| Subframe 1 and 6 for frame structure type 2 or a subframe for frame structure type 3 with the same duration as the DwPTS duration of a special subframe configuration | 1, 2 | 2 |
| MBSFN subframes with and configured with 1 or 2 cell-specific antenna ports | 1, 2 | 2 |
| MBSFN subframes with and configured with 4 cell-specific antenna ports | 2 | 2 |
| MBSFN subframes with , , or | 0 | 0 |
| Non-MBSFN subframes (except subframe 6 for frame structure type 2) configured with positioning reference signals | 1, 2, 3 | 2, 3 |
| All other cases | 1, 2, 3 | 2, 3, 4 |

The UE may assume the PCFICH is transmitted when the number of OFDM symbols for PDCCH is greater than zero unless stated otherwise in [4, clause 12].

</TP>

<TP 2.1 Alt2, TS36.211>

## 6.7	Physical control format indicator channel

The physical control format indicator channel carries information about the number of OFDM symbols used for transmission of PDCCHs in a subframe. The set of OFDM symbols possible to use for PDCCH in a subframe is given by Table 6.7-1.

Table 6.7-1: Number of OFDM symbols used for PDCCH

| Subframe | Number of OFDM symbols for PDCCH when | Number of OFDM symbols for PDCCH when |
| --- | --- | --- |
| Subframe 1 and 6 for frame structure type 2 or a subframe for frame structure type 3 with the same duration as the DwPTS duration of a special subframe configuration | 1, 2 | 2 |
| MBSFN subframes with and configured with 1 or 2 cell-specific antenna ports | 1, 2 | 2 |
| MBSFN subframes with and configured with 4 cell-specific antenna ports | 2 | 2 |
| MBSFN subframes with , Δf=2.5 kHz , or MBSFN slots with Δf≈0.37 kHz | 0 | 0 |
| Non-MBSFN subframes (except subframe 6 for frame structure type 2) configured with positioning reference signals | 1, 2, 3 | 2, 3 |
| All other cases | 1, 2, 3 | 2, 3, 4 |

The UE may assume the PCFICH is transmitted when the number of OFDM symbols for PDCCH is greater than zero unless stated otherwise in [4, clause 12].

</TP>

Additionally, x3535 proposes the following TP to correct the determination of starting symbol:

<TP 2.2, TS36.211>

## 6.5 		Physical multicast channel

The physical multicast channel shall be processed and mapped to resource elements as described in clause 6.3 with the following exceptions:

-	No transmit diversity scheme is specified.

-	Layer mapping and precoding shall be done assuming a single antenna port and the transmission shall use antenna port 4.

-	The PMCH can only be transmitted in the MBSFN region.

-	For PMCH with  other than , the index  in the first slot in the MBSFN subframe fulfils l≥lPMCHStart,

-	For PMCH with , the index  in the slot of the MBSFN region fulfils l≥lPMCHStart.

-	where lPMCHStart is equal to the value given by the higher layer parameter non-MBSFNregionLength [9].

-	The PMCH shall use extended cyclic prefix.

-	The PMCH is not mapped to resource elements used for transmission of MBSFN reference signals.

-	In clause 6.3.1, for Δf = 1.25 kHz and Δf ≈ 0.37 kHz, the scrambling generator shall be initialised at the start of each slot.

-	For Δf=182944Ts≈0.37 kHz the following exception applies to clause 6.3.5:

-	The text “which meet all of the following criteria in the current subframe” shall be replaced by “which meet all of the following criteria in the current slot”

-	The mapping to resource elements k,l on antenna port p not reserved for other purposes shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l.

</TP>

# Issue #3: Semistatic CFI

In x3786 and x4667, the following TPs are proposed:

- “Shall” vs “may” (x3786) for applicability of semistatic CFI

- Alignment with RAN2 specs (x4667)

<TP 3.1 36.213>

For a MBMS-dedicated cell, if a UE is configured with higher layer parameter semistaticCFI included in MasterInformationBlock-MBMS, the UE may assume the CFI is equal to the value of the higher layer parameter semistaticCFI for non-MBSFN subframes.

</TP>

<TP 3.2 36.213>

9.1.3	Control Format Indicator (CFI) assignment procedure

For a serving cell, if a UE is configured with higher layer parameter semiStaticCFI-SlotSubslotNonMBSFN, the UE shall assume the CFI is equal to the value of the higher layer parameter semiStaticCFI-SlotSubslotNonMBSFN for non-MBSFN subframes for receiving physical downlink shared channel with slot/subslot duration.

-------------------------- Other parts are omitted -----------------------------

For a MBMS-dedicated cell, if a UE is configured with higher layer parameter semiStaticCFI-MBMS included in MasterInformationBlock-MBMS, the UE shall assume the CFI is equal to the value of the higher layer parameter semiStaticCFI-MBMS for non-MBSFN subframes if a non-zero value is indicated by semiStaticCFI-MBMS.

</TP>

# Issue #4: Categories for 0.37kHz SCS

In x3785, it is proposed to send an LS to RAN2 to update TS 36.306 with the new larger TBS value. RAN1 would need to endorse a TP and send it to RAN2 to implement the CR. The proposed TP is as follows:

<===================================== TP 4.1 36.306 =============================>

## 4.1	ue-Category

<Unchanged parts are omitted>

Table 4.1-4: Maximum number of bits of a MCH transport block received within a TTI set by the field ue-Category for an MBMS capable UE capable of reception via MBSFN

| UE Category | Maximum number of bits of a MCH transport block received within a TTI |
| --- | --- |
| Category 1 | 10296 |
| Category 2 | 51024 |
| Category 3 | 75376 226416 (0.37kHz) |
| Category 4 | 75376 226416 (0.37kHz) |
| Category 5 | 75376 226416 (0.37kHz) |
| Category 6 | 75376 226416 (0.37kHz) |
| Category 7 | 75376 226416 (0.37kHz) |
| Category 8 | 75376 226416 (0.37kHz) |
| Category 9 | 75376 226416 (0.37kHz) |
| Category 10 | 75376 226416 (0.37kHz) |
| Category 11 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| Category 12 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |

<Unchanged parts are omitted>

## 4.1A	ue-CategoryDL and ue-CategoryUL

<Unchanged parts are omitted>

Table 4.1A-4: Maximum number of bits of a MCH transport block received within a TTI set by the field ue-CategoryDL for an MBMS capable UE capable of reception via MBSFN

| UE DL Category | Maximum number of bits of a MCH transport block received within a TTI |
| --- | --- |
| DL Category M1 | NA |
| DL Category M2 | NA |
| DL Category 0 | 4584 |
| DL Category 1bis | 10296 |
| DL Category 4 | 75376 226416 (0.37kHz) |
| DL Category 6 | 75376 226416 (0.37kHz) |
| DL Category 7 | 75376 226416 (0.37kHz) |
| DL Category 9 | 75376 226416 (0.37kHz) |
| DL Category 10 | 75376 226416 (0.37kHz) |
| DL Category 11 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 12 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 13 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 14 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 15 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 16 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 17 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 18 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 19 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 20 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |
| DL Category 21 | 75376 (64QAM) 97896 (256QAM) 226416 (64QAM 0.37kHz) 293736 (256QAM 0.37kHz) |

<=====================================End TP 4.1 =============================>

# Proposed scope of email discussions

Issues 2 and 3 are related to PDCCH/CFI, and it is the moderator’s view that they should not be very controversial (probably we just need to discuss wording, the proposals should implement the common understanding in RAN1).

Issue 4 is new and may require some additional discussion in RAN1 before liaising with RAN2.

Thus, it is proposed to have two email discussions as follows.

Proposal: The scope of email discussion for RAN1#101 is as follows:

- Email discussion #1: Issues related to control region and CFI (including TP 2.1, 2.2, 3.1, 3.2).

  - Moderator will provide consolidated TPs based on the input to this meeting.

- Email discussion #2: UE categories and 0.37kHz numerology.

  - Discuss TP 4.1 and potential LS to RAN2.

- Approval of the editor CR in x4200 will be handled with the endorsement of other editor CRs after the meeting.

# References

| R1-2004667 (revision of R1-2003337) | Control region for the new numerologies of LTE-based 5G terrestrial broadcast | ZTE, ABS |
| --- | --- | --- |
| R1-2003535 | Corrections to 36.211 for the 0.37 kHz subcarrier spacing MBSFN | Huawei, HiSilicon |
| R1-2003785 | Support of longer numerologies for rooftop reception | Qualcomm Incorporated |
| R1-2003786 | Corrections for CAS reception | Qualcomm Incorporated |
| R1-2004163 | Corrections to 36.211 for the 2.5 kHz subcarrier spacing MBSFN | Huawei, HiSilicon |
| R1-2004200 | Corrections to LTE-based 5G terrestrial broadcast | Nokia, Nokia Shanghai Bell |