3GPP TSG RAN WG1 #101	R1-2004700

e-Meeting, May 25th – June 5th, 2020

Source:	Sharp

Title:	Preliminary evaluation for FR1 Urban scenario

Agenda Item:	8.4.1.1

Document for:	Discussion and Decision

Background

SID for coverage enhancement in RP193240 [1] is approved at RAN#86.

Coverage evaluation

As described in [1], one of the targets for NR coverage enhancement is the fact that NR even in FR1 will be deployed in higher frequency than LTE (e.g., 3.5 GHz). Since it is expected that NR deployed in 3.5 GHz may serve eMBB services as well as voice and low-rate data services, ensuring certain level of data rate is required (e.g., 10 Mbps for DL, 1 Mbps for UL, in Urban scenario). Thus, supported functionalities should be carefully determined based on evaluating each scenario and candidate solutions to identify the functionalities with minimum specification impact and quality of services.

In the following section, we provide a preliminary evaluation for coverage estimation targeting FR1 Urban scenario, based on the link budget analysis.

Evaluation assumption

Table 1 shows a preliminary evaluation assumption for coverage estimation. Most of them are inherited from the self-evaluation for link budget analysis [2], with change on the target data rate as indicated by [1]. Table 2-1 to 2-5 shows additional parameters for link level simulation. Full parameters for the link budget analysis can be found in the attached spread sheet.

Table 1: Initial evaluation assumption for coverage estimation

| Carrier frequency | 4 GHz |
| --- | --- |
| TDD configuration | DDDSUUUU |
| Subcarrier spacing | 30 kHz |
| BS antenna height | 25 m |
| UE antenna height | 1.5 m |
| coverage reliability for control channel | 95% |
| coverage reliability for data channel | 90% |
| shadow fading standard deviation | 7 dB |
| Pathloss model | NLOS O-to-I |
| Penetration margin | 26.25 dB |
| Target data rate for PDCCH | 128000 bps |
| Target data rate for PUCCH | 4000 bps |
| Target data rate for PDSCH | 10 Mbps for Urban scenario [1] |
| Target data rate for PUSCH | 1 Mbps for Urban scenario [1] |
| Target error rate for control channel | 0.01 |
| Target error rate for data channel | 0.1 |
| Number of transmit antennas | 128 for model A, 192 for model B |
| Number of antenna ports | 2 |
| Transmission power | 44 dBm per 20 MHz for DL, 23 dBm for UL |
| Transmitter antenna gain | 8 dBi for DL, 0 dBi for UL |
| Cable, connector, body losses | 3 dB for DL, 1 DB for UL |
| Number of receive antennas | 4 for model A/B |
| Receiver noise figure | 7 dB for DL, 5 dB for UL |
| Thermal noise density | -174 dB |
| Receiver interference density | -169.30 dBm/Hz |
| Receiver implementation margin | 2 dB |
| HARQ gain for control channel | 0 dB |
| HARQ gain for data channel | 0.5 dB |

Table 2-1: Initial link level simulation assumption for all channels

| Channel estimation | 2D-MMSE |
| --- | --- |
| Receiver | MMSE |
| Channel model | TDL-C |

Table 2-2: Initial link level simulation assumption for PDCCH

| Number of OFDM symbols | 2 OFDM symbols |
| --- | --- |
| Aggregation level | 16 |
| DCI size | 64 bits including CRC |
| Channel coding | polar code |
| Allocated RBs | 48 RBs |

Table 2-3: Initial link level simulation assumption for PUCCH

| PUCCH format | PUCCH format 1 |
| --- | --- |
| Number of OFDM symbols | 14 OFDM symbols |
| UCI size | 2 bits |
| Allocated RBs | 1 RB |

Table 2-4: Initial link level simulation assumption for PDSCH

| Number of OFDM symbols | 12 OFDM symbols |
| --- | --- |
| MCS index | MCS 2 in Table 5.1.3.1-1 in [3] |
| Allocated RBs | 250 RBs |

Table 2-5: Initial link level simulation assumption for PUSCH

| Number of OFDM symbols | 14 OFDM symbols |
| --- | --- |
| MCS index | MCS 0 in Table 5.1.3.1-1 in [3] |
| Allocated RBs | 34 RBs |

Evaluation results

Table 3 shows the maximum range for each channel for Model A and Model B.

Table 4: Maximum range for each channel

|  | Model A | Model B |
| --- | --- | --- |
| PDCCH | 885.18 | 1597.01 |
| PUCCH | 264.15 | 477.58 |
| PDSCH | 760.81 | 1372.61 |
| PUSCH | 248.81 | 449.85 |

As shown above, the PUSCH has the smallest coverage and the PDCCH has the largest coverage among the 4 channels. Considering the PCell, or PSCell operation with NR, the coverage for the PUSCH should be enhanced to support the same coverage for the PDSCH. In addition, we see that the coverage range for the PDCCH should be enhanced for a ubiquitous coverage.

Proposal 1: The coverage for the PUSCH should be enhanced to support at least the same coverage as the PDSCH.

Proposal 2: PDCCH, PDSCH and PUCCH should be further studied for enhancement to support a ubiquitous coverage.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: The coverage for the PUSCH should be enhanced to support the same coverage for the PDCCH.

Proposal 2: PDCCH, PDSCH and PUCCH should be further studied for enhancement to support a ubiquitous coverage.

References

- RP-193240, December 2019

- RP-191533, June 2019