3GPP TSG RAN WG1 Meeting #101-e	  R1-2004763

E-meeting, 25 May – 5 June, 2020

Agenda Item:	7.2.5.1

Source:	Moderator (Huawei)

Title:	Summary #1 of email discussion [101-e-NR-L1enh-URLLC-PDCCH enhancements-01] on remaining issues on DCI format design

Document for:	Discussion and Decision

Introduction

The email discussion is to discuss the remaining issues on DCI format design.

[101-e-NR-L1enh-URLLC-PDCCH enhancements-01] Remaining issues on DCI format design and proposed conclusion#1 by 5/29 and corresponding TP (if any) by 6/5 – Chengyan (Huawei) including

- Issue A-1: Further extension of DCI size alignment due to the introduction of DCI format 0_2/1_2

- Issue A-3: Determination of DCI field sizes for the case of two HARQ-ACK codebooks

- Issue A-5: Completion of DMRS and PTRS reception procedure for PDSCH scheduled by DCI formats 1_2

- Issue A-13 (from UCI): Type2 HARQ-ACK codebook construction related to DAI bit width

- Issue A-4: Whether to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2

- Issue A-12: Correction on bandwidth part operation

- Proposed conclusion #1:

  - Leave it to gNB implementation to configure the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1 in the granularity of RBG.

This document summarizes the above issue and provide some initial proposals for discussion. Companies are encouraged to provide the first round views by 5/26, then we can adjust the proposals and prepare the TPs for the next step discussions.

Proposed conclusion #1

During the preparation phase, the following conclusion was proposed corresponding to issue A-10:

Proposed conclusion #1:

Leave it to gNB implementation to configure the starting position and hopping offset for PUSCH scheduled by DCI format 0_2 with resource allocation type 1 in the granularity of RBG.

Please comment if you have strong concern on the above proposed conclusion #1.

| Company | View |
| --- | --- |
| Feature lead | During the preparation phase, no any strong concern was raised on the proposed conclusion. |
| HW/HiSi | In case that this is concluded, we should at least make a note to further check whether a spec update in 38.214 is needed to translate RBG granularity into the PRB granularity of the start and offset values. |
| Qualcomm | We do not think that this conclusion is needed, and also don’t see why we need to restrict the FH offset to be RBG granularity. The main concern in R1-2003525 is for case (b) in the figure below. However, it’s not necessary that gNB always multiplex UEs that are scheduled using DCI format 0_2 with same RBG granularity in the same slot. The case (b) below is fine if other UEs are scheduled using DCI format 0_1 or DCI format 0_2 with smaller RBG size. Moreover, if gNB wants, it can configure FH offset as in case (c). But if we restrict all configurations to be compatible with case (c), then this may limit the flexibility of gNB scheduling (e.g., multiplex rel-15 and rel-16 UEs). Figure 1 - Frequency resource before and after FH |

- Summary of the status for proposed conclusion #1

- Has concern: Qualcomm

- Reason: Limit the flexibility of gNB scheduling

- Feature lead recommendation: Take the proposed conclusion #1. Firstly, for PUSCH scheduled by DCI format 0_2 with resource allocation type 1, we already agreed to indicate the resource allocation in the granularity of RBG to align with the common PRB grid, in order to provide better multiplexing with resource allocation type 0. If the starting position and hopping offset is not in the granularity of RBG, the benefit may not be achieved. Secondly, with the conclusion, it is easier for gNB to know that to achieve the benefit then gNB should configure the starting position and hopping offset in the granularity of the RBG. Finally, since it is leave to gNB implementation, gNB still has the flexibility not to configure it in this way if gNB feels more beneficial to do other way, that is there is no limitation on the flexibility at gNB side.

DCI format scheduling Rel-16 URLLC

This section summarize the issues on to be discussed on DCI format design.

- Issue A-1: Further extension of DCI size alignment due to the introduction of DCI format 0_2/1_2

In the current TS 38.212, DCI size alignment is extended due to the introduction of DCI format 0_2/1_2 with a few places in bracket due to lack of explicit agreements. WILUS (R1-2002634) provides a very nice table to summarize the DCI format sizes after each step as below:

Table 1. DCI format size after each step (TS38.212 v16.1.0)

|  | After Step 0 | After Step 1 | After Step 2 | After Step 2A | After Step 3 Check | After Step 4A | After Step 4B | After Step 4C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCI format 0_0 in CSS | Size A (padding) |  |  |  | 3 different DCI sizes with C-RNTI ? If no, go to step 4A~4C Otherwise, finish | Size A | Size A | Size A |
| DCI format 1_0 in CSS | Size A (truncation of FDRA field) |  |  |  |  | Size A | Size A | Size A |
| DCI format 0_0 in USS |  | Size B (padding) |  |  |  | Size B Size A | Size A | Size A |
| DCI format 1_0 in USS |  | Size B (padding) |  |  |  | Size B Size A | Size A | Size A |
| DCI format 0_1 |  |  | Size C (one bit insertion to differentiate Size B is needed) |  |  | Size C | Size C | Size C Size H |
| DCI format 1_1 |  |  | Size D (one bit insertion to differentiate Size B is needed) |  |  | Size D | Size D | Size D Size H |
| DCI format 0_2 |  |  |  | Size E (one bit insertion to differentiate Size B is needed) |  | Size E | Size E Size G | Size G |
| DCI format 1_2 |  |  |  | Size F (one bit insertion to differentiate Size B is needed) |  | Size F | Size F Size G | Size G |

The remaining issue is how to distinguish the following cases.

Case 1: Fallback DCI format (0_0, 1_0) and new DCI format (0_2, 1_2)

Case 2: Non-fallback DCI format (0_1, 1_1) and new DCI format (0_2, 1_2)

The issue of DCI size alignment (i.e. appending zero padding bit(s) when necessary, or if gNB should guarantee the size to be different) has been discussed in the RAN1#100-e email discussion thread [100e-NR-L1enh_URLLC_PDCCH-01]. Unfortunately, consensus was not achieved.

Based on the inputs from the contributions, company positions are summarized as below:

- Option 1:

  - One zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_1/1_1 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same USS.

  - Support (11 companies): FUTUREWEI, WILUS, Ericsson, ZTE, Huawei/HiSilicon, Vivo, NTT DOCOMO, CATT, Spreadtrum, Panasonic, OPPO

  - Reason

    - Same principle in Rel-15 when both DCI formats 0_0/1_0 and DCI formats 0_1/1_1 are configured to be monitored in USS

    - Cons from option 2 and option 3

      - Restrictions on configured functionality: it is not guaranteed that there will be many fields which can be easily adjusted without impacting desired functionality

        - e.g., if a large number of processes are configured to differentiate the DCI size, then the UE will not know that only a few processes that could possibly be used, thus potentially degrading performance. The performance difference will increase if the gNB had desired to configure FBRM along with the reduced processes, assuming that the extra memory could be used to improve performance of that ultra-reliable application.

- Option 2:

  - A UE is not expected to monitor a first decoding candidate with DCI format 0_0/1_0  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_0/1_0 and 0_2/1_2 have the same size.

  - A UE is not expected to monitor a first decoding candidate with DCI format 0_1/1_1  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_1/1_1 and 0_2/1_2 have the same size.

  - Support (8 companies): Intel, Qualcomm, Huawei/HiSilicon, MTK, LG, Apple, Nokia (2nd), Panasonic (2nd)

  - Reasons

    - It would be feasible to have different DCI sizes by gNB configuration

- Option 3: Supporting zero padding in case the DCI size budget is not exceeded (i.e. in steps 2A, 2B) and not supporting zero padding in case the DCI size budget is exceeded (i.e. in steps 4X), i.e.

  - For step 2A, one zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS, i.e. remove the brackets in step 2A of the DCI size alignment of Sec. 7.3.1.0 of TS 38.213.

  - For step 2B, one zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same or a different USS when having the same size after step 2A, i.e. support the intention of step 2B as outlined in the RAN1#100-e FL summary in R1-2001404 with a slight change to include the same or different search space.

  - For steps 4X, do not support zero padding to distinguish the DCI sizes as in case of Rel-15, i.e. no changes to steps 4A to 4C are seen as needed.

  - Support: Nokia, ZTE (fine), Huawei/HiSilicon (compromise)

- Option 4 There is no need to specify any mechanism to distinguish DCI formats 0_2/1_2 and DCI formats 0_0/1_0.

  - Support: Samsung

  - Reasons

    - It would be feasible to have different DCI sizes by gNB configuration

Feature lead view: We had intensive discussion in RAN1#100-e meeting, and unfortunately we was not able to achieve agreement due to objections to both option 1 and option 2. From feature lead perspective, I do agree with some companies that this should not be considered as an objectionable issue. In addition, we did have tried different ways to progress the discussion but in the end stuck still. At this stage, I don’t have a good recommendation but go with the majority view, i.e. option 1.

Proposal 3-1:

  - One zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_1/1_1 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same USS

Please comment if you have strong concern on the above proposal 3-1. Please also share if you think option 3 from Nokia can be a potential compromise solution if you don’t accept the above proposal 3-1 or if you support the proposal but will object option 2.

| Company | View |
| --- | --- |
| CATT | We support the proposal. As summarized by FL, the padding operation is already applied to the case when DCI format 0-1/1-1 has same size as that of DCI format 0-0/1-0 in the current specification and the configured functionality should be guaranteed. |
| Nokia, NSB | In principle we support the zero-padding, but would like to note here two things: - The zero-padding in Rel-15 is not supported in case the DCI size budget is exceeded (i.e. in step 4) - Having the zero padding also for this case, will create a quite complicated structure, as you will need to add and potentially remove the zero padding again when checking the number of applicable DCI sizes in each of the steps 4. Therefore, we don’t think we should do this at least in step 4, as this will become very easily very complicated. So in short, having the zero padding unconditionally (in both steps 2 & 4) should not be supported. We prefer either to do this only in step 2 (Option 3) – or then not do it at all (Option 2/4). |
| Panasonic | When BWP is switched or some parameter is changed, reconfiguration of other parameter is also necessary for size alignment. Therefore, our first preference is option 1 (proposal 3-1). However, if there is no consensus of option 1, we can accept option 2, “A UE is not expected to monitor …” should be specified in spec clearly in order to understand spec correctly. |
| Samsung | Object to the proposal. No specification change is needed (or at most a “UE does not expect” if a network misconfiguration is to be captured for this case). The reasons are (a) increased specification and UE complexity and (b) worse outcome (useless padding) than the network avoiding a same DCI size when configuring the (practically fully configurable) fields of DCI format 0_2/1_2. |
| WILUS | Support the proposal. When considering up to 4 DL BWPs and 4 UL BWPs, it may be complicate for gNB to ensure 3+1 DCI budget for every BWP pairs. Even if it is possible, gNB may configure unnecessary bit(s) to make different DCI size but UE does not know which bit(s) are useless. However, option 1 gives a way to ensure DCI size budget and also UE can use the zero-padding bit(s) as virtual CRC. |
| ZTE | Support the proposal, and also fine with Option 3. |
| vivo | We support the proposal 3-1. |
| HW/HiSi | We support the proposal. This discussion should come to an end now and it seems majority view is the only way out. However, we would also be fine with the Nokia proposal as a compromise. |
| Qualcomm | We object to the proposal, and we support Option 2. In our view, gNB has full flexibility to configure a different DCI size for format 0_2/1_2 from other DCI formats. Zero-padding based solution increases both the UE implementation and the specification complexity. We do not see a strong benefit for the gNB to go with Option 1 either: it is more complicated for the gNB to go through the lengthy alignment procedure summarized in Table 1 than to configure 1 more bit in DCI 0_2/1_2 (e.g., using finer granularity for RV/MCS/TDRA/FDRA etc.) |
| Ericsson | We support the proposal. This is a simple solution with minimal standardization and implementation impact and is based on the already established principle in Rel-15 for the case where UE is configured with both DCI formats 0_0/1_0 and DCI formats 0_1/1_1 having the same size in USS. |
| Apple | We support Option 2 |

- Summary of the status for issue A-1

Company position on proposal 3-1 is summarized as below, and some additional support for other options are added above also.

- Support: CATT, Panasonic, WILUS, ZTE, Vivo, Huawei/HiSilicon, Ericsson

- Object: Samsung, Qualcomm

- Feature lead recommendation: The situation seems same as in RAN1#100-e, we will need some help from Chairman on the conference call.

In addition, the following two questions were raised due to the discussion of DCI format alignment. Companies are encouraged to share your views.

Question A-1-1: Whether to use FBRM on the downlink with configurable size for HARQ process number field to improve the reliability of URLLC

As a UE may implement its soft buffer management according to the maximum number of HARQ processes, when a smaller number of HARQ processes are configured, it can be expected that the average memory per process can increase. However, if a large number of processes are configured to differentiate the DCI size, then the UE will not know that only a few processes that could possibly be used, thus potentially degrading performance.

As discussed in R1-2003722, with the configurable size number of HARQ channels, the performance will increase if the gNB uses FBRM instead of LBRM along with the reduced processes, with the UE using the extra memory for FBRM to improve performance of that ultra-reliable application. In Rel-15 the number of HARQ processes is not configurable, and LBRM is always used on the downlink. In Rel-16 LBRM or FBRM may not have been discussed. When the number of bits is smaller than the maximum, FBRM should be possible. Therefore, R1-2003722 proposes that FBRM is used on the downlink to improve the reliability of URLLC.

Feature lead view: More inputs are needed on this aspect before being able to make any proposal or conclusion.

Companies are encouraged to provide your views on whether to use FBRM on the downlink with configurable size for HARQ process number field.

| Company | View |
| --- | --- |
| CATT | A UE has to monitor fallback DCI formats which contain 4 bits HPN indicating 16 HARQ processes. If FBRM is applied, it seems a UE has to support LBRM and FBRM simultaneously depending on which DCI format is received. Furthermore, the TBS of URLLC traffic is typically small, LBRM may not be a limiting factor for URLLC. All in all, we don’t think it’s relevant to the PDCCH discussion here. |
| Nokia, NSB | As CATT pointed out, there may be other DCI formats using all up to 16 HARQ processes. Moreover, this is a unnecessary optimization from our perspective, as the difference in performance will only happen for very high coderates (not typical for URLLC). So we do not see a need for this. |
| FUTUREWEI | LBRM or FBRM for the DL may not have been discussed yet for URLLC in rel-16. The mother code rate is effectively limited to 2/3 rather than 1/3 (BG1) or 1/5 (BG2). Performance loss will be more apparent for smaller packet sizes and lower (more reliable) target codes than 2/3, which as CATT/Nokia point out are more typical for URLLC. Using FBRM should be more important for URLLC as every part of a dB is important. If people are fine to always use FBRM, or always want to use LBRM, then can conclude that and should not impact the alignment discussion. If people are ok to use FBRM but only for a small configured number of processes (as there may be enough memory to go around) then there is some relation to alignment as it is an example of where you may want to pad (and use say 4 HARQ and FBRM) rather than configure say 8 processes, use half of them, but then have to use LBRM. |
| Samsung | The proposal is neither relevant nor does it provide any necessary or beneficial correction. |
| Spreadtrum | We share the similar view as CATT. High code rate is not a typical use case for URLLC. |
| DOCOMO | Share the similar view as CATT. We think it is an optimization and not necessary at this stage. |
| vivo | Share the similar view as CATT. It is an optimization and not necessary to consider. |
| Intel | Updates in green and red: Similar views as CATT and Nokia. The benefits may not be realizable in practice with other DCI formats, and more importantly, use cases with small TBS values being typical for URLLC (i.e., TBS ≤ 3*TBS_max/210, i.e., TBS ≤ 0.3*TBSLBRM using NR terminology) imply negligible performance differences between LBRM and FBRM. |
| Qualcomm | We have similar view as CATT and others above. UE has to support other DCI formats with 16 HARQ processes on the same cell. So we don’t think the proposal is necessary. |
| Ericsson | While we are sympathetic towards the spirit of the proposal, we see difficulty to discuss an incomplete proposal at this stage. - If following UL configuration (i.e., using a RRC parameter to switch between FBRM and LBRM), a new RRC parameter is needed. We don’t think the proposal passes the high bar for introducing new RRC parameter at this stage. - Otherwise, the condition for applying FBRM behavior needs to be defined. The proposal above does not clearly describe such condition. Also we think it is difficult to sort out the condition at this stage. |

- Summary of the status for Question A-1-1

Companies provide some views on question A-1-1 above. Some companies provide the view that FBRM is mainly beneficial for larger packet size, and smaller packet size is the typical case for URLLC. Some company provide the view that if FBRM is used for small HARQ process number field then the performance loss will be more apparent for smaller packet size if padding HARQ process field to a larger size due to ensure different DCI size for different DCI format. Therefore, it seem the more critical question is that we don’t have any conclusion yet on the usage of LBRM and FBRM for URLLC, e.g. always use LBRM, or always use FBRM, or can be flexible. As some company pointed out that we may not be able to get sufficient time to discuss this during the CR phase. From feature lead perspective, I may recommend not to touch this in Rel-16, unless there is any problem. If there is any problem without any conclusion here, happy to see the views also.

Question A-1-2: Whether DCI size alignment is performed slot-by-slot basis or in a BWP across slots

## Apple (R1-2004221) In offline discussions with companies, it also appears that different companies have different understandings concerning the DCI size alignment procedure in Rel-15: - Understanding 1: DCI size alignment is performed over all search spaces configured at a BWP and across all slots, then for a given DCI format at a given search space, the DCI size is the same across slots; - Understanding 2: DCI size alignment is performed on a slot by slot basis over search spaces present in each slot, then for a given DCI format at a given search space, the DCI size may vary across slots. We have checked the discussion history on DCI size alignment in Rel-15: At RAN1 NR Ad Hoc meeting in Jan. 2018, the following working assumptions were reached: Agreements: For one carrier: - (working assumption) Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP - (working assumption) At most 4 different DCI sizes are monitored by the UE per slot - At most 3 different DCI sizes are monitored per C-RNTI per slot - Payload size for formats 0-1 and 1-1 may differ Working assumption: - The number of bits in the resource allocation field for format 0-0 and 1-0 depends on search space: - In CSS(s) in CORESET 0, use initial DL BWP for DCI size determination and RB numbering - FFS If a UE monitors 0-0/1-0 in CSS in CORESET 0 in a slot, it does not monitor formats 0-0 or 1-0 (or 2-x family in case they have a size aligned with 0-0/1-0) in any other search space - Otherwise, use active BWP for DCI size determination and RB numbering At RAN1 #92, the following were reached: Agreement: Above working assumption is replaced by the following: - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - FFS: Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP - FFS: how to meet the C-RNTI size and DCI size budget per slot - align 0-1 and 1-1 - configure active BWP such that the DCI size is the same as of the initial BWP - do not configure 0-1 and 1-1 - do not configure 0-0/1-0 in USS - other are not precluded - FFS: for format 0-0/1-0, how to interpret the frequency-domain field in a DCI with a size defined from a BWP with a different size than the BWP it is applied to At RAN1 #92bis, the following were reached: Agreement: - Confirm the following working assumption: - (working assumption) At most 4 different DCI sizes are monitored by the UE per slot - At most 3 different DCI sizes are monitored per C-RNTI per slot Conclusion: - It is understood that DCI sizes to monitor do not vary dynamically from slot-to-slot (other than impact due to BWP switching) but are based on RRC configuration, CSS vs. USS, and/or active BWP. Agreements: - To confirm the following working assumption with update Working assumption: - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - FFS: Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP From the discussion history, it can be seen while the early working assumptions might leave a room for different readings on how to ensure at most 4 DCI sizes are monitored by a UE per slot: - DCI size alignment is per slot, the limit is checked and met on a slot by slot basis; - DCI size alignment is per BWP, the limit is checked and met by examining all search spaces and across slots, and consequently there are at 4 DCI sizes per slot; The conclusion from RAN1 #92bis indicates unambiguously that DCI size alignment is not performed on a slot-by-slot basis (otherwise DCI sizes to monitor can vary dynamically from slot to slot). We conclude previous RAN1 decisions support the position that DCI size alignment is conducted for a BWP across slots rather than on a slot-by-slot basis. To facilitate discussion on DCI size alignment, we have Proposal 4: Note it in the chairman’s notes that DCI size alignment is conducted for search spaces in a BWP across slots rather than on a slot-by-slot basis.

Feature lead view: It would be good to clarify since it was observed that different companies may have different understanding. However, since it will have impact on Rel-15 also, it would be better to clarify it under Rel-15 maintenance first. However, if companies would like to share views here also, you can share it in the following table though we may not try to conclude here.

| Company | View |
| --- | --- |
| CATT | We share understanding#1, i.e. DCI size alignment is performed over all search spaces configured at a BWP and across all slots, then for a given DCI format at a given search space, the DCI size is the same across slots. |
| Nokia, NSB | Understanding #1, i.e. across all search spaces of a BWP across all slot |
| Panasonic | We support understanding 1 above as “DCI size alignment is performed over all search spaces configured at a BWP and across all slots, then for a given DCI format at a given search space, the DCI size is the same across slots”. Slot by Slot basis DCI size alignment is very complicated. |
| Samsung | In Rel-15, DCI size budget is determined according to the configured search space sets for the UE as described in TS 38.213 – there is no slot-by-slot dependence (other than for BWP switching or when some RRC configurations become invalid based on specifications). There is no additional aspect to consider in Rel-16. |
| Spreadtrum | Understanding #1 for Rel-15. |
| WILUS | Understanding #1 |
| Sharp | Understanding #1. Agree with Samsung. TS 38.213 cited below has already described that UE counts DCI format size per serving cell. Consequently, the UE would not count DCI size on a slot by slot and perform DCI size alignment on slot by slot. 38.213 Section 10.1 A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP. |
| DOCOMO | Share Understanding #1. |
| ZTE | Understanding #1 |
| vivo | Understanding #1 |
| Intel | Understanding #1. |
| HW/HiSi | Understanding #1. This is also simpler for the UE implementation |
| Qualcomm | Understanding #1. |
| Ericsson | Our understanding of Rel-15 behavior on DCI size alignment procedure is according to understanding #1. |
| Apple | Our understanding is Understanding #1. |

- Summary of the status for Question A-1-2

- Understanding 1: DCI size alignment is performed over all search spaces configured at a BWP and across all slots, then for a given DCI format at a given search space, the DCI size is the same across slots;

  - Support: CATT, Nokia, Panasonic, Samsung, Spreadtrum, WILUS, Sharp, DOCOMO, ZTE, Vivo, Intel, Huawei/HiSilicon, Qualcomm, Ericsson, Apple

- Understanding 2: DCI size alignment is performed on a slot by slot basis over search spaces present in each slot, then for a given DCI format at a given search space, the DCI size may vary across slots.

  - Support:

Based on the above views, it seems that understanding 1 is the common understanding. However, as I expressed above, since it will have impact on Rel-15 also, we will not try to make any conclusion here. Good to know the views from companies though.

- Issue A-3: Determination of DCI field sizes for the case of two HARQ-ACK codebooks (38.212, Section 7.3.1.2.2 and 7.3.1.2.3)

In the RAN1#99 meeting, the following agreement was made:

Working assumption:

When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

Ericsson, Vivo and CATT propose to confirm the above working assumption. Some companies propose to confirm the above working assumptions with some updates as summarized as below:

Potential update #1:

If the UE is configured with dynamic priority indication for DCI formats 0_1, 0_2, 1_1 or 1_2 (using PriorityIndicator-ForDCIFormat0_1/1_1/0_2/1_2)When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following DCI fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

  - Support: Nokia, Ericsson, MTK, NTT DOCOMO,CATT

  - Reasons:

    - Configuration of two HARQ-ACK codebooks does not necessarily mean that a particular DCI format can be used to schedule both HARQ-ACK priorities.

    - A more precise condition is to check whether the priority indicator field is present in the format, i.e. if more than one priority is scheduled by the respective DCI format as otherwise, the DCI format size may be unnecessarily large.

Feature lead view: When the agreements and TS 38.212 was made, it was not clear whether configuring priority indicator in DCI is the only way to enable one DCI format scheduling different priorities, e.g. some RRC configured way can be considered. Based on the discussion for the recent meetings, it seems the proposal is reasonable and the change can be update accordingly.

Potential update #2:

- When the UE is configured with two HARQ-ACK codebooks and for the case when any of DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

- PRI

  - Support: Vivo，Panasonic

  - Reasons:

    - PUCCH resources are also separately configured for HARQ-ACK codebooks.

  - Not support: Spreadtrum

  - Reasons:

    - The bit width is explicitly configured by gNB, not relying on any other configuration.

Feature lead view: PRI was actually discussed also when we made the agreement, the reason we didn’t include it is similar as what Spreadtrum gave here, since for PRI RRC will configure the number of bits for PRI directly, it seems not really necessary to have the alignment for different HARQ-ACK codebook, since gNB should be able to configure the same number of bits directly.

Potential update #3:

When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for the low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

- PUCCH resource indicator

  - Support: LG

  - Reasons:

    - The CBG operation for PUSCH is nothing to do with two HARQ-ACK codebook constructions.

Feature lead view: For PRI, similar views as that for proposed update #2. For deleting uplink DCI format 0_1, it seems reasonable. Similar issue raised under issue A-6 also.

Proposal 3-2: Confirm the working assumption below with updates:

Working assumption:

If the UE is configured with dynamic priority indication for DCI formats 0_1, 0_2, 1_1 or 1_2 (using PriorityIndicator-ForDCIFormat0_1/1_1/0_2/1_2)When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following DCI fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

Please provide your views and the corresponding reasons on the above proposal 3-2.

| Company | View |
| --- | --- |
| CATT | We support the proposal. |
| Nokia, NSB | We support the proposal |
| Panasonic | The argument from Spreadtrum seems reasonable. We are OK not to include PRI and support the proposal. |
| Samsung | Revisit after other agreements, at least ones related to priority indication. |
| Spreadtrum | We support the proposal |
| Sharp | Support the proposal. |
| DOCOMO | We support the proposal |
| ZTE | Support the proposal. |
| vivo | We are fine with the proposal. |
| Intel | Support the proposal. |
| HW/HiSi | Support the proposal |
| Qualcomm | We share the same view as Samsung; we suggest to revisit the issue after the ones related to priority indication with two DCI formats are settled. |
| Ericsson | We support the proposal. |
| Apple | We support the FL proposal |

Note that the corresponding TPs will be discussed in the TP phase.

- Summary of the status for issue A-3 (i.e. proposal 3-2)

- Support: CATT, Nokia, Panasonic, Spreadtrum, Sharp, DOCOMO, ZTE, Vivo, Intel, Huawei/HiSilicon, Ericsson, Apple

- 2 companies (Samsung and Qualcomm) prefer to revisit after achieving the agreement under UCI 02.

- Feature lead recommendation: Take proposal 3-2. It seems no matter what the outcome we would have under UCI 02, the conclusion here is still valid, since anyway we will have the case that a DCI format will be configured with the priority indicator field.

- Issue A-4: Whether to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2?

Ericsson R1-2003439 For DCI format 1_2 scheduling PDSCH, if only one bit is signalled, the redundancy version to be applied is either 0 or 3. This is a reasonable choice for PDSCH since both RV 0 and 3 are self-decodable for high code rate, and error cases exist where the gNB cannot tell whether the UE received the first transmission and stored the corresponding soft values or not. This is not the case for PUSCH. If the UE does not transmit the PUSCH correctly due to a missed grant, it is possible for the gNB to detect this, e.g. by looking at the noise level estimate based on DMRS. In this case the gNB can schedule the retransmission using RV 0 (basically treating it as the first transmission), which gives better performance than using RV 3 for a first transmission. On the other hand, if the first PUSCH transmission is transmitted correctly, but not decoded at the gNB due to a noisy transmission, the gNB would like to schedule the retransmission using RV 2, and soft combine with the first transmission. This gives better performance than using RV 3, as can be seen in [3] where Figure 1 appears. For this case, LDPC base graph (BG) #1 is used for information block size of K=1056 bits, and two consecutive transmissions are soft combined before decoding. As can be observed from Figure 1, for medium to high code rates above 2/3 (=0.67), the difference between using RV 3 and RV 2 for the second transmission is more than 1.5 dB over an AWGN channel. Figure 3 Required SNR for decoding after two transmissions for different RV orders for BG1. K is the TBS including CRC bits. Dynamically scheduled PUSCH is a case where there is no ambiguity about whether transmission occurred, or which instance of a transmission occurred. Self-decodability is not important for an individual retransmission. Hence the gNB should be able to schedule for best performance, i.e. it should be able to signal RV 2. For dynamically scheduled PUSCH, there is no ambiguity at the gNB whether the first transmission occurred or not, and RV should be chosen to maximize performance. The note above did not capture these aspects and thus cannot be used to conclude on the RV field for DCI format 0_2. For performance reasons and for alignment with NR-U agreement, we have the following proposal. The text proposal for TS 38.212 is also provided below. When only one bit is used to signal RV in DCI format 0_2, it indicates either RV 0 or RV 2. ------------------ Text Proposal for 38.212 Section 7.3.1.1.3 ------------------ - Redundancy version – 0, 1 or 2 bits determined by higher layer parameter NumberofbitsforRV-ForDCIFormat0_2 - If 0 bit is configured, rvid to be applied is 0; - 1 bit according to Table 7.3.1.2.3-1 7.3.1.1.2-34; - 2 bits according to Table 7.3.1.1.1-2. ----------------------------------------------End of proposed TP --------------------------------------------------.

Samsung (R1-2003865), MTK (R1-2003684) has the same view as Ericsson. More views are needed before making any decision on this issue. Companies are encouraged to provide their views on this.

Feature lead view: It looks beneficial to make the change. However, more inputs are needed before we can make any proposal here.

Question A-4-1: Is it necessary to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2?

Please provide your views and the corresponding reasons on the above question A-4-1.

| Company | View |
| --- | --- |
| CATT | We are fine to make such change considering the benefits. |
| Nokia, NSB | We are fine to make the change. Although not absolutely needed, there seems to be some benefit (as discussed in NR-U) and would align the operation here between ‘NR-U’ and ‘URLLC’ operation. |
| Panasonic | We see a merit on it. However, it is rather optimization. If gNB consider usage of RV 2 is important, bits of RV could be 2 bits in DCI. |
| Samsung | Support the change – both for performance and for having coherent specifications (with NR-U) |
| Sharp | Fine to support the change. |
| DOCOMO | We are fine with the change considering the benefits presented above contribution. |
| ZTE | We are fine to make such change, but we think it should also apply to PDSCH transmission scheduled by DCI format 1_2. The reason is follows. For DL, if RV{0, 2} is adopted, similar situation happens as UL, i.e., - If gNB receives no HARQ-ACK feedback from UE, gNB will schedule a re-transmission using RV0 (treating as the initial transmission), which gives better performance than using RV 3. - If gNB receives a NACK from UE, gNB will schedule a re-transmission using RV2. The claimed gain from RV2 over RV3 also exists for DL. The only potential issue is that, if UE misses the UL grant while gNB detects DTX as NACK, gNB would schedule RV2 which is not self-decodable. In such case, using RV3 is better. But such case is really rare (DL miss detection plus UL false alarm), and is not considered in gNB scheduling. |
| Intel | We are also fine with the aligning the behavior between NR-U and URLLC. |
| HW/HiSi | It seems reasonable to make the change. We would not object to it. |
| Qualcomm | We understand the intention of the proposal, and we are open to discuss the issue. However, we have the following concerns: - In case PUSCH is scheduled with repetition, then it’s better to use [0,3] than [0,2]. For example, suppose 2 repetitions are scheduled (which is the most typical case for type B PUSCH repetition), then the first transmission will use RV 0,2. For the retransmission, if we go with the proposal, then it will use RV 2,3. On the other hand, according to the current spec, RV 3,1 will be used, which has better performance. - The proposal requires the gNB to perform a DTX detection on the PUSCH. But not all gNBs has such capability. For gNB that does not have the DTX detection capability for PUSCH, it is better to use RV [0,3] due to self-decodability of RV3. |
| Ericsson | We support the proposal. |
| Apple | We are open to discuss the issue. |

- Summary of the status for question A-4-1

- Change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2

  - Support or fine with it: CATT, Nokia, Panasonic, Samsung, Sharp, DOCOMO, Intel, Huawei/HiSilicon, Ericsson

  - Reason:

    - Better performance

    - Same mechanism as that in NR-U

  - Have concerns: Qualcomm, ZTE

    - Qualcomm:

      - {0, 3} is better in case PUSCH is scheduled with repetition, e.g. for the case of 2 repetitions, with the current mechanism in the spec then {3, 1} will be used for the retransmission of the PUSCH with 2 repetitions, which with the proposal here then only {2, 3} can be used, in this case the current spec is better.

      - The scheme here relies on gNB to perform DTX detection on the PUSCH. If gNB doesn’t have this capability or the DTX performance is not reliable, then {0, 3} is better.

    - ZTE:

      - If we make the change, then it should be applied to both UL (i.e. DCI format 0_2 and DL (i.e. DCI format 1_2)

  - Feature lead recommendation: It seems the issue raised by Qualcomm on PUSCH with repetition is a valid point. Therefore, it seems making the change or not both have some beneficial scenario. ZTE’s point on DL is valid also. Since no company provide the response to Qualcomm and ZTE, I would make some tentative proposal to take the change based on the majority view right now and see more views from companies on the issue raised by Qualcomm. Simultaneously, companies should also check if they are ok to apply it to DL also.

- Issue A-5: Completion of DMRS and PTRS reception procedure for PDSCH scheduled by DCI formats 1_2 (Sec. 7.3.1.2.3 in 38.212, Sec. 5.1.6.2 & 5.1.6.3 in 38.214)

R1-2003577 proposed to simply follow the same logic for PDSCH with DCI format 1_2 as agreed for PUSCH with DCI format 0_2 by agreeing several related TPs (which are the DL / PDSCH / DCI format 1_2 equivalent of the related TPs agreed as part of Email discussion [100b-e-NR-L1enh-URLLC-eCG-01] with the outcome reported in R1-2002804.

TP2-2a:

- 5.1.6.2 DM-RS reception procedure ## <Unchanged text is omitted> If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB DMRS-DownlinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH: - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and - PT-RS is transmitted to the UE. ## <Unchanged text is omitted> - 5.1.6.3 PT-RS reception procedure The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and to a UE receiving PDSCH scheduled by DCI format 1_0 or format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support. ## <Unchanged text is omitted>

TP 2-2b:

- 7.3.1.2.3 Format 1_2 ## <Unchanged text is omitted> - Antenna port(s) – 0, 4, 5, or 6 bits - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 is not configured; - Otherwise 4, 5 or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and is configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 but configured with one or more of dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4. ## <Unchanged text is omitted>

Feature lead view: The issue is valid and needs to be corrected. However, for the TP 2-2b above for DCI format 1_2, it seems not necessary. This is different from the case for PUSCH, where the sentence is deleted to cover configured grant case. However, for downlink, there is no similar issue, and if none of the parameter dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 is configured, it should follow the behavior for DCI format 1_0.

Updated feature lead view: As Nokia pointed that if we don’t take the TP 2-2b then we need some additional TP for TS 38.214 also. My previous worry is that with the TP 2-2b, it would require gNB to always configure either dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 in order to determine which table among Tables 7.3.1.2.2-1/2/3/4 to use to get the bit field index value 0, while following the procedure for fallback DCI 1_0 can avoid this. But after further checking TS 38.331, even neither dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 is configured, the default values are defined in TS 38.331, which can be used to determine the table accordingly. Therefore, it is ok to take the TP 2-2b also instead of including another TP for TS 38.214.

Revised Proposal 3-3: Adopt the following text proposals with changes marked in red for PTRS reception procedures with DCI format 1_2:

#### 5.1.6.2 DM-RS reception procedure ## <Unchanged text is omitted> If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB DMRS-DownlinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH: - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and - PT-RS is transmitted to the UE. ## <Unchanged text is omitted> #### 5.1.6.3 PT-RS reception procedure The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and to a UE receiving PDSCH scheduled by DCI format 1_0 or format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support. ## <Unchanged text is omitted>

- 7.3.1.2.3 Format 1_2 ## <Unchanged text is omitted> - Antenna port(s) – 0, 4, 5, or 6 bits - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 is not configured; - Otherwise 4, 5 or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and is configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 but configured with one or more of dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4. ## <Unchanged text is omitted>

Please provide your views on the above proposal 3-3.

| Company | View |
| --- | --- |
| CATT | We are fine with the proposal |
| Nokia, NSB | Do not agree - the 38.214 TP alone is not complete without the TP in 212 especially considering the DM-RS reception procedure. Reason: If the FL thinks we do not want to change 38.212 and then DCI format 1_2 would then follow DCI format 1_0, we are then in trouble otherwise – e.g. with the DM-RS transmission which would require further changes (also for DM-RS overhead assumption, please see our related TPs to the last meeting – which shows which additional changes would be needed if we follow the fallback DCI, see Sec. 2.2 of our RAN1#100bis-e meeting in R1-2001694). We did not include it this time, as we thought that we have the same operation for PDSCH and PUSCH – i.e. 1_2 always follows basically 1_1 with potentially having fixed mapping to antenna port value 0. So either we go also for TP 2-2b – or the changes in Sec. 2.2 of R1-2001694 are needed! Update in v12 based on revised proposal: Agree with revised proposal 3-3 |
| Samsung | OK with the proposal. Can further discuss if any other change is needed. |
| Spreadtrum | We support the revised proposal. When UE is not configured AntennaPorts-FieldPresence-ForDCIFormat1_2 and is configured with either dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2, the UE should following the first index in respective table according to DMRS configurations. When without DMRS configurations, UE assumes a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 as DCI format 1_0 is transmitted. |
| Sharp | Support the revised proposal. |
| DOCOMO | Support the revised proposal. |
| ZTE | Support the revised proposal. |
| vivo | Support the revised proposal. |
| Intel | We support the revised proposal. |
| HW/HiSi | Support the revised proposal. |
| Qualcomm | We agree that the identified issues need to be fixed. However, we have some questions about TP 2-2b regarding the default behavior when neither dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 is configured. Can we clarify what is the default behavior when the two parameters are not configured? Is there any spec update (e.g., in 38.214) needed to capture the default behavior for PDSCH scheduled by DCI format 1_2? |
| Ericsson | We support the proposal. |
| Apple | We support this proposal |

- Summary of the status for issue A-5 (i.e. proposal 3-3)

- Support: CATT, Nokia, Samsung, Spreadtrum, Sharp, DOCOMO, ZTE, Vivo, Intel, Huawei/HiSilicon, Ericsson, Apple

- May have concern: Qualcomm

  - What’s the default behavior when the two parameters are not configured? Is there any spec update (e.g., in 38.214) needed to capture the default behavior for PDSCH scheduled by DCI format 1_2?

- Feature lead recommendation: Take the proposal 3-3. As to the question from Qualcomm, as shown in the TP, when both parameters are not configured, the UE will use bit field index value 0 of the corresponding table based on the default values of the DMRS related parameter (e.g. dmrs-Type and maxLength) defined in TSS 38.331.  No spec updated needed in TS 38.214.

- Issue A-12: Correction on bandwidth part operation

In the RAN1#98bis meeting [1], the “bandwidth part indicator field” has been agreed to support BWP switching with DCI formats 0_2 and 1_2. The same RRC parameters are used for both DCI format 0_1/1_1 and DCI format 0_2/1_2. However, the corresponding specification for DCI format 0_2/1_2 are missing in TS 38.213. Huawei (R1-2003525) proposed a TP for it.

Feature lead view: The issue is valid and a TP is needed.

Proposal 3-5: Adopt the following text proposal to capture BWP switching with DCI format 0_2 and DCI format 1_2 in section 12 in TS 38.213.

- 12 Bandwidth part operation *** Unchanged text is omitted *** If a bandwidth part indicator field is configured in DCI format 1_1 or DCI format 1_2, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1 or DCI format 0_2, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1/0_2 or DCI format 1_1/1_2 and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the received DCI format 0_1 or DCI format 1_1 or DCI format 0_2 or DCI format 1_2 - if the size of the information field is smaller than the one required for the DCI format 0_1/0_2 or DCI format 1_1/1_2 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format 0_1/0_2 or DCI format 1_1/1_2 information fields, respectively - if the size of the information field is larger than the one required for the DCI format 0_1/0_2 or DCI format 1_1/1_2 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE uses a number of least significant bits of DCI format 0_1/0_2 or DCI format 1_1/1_2 equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format 0_1/0_2 or DCI format 1_1/1_2 information fields, respectively - set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format 0_1 or DCI format 1_1 or DCI format 0_2 or DCI format 1_2, respectively A UE does not expect to detect a DCI format 1_1/1_2 or a DCI format 0_1/0_2 indicating respectively an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change [10, TS 38.133]. If a UE detects a DCI format 1_1 or a DCI format 1_2 indicating an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format 1_1 or a DCI format 1_2 in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format 1_1 or a DCI format 1_2. If a UE detects a DCI format 0_1 or a DCI format 0_2 indicating an active UL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format 0_1 or a DCI format 0_2 in the scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format 0_1 or a DCI format 0_2. A UE does not expect to detect a DCI format 1_1 or a DCI format 1_2 indicating an active DL BWP change or a DCI format 0_1 or a DCI format 0_2 indicating an active UL BWP change for a scheduled cell within FR1 (or FR2) in a slot other than the first slot of a set of slots for the DL SCS of the scheduling cell that overlaps with a time duration where the UE is not required to receive or transmit for an active BWP change in a different cell from the scheduled cell within FR1 (or FR2). A UE expects to detect a DCI format 0_1 or a DCI format 0_2 indicating active UL BWP change, or a DCI format 1_1 or a DCI format 1_2 indicating active DL BWP change, only if a corresponding PDCCH is received within the first 3 symbols of a slot. *** Unchanged text is omitted ***

Please provide your views and the corresponding reasons on the above proposal 3-4.

| Company | View |
| --- | --- |
| CATT | Support |
| Nokia, NSB | Support |
| Panasonic | Support |
| Samsung | OK |
| Spreadtrum | Support |
| WILUS | Support |
| Sharp | Support |
| DOCOMO | Support |
| ZTE | Support |
| vivo | Support |
| Intel | Support |
| HW/HiSi | Support |
| Qualcomm | Support. |
| Ericsson | We support the proposal. |
| Apple | We support the proposal. |

- Summary of the status for issue A-12 (i.e. proposal 3-5)

- Support: CATT, Nokia, Panasonic, Samsung, Spreadtrum, WILUS, Sharp, DOCOMO, ZTE, Vivo, Intel, Huawei/HiSilicon, Qualcomm, Ericsson, Apple

- Feature lead recommendation: Take proposal 3-5 which is stable.

## Issue A-13: Type2 HARQ-ACK codebook construction related to DAI bit width

The C-DAI field in DCI format 1_1 is 2 bits if only one serving cell is configured in the DL, and in DCI format 1_2 is configured as 0, 1, or 2 bit. The C-DAI is used to find HARQ-ACK bit position in Type 2 HARQ-ACK codebook. If the first HARQ-ACK bit corresponding to a first PDSCH scheduled by DCI format 1_1 and the second HARQ-ACK bit corresponding to a second PDSCH scheduled by DCI format 1_2 are multiplexed into a same dynamic HARQ-ACK codebook, and bit length of C-DAI in DCI format 1_1 and in DCI format 1_2 is different, according to current pseudo-code in TS38. 213, the gNB cannot correctly find the HARQ-ACK bit position for the first HARQ-ACK bit and the second HARQ-ACK bit.

This issues was discussed in RAN1#100bis-e under UCI enhancements but not be able to conclude. Companies provide further views in the contributions this meeting, the candidate options and positions are summarized as below:

- Option 1: If the size of counter DAI between DCI format 1_2 and 1_0/1_1 is different, a UE is not expected to multiplex HARQ-ACK for PDSCH scheduled by DCI format 1_2 and HARQ-ACK for PDCSH scheduled by DCI format 1_0/1_1 in the same HARQ-ACK codebook.

  - UE does not expect to multiplex HARQ-ACK information scheduled by a DCI format without counter DAI field in a same HARQ-ACK codebook

  - Support: Vivo, Huawei, CATT, NEC, Spreadtrum, OPPO, DCM, LGE, Qualcomm, LGE, Samsung, ZTE

  - Reasons

    - Enable smaller DCI size for high priority scheduling.

    - gNB can avoid multiplexing PDSCH scheduled by different DCI formats with different DAI bit width to the same HARQ-ACK codebook

- Option 2: The size of DAI field is the maximum of the bit widths for the DCI formats.

  - Support: LGE

- Option 3: When a UE is configured with DCI format 1_0 and/or with DCI format 1_1 and DCI format 1_2 for scheduling same priority traffic, the bit-width of the DAI fields is same.

  - Support: ZTE, Panasonic, Samsung, Spreadtrum, Apple

  - Reasons

    - The overall flexibility of Option 1 is questionable since the reliance on gNB scheduling is via scheduling restrictions.

    - No need to complicate DAI calculation and tracking unnecessarily complicated. There is no incentive for the NW to configure different sizes of DAI field to gain 1 bit DCI saving at the cost of increased complexity. So, the reasonable configuration would be to have the same DAI field bit-sizes for the DCIs contributing to the same CB

- Option 4: Modify the procedure of generation of codebook. 2-bit DAI and 1-bit DAI is transformed to standard DAI value and the standard DAI value is used to determine HARQ-ACK bit location. For example, 2-bit DAI value {1, 2, 3, and 4} is transformed to {0, 1, 0, and 1} to align 1-bit DAI value. In general way, standard DAI value = mod (2-bit DAI value or 1-bit DAI value -1, 2) +1

  - Support: OPPO, Nokia, WILUS

  - Reasons

    - Unnecessarily restrictive for option 1, 3 and 5

- Option 5: For type-2 HARQ-ACK codebook, if the size of DAI field between DCI format 1_2 and 1_0/1_1 is different, and they can schedule HARQ transmission using the same PUCCH-Config then the minimum DAI bit width over DCI_1_1 and DCI_1_2 is to be taken (Option 4 in R1-2002784)

  - Support: MTK, WILUS, Nokia (2rd), Intel

  - Reasons

    - More flexibility than mandating that the same DAI bitwidth be configured

Feature lead view: Since it was already discussed in RAN1#100b-e meeting, I assumed that people already understand well the pros and cons of each option, but it seems still not much difference with the positions. In this case, I would like to make the tentative proposal below to follow the majority view first, though I do agree that we can support it better with some modification in the spec.

Proposal 3-6: If the size of counter DAI between DCI format 1_2 and 1_0/1_1 is different, a UE is not expected to multiplex HARQ-ACK for PDSCH scheduled by DCI format 1_2 and HARQ-ACK for PDCSH scheduled by DCI format 1_0/1_1 in the same HARQ-ACK codebook.

- UE does not expect to multiplex HARQ-ACK information scheduled by a DCI format without counter DAI field in a same HARQ-ACK codebook

Please provide your views on the above proposal 3-6.

| Company | View |
| --- | --- |
| CATT | Support. It can be easily handled by gNB and guarantee a smaller payload size as expected. In addition, the intention of the sub-bullet seems to make sure UE will not multiplex different HARQ-ACK information scheduled by different DCI formats without C-DAI field in a same HARQ-ACK codebook. We propose the following updates: If the size of counter DAI between DCI format 1_2 and 1_0/1_1 is different, a UE is not expected to multiplex HARQ-ACK for PDSCH scheduled by DCI format 1_2 and HARQ-ACK for PDCSH scheduled by DCI format 1_0/1_1 in the same HARQ-ACK codebook. - UE does not expect to multiplex HARQ-ACK information scheduled by a DCI formats without counter DAI field in a same HARQ-ACK codebook |
| Nokia, NSB | We cannot agree to this (i.e. Option 1), as the argument for doing this is not working. We are not having any default ‘high priority’ for DCI format 1_2 (the DCI format can schedule both) but dynamic indication. This would basically mean, the gNB would anyhow need to align then the DAI size (as for Option 3). Would be nice to hear from Option 1 camp some clarification how the saving is there! We would prefer either Option 4 (for optimal support and DAI field size saving) or then something along the lines of Option 5 as a compromise (which seems to be a better version of Option1). |
| Panasonic | We are supportive of Option 1 or Option 3. Although Option 3 might have more flexibility on scheduling, Option 1 is also OK to us. |
| Samsung | Option 1 or Option 3. |
| Spreadtrum | We support the proposal, and also fine with option 3. The different size of counter DAI between DCI format 1_2 and 1_0/1_1 only exits in single carrier configuration. For this one case, we don’t prefer to change the UE behaviors for DAI such as Option 4 and 5, especially it only save 1 bit for DCI 1_2 but with potential performance loss and increased complexity for both of UE and gNB side. |
| WILUS | Option 4 or 5 provide a rule to multiplex HARQ-ACK bits even in the case of different counter DAI bit width. Additional UE complexity is marginal at least in option 5 because a UE can generate type-2 CB by taking the minimum DAI bit width account into. If this additional complexity is still concern, it would be beneficial to adopt option 3 as a compromise. |
| Sharp | We are fine with either the proposal or the option 3. |
| DOCOMO | Support the proposal |
| ZTE | Option 1 or Option 3. |
| vivo | Support the proposal |
| Intel | Agree with Nokia that with Options 1or 3, effectively the benefits and flexibility from introduction of flexible DAI bit-field size for DCI format 1_2 is nullified. In this regard, we further agree with Nokia that Option 5 that resembles a truncation-based approach is the most reasonable compromise that provides the DCI format size flexibility without forcing gNB to align DAI bit-fields. |
| HW/HiSi | Option 1 |
| Qualcomm | We support the proposal. |
| Ericsson | We can support the proposal. |
| Apple | Option 3 is preferred, excessive flexibility is not needed. |

- Summary of the status for issue A-13

- Proposal 3-6:

  - Support: CATT, Panasonic, Samsung, Spreadtrum, Sharp, DOCOMO, ZTE, Vivo, Huawei/HiSilicon, Ericsson, Qualcomm

  - Not support: Nokia, WILUS, Intel

    - Nokia: When DCI 1_2 is used to schedule both priorities, then the DAI field needs to go to the larger one, thus no bit reduction

    - FL: Companies think in this case will still go with the smaller one if really need to reduce the DCI size for high priority

    - WILUS: Additional UE complexity is marginal by option 5

    - Intel: Option 5 that resembles a truncation-based approach is the most reasonable compromise that provides the DCI format size flexibility without forcing gNB to align DAI bit-fields

In addition, WILUS (R1-2004523) also provide some views on how to handle 1-bit counter DAI and 2 bits UL DAI as below.

WILUS R1-2004523 2-bit total-DAI and 2-bit UL DAI There exists 2-bit UL DAI for dynamic HARQ-ACK codebook in DCI format 0_1 and 0_2 and similarly, there are 2-bit total DAI field for dynamic HARQ-ACK codebook in DCI format 1_1 and 1_2. The possible value range of the 2-bit total DAI and UL DAI can be from 1 to 4. The total DAI and UL DAI can be used to determine HARQ-ACK codebook size. For example, by definition of UL DAI, the value of UL DAI, denoted by VDAIUL, is determined by VDAIUL=(X-1) mod 4 + 1 , where X is the number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release is present. In other words, if a UE detects a DCI format with VDAIUL, then the possible HARQ-ACK codebook size will be 4*n+VDAIUL, where n is non-negative integer. Consider the case where a UE receives 3 DCI formats having 1-bit counter-DAI among 3 monitoring occasions as shown in figure 3. The counter-DAI value is 1, 2, and 1 in DCI formats detected in MO#0, MO#1, and MO#2, respectively. In this case, 2-bit UL DAI in DCI format scheduling PUSCH indicates the UL-DAI = 3. Figure 3. 2-bit UL DAI and 1-bit counter-DAI Based on the current pseudo-code for Type-2 HARQ-ACK codebook, the HARQ-ACK codebook size is determine by OACK=TD⋅j+Vtemp2. And in this example, TD=2, j=1, Vtemp2=3 so that OACK= 2*1+3=5, which results in incorrect HARQ-ACK codebook size. To address this issue, the HARQ-ACK codebook size should be OACK=TD⋅j+Vtemp2-1mod TD+1. The same way can be also applied to the 2-bit total DAI case. - Proposal 5: To support 2-bit UL DAI, 2-bit total-DAI and 1-bit counter-DAI, the HARQ-ACK codebook size should be determined by OACK=TD⋅j+Vtemp2-1mod TD+1.

Feature lead view: The issue is valid and we need to discuss and fix it. The situation is different from counter DAI, since even when only DCI format 0_2/1_2 is configured to monitor, the issues still exits, therefore better to solve it. It seems the proposal from WILUS is reasonable. The following tentative proposal is made for further discussion. Note that the case of 1-bit counter DAI and 2-bits total DAI doesn’t exist, since 1-bit counter DAI is only applied to single carrier case.

Proposal 3-7: If UE is configured to monitor DCI format 1_2/0_2, the HARQ-ACK codebook size for type-2 HARQ-ACK codebook is determined by OACK=TD⋅j+Vtemp2-1mod TD+1

Please provide your views on how to solve the above issue.

| Company | View |
| --- | --- |
| Feature lead | Since the above formula can be applied to the case of 2-bit counter DAI also, it seems simpler to use it for all cases when DCI format 1_2/0_2 is configured to monitor. |
| CATT | We agree this issue should be fixed as 2 bits UL DAI and 1 bit C-DAI is a possible combination. |
| Nokia, NSB | Issue is valid and needs to be addressed. |
| Samsung | Can discuss further – proposed correction can result to modulo of a negative number which is problematic. |
| WILUS | Need to discuss the issue 2-bit UL DAI and 1-bit C-DAI. Note that since Vtemp2 = VDAIUL has one value among 1,2,3, or 4, Vtemp2 is non-negative. So, there are no problems in the modulo operation in the proposal. |
| Sharp | Support. Agree with WILUS. The value of (Vtemp2 -1) would not be a negative number in the proposed correction. |
| ZTE | Agree |
| vivo | We agree this issue should be fixed and can be further discussed. |
| Intel | Support the proposal. |
| HW/HiSi | Agree |
| Qualcomm | We support the proposal. |
| Ericsson | We do not support the proposal. We are supportive to remove the possibility of 1-bit DAI field in DCI format 1_2. We see no motivation for the network to configure different bit-widths for DAI fields in different DCI formats, e.g., 1-bit counter DAI in DCI format 1_2. To save 1 bit in DCI at the cost of increased complexity is not justified in our opinion. Any reasonable configuration would have the same DAI field bit-sizes for the DCIs contributing to the same HARQ-ACK codebook. With the removal of the possibility of 1-bit DAI field for DCI format 1_2, the issue would no longer exist and there is no need for the correction. |
| Apple | We agree with Ericsson |

- Summary of the status for issue A-13 (i.e. proposal 3-7)

- Proposal 3-6:

  - Support: CATT, Nokia, WILUS, Sharp, ZTE, Vivo, Intel, Huawei/HiSilicon, Qualcomm

  - May have concern: Samsung, Ericsson, Apple

    - Samsung: May result to modulo of a negative number

    - Feature Lead: As WILUS and Sharp mentioned, since Vtemp2 = VDAIUL has one value among 1,2,3, or 4, Vtemp2 is non-negative

    - Ericsson&Apple: Prefer to remove 1-bit DAI in DCI format 1_2. No motivation for the network to configure different bit-widths for DAI fields in different DCI formats, e.g., 1-bit counter DAI in DCI format 1_2.

    - Feature Lead: We should try to avoid reverting the agreement we made. In addition, the total DCI size of DCI format 0_2 and DCI format 1_2 depends on many other fields also, not just this DAI fields.

  - Feature lead recommendation: Take the proposal 3-7.

Summary of the status and potential proposals for second round discussion

- Issue A-1: Further extension of DCI size alignment due to the introduction of DCI format 0_2/1_2

Proposal 3-1:

  - One zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_1/1_1 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same USS

Summary of the status for issue A-1

Company position on proposal 3-1 is summarized as below, and some additional support for other options are added above also.

- Support: CATT, Panasonic, WILUS, ZTE, Vivo, Huawei/HiSilicon, Ericsson

- Object: Samsung, Qualcomm

- Feature lead recommendation: The situation seems same as in RAN1#100-e, we will need some help from Chairman on the conference call.

Details of company positions for issue A-1

- Option 1:

  - One zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_1/1_1 monitored in another USS.

  - One zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same USS.

  - Support (11 companies): FUTUREWEI, WILUS, Ericsson, ZTE, Huawei/HiSilicon, Vivo, NTT DOCOMO, CATT, Spreadtrum, Panasonic, OPPO

  - Reason

    - Same principle in Rel-15 when both DCI formats 0_0/1_0 and DCI formats 0_1/1_1 are configured to be monitored in USS

    - Cons from option 2 and option 3

      - Restrictions on configured functionality: it is not guaranteed that there will be many fields which can be easily adjusted without impacting desired functionality

        - e.g., if a large number of processes are configured to differentiate the DCI size, then the UE will not know that only a few processes that could possibly be used, thus potentially degrading performance. The performance difference will increase if the gNB had desired to configure FBRM along with the reduced processes, assuming that the extra memory could be used to improve performance of that ultra-reliable application.

- Option 2:

  - A UE is not expected to monitor a first decoding candidate with DCI format 0_0/1_0  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_0/1_0 and 0_2/1_2 have the same size.

  - A UE is not expected to monitor a first decoding candidate with DCI format 0_1/1_1  and a second candidate with DCI format 0_2/1_2, where the two decoding candidates are mapped to the same resource and the DCI formats 0_1/1_1 and 0_2/1_2 have the same size.

  - Support (8 companies): Intel, Qualcomm, Huawei/HiSilicon, MTK, LG, Apple, Nokia (2nd), Panasonic (2nd)

  - Reasons

    - It would be feasible to have different DCI sizes by gNB configuration

- Option 3: Supporting zero padding in case the DCI size budget is not exceeded (i.e. in steps 2A, 2B) and not supporting zero padding in case the DCI size budget is exceeded (i.e. in steps 4X), i.e.

  - For step 2A, one zero-padding bit is added to DCI format 0_2/1_2 to differentiate DCI format 0_2/1_2 monitored in USS and DCI format 0_0/1_0 monitored in another USS, i.e. remove the brackets in step 2A of the DCI size alignment of Sec. 7.3.1.0 of TS 38.213.

  - For step 2B, one zero-padding bit is added to DCI format 0_1/1_1 to differentiate DCI format 0_2/1_2 and DCI format 0_1/1_1 monitored in the same or a different USS when having the same size after step 2A, i.e. support the intention of step 2B as outlined in the RAN1#100-e FL summary in R1-2001404 with a slight change to include the same or different search space.

  - For steps 4X, do not support zero padding to distinguish the DCI sizes as in case of Rel-15, i.e. no changes to steps 4A to 4C are seen as needed.

  - Support: Nokia, ZTE (fine), Huawei/HiSilicon (compromise)

- Option 4 There is no need to specify any mechanism to distinguish DCI formats 0_2/1_2 and DCI formats 0_0/1_0.

  - Support: Samsung

  - Reasons

    - It would be feasible to have different DCI sizes by gNB configuration

Feature lead view: We had intensive discussion in RAN1#100-e meeting, and unfortunately we was not able to achieve agreement due to objections to both option 1 and option 2. From feature lead perspective, I do agree with some companies that this should not be considered as an objectionable issue. In addition, we did have tried different ways to progress the discussion but in the end stuck still. At this stage, I don’t have a good recommendation but go with the majority view, i.e. option 1.

- Issue A-3: Determination of DCI field sizes for the case of two HARQ-ACK codebooks (38.212, Section 7.3.1.2.2 and 7.3.1.2.3)

Proposal 3-2: Confirm the working assumption below with updates:

Working assumption:

If the UE is configured with dynamic priority indication for DCI formats 0_1, 0_2, 1_1 or 1_2 (using PriorityIndicator-ForDCIFormat0_1/1_1/0_2/1_2)When the UE is configured with two HARQ-ACK codebooks at least for the case when only one of the two DCI formats (1_1 and 1_2 for DL, 0_1 and 0_2 for UL), configured to support two HARQ-ACK codebooks, is configured to be monitored by the UE, the bit width of the following DCI fields is the maximum of the bit widths for the two configurations corresponding to the two HARQ-ACK codebooks. The necessary number of most significant zero bits can be added to a field to achieve the alignment.

- PDSCH-to-HARQ_feedback timing indicator

- Beta offset indicator

- DAI

- CBGTI & CBGFI (if configured for low priority HARQ-ACK codebook for DCI format 1_1 and DCI format 0_1)

Please comment if you still cannot accept the above proposal 3-2.

| Company | View |
| --- | --- |

Summary of the status for issue A-3 (i.e. proposal 3-2)

- Support: CATT, Nokia, Panasonic, Spreadtrum, Sharp, DOCOMO, ZTE, Vivo, Intel, Huawei/HiSilicon, Ericsson, Apple

- 2 companies (Samsung and Qualcomm) prefer to revisit after achieving the agreement under UCI 02.

- Feature lead recommendation: Take proposal 3-2. It seems no matter what the outcome we would have under UCI 02, the conclusion here is still valid, since anyway we will have the case that a DCI format will be configured with the priority indicator field.

- Issue A-4: Whether to change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2?

Summary of the status for question A-4-1

- Change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2

  - Support or fine with it: CATT, Nokia, Panasonic, Samsung, Sharp, DOCOMO, Intel, Huawei/HiSilicon, Ericsson

  - Reason:

    - Better performance

    - Same mechanism as that in NR-U

  - Have concerns: Qualcomm, ZTE

    - Qualcomm:

      - {0, 3} is better in case PUSCH is scheduled with repetition, e.g. for the case of 2 repetitions, with the current mechanism in the spec then {3, 1} will be used for the retransmission of the PUSCH with 2 repetitions, which with the proposal here then only {2, 3} can be used, in this case the current spec is better.

      - The scheme here relies on gNB to perform DTX detection on the PUSCH. If gNB doesn’t have this capability or the DTX performance is not reliable, then {0, 3} is better.

    - ZTE:

      - If we make the change, then it should be applied to both UL (i.e. DCI format 0_2 and DL (i.e. DCI format 1_2)

  - Feature lead recommendation: It seems the issue raised by Qualcomm on PUSCH with repetition is a valid point. Therefore, it seems making the change or not both have some beneficial scenario. ZTE’s point on DL is valid also. Since no company provide the response to Qualcomm and ZTE, I would make some tentative proposal to take the change based on the majority view right now and see more views from companies on the issue raised by Qualcomm. Simultaneously, companies should also check if they are ok to apply it to DL also.

Proposal A: Change the candidate RV values from {0, 3} to {0, 2} in case of 1 bit for Redundancy version for DCI format 0_2.

Please provide your views on the above proposal. If you agree with the proposal A here, please also indicate if you are ok to apply similar change to DCI format 1_2.

| Company | View |
| --- | --- |

- Issue A-5: Completion of DMRS and PTRS reception procedure for PDSCH scheduled by DCI formats 1_2 (Sec. 7.3.1.2.3 in 38.212, Sec. 5.1.6.2 & 5.1.6.3 in 38.214)

Revised Proposal 3-3: Adopt the following text proposals with changes marked in red for PTRS reception procedures with DCI format 1_2:

#### 5.1.6.2 DM-RS reception procedure ## <Unchanged text is omitted> If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB DMRS-DownlinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH: - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and - PT-RS is transmitted to the UE. ## <Unchanged text is omitted> #### 5.1.6.3 PT-RS reception procedure The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and to a UE receiving PDSCH scheduled by DCI format 1_0 or format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB. A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support. ## <Unchanged text is omitted>

- 7.3.1.2.3 Format 1_2 ## <Unchanged text is omitted> - Antenna port(s) – 0, 4, 5, or 6 bits - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 is not configured; - Otherwise 4, 5 or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports p0,…,pv-1 shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2 and is configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat1_2 but configured with one or more of dmrs-DownlinkForPDSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCIFormat1_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.2.2-1/2/3/4. ## <Unchanged text is omitted>

Summary of the status for issue A-5 (i.e. proposal 3-3)

- Support: CATT, Nokia, Samsung, Spreadtrum, Sharp, DOCOMO, ZTE, Vivo, Intel, Huawei/HiSilicon, Ericsson, Apple

- May have concern: Qualcomm

  - What’s the default behavior when the two parameters are not configured? Is there any spec update (e.g., in 38.214) needed to capture the default behavior for PDSCH scheduled by DCI format 1_2?

- Feature lead recommendation: Take the proposal 3-3. As to the question from Qualcomm, as shown in the TP, when both parameters are not configured, the UE will use bit field index value 0 of the corresponding table based on the default values of the DMRS related parameter (e.g. dmrs-Type and maxLength) defined in TSS 38.331.  No spec updated needed in TS 38.214.

- Issue A-12: Correction on bandwidth part operation

Proposal 3-5: Adopt the following text proposal to capture BWP switching with DCI format 0_2 and DCI format 1_2 in section 12 in TS 38.213.

- 12 Bandwidth part operation *** Unchanged text is omitted *** If a bandwidth part indicator field is configured in DCI format 1_1 or DCI format 1_2, the bandwidth part indicator field value indicates the active DL BWP, from the configured DL BWP set, for DL receptions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1 or DCI format 0_2, the bandwidth part indicator field value indicates the active UL BWP, from the configured UL BWP set, for UL transmissions as described in [5, TS 38.212]. If a bandwidth part indicator field is configured in DCI format 0_1/0_2 or DCI format 1_1/1_2 and indicates an UL BWP or a DL BWP different from the active UL BWP or DL BWP, respectively, the UE shall - for each information field in the received DCI format 0_1 or DCI format 1_1 or DCI format 0_2 or DCI format 1_2 - if the size of the information field is smaller than the one required for the DCI format 0_1/0_2 or DCI format 1_1/1_2 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE prepends zeros to the information field until its size is the one required for the interpretation of the information field for the UL BWP or DL BWP prior to interpreting the DCI format 0_1/0_2 or DCI format 1_1/1_2 information fields, respectively - if the size of the information field is larger than the one required for the DCI format 0_1/0_2 or DCI format 1_1/1_2 interpretation for the UL BWP or DL BWP that is indicated by the bandwidth part indicator, respectively, the UE uses a number of least significant bits of DCI format 0_1/0_2 or DCI format 1_1/1_2 equal to the one required for the UL BWP or DL BWP indicated by bandwidth part indicator prior to interpreting the DCI format 0_1/0_2 or DCI format 1_1/1_2 information fields, respectively - set the active UL BWP or DL BWP to the UL BWP or DL BWP indicated by the bandwidth part indicator in the DCI format 0_1 or DCI format 1_1 or DCI format 0_2 or DCI format 1_2, respectively A UE does not expect to detect a DCI format 1_1/1_2 or a DCI format 0_1/0_2 indicating respectively an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change [10, TS 38.133]. If a UE detects a DCI format 1_1 or a DCI format 1_2 indicating an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format 1_1 or a DCI format 1_2 in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format 1_1 or a DCI format 1_2. If a UE detects a DCI format 0_1 or a DCI format 0_2 indicating an active UL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format 0_1 or a DCI format 0_2 in the scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format 0_1 or a DCI format 0_2. A UE does not expect to detect a DCI format 1_1 or a DCI format 1_2 indicating an active DL BWP change or a DCI format 0_1 or a DCI format 0_2 indicating an active UL BWP change for a scheduled cell within FR1 (or FR2) in a slot other than the first slot of a set of slots for the DL SCS of the scheduling cell that overlaps with a time duration where the UE is not required to receive or transmit for an active BWP change in a different cell from the scheduled cell within FR1 (or FR2). A UE expects to detect a DCI format 0_1 or a DCI format 0_2 indicating active UL BWP change, or a DCI format 1_1 or a DCI format 1_2 indicating active DL BWP change, only if a corresponding PDCCH is received within the first 3 symbols of a slot. *** Unchanged text is omitted ***

Summary of the status for issue A-12 (i.e. proposal 3-5)

- Support: CATT, Nokia, Panasonic, Samsung, Spreadtrum, WILUS, Sharp, DOCOMO, ZTE, Vivo, Intel, Huawei/HiSilicon, Qualcomm, Ericsson, Apple

- Feature lead recommendation: Take proposal 3-5 which is stable.

## Issue A-13: Type2 HARQ-ACK codebook construction related to DAI bit width

Proposal 3-6: If the size of counter DAI between DCI format 1_2 and 1_0/1_1 is different, a UE is not expected to multiplex HARQ-ACK for PDSCH scheduled by DCI format 1_2 and HARQ-ACK for PDCSH scheduled by DCI format 1_0/1_1 in the same HARQ-ACK codebook.

- UE does not expect to multiplex HARQ-ACK information scheduled by a DCI format without counter DAI field in a same HARQ-ACK codebook

Please comment if you still cannot accept the above proposal 3-6.

| Company | View |
| --- | --- |

Summary of the status for issue A-13 for proposal 3-6

  - Support: CATT, Panasonic, Samsung, Spreadtrum, Sharp, DOCOMO, ZTE, Vivo, Huawei/HiSilicon, Ericsson, Qualcomm

  - Not support: Nokia, WILUS, Intel

    - Nokia: When DCI 1_2 is used to schedule both priorities, then the DAI field needs to go to the larger one, thus no bit reduction. Or if saving bit is target, option 5 is better.

    - WILUS: Additional UE complexity is marginal by option 5

    - Intel: Option 5 that resembles a truncation-based approach is the most reasonable compromise that provides the DCI format size flexibility without forcing gNB to align DAI bit-fields

  - Feature lead recommendation: Have to recommend to take the majority view. The issue was already discussed in RAN1#100b-e meeting under UCI agenda, but it seems still not much difference with the positions on the candidate options. Personally, the arguments from companies who don’t support the proposal looks reasonable to me, and it seems option 5.

Details of company positions for issue A-13

- Option 1: If the size of counter DAI between DCI format 1_2 and 1_0/1_1 is different, a UE is not expected to multiplex HARQ-ACK for PDSCH scheduled by DCI format 1_2 and HARQ-ACK for PDCSH scheduled by DCI format 1_0/1_1 in the same HARQ-ACK codebook.

  - UE does not expect to multiplex HARQ-ACK information scheduled by a DCI format without counter DAI field in a same HARQ-ACK codebook

  - Support: Vivo, Huawei, CATT, NEC, Spreadtrum, OPPO, DCM, LGE, Qualcomm, LGE, Samsung, ZTE

  - Reasons

    - Enable smaller DCI size for high priority scheduling.

    - gNB can avoid multiplexing PDSCH scheduled by different DCI formats with different DAI bit width to the same HARQ-ACK codebook

- Option 2: The size of DAI field is the maximum of the bit widths for the DCI formats.

  - Support: LGE

- Option 3: When a UE is configured with DCI format 1_0 and/or with DCI format 1_1 and DCI format 1_2 for scheduling same priority traffic, the bit-width of the DAI fields is same.

  - Support: ZTE, Panasonic, Samsung, Spreadtrum

  - Reasons

    - The overall flexibility of Option 1 is questionable since the reliance on gNB scheduling is via scheduling restrictions.

    - No need to complicate DAI calculation and tracking unnecessarily complicated. There is no incentive for the NW to configure different sizes of DAI field to gain 1 bit DCI saving at the cost of increased complexity. So, the reasonable configuration would be to have the same DAI field bit-sizes for the DCIs contributing to the same CB

- Option 4: Modify the procedure of generation of codebook. 2-bit DAI and 1-bit DAI is transformed to standard DAI value and the standard DAI value is used to determine HARQ-ACK bit location. For example, 2-bit DAI value {1, 2, 3, and 4} is transformed to {0, 1, 0, and 1} to align 1-bit DAI value. In general way, standard DAI value = mod (2-bit DAI value or 1-bit DAI value -1, 2) +1

  - Support: OPPO, Nokia, WILUS

  - Reasons

    - Unnecessarily restrictive for option 1, 3 and 5

- Option 5: For type-2 HARQ-ACK codebook, if the size of DAI field between DCI format 1_2 and 1_0/1_1 is different, and they can schedule HARQ transmission using the same PUCCH-Config then the minimum DAI bit width over DCI 1_1 and DCI 1_2 is to be taken (Option 4 in R1-2002784)

  - Support: MTK, WILUS, Nokia (2rd), Intel

  - Reasons

    - More flexibility than mandating that the same DAI bit width be configured as proposed by option 3

Proposal 3-7: If UE is configured to monitor DCI format 1_2/0_2, the HARQ-ACK codebook size for type-2 HARQ-ACK codebook is determined by OACK=TD⋅j+Vtemp2-1mod TD+1

Please comment if you still cannot accept the above proposal 3-7.

| Company | View |
| --- | --- |

Summary of the status for issue A-13 (i.e. proposal 3-7)

- Proposal 3-6:

  - Support: CATT, Nokia, WILUS, Sharp, ZTE, Vivo, Intel, Huawei/HiSilicon, Qualcomm

  - May have concern: Samsung, Ericsson, Apple

    - Samsung: May result to modulo of a negative number

    - Feature Lead: As WILUS and Sharp mentioned, since Vtemp2 = VDAIUL has one value among 1,2,3, or 4, Vtemp2 is non-negative

    - Ericsson&Apple: Prefer to remove 1-bit DAI in DCI format 1_2. No motivation for the network to configure different bit-widths for DAI fields in different DCI formats, e.g., 1-bit counter DAI in DCI format 1_2.

    - Feature Lead: We should try to avoid reverting the agreement we made. In addition, the total DCI size of DCI format 0_2 and DCI format 1_2 depends on many other fields also, not just this DAI fields.

  - Feature lead recommendation: Take the proposal 3-7.