3GPP TSG RAN WG1 #101		R1-2004770

e-Meeting, May 20th – June 5th, 2020

Source:	Moderator (OPPO)

Title:	Text Proposal for TS 38.211 in [101-e-NR-eMIMO-multiTRP-03]

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

- Introduction

Rel-16 enhancement on MIMO WID includes objectives of enhancing multi-TRP/Panel transmission with ideal and non-ideal backhaul. During the work of rel-16, designs for multiple-PDCCH based and single-PDCCH based multi-TRP/Panel transmission were discussed and specified. This document provides the discussion for Issue #b-10 in multi-TRP email thread 3:

- Issue #b-10 to correct Description on QCL of DMRS ports of M-TRP PDSCH in 38.211

- Text Proposal

Reason for changes:

The description on the QCL of DMRS in TS 38.211 can only apply to the UE when the PDSCH is associated with one TCI state, for instance, single-TRP transmission of rel-15. But the description does not apply to some multi-TRP transmission where the PDSCH can be indicated with two TCI states. With that description, the UE behavior could be wrong for receiving PDSCH of multi-TRP transmission.

Summary of changes:

Change the text description in TS 38.211 to specify that the QCL description only applies to the DMRS ports associated with one TCI state of PDSCH.

Specs/Sections impacted:

TS 38.211 V16.1.0 /7.4.1.1.2

Consequences if not approved:

The UE behavior on receiving DMRS is ambiguous.

The text proposal for TS 38.211 is:

##### 7.4.1.1.2 Mapping to physical resources <Unchanged parts are omitted> In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. Unless specified otherwise, the UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx (when applicable). UE may assume that DMRS ports associated with a TCI state as described in clause 5.1.6.2 of [6, TS 38.214] of a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain. The UE may assume that no DM-RS collides with the SS/PBCH block.

- Reference

- R1-2003397	On remaining issues on M-TRP	vivo

- R1-2003469	Maintenance of multi-TRP enhancements	ZTE

- R1-2003531	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

- R1-2003627	Discussion on remaining issues of multi-TRP/panel transmission	CATT

- R1-2003660	Remaining issues on multi-TRP transmission	MediaTek Inc.

- R1-2003742	Corrections to multi-TRP	Intel Corporation

- R1-2003819	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

- R1-2003881	On Rel.16 multi-TRP/panel transmission	Samsung

- R1-2003928	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2003954	Remaining issues on multi-TRP/panel transmission	CMCC

- R1-2003987	Discussion on remaining issues of multi-TRP operation	Spreadtrum Communications

- R1-2004047	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

- R1-2004229	Remaining issues for Multi-TRP enhancement	Apple

- R1-2004265	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

- R1-2004311	Remaining issues on multi-TRP transmission	NEC

- R1-2004395	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

- R1-2004432	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

- R1-2004463	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2004592	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

- R1-2004719 FL summary #2 for Multi-TRP/Panel Transmission Moderator (OPPO)