3GPP TSG RAN WG1 #101	   	 	  	                 		          R1-2004793

e-Meeting, May 25th – June 5th, 2020

Agenda item: 7.2.6.3.

Source: Moderator (LG Electronics)

Title: TP for capturing outcome of email thread [101-e-NR-eMIMO-MB1-01]

Document for: Discussion and Decision

- Analysis

| # Reasons for change | # The enabling condition for default PL RS/spatial relation was agreed as “at least for UEs supporting beam correspondence”. The related descriptions for PUCCH in TS38.213 and for SRS in TS38.214 are misaligned. In addition, a separated UE capability reporting has been defined with respect to the support of default PL RS/spatial relation. |
| --- | --- |
| # Summary of changes | # Delete the condition of UE reporting beamCorrespondenceWithoutUL-BeamSweeping from the enabling condition of the default PL RS/spatial relation for PUCCH |
| # Specs/Sections impacted | # TS 38.213, clause 9.2.2 |
| # Consequences if not approved | # Enabling conditions for default PL RS/spatial relation for PUCCH and SRS are misaligned. |

- Text Proposal

TP for clause 9.2.2 of TS 38.213

9.2.2 PUCCH Formats for UCI transmission *** Unchanged text is omitted *** If a UE - reports beamCorrespondenceWithoutUL-BeamSweeping, - is not provided pathlossReferenceRSs in PUCCH-PowerControl, - is provided enableDefaultBeamPlForPUCCH, and - is not provided PUCCH-SpatialRelationInfo, and - is not provided CORESETPoolIndex value of 1 for any CORESET, or is provided CORESETPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] a spatial setting for a PUCCH transmission from the UE is same as a spatial setting for PDCCH receptions by the UE in the CORESET with the lowest ID on the active DL BWP of the PCell.