3GPP TSG RAN WG1 #101			R1-2004813

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary on [101-e-LTE-UEFeatures-NBIoT-01]

Agenda Item:	6.2.5.2

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion/approval regarding UE features for additional enhancements for NB-IoT.

[101-e-LTE-UEFeatures-NBIoT-01] Email discussion/approval on remaining issues on UE features for NB-IoT (25th – 29th May) – (DCM, Hiroki)

- Discuss and decide any necessary update for the UE features list for NB-IoT based on identified issues/proposals in R1-2004379

- Note that all the LTE-MTC/NB-IoT alignment issues will be discussed in this email discussion

- Discussion on UE features for additional enhancements for NB-IoT

2.1	FG2-1

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-1 | UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | Rel-15 NWUS | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-1 with those for 1-1: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | It is noted that many of the same features exist in both LTE_eMTC5 and NB_IOTenh3 WI, therefore it is suggested to align the naming and the corresponding description of these FGs in the feature file. 1-1; Group WUS without group resource alternation; 1. Group WUS without group resource alternation. 1-2; Group WUS with group resource alternation; 1. Group WUS with group resource alternation. 2-1; UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation); - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation). 2-2; UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation). - Either add UE-group wake-up signal in 1-1/2 or remove this term and use Group WUS in 2-1/2. |
| --- | --- |

Based on above, following FL proposals are made.

FL proposal 1:

- FG name and components for FG1-1

  - Change “Group WUS” to “UE-group wake-up signal (Group WUS)”

- FG name and components for FG1-2

  - Change “Group WUS” to “UE-group wake-up signal (Group WUS)”

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. LTE_eMTC5 | 1-1 | UE-group wake-up signal (Group WUS) without group resource alternation | 1. UE-group wake-up signal (Group WUS) without group resource alternation | Rel-15 MWUS | Yes | N/A | The UE will be paged with Rel-15 MWUS or without MWUS instead of Group WUS. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-2 | UE-group wake-up signal (Group WUS) with group resource alternation | 1. UE-group wake-up signal (Group WUS) with group resource alternation | 1-1 | Yes | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 MWUS when Rel-15 MWUS is configured or no MWUS when Rel-15 MWUS is not configured. | Per UE | Yes | N/A |  | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change |
| Huawei, HiSilicon | We are fine with the change. |

2.2	FG2-2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-2 | UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | 2-1 | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured. | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-2 with those for 1-2: [3]

- Consequence if the feature is not supported by the UE

  - Add “and group resource alternation”: [2]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | 2-2; UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); 2-1; Yes; N/A; The network cannot wake-up a group of users with one wake-up signal and group resource alternation.; Per UE; FDD only; N/A; If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured.; Optional with capability signalling. |
| --- | --- |
| [3] | It is noted that many of the same features exist in both LTE_eMTC5 and NB_IOTenh3 WI, therefore it is suggested to align the naming and the corresponding description of these FGs in the feature file. 1-1; Group WUS without group resource alternation; 1. Group WUS without group resource alternation. 1-2; Group WUS with group resource alternation; 1. Group WUS with group resource alternation. 2-1; UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation); - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation). 2-2; UE-group WUS with a wake-up time before the first associated PO (with group resource alternation); - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation). Either add UE-group wake-up signal in 1-1/2 or remove this term and use Group WUS in 2-1/2. |

Based on above, following FL proposals are made.

FL proposal 2:

- Consequence if the feature is not supported by the UE for FG2-2

  - Add “with group resource alternation”

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-2 | UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | 2-1 | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal with group resource alternation | Per UE | FDD only | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured. | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change |
| Huawei, HiSilicon | We are fine with the change. |
| ZTE,Sanechips | Is it better to say " The network cannot wake-up a group of users with one wake-up signal and with group resource alternation"? The change is editorial nature anyway, but it is aligned with with how the term is used in the feature group and components fields. |
| Ericsson 2 | We are fine with ZTE’s suggestion. |
| Moderator (NTT DOCOMO) | The proposal is updated to apply ZTE’ suggestion |

2.3	FG2-3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-3 | Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) | - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) |  | Yes | N/A | UE cannot transmit without an UL grant | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-3 with those for 1-3: [3]

- Consequence if the feature is not supported by the UE

  - Change to “UL data transmission will use EDT or connected mode instead of PUR”: [2]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | 2-3; Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS); - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS); Yes; N/A; UL data transmission will use EDT or connected mode instead of PUR; Per UE; FDD only; N/A; RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list.; Optional with capability signalling. |
| --- | --- |
| [3] | 1-3; PUR for full-PRB in CEmodeA. 2-3; Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS). - Align the description for PUR. Either change the name of FG 1-3 to 'Transmission in preconfigured UL resources (PUR) for full-PRB in CEmodeA' , or change the name of FG 2-3 to ' PUR (with potential UE-specific cyclic shift for DMRS ) |

Based on above, following FL proposals are made.

FL proposal 3:

- FG name and components for FG1-3

  - Change “PUR” to “Transmission in preconfigured UL resources (PUR)”

    - Same change is also made for 1-4/1-5/1-6

|  | 1-3 | Transmission in preconfigured UL resources (PUR) for full-PRB in CEmodeA | 1. Transmission in preconfigured UL resources (PUR) for full-PRB in CEmodeA | CEmodeA | Yes | N/A | UL data transmission will use EDT or connected mode instead of PUR in CEmodeA. | Per UE | Yes | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change, but the same change should then be made for 1-4/1-5/1-6 as well. It’s probably not necessary to make it for the other PUR feature groups. |
| Moderator (NTT DOCOMO) | The proposal is updated to apply the same change to 1-4/1-5/1-6 according to above comment. |
| Huawei, HiSilicon | We are fine with the change. |
| Ericsson 2 | We are fine with the change. |

FL proposal 4:

- Consequence if the feature is not supported by the UE for FG2-3

  - Change to “UL data transmission will use EDT or connected mode instead of PUR”

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-3 | Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) | - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) |  | Yes | N/A | UL data transmission will use EDT or connected mode instead of PUR | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change |
| Huawei, HiSilicon | We are fine with the change. |

2.4	FG2-4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-4 | PUR with serving cell RSRP for TA validation | - PUR with serving cell RSRP for TA validation | 2-3 | Yes | N/A | PUR will not use serving cell RSRP for TA validation | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. TA validation mechanisms based on ‘Serving cell changes’, ‘TA timer for idle mode’ and ‘TA always valid’ are mandatory for PUR UEs | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-4 with those for 1-7: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-7; PUR serving cell RSRP TA validation. 2-4; PUR with serving cell RSRP for TA validation. - For 1-7 change to 'PUR with serving cell RSRP for TA validation' |
| --- | --- |

Based on above, following FL proposals are made.

FL proposal 5:

- FG name and components for FG1-7

  - Change to “PUR with serving cell RSRP for TA validation”

|  | 1-7 | PUR with serving cell RSRP TA validation | 1. PUR with serving cell RSRP for TA validation | 1-3 | Yes | N/A | PUR will not use serving cell RSRP for TA validation. | Per UE | Yes | N/A | TA validation mechanisms based on ‘Serving cell changes’ and ‘TA timer for idle mode’ (and ‘TA always valid’) are mandatory for PUR UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change |
| Huawei, HiSilicon | We are fine with the change. |

2.5	FG2-6

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-6 | Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-6 with those for 1-10: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-10; Multi-TB unicast for DL in CEmodeA. 2-6; Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission). - Change the name 'Multi-TB' to 'Multi-TB scheduling' in LTE_eMTC5 |
| --- | --- |

Based on above, following FL proposals are made.

FL proposal 6:

- FG name and components for FG1-10

  - Change to “Multi-TB scheduling for unicast in DL in CE mode A”

    - Same change is also made for FGs 1-11 through 1-22

|  | 1-10 | Multi-TB scheduling for unicast in DL in CEmodeA | 1. Multi-TB scheduling for unicast in DL in CEmodeA | CEmodeA | Yes | N/A | Each DCI will schedule a single TB instead of multiple TBs in DL in CEmodeA. | Per UE | Yes | N/A | Following legacy capabilities are reused to support combinations. o Rel-14 feature for 2984 bits max UL TBS in 1.4 MHz in CE mode A o Rel-14 feature for new numbers of repetitions for PUSCH in CE mode A o Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A o Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change, but the same change should then be made for at least some of FGs 1-11 through 1-22 as well. |
| Moderator (NTT DOCOMO) | The proposal is updated to apply the same change to 1-11~1-22 according to above comment. |
| Huawei, HiSilicon | We are fine with the change. |
| Ericsson 2 | We are fine with the change. |

2.6	FG2-12

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-12 | DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-12 with those for 1-23: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-23; Subframe level resource reservation for DL in CEmodeA. 2-12; DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers.. - Align the description for resource reservation. |
| --- | --- |

Based on above, following FL proposals are made.

FL proposal 7:

- FG name and components for FG1-23

  - Change component 1 to “DL time-domain resource reservation with subframe-level granularity in CE mode A”

  - Change component 2 to “DL frequency-domain resource reservation with RBG-level granularity in CE mode A”

  - Similar changes are applied to FGs 1-23a through 1-26a

|  | 1-23 | DL resource reservation with subframe-level granularity in CEmodeA | 1. DL resource reservation with subframe-level granularity in CEmodeA 2. RBG-level frequency-domain resource reservation in DL in CEmodeA | CEmodeA | Yes | N/A | Whole DL subframe(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change, but a similar change should then be made for the 2nd component in the FG, and in FGs 1-23a through 1-26a. At the same time, FGs 1-27/1-28 can be changed from “Subcarrier puncturing in DL” to “DL subcarrier puncturing” to align with other FGs. |
| Moderator (NTT DOCOMO) | The proposal is updated to apply similar change to 2nd component of FG1-23 and FGs 1-23a through 1-26a. In addition, new proposal 9 is added to apply the suggested change from Ericsson to FG1-27/1-28. |
| Huawei, HiSilicon | We are fine with the change. |
| Ericsson 2 | We are fine with the change. |

FL proposal 9:

- FG name and component for FG1-27 (1-28)

  - Change to “DL subcarrier puncturing in CE mode A (CE mode B)”

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We are fine with the change. |
| Ericsson 2 | We are fine with the change. |

2.7	FG2-14/15

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-14 | Quality report in Msg3 for non-anchor access in idle | - Quality report in Msg3 for non-anchor access in IDLE mode | multicarrier-NPRACH-r14 | No | N/A | The network cannot receive the quality report for non-anchor carriers in IDLE mode | Per-UE | FDD only | N/A |  | Optional without capability signalling |
|  | 2-15 | Quality report in connected mode | 1. Quality report in connected mode other than Msg3 for anchor and non-anchor carriers |  | Yes | N/A | The network cannot receive the quality report other than in Msg3 | Per-UE | FDD only | N/A |  | Optional with capability signalling |

- FG name and components

  - Align the naming and description for 2-14/15 with those for 1-29/30: [3]

Above remaining issue and proposal are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | 1-29; DL quality report in Msg3 in Idle. 1-30; DL quality report in Connected. 2-14; Quality report in Msg3 for non-anchor access. 2-15; Quality report in connected mode. - Align the description for quality report. |
| --- | --- |

Based on above, following FL proposals are made.

FL proposal 8:

- FG name and components for FG2-14

  - Change to “DL quality report in Msg3 for non-anchor access in Idle”

- FG name and components for FG2-15

  - Change to “DL quality report in Connected”

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-14 | DL quality report in Msg3 for non-anchor access in Idle | - DL quality report in Msg3 for non-anchor access in Idle | multicarrier-NPRACH-r14 | No | N/A | The network cannot receive the quality report for non-anchor carriers in IDLE mode | Per-UE | FDD only | N/A |  | Optional without capability signalling |
|  | 2-15 | DL quality report in Connected | 1. DL quality report in Connected other than Msg3 for anchor and non-anchor carriers |  | Yes | N/A | The network cannot receive the quality report other than in Msg3 | Per-UE | FDD only | N/A |  | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Qualcomm | OK with change |
| Ericsson | OK with change |
| Huawei, HiSilicon | We are fine with the change. |

- Conclusion

Agreement:

- FG name and components for FG1-1

  - Change “Group WUS” to “UE-group wake-up signal (Group WUS)”

- FG name and components for FG1-2

  - Change “Group WUS” to “UE-group wake-up signal (Group WUS)”

Agreement:

- Consequence if the feature is not supported by the UE for FG2-2

  - Add “and group resource alternation”

Agreement:

- FG name and components for FG1-3

  - Change “PUR” to “Transmission in preconfigured UL resources (PUR)”

    - Same change is also made for 1-4/1-5/1-6

Agreement:

- Consequence if the feature is not supported by the UE for FG2-3

  - Change to “UL data transmission will use EDT or connected mode instead of PUR”

Agreement:

- FG name and components for FG1-7

  - Change to “PUR with serving cell RSRP for TA validation”

Agreement:

- FG name and components for FG1-10

  - Change to “Multi-TB scheduling for unicast in DL in CE mode A”

    - Same change is also made for FGs 1-11 through 1-22

Agreement:

- FG name and components for FG1-23

  - Change component 1 to “DL time-domain resource reservation with subframe-level granularity in CE mode A”

  - Change component 2 to “DL frequency-domain resource reservation with RBG-level granularity in CE mode A”

  - Similar changes are applied to FGs 1-23a through 1-26a

Agreement:

- FG name and components for FG2-14

  - Change to “DL quality report in Msg3 for non-anchor access in Idle”

- FG name and components for FG2-15

  - Change to “DL quality report in Connected”

Agreement:

- FG name and component for FG1-27 (1-28)

  - Change to “DL subcarrier puncturing in CE mode A (CE mode B)”

# Reference

[1]	R1-2003196	Summary on email discussion [100b-e-LTE-UEFeatures-Remaining]	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2003788	UE features for NB-IoT	Qualcomm Incorporated

[3]	R1-2003800	Discussion on UE features for additional enhancements for NB-IoT	ZTE

# Appendix: latest version of UE features list for additional enhancements for NB-IoT [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. NB_IOTenh3 | 2-1 | UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | - UE-group wake-up signal (Group WUS) with a wake-up time before the first associated PO (without group resource alternation) | Rel-15 NWUS | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-2 | UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | - UE-group WUS with a wake-up time before the first associated PO (with group resource alternation) | 2-1 | Yes | N/A | The network cannot wake-up a group of users with one wake-up signal | Per UE | FDD only | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 NWUS when Rel-15 NWUS is configured or no NWUS when Rel-15 NWUS is not configured. | Optional with capability signalling |
|  | 2-3 | Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) | - Transmission in preconfigured UL resources (PUR) (with potential UE-specific cyclic shift for DMRS) |  | Yes | N/A | UE cannot transmit without an UL grant | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 2-4 | PUR with serving cell RSRP for TA validation | - PUR with serving cell RSRP for TA validation | 2-3 | Yes | N/A | PUR will not use serving cell RSRP for TA validation | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. TA validation mechanisms based on ‘Serving cell changes’, ‘TA timer for idle mode’ and ‘TA always valid’ are mandatory for PUR UEs | Optional with capability signalling |
|  | 2-5 | PUR with L1 ACK | - PUR with L1 ACK | 2-3 | Yes | N/A | PUR will not use L1 ACK | Per UE | FDD only | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 2-6 | Multi-TB scheduling for unicast in DL with a single DCI(Interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-7 | Multi-TB scheduling for unicast in DL with a single DCI (Non-interleaved transmission) | - Multi-TB scheduling for unicast in DL with a single DCI (Non-interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (non-interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-8 | Multi-TB scheduling for unicast in UL with a single DCI (Interleaved transmission) | - Multi-TB scheduling for unicast in UL with a single DCI (Interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in UL with a single DCI (interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-9 | Multi-TB scheduling for unicast in UL with a single DCI(Non-interleaved transmission) | - Multi-TB scheduling for unicast in UL with a single DCI(Non-interleaved transmission) | Two HARQ processes | Yes | N/A | The network cannot schedule transmission of multiple TBs in UL with a single DCI (non-interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-10 | Multi-TB scheduling for unicast in DL in a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | - Multi-TB scheduling for unicast in DL in a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | 2-6 | Yes | N/A | The network cannot schedule transmission of multiple TBs in DL with a single DCI (HARQ bundling for HARQ-ACK feedback to interleaved transmission) | Per UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-11 | Multi-TB scheduling for SC-MTCH | 1. Scheduling of multiple transport blocks for SC-MTCH in a single DCI 2. Scheduling of multiple transport blocks for SC-MTCH in a single DCI with scheduling gaps | SC-PTM | No | N/A | The network cannot schedule transmission of multiple TBs with a single DCI | Per UE | FDD only | N/A | The basic multicast (SC-PTM) functionality was introduced for LTE-M/NB-IoT in Rel-14 without capability signaling. | Up to RAN2 |
|  | 2-12 | DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-12a | DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | - DL resource reservation with slot-level and symbol-level granularity of NB-IoT non-anchor carriers. | 2-12 | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-13 | UL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. | - UL resource reservation with subframe-level granularity of NB-IoT non-anchor carriers. |  | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-13a | UL resource reservation with slot-level and symbol(s)-level granularity of NB-IoT non-anchor carriers. | - UL resource reservation with slot-level and symbol(s)-level granularity of NB-IoT non-anchor carriers. | 2-13 | Yes | N/A | NB-IoT transmission may collide with NR transmission | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 2-14 | Quality report in Msg3 for non-anchor access in idle | - Quality report in Msg3 for non-anchor access in IDLE mode | multicarrier-NPRACH-r14 | No | N/A | The network cannot receive the quality report for non-anchor carriers in IDLE mode | Per-UE | FDD only | N/A |  | Optional without capability signalling |
|  | 2-15 | Quality report in connected mode | 1. Quality report in connected mode other than Msg3 for anchor and non-anchor carriers |  | Yes | N/A | The network cannot receive the quality report other than in Msg3 | Per-UE | FDD only | N/A |  | Optional with capability signalling |
|  | 2-16 | NRS on a non-anchor carrier for paging | 1. Presence of NRS on a set of subframes on a non-anchor carrier when no paging NPDCCH is transmitted | Paging non-anchor carrier | No | N/A | UE does not know whether NRS is transmitted on a non-anchor carrier when paging is not present | Per UE | FDD only | N/A |  | Optional without capability signalling |