3GPP TSG RAN WG1 #101-e			R1-2004828

e-Meeting, May 25th – June 5th, 2020

Source:	NTT DOCOMO, INC.

Title:	Summary on [101-e-NR-UEFeatures-Others-01]

Agenda Item:	7.2.11.13

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the following email discussion/approval regarding UE features that are not dedicated to a specific Rel-16 WI/TEI.

[101-e-NR-UEFeatures-Others-01] Email discussion/approval on potential new FGs that are not dedicated to a specific Rel-16 work item/TEI (25th – 27th May) – (DCM, Hiroki)

- Discuss and decide whether or not to introduce any new FG(s) (or which WI will handle the new FG) based on identified issues/proposals in R1-2004415

- Discuss and decide whether/how to introduce a new FG for uplink Tx switching according to the agreements made at RAN1#100bis-e

- Discuss and decide capability signaling design for FG(s) decided to be added in this email discussion (if any)

- Proposals for potential new FGs or update of existing FGs

2.1	a new FG for UL Tx switching

At the RAN1#100bis-e meeting, following agreements were made.

Agreements: For inter-band UL CA, if UE reports via capability signaling to support uplink Tx switching, UE further reports via capability signaling which option (between Option 1 and Option 2) is supported. Option 1: If uplink Tx switching is configured, UE is not expected to be scheduled or configured with UL transmission on carrier 2 for case 1. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 1; 1T+1T; 1P+0P. Case 2; 0T+2T; 0P+2P, 0P+1P. Option 2: If uplink Tx switching is configured, UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 for case 1. o UE can be scheduled or configured with UL transmission on either carrier 1 or carrier 2. o UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 simultaneously. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 1; 1T+1T; 1P+0P, 1P+1P, 0P+1P. Case 2; 0T+2T; 0P+2P, 0P+1P.

Based on above, following FL proposals are made.

FL proposal 1:

- A new FG for indicating supported option for UL Tx switching for inter-band UL CA is added in the UE features list

  - Candidate values set is {option1, option2} (depending on [101-e-LS-TxSwitching-01])

  - Type of the FG is “Per BC”

  - The FG is “Optional with capability signaling”

| 22. NR Others | 22-1 | Indicating supported option for UL Tx switching for inter-band UL CA | - Indicating supported option for UL Tx switching for inter-band UL CA |  | Yes | N/A |  | Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Moderator (NTT DOCOMO) | Whether UE can report support of both option 1 and option 2 will be discussed in [101-e-LS-TxSwitching-01], and then the outcome of the discussion can be reflected here. |
| ZTE | The prerequisite feature group should be 6-6. Should add candidate values of {Option1, Option2, [Option1 and Option2]}. Whether it is “Optional with capability signalling” should be discussed and decided together with other FGs for UL Tx switching since this FG should be required for the UEs supporting UL Tx switching for CA. |
| Ericsson | Agree that 6-6 is prerequisite. There needs to be a signaling option that indicates no support. |
| Nokia, NSB | We agree with the moderator proposal and that 6-6 should be pre-requisite. |

Based on the discussion in Tuesday GTW session, following agreements were made.

Agreements

- A new FG for indicating supported option for UL Tx switching for inter-band UL CA is added in the UE features list

  - Candidate values set is {option1, option2, [both option 1 and option 2]} (depending on [101-e-LS-TxSwitching-01])

  - FFS: Type of the FG is “Per UE” or “Per BC”

  - FFS: The FG is “Optional with capability signaling”

  - 6-6 and [RAN4 cap] are prerequisite feature groups for the new FG

  - Further discussion on FFS parts – Ruyue (ZTE)

Updated FL proposal 1:

Companies are encouraged to discuss further on the FFS parts of above agreements.

-Type

Support “Per UE”:

Support “Per BC”:

-Relationship with [RAN4 cap]

| Company | Comment |
| --- | --- |
| Qualcomm | There needs to be a way to signal no support but that may be done with the RAN4 defined capability. Ok to add 6-6 as prerequisite. Reporting type “per BC”. |

Based on the discussion in Friday GTW session, agreements were updated as below.

Agreements

- A new FG for indicating supported option for UL Tx switching for inter-band UL CA is added in the UE features list

  - Candidate values set is {option1, option2, [both option 1 and option 2]}

  - Type of the FG is “Per BC”

  - This FG is "Conditional mandatory with capability signalling".  Signaling of this FG is mandatory conditioned on the support of switching time capability for Tx switching between two uplink carriers in inter-band UL CA band combinations in RAN4 FG 7-1 (i.e. Tx switching period between two uplink carriers).

  - 6-6 and RAN4 FG 7-1 are prerequisite feature groups for the new FG

  - Note “[it is up to RAN2 how to report support of both option 1 and option 2]”

  - Add "FR1 only" in the column of FR1/FR2 differentiation

Based on the discussion in GTW session, following additional agreements were made.

Agreements:

- A new FG 22-2 for indicating supported option for UL Tx switching for EN-DC is added in the UE features list

  - Candidate values set is {option1, option2, [both option 1 and option 2]}

  - Type of the FG is “Per BC”

  - This FG is "Conditionally mandatory with capability signalling".  Signaling of this FG is mandatory conditioned on the support of switching time capability for Tx switching between two uplink carriers in EN-DC in RAN4 FG 7-1 (i.e. Tx switching period between two uplink carriers).

  - EN-DC and RAN4 FG 7-1 are prerequisite feature groups for the new FG

  - Add "FR1 only" in the column of FR1/FR2 differentiation

- Add a note depending on the outcome from [101-e-LS-TxSwitching-01] to both FG 22-1 and FG 22-2 - e.g. ["It has been agreed in RAN1 that UE can report support of one of the three candidates {option1, option2, both option1 and option2}.  It is up to RAN2 to design the corresponding UE capability signalling."]

2.2	new FGs for up to 3 unicast PDSCHs/PUSCHs per slot per CC for different TBs

- Necessity of new FGs for up to 3 unicast PDSCHs/PUSCHs per slot per CC for different TBs

  - FGs are necessary: [5]

    - This should take msgB into account: [7]

  - FGs are not necessary: [3]

Above discussion points and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | During the email discussion, the following yellow highlighted parts were proposed. Index; Feature group; Components; Prerequisite feature groups. 8-1; Dynamic power sharing for LTE-NR DC; When total transmission power exceeds Pcmax, UE scales NR transmission power.; EN-DC. [5-11c]; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included. [5-12c]; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1. [5-13g]; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5a or 5-5b. [5-13h]; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5c. [5-35]; Simultaneously enable CBG and multiple PDSCHs per slot; Simultaneously enable CBG and multiple PDSCHs per slot; 5-11,5-11a, 5-11b, 5-13. 5-13a. 5-13c, 5-22, 5-23, 5-24. For FGs 5-11c/5-12c/5-13g/5-13h above, there are already capabilities for UE to receive up to 2 or 4 PDSCH/PUSCH, respectively, while the proposed FGs are for UE to receive up to 3 PDSCH/PUSCH. This would bring UE fragmentation with no clear benefits. Also for FG 5-35, this seems a signaling to indicate less capability than Rel-15. It is also not clear to have this signaling. Observation 1: There is no clear benefit to introduce new FGs 5-11c/5-12c/5-13g/5-13h, and 5-35. Processing time capability#2 In rel-15, declaration of support of cap#2 is per-FS, which means that a UE declare the support of cap#2 for all CC’s in certain band of certain band combination. However, some rel-16 features may be difficult to be supported simultaneously in the same CC’s together with cap#2 due to UE implementation complexity. One example of such rel-16 feature is multi DCI-based multi-TRP operation with out of order operation and/or overlapping PDSCH’s. Another example is span-based PDCCH monitoring. Under the current UE capability signaling, a UE may need to avoid simultaneously signaling support of cap#2 and those rel-16 features for certain band in the band combination. This means that none of CC’s in that band will support that feature, and such under-reporting is not desirable. A straightforward way to improve this situation is to define FSPC cap#2 support signaling for rel-16 UE’s. In this case, the existing rel-15 signaling may be ignored by network when rel-16 signaling is utilized. Another possible solution is to define new capability signaling to indicate support of simultaneous cap#2 and certain rel-16 features in the same CC, and this may need to be done case-by-case manner for each rel-16 feature. Proposal 3: Following 2 options can be considered. Option 1: define FSPC cap#2 support signaling for rel-16 UE’s, option 2: define new capability signaling to indicate support of simultaneous cap#2 and certain rel-16 features in the same CC in case-by-case manner. In rel-15, cap#2 processing time is only supported for self-scheduling case, but support of cap#2 with cross-carrier scheduling (CCS) is being considered in rel-16. Under the current signaling, it is possible that CCS can be configured between cap#1 and cap#2 scheduling cell and scheduled cell. It would be natural to think that at least scheduled cell should be cap#2 to apply cap#2 processing time, but scheduling cell may also need to be cap#2 since PDCCH decoding on cap#1 could take longer time. To handle this, several possible solutions can be considered. Proposal 4: Following 2 options can be considered. - Option 1: Capability#2 processing time is applied for CCS only if both scheduling and scheduled cell are configured with cap#2. Otherwise, cap#1 processing time is applied, - Option 2: Define UE capability to support cap#2 processing time for CCS even if only one of scheduling and scheduled cell is configured with cap#2. |
| --- | --- |
| [5] | Furthermore, we propose new FGs for Rel. 15 NR. The proposed changes in 5-11c, 5-12c, 5-13d and 5-13g are to allow for scheduling 3 TBs in both UL and DL. Considering the (4,3) span pattern of FG 3-5b, these additions would allow for matching the number of TBs and the spans in each slot. In addition, the proposed FG 11-3a-e would allow for capability signalling for the simultaneous use of CBG-based UL transmission and minimum processing capability 2. Proposal 4: Add Rel-15 FGs to allow for scheduling 3 TBs in both UL and DL in order to match the number of control spans per slot Proposal 5: Add Rel-15 FGs for the simultaneous use of CBG-based UL transmission and UE processing time capability 2. 5-11c; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 PDSCH(s) for Msg. 4 is included; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS; Optional with capability signalling. 5-12c; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS; Optional with capability signalling. 5-13d; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5a or 5-5b; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination; Optional with capability signalling. 5-13g; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5c; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination; Optional with capability signalling. |
| [7] | The maximum data rate which UE supports is determined by the supported maximum data rate defined in subclause 4.1.2 of TS 38.306 [1], at both physical layer and high layer. At the physical layer, the statements in subclause 5.1.3 of TS 38.214 [2] determine the maximum amount of data required UE to be handled per cell group or per cell respectively. Then, the successfully decoded TB will be delivered to high layer for further processing. It is common understanding that high layer capabilities pipeline, including buffer size, processing capability, bus capacity, etc., are also designed based on the maximum supported data rate defined in subclause 4.1.2 of TS 38.306 [1]. However, when CBG based retransmission is scheduled, the amount of data delivered to high layer may exceed the maximum data rate calculated as above. This issue was discussed in [3] at RAN1 #97. Given the potential NBC change, it was understood that for Rel-15 when the scenarios described in [3] happens, it is up to UE implementation, for example, UE may have to drop correctly decoded TB(s) during peak data rate reception and the performance will be degraded accordingly. The following was concluded: Conclusion: - The scenario described in the draft CR (R1-1907505) can happen but can be handled via implementation in which case the UE performance may not be optimal The conclusion was made with compromise of peak rate not reaching the NR design targeted. However, it is worth noting that configuration of CBG operation is actually one of the scheme introduced for higher data rate traffic. Thus the described case is not a corner case, rather can be a typical scenario that data rate is the KPI. As one example assuming two CBGs are configured as in Figure 1 below, in slot n, TB0 is received at UE side with decoding failure for CBG1. In slot n+3, CBG1 of TB0 is retransmitted and a new TB is also scheduled by a separate DCI, TB1 in the same slot. At physical layer, both CBG1 of TB0 and TB1 are correctly decoded and sent to high layer. That means, UE needs to handle the amount of data of TB0 and TB1 at corresponding processing time to physical layer slot n+3. Assume UE is under peak data rate required service, TB0 and TB1 are scheduled with full channel bandwidth, highest code rate, highest number of layer, and highest modulation order, in order to not block the pipeline, UE needs to increase its high layer memory and processing capability 1.5 times than those of non-CBG based retransmission. In the worst case if the number of CBGs is 8, then UE needs to increase its capability 7 times than CBG is disabled if all unsuccessfully decoded CBGs are retransmitted in one slot assuming the maximum number of DL TB per slot is 7. On the other hand, if peak rate performance is sacrificed as concluded in RAN1, significant data rate degrade (more than 10%) can be expected due to e.g. dropping one or more successfully decoded TBs, which further leads to more potential retransmissions consequently and is highly undesirable. Figure 1. One CBG based retransmission example In order to maintain the NR designated peak rate performance, it is desired to solve this problem in Rel-16 and beyond. Several approaches can be considered: - Option 1: Change C' to C in Vj,mfor DataRate calculation and DataRateCC calculation in subcluase 5.1.3 and 6.1.4 of TS 38.214 as described in R1-1907505. However, this requires specification changes and needs to consider impact on Rel-15 implementations. - Option 2: When CBG based retransmission is enabled, only one unicast PDSCH is scheduled per slot. However, this restricts the network scheduling and applicable scenarios for CBG based operation especially for traffic heavy/data rate oriented cases. - Option 3: UE reports newtork that whether UE supports more than one unicast PDSCH reception per slot on this cell, when CBG based retransmission is configured for a cell. This has benefits of no specification impact and actually relying on UE implementation evolution with sufficient flexibility, if the device has the capability to work under the enhanced operation. Proposal 1: Agree on FG 5-35 as an optional per-UE reported capability for Rel-16 to indicate whether UE supports more than one unicast PDSCH reception with UE processing capability 1 per slot on a carrier when CBG based (re)transmission is configured for that carrier. ~ Since msgB is introduced from 2-step RACH WI in response to successfully decoding of msgA, which can similar to msg4 when carrying successRAR, further discussion would be needed for relevant UE capabilities in DL. This may be either handled in specific 2-step RACH WI, or as enhanced UE capabilities of DL of FG 5-11 ~ 5-13h. Proposal 2: UE capabilities for multiple PDSCHs reception should take msgB into account. |

Based on above, following FL proposals are made.

FL proposal 2:

- New FGs for up to 3 unicast PDSCHs/PUSCHs per slot per CC for different TBs are introduced

  - 5-11c: Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1

  - 5-12c: Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1

  - 5-13d: Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2

  - 5-13g: Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2

| 5-11c | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 PDSCH(s) for Msg. 4 is included |  | Yes | N/A |  | Per FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-12c | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  | Yes | N/A |  | Per FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 5-13d | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a or 5-5b | Yes | N/A |  | Per FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |
| 5-13g | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c | Yes | N/A |  | Per FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| ZTE | Given that “up to 2” and “up to 4” PDSCHs/PUSCHs have already been defined in Rel-15, the use case of “up to 3” PDSCHs/PUSCHs may be marginal from our perspective. We slightly prefer not to introduce these UE capabilities, but we are also ok with the FL proposal if majority companies prefer to have these UE capabilities. |
| Ericsson | As indicated in the prep discussion, this was already discussed in RAN1#100bis-e and outcome is likely to be the same this time. |
| Huawei, HiSi | Clarify that for 5-11c as well as other number of PDSCH with processing capability 1, also include msgB PDSCH. |
| Nokia, NSB | We agree with the observation that this has been discussed already in the previous meeting and our views have not changed in the meantime. |
| Qualcomm | The motivation for the proposal is to correct the misalignment between the PDSCH/PUSCH capability and the PDCCH FG3-5b capability. Haven’t received feedback on how to mitigate the misalignment without the new FG. |
| Moderator (NTT DOCOMO) | Based on the discussion in GTW sessions and the current situation, my suggestion as moderator is to focus on FL proposal 3/3b and to give up on FL proposal 2/3a. |
| Huawei, HiSi02 | We further checked the R15 capability and think proposal 2 is not necessary. It does not relates to DCI span as more than 1 DCI can be used for scheduling TBs. There is 5-32 ensuring no more than 3 TBs already. |
| Ericsson | This is fine |

2.3	new FG(s) for simultaneous use of CBG-based transmission and multiple PDSCHs/PUSCHs per slot per CC for different TBs

- Necessity of new FG(s) for simultaneous use of CBG-based transmission and multiple PDSCHs/PUSCHs per slot per CC for different TBs

  - FG for simultaneous use of CBG and multiple PDSCHs is necessary: [7]

  - FGs for simultaneous use of CBG based UL and UE processing time capability 2 are necessary: [5]

  - FGs are not necessary: [3]

Above discussion points and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | During the email discussion, the following yellow highlighted parts were proposed. Index; Feature group; Components; Prerequisite feature groups. 8-1; Dynamic power sharing for LTE-NR DC; When total transmission power exceeds Pcmax, UE scales NR transmission power.; EN-DC. [5-11c]; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included. [5-12c]; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1. [5-13g]; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5a or 5-5b. [5-13h]; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5c. [5-35]; Simultaneously enable CBG and multiple PDSCHs per slot; Simultaneously enable CBG and multiple PDSCHs per slot; 5-11,5-11a, 5-11b, 5-13. 5-13a. 5-13c, 5-22, 5-23, 5-24. For FGs 5-11c/5-12c/5-13g/5-13h above, there are already capabilities for UE to receive up to 2 or 4 PDSCH/PUSCH, respectively, while the proposed FGs are for UE to receive up to 3 PDSCH/PUSCH. This would bring UE fragmentation with no clear benefits. Also for FG 5-35, this seems a signaling to indicate less capability than Rel-15. It is also not clear to have this signaling. Observation 1: There is no clear benefit to introduce new FGs 5-11c/5-12c/5-13g/5-13h, and 5-35. Processing time capability#2 In rel-15, declaration of support of cap#2 is per-FS, which means that a UE declare the support of cap#2 for all CC’s in certain band of certain band combination. However, some rel-16 features may be difficult to be supported simultaneously in the same CC’s together with cap#2 due to UE implementation complexity. One example of such rel-16 feature is multi DCI-based multi-TRP operation with out of order operation and/or overlapping PDSCH’s. Another example is span-based PDCCH monitoring. Under the current UE capability signaling, a UE may need to avoid simultaneously signaling support of cap#2 and those rel-16 features for certain band in the band combination. This means that none of CC’s in that band will support that feature, and such under-reporting is not desirable. A straightforward way to improve this situation is to define FSPC cap#2 support signaling for rel-16 UE’s. In this case, the existing rel-15 signaling may be ignored by network when rel-16 signaling is utilized. Another possible solution is to define new capability signaling to indicate support of simultaneous cap#2 and certain rel-16 features in the same CC, and this may need to be done case-by-case manner for each rel-16 feature. Proposal 3: Following 2 options can be considered. Option 1: define FSPC cap#2 support signaling for rel-16 UE’s, option 2: define new capability signaling to indicate support of simultaneous cap#2 and certain rel-16 features in the same CC in case-by-case manner. In rel-15, cap#2 processing time is only supported for self-scheduling case, but support of cap#2 with cross-carrier scheduling (CCS) is being considered in rel-16. Under the current signaling, it is possible that CCS can be configured between cap#1 and cap#2 scheduling cell and scheduled cell. It would be natural to think that at least scheduled cell should be cap#2 to apply cap#2 processing time, but scheduling cell may also need to be cap#2 since PDCCH decoding on cap#1 could take longer time. To handle this, several possible solutions can be considered. Proposal 4: Following 2 options can be considered. - Option 1: Capability#2 processing time is applied for CCS only if both scheduling and scheduled cell are configured with cap#2. Otherwise, cap#1 processing time is applied, - Option 2: Define UE capability to support cap#2 processing time for CCS even if only one of scheduling and scheduled cell is configured with cap#2. |
| --- | --- |
| [5] | Furthermore, we propose new FGs for Rel. 15 NR. The proposed changes in 5-11c, 5-12c, 5-13d and 5-13g are to allow for scheduling 3 TBs in both UL and DL. Considering the (4,3) span pattern of FG 3-5b, these additions would allow for matching the number of TBs and the spans in each slot. In addition, the proposed FG 11-3a-e would allow for capability signalling for the simultaneous use of CBG-based UL transmission and minimum processing capability 2. Proposal 4: Add Rel-15 FGs to allow for scheduling 3 TBs in both UL and DL in order to match the number of control spans per slot Proposal 5: Add Rel-15 FGs for the simultaneous use of CBG-based UL transmission and UE processing time capability 2. 11-3a; CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-5c; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3b; CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3c; CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13a; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3d; CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13c; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3e; CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13d; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. |
| [7] | The maximum data rate which UE supports is determined by the supported maximum data rate defined in subclause 4.1.2 of TS 38.306 [1], at both physical layer and high layer. At the physical layer, the statements in subclause 5.1.3 of TS 38.214 [2] determine the maximum amount of data required UE to be handled per cell group or per cell respectively. Then, the successfully decoded TB will be delivered to high layer for further processing. It is common understanding that high layer capabilities pipeline, including buffer size, processing capability, bus capacity, etc., are also designed based on the maximum supported data rate defined in subclause 4.1.2 of TS 38.306 [1]. However, when CBG based retransmission is scheduled, the amount of data delivered to high layer may exceed the maximum data rate calculated as above. This issue was discussed in [3] at RAN1 #97. Given the potential NBC change, it was understood that for Rel-15 when the scenarios described in [3] happens, it is up to UE implementation, for example, UE may have to drop correctly decoded TB(s) during peak data rate reception and the performance will be degraded accordingly. The following was concluded: Conclusion: - The scenario described in the draft CR (R1-1907505) can happen but can be handled via implementation in which case the UE performance may not be optimal The conclusion was made with compromise of peak rate not reaching the NR design targeted. However, it is worth noting that configuration of CBG operation is actually one of the scheme introduced for higher data rate traffic. Thus the described case is not a corner case, rather can be a typical scenario that data rate is the KPI. As one example assuming two CBGs are configured as in Figure 1 below, in slot n, TB0 is received at UE side with decoding failure for CBG1. In slot n+3, CBG1 of TB0 is retransmitted and a new TB is also scheduled by a separate DCI, TB1 in the same slot. At physical layer, both CBG1 of TB0 and TB1 are correctly decoded and sent to high layer. That means, UE needs to handle the amount of data of TB0 and TB1 at corresponding processing time to physical layer slot n+3. Assume UE is under peak data rate required service, TB0 and TB1 are scheduled with full channel bandwidth, highest code rate, highest number of layer, and highest modulation order, in order to not block the pipeline, UE needs to increase its high layer memory and processing capability 1.5 times than those of non-CBG based retransmission. In the worst case if the number of CBGs is 8, then UE needs to increase its capability 7 times than CBG is disabled if all unsuccessfully decoded CBGs are retransmitted in one slot assuming the maximum number of DL TB per slot is 7. On the other hand, if peak rate performance is sacrificed as concluded in RAN1, significant data rate degrade (more than 10%) can be expected due to e.g. dropping one or more successfully decoded TBs, which further leads to more potential retransmissions consequently and is highly undesirable. Figure 1. One CBG based retransmission example In order to maintain the NR designated peak rate performance, it is desired to solve this problem in Rel-16 and beyond. Several approaches can be considered: - Option 1: Change C' to C in Vj,mfor DataRate calculation and DataRateCC calculation in subcluase 5.1.3 and 6.1.4 of TS 38.214 as described in R1-1907505. However, this requires specification changes and needs to consider impact on Rel-15 implementations. - Option 2: When CBG based retransmission is enabled, only one unicast PDSCH is scheduled per slot. However, this restricts the network scheduling and applicable scenarios for CBG based operation especially for traffic heavy/data rate oriented cases. - Option 3: UE reports newtork that whether UE supports more than one unicast PDSCH reception per slot on this cell, when CBG based retransmission is configured for a cell. This has benefits of no specification impact and actually relying on UE implementation evolution with sufficient flexibility, if the device has the capability to work under the enhanced operation. Proposal 1: Agree on FG 5-35 as an optional per-UE reported capability for Rel-16 to indicate whether UE supports more than one unicast PDSCH reception with UE processing capability 1 per slot on a carrier when CBG based (re)transmission is configured for that carrier. ~ Since msgB is introduced from 2-step RACH WI in response to successfully decoding of msgA, which can similar to msg4 when carrying successRAR, further discussion would be needed for relevant UE capabilities in DL. This may be either handled in specific 2-step RACH WI, or as enhanced UE capabilities of DL of FG 5-11 ~ 5-13h. Proposal 2: UE capabilities for multiple PDSCHs reception should take msgB into account. |

Based on above, following FL proposals are made.

FL proposal 3:

- New FGs for simultaneous use of CBG-based transmission for PUSCH(s) with UE processing time capability 2 are introduced

  - 11-3a: CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 11-3b: CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 11-3c: CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 11-3d: CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

FL proposal 3a:

  - 11-3e: CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

| 11-3a | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-5c | Yes | N/A |  | FS | No | FR1 only |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11-3b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13 | Yes | N/A |  | FS | No | FR1 only |  |  | Optional with capability signalling |
| 11-3c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13a | Yes | N/A |  | FS | No | FR1 only |  |  | Optional with capability signalling |
| 11-3d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13c | Yes | N/A |  | FS | No | FR1 only |  |  | Optional with capability signalling |
| 11-3e | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13d | Yes | N/A |  | FS | No | FR1 only |  |  | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| ZTE | We are open for these UE features except for FG 11-3e. FG 11-3e is not that desirable as commented in Section 2.2. We are not sure why FG 11-3a ~ FG 11-3e for PUSCH transmissions need to make 5-5a, 5-5b, 5-13, 5-13a, 5-13c, 5-13d as prerequisites, which are intended for PDSCH transmissions. |
| Ericsson | As indicated in the prep discussion, this was already discussed in RAN1#100bis-e and outcome is likely to be the same this time. |
| Moderator (NTT DOCOMO) | Prerequisite FGs for proposed 11-3a are updated according to [5]. |
| Huawei, HiSi | Cannot accept the proposals. We are not OK with dropping capabilities for CBG with processing capability 1 and for DL, which are more typical scenario targeting data rate with large packet size. CBG based operation with processing capability 2 can be kept or even not supported in Rel-16. Propose the following for consistence with latest 306 capability name. - Introduce a FG, which defines whether the UE, when configured with DL CBG-based operation and capable of processing time capability 1, supports reception of up to one, two, three four or seven unicast PDSCHs for several transport blocks with PDSCH scrambled using C-RNTI, TC-RNTI, or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. Introduce a FG, which defines whether the UE, when configured with UL CBG-based operation and capable of processing time capability 1, supports reception of up to one, two, three four or seven unicast PUSCHs for several transport blocks with PUSCH scrambled using C-RNTI, TC-RNTI, or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. |
| Nokia, NSB | We agree with the observation that this has been discussed already in the previous meeting and our views have not changed in the meantime. |
| Qualcomm | The FGs should be introduced. |
| Huawei, HiSilicon-02 | Unclear why separate 11-3a~11-3d are intended. For example, it does not make sense to me if the UE reports support 11-3c but not support 11-3b; or in other words, if the UE reports support of 11-3c why does it need to report 11-3a/-3b additionally. Note for a UE report Cap#2 it can also fallback to Cap#1. It is unclear to us what is the assumption for 11-3x with fallback and/or multiple-CC, if without defining capability for Cap#1. The previous example of R15 UE “ FG5-13x: ‘supported’, FG5-25: ‘not supported’ ” is also not clear to us, since such report which tells CBG is not supported while Cap#2 is supported, will further disable the operation of CBG+Cap#1 - which may not be the intention of the UE, and can be resolved only if separate capability for CBG+Cap#1 is defined as well. |
| Moderator (NTT DOCOMO) | Based on the discussion in GTW sessions and the current situation, my suggestion as moderator is to focus on FL proposal 3/3b and to give up on FL proposal 2/3a. |
| Ericsson | Our understanding of this proposal is that for capability 2: - There is no change for Rel-15. A Rel-15 UE indicating individually support for both CBG-based transmission and multiple PDSCHs/PUSCHs per slot per CC for different TBs also supports simultaneous use of the two features - The Rel-16 UE indicating support for the new FG, behaves exactly like a Rel-15 UE indicating support individually for both as described above |
| Intel | If these FGs are to be introduced, - In our view, PDSCH cases also need to be addressed (currently 11-3x shows PUSCH only). We slightly prefer to merge PDSCH cases to PUSCH cases in FG 11-3x rather than having separate FGs. - We prefer to introduce one bit Rel-16 RRC parameter to confirm Rel-16 configurations due to these new Rel-16 FGs. This can address backward compatibility issue for the network having not upgraded to Rel-16, or can prevent under-utilization of actual UE capabilities of CBG retransmission, multi-TBs, and Cap#1/2. |
| Moderator (NTT DOCOMO) | Based on the discussion so far, three alternatives for updated FL proposal 3 with all the details are provided below. |

Agreements:

- New FGs for simultaneous use of CBG-based transmission for PUSCH(s) with UE processing time capability 2 are introduced

  - 22-3a: CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3b: CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3c: CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3d: CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

- New FGs for simultaneous use of CBG-based transmission for PDSCH(s) with UE processing time capability 2 are introduced

  - 22-3e: CBG based transmission for DL with 1 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3f: CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3g: CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3h: CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

- Introduce one bit Rel-16 RRC parameter to confirm Rel-16 configuration

Updated FL proposal 3:

- Detailed design of FG22-3a to 3h is as below.

| 22-3a | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 2 | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22-3b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3e | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 2 | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3f | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3g | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3h | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Samsung | Based on our understanding on the discussion, both 5-22/5-25 and 5-5x/5-13x cannot be pre-requisite since this feature is for a rel-15 UE which will not declare support for one of cap#2 or CBG. We suggest to remove 5-22 and 5-25 from pre-requisite, and indication of this feature implies that a UE supports CBG in the scenarios mentioned in the description. As I mentioned before, this signaling becomes very similar to per-FS CBG support, but not exactly the same since a UE can selectively signal support of CBG for #1/#2/#4/#7 differently with this signaling. Overall, in our understanding, this is per-FS 1-bit signal for each #TB relying on rel-15 signaling on 5-5x and 5-13x. |
| ZTE | The description “1 unicast PUSCHs per slot per CC for different TBs” could be misleading as 1 unicast PUSCH/PDSCH corresponds to one TB instead of “different TBs”. We propose to update all the “1 unicast PUSCHs per slot per CC for different TBs” to “1 unicast PUSCH per slot per CC”; all the “1 unicast PDSCHs per slot per CC for different TBs” to “1 unicast PDSCH per slot per CC”. |
| Intel | We partially agree with Samsung that 5-22/5-25 cannot be pre-requisite. The CBG capability by 5-22/5-25 didn’t distinguish Cap#1 from Cap#2. We are okay to keep 5-5x/5-13x as pre-requisite. |
| Moderator (NTT DOCOMO) | Based on above feedbacks, FL proposal are updated as above. |
| Huawei, HiSilicon | - Per FS is fine, to align with 5-11x/5-13x. - Do we need to say “This capability is necessary for each SCS” as 5-11x/-13x in Rel-15? Some side comments but since we already agreed Alt1, this is just for understanding and may be helpful for RAN2 colleagues design the signalling: - Still think the Alt2 discussed in original FL 3b is the effectively the sam as Alt1. In a particular CC of a band, a UE just report one from 5-11x in R15. That’s the reason that for Cap#1, we defined 5-11/-11a/-11b ect in 38822, but in the end in 306, there is only one IE defined. Similar to Cap#2. Thus no need to have 4 capability bits for different number of TBs. |

FL proposal 3b:

- Introduce a FG, which defines whether the UE, when configured with DL CBG-based operation and capable of processing time capability 1, supports reception of up to one, two, four or seven unicast PDSCHs for several transport blocks with PDSCH scrambled using C-RNTI or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only.

- Introduce a FG, which defines whether the UE, when configured with UL CBG-based operation and capable of processing time capability 1, supports reception of up to one, two, four or seven unicast PUSCHs for several transport blocks with PUSCH scrambled using C-RNTI or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only.

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We think either of the below approach is needed, given no difference has been clarified compared to that in Rel-15 (for Approach 1) or no specific concerns has been expressed with introducing new capabilities in Rel-16 (for Approach 2). Approach 1 - A UE is not required to support the following - A UE is scheduled/configured with multiple TBs ( in DL or UL, processing capability 1 or capability 2) for unicast transmissions within a slot per CC simultaneously with CBG operation Approach 2 - Introduce a FG, which defines whether the UE, when configured with DL CBG-based operation and capable of processing time capability 1, supports reception of up to one, two, four or seven unicast PDSCHs for several transport blocks with PDSCH scrambled using C-RNTI or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. - Introduce a FG, which defines whether the UE, when configured with UL CBG-based operation and capable of processing time capability 1, supports reception of up to one, two, four or seven unicast PUSCHs for several transport blocks with PUSCH scrambled using C-RNTI or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. - Introduce a FG, which defines whether the UE, when configured with DL CBG-based operation and capable of processing time capability 2, supports reception of up to one, two, four or seven unicast PDSCHs for several transport blocks with PDSCH scrambled using C-RNTI or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. - Introduce a FG, which defines whether the UE, when configured with UL CBG-based operation and capable of processing time capability 2, supports reception of up to one, two, four or seven unicast PUSCHs for several transport blocks with PUSCH scrambled using C-RNTI or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. We are not ok with only introducing capability for FL proposal 3 in its current form (1-3a~11-3d) , given that - No clear reasons justified for difference between Cap#1 and Cap#2 with CBG, in terms spec support, RRC support and capability design in Rel-15 - Cap#1 would be eventually widely supported and can be used in more scenarios e.g. FR2, X-carrier scheduling thus more fit the market - Unclear reasons to have separate report for each # of TBs, e.g. why if the UE reports support 11-3c but not support 11-3b; or in other words, if the UE reports support of 11-3c why does it need to report 11-3a/-3b additionally. And what is the assumption if a UE report Cap#2 with fallback, and how it is related to # of CCs as defined in pdsch-ProcessingType2. pdsch-ProcessingType1-DifferentTB-PerSlot Defines whether the UE capable of processing time capability 1 supports reception of up to two, four or seven unicast PDSCHs for several transport blocks with PDSCH scrambled using C-RNTI, TC-RNTI, or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. Note PDSCH(s) for Msg.4 is included.; FS; No; No; No. pdsch-ProcessingType2 Indicates whether the UE supports PDSCH processing capability 2. The UE supports it only if all serving cells are self-scheduled and if all serving cells in one band on which the network configured processingType2 use the same subcarrier spacing. This capability signalling comprises the following parameters for each sub-carrier spacing supported by the UE. - fallback indicates whether the UE supports PDSCH processing capability 2 when the number of configured carriers is larger than numberOfCarriers for a reported value of differentTB-PerSlot. If fallback = 'sc', UE supports capability 2 processing time on lowest cell index among the configured carriers in the band where the value is reported, if fallback = 'cap1-only', UE supports only capability 1, in the band where the value is reported; - differentTB-PerSlot indicates whether the UE supports processing type 2 for 1, 2, 4 and/or 7 unicast PDSCHs for different transport blocks per slot per CC; and if so, it indicates up to which number of CA serving cells the UE supports that number of unicast PDSCHs for different TBs. The UE shall include at least one of numberOfCarriers for 1, 2, 4 or 7 transport blocks per slot in this field if pdsch-ProcessingType2 is indicated.; FS; No; No; FR1 only. |
| Moderator (NTT DOCOMO) | Based on the discussion in GTW sessions and the current situation, my suggestion as moderator is to focus on FL proposal 3/3b and to give up on FL proposal 2/3a. Based on above comments, Approach #1 and #2 above and potentially #3 based on proposal 3 (separate FGs for each # of TBs) can be further discussed. |
| Ericsson | Our understanding is that this proposal for capability 1 is similar to what we describe above for capability 2 and no new UE behavior is introduced in Rel-16. |
| Moderator (NTT DOCOMO) | Based on the discussion so far, similar to FL proposal 3, three alternatives for updated FL proposal 3a with all the details are provided below. |

Agreements:

- New FGs for simultaneous use of CBG-based transmission for PUSCH(s) with UE processing time capability 1 are introduced

  - 22-4a: CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4b: CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4c: CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4d: CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

- New FGs for simultaneous use of CBG-based transmission for PDSCH(s) with UE processing time capability 1 are introduced

  - 22-4e: CBG based transmission for DL with 1 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4f: CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4g: CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4h: CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

- Introduce one bit Rel-16 RRC parameter to confirm Rel-16 configuration

Updated FL proposal 3b:

- Detailed design of FG22-4a to 4h is as below.

| 22-4a | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 1 | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22-4b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4e | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 1 | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4f | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4g | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4h | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Samsung | Not clear on the pre-requisite. If we follow, proposal 3, then 5-22 and 5-25 need to be removed. Then, this is nothing but per-FS CBG support for cap#1 . The actual signaling can just be 1-bit per-FS signaling indicating application of CBG for the relevant cap#1 scenario declared in 5-11x and 5-12x. |
| ZTE | The same comments as for proposal. The description “1 unicast PUSCHs per slot per CC for different TBs” could be misleading as 1 unicast PUSCH/PDSCH corresponds to one TB instead of “different TBs”. We propose to update all the “1 unicast PUSCHs per slot per CC for different TBs” to “1 unicast PUSCH per slot per CC”; all the “1 unicast PDSCHs per slot per CC for different TBs” to “1 unicast PDSCH per slot per CC”. |
| Intel | We partially agree with Samsung that 5-22/5-25 cannot be pre-requisite. The CBG capability by 5-22/5-25 didn’t distinguish Cap#1 from Cap#2. |
| Moderator (NTT DOCOMO) | Based on above feedbacks, FL proposal are updated as above. |
| Huawei, HiSilicon | Similar comments as in above for FL proposal 3. |

2.4	new FGs for simultaneous use of UE processing time capability 2 and a certain Rel-16 features in the same CC

- Necessity of new FG(s) for simultaneous use of UE processing time capability 2 and a certain Rel-16 features in the same CC

  - Define Rel-16 UE processing time capability 2 as per FSPC reporting: [3]

  - Define new FG(s) to indicate support of simultaneous cap2 and a certain Rel-16 features in the same CC in case-by-case manner: [3]

    - FGs for simultaneous use of CBG based UL and UE processing time capability 2 are necessary: [5]

Above discussion points and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | During the email discussion, the following yellow highlighted parts were proposed. Index; Feature group; Components; Prerequisite feature groups. 8-1; Dynamic power sharing for LTE-NR DC; When total transmission power exceeds Pcmax, UE scales NR transmission power.; EN-DC. [5-11c]; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included. [5-12c]; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1. [5-13g]; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5a or 5-5b. [5-13h]; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5c. [5-35]; Simultaneously enable CBG and multiple PDSCHs per slot; Simultaneously enable CBG and multiple PDSCHs per slot; 5-11,5-11a, 5-11b, 5-13. 5-13a. 5-13c, 5-22, 5-23, 5-24. For FGs 5-11c/5-12c/5-13g/5-13h above, there are already capabilities for UE to receive up to 2 or 4 PDSCH/PUSCH, respectively, while the proposed FGs are for UE to receive up to 3 PDSCH/PUSCH. This would bring UE fragmentation with no clear benefits. Also for FG 5-35, this seems a signaling to indicate less capability than Rel-15. It is also not clear to have this signaling. Observation 1: There is no clear benefit to introduce new FGs 5-11c/5-12c/5-13g/5-13h, and 5-35. Processing time capability#2 In rel-15, declaration of support of cap#2 is per-FS, which means that a UE declare the support of cap#2 for all CC’s in certain band of certain band combination. However, some rel-16 features may be difficult to be supported simultaneously in the same CC’s together with cap#2 due to UE implementation complexity. One example of such rel-16 feature is multi DCI-based multi-TRP operation with out of order operation and/or overlapping PDSCH’s. Another example is span-based PDCCH monitoring. Under the current UE capability signaling, a UE may need to avoid simultaneously signaling support of cap#2 and those rel-16 features for certain band in the band combination. This means that none of CC’s in that band will support that feature, and such under-reporting is not desirable. A straightforward way to improve this situation is to define FSPC cap#2 support signaling for rel-16 UE’s. In this case, the existing rel-15 signaling may be ignored by network when rel-16 signaling is utilized. Another possible solution is to define new capability signaling to indicate support of simultaneous cap#2 and certain rel-16 features in the same CC, and this may need to be done case-by-case manner for each rel-16 feature. Proposal 3: Following 2 options can be considered. Option 1: define FSPC cap#2 support signaling for rel-16 UE’s, option 2: define new capability signaling to indicate support of simultaneous cap#2 and certain rel-16 features in the same CC in case-by-case manner. In rel-15, cap#2 processing time is only supported for self-scheduling case, but support of cap#2 with cross-carrier scheduling (CCS) is being considered in rel-16. Under the current signaling, it is possible that CCS can be configured between cap#1 and cap#2 scheduling cell and scheduled cell. It would be natural to think that at least scheduled cell should be cap#2 to apply cap#2 processing time, but scheduling cell may also need to be cap#2 since PDCCH decoding on cap#1 could take longer time. To handle this, several possible solutions can be considered. Proposal 4: Following 2 options can be considered. - Option 1: Capability#2 processing time is applied for CCS only if both scheduling and scheduled cell are configured with cap#2. Otherwise, cap#1 processing time is applied, - Option 2: Define UE capability to support cap#2 processing time for CCS even if only one of scheduling and scheduled cell is configured with cap#2. |
| --- | --- |
| [5] | Furthermore, we propose new FGs for Rel. 15 NR. The proposed changes in 5-11c, 5-12c, 5-13d and 5-13g are to allow for scheduling 3 TBs in both UL and DL. Considering the (4,3) span pattern of FG 3-5b, these additions would allow for matching the number of TBs and the spans in each slot. In addition, the proposed FG 11-3a-e would allow for capability signalling for the simultaneous use of CBG-based UL transmission and minimum processing capability 2. Proposal 4: Add Rel-15 FGs to allow for scheduling 3 TBs in both UL and DL in order to match the number of control spans per slot Proposal 5: Add Rel-15 FGs for the simultaneous use of CBG-based UL transmission and UE processing time capability 2. 5-11c; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 PDSCH(s) for Msg. 4 is included; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS; Optional with capability signalling. 5-12c; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS; Optional with capability signalling. 5-13d; Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5a or 5-5b; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination; Optional with capability signalling. 5-13g; Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2; Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz; 5-5c; Yes; N/A; FS; N/A; N/A; This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination; Optional with capability signalling. 11-3a; CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-5a or 5-5b; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3b; CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3c; CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13a; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3d; CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13c; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. 11-3e; CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2; 5-13d; Yes; N/A; FS; No; FR1 only; [Modification of Rel-15 capability]; Optional with capability signalling. |

Based on above, following FL proposals are made.

FL proposal 4:

- No more new FGs for simultaneous use of UE processing time capability 2 with a certain Rel-16 features in the same CC is introduced

  - Adopting “Per FSPC” for UE processing time capability 2 for Rel-16

  - FFS: Cap2 with PUSCH repetition

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| ZTE | We are ok with the FL proposal. |
| Ericsson | Proposal is fine |
| Nokia, NSB | We are OK with FL proposal. |
| Qualcomm | We do not agree with a generic proposal on not introducing future capabilities. |
| Moderator (NTT DOCOMO) | Based on the discussion in GTW sessions and the current situation, my suggestion as moderator is to focus on FL proposal 3/3b and to give up on FL proposal 2/3a and any other new FG proposals for simultaneous use of UE processing time capability 2 with a certain Rel-16 features in the same CC. |
| Ericsson | Agree with moderator proposal |

2.5	Others

Proposal 6: Add a Rel-15 FG for supporting offset between the end of PDCCH triggering A-SRS and the SRS transmission for CB PUSCH and antenna switching for UEs supporting PDCCH capabilities besides FG 3-1. Proposal 7: Add a Rel. 15 FG for supporting partial cancellation of configured PUCCH/PUSCH/PRACH due to dynamic SFI, dynamically granted PDSCH and CSI-RS. Regarding Proposal 6: If a UE does not support FG 2-58, a UE expects a gap between the last symbol of PDCCH carrying SRS triggering and the earliest SRS symbol to be offset by at least 1 slot, i.e., SRS triggering and resource for transmission cannot be in the same slot. This gap helps the UEs supporting only PDCCH based on FG 3-1. In particular, assuming a 3-symbol PDCCH at the beginning of slot n, the earliest time for SRS transmission is over the last 6 symbols of slot n+1, i.e., the gap between the end of PDCCH triggering SRS and the first potential symbol for SRS transmission is 19 symbols. However, the same gap cannot be maintained for UEs supporting any other PDCCH capability, e.g., 3-2, 3-5/a/b or the new Rel. 16 PDCCH. To address the issue for the UEs supporting PDCCH monitoring capabilities besides FG 3-1, we propose to adopt the following FG: 2-58a; For SRS for CB PUSCH and antenna switching on FR1 with symbol level offset for aperiodic SRS transmission; For UEs supporting a PDCCH monitoring capability in addition to FG 3-1: - For SRS for CB PUSCH and antenna switching on FR1, support d symbols offset between aperiodic SRS triggering and transmission; 2-53; Yes; N/A; FS; n/a; n/a; n/a; Optional with capability signalling The value range for component 1 = {0, 19}. Regarding Proposal 7: A partial cancellation according to the following clause from Section 11.1.1 of 38.213 (f90) was included in Rel. 15 specification: If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0, DCI format 1_0, DCI format 1_1 or DCI format 0_1 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0 - the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols. The behavior defined according to the text basically means that the UE should be able to cancel an ongoing configured uplink transmission if it detects a DCI scheduling PDSCH or CSI-RS or SFI. The cancellation could be partial based on the timeline. (A similar text with new DCI formats exists in the Rel. 16 spec.) Based on this text, e.g., a DL DCI scheduling a PDSCH can force a UE to interrupt an ongoing P-CSI transmission. However, the Rel. 15 UEs are not able to partially cancel an ongoing uplink transmission. Hence, we propose to add the following FG to support the feature: XX; XX; Cancellation of PUCCH, PUSCH or PRACH with a DCI scheduling a PDSCH or CSI-RS or a DCI format 2_0 for SFI; A UE supports the partial cancellation of the SRS or PUCCH or PUSCH or PRACH configured transmission: - The UE cancels the configured PUCCH or PUSCH or PRACH in a set of symbols of a slot due to detection of a DCI format 2_0 with a slot format value other than 255 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible - The UE cancels the configured PUCCH or PUSCH or PRACH in a set of symbols of a slot due to the detection of a DCI format 1_0, DCI format 1_1, DCI format 1_2 or DCI format 0_1 and DCI format 0_2 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols.; Yes; N/A; FS; N/A; N/A; N/A; Optional with capability signaling. Component-1 is subjected to FG 3-6. If the UE does not support this feature, then cancellation of the configured PUCCH/PUSCH/PRACH is possible if their starting symbol is at least after a number of symbols that is larger or equal to the PUSCH preparation time T_proc,2 from the last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI formats 1_0, 1_1, 1_2 or the DCI format 0_1/0_2. In such a case, the entire duration of the configured transmission is cancelled. Proposed Conclusion: A UE not supporting the proposed FG, including the Rel. 15 UEs, is not expected to cancel a transmission of a configured PUCCH/PUSCH/PRACH over a subset of symbols if the gap between the starting symbol of the PUCCH/PUSCH/PRACH transmissions and the last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1or DCI format 1_2 or the DCI format 0_1 or DCI format 0_2, is smaller than the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming and corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0, DCI format 1_0, DCI format 1_1, DCI format 1_2, DCI format 0_1 or DCI format 0_2 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0.

Companies are encouraged to check above proposals and to provide feedbacks if any.

| Company | Comment |
| --- | --- |
| Apple | We support proposal 6 and 7 |
| Ericsson | For proposal 6, our understanding is that this is adding an “in-between” capability that would reduce the delay somewhat, but not as significant as for UEs supporting 2-58. We are skeptical that network implementations would cater for such UEs, especially given the large range of values that can be reported for the component. For proposal 7, we may have some sympathy for such an FG, but in you proposed conclusion you are basically stating that it also applies to Rel-15 even if a Rel-15 network would not recognize this UE capability. Isn’t this in practice then an incapability indication that RAN2 has warned about? |
| Qualcomm | We support proposal 6 and 7. To respond to Ericsson: - Proposal 6 does not intend to reduce the delay; it is to ensure that regardless of the PDCCH monitoring capability supported by the UE, the gap between the end of PDCCH triggering A-SRS for CB PUSCH and antenna switching and the SRS transmission remains the same. This is not the case currently for UEs supporting PDCCH monitoring capabilities in addition to FG 3-1. As an example, if the UE only supports FG 3-1 PDCCH and does not support 2-58, the gap between the end of PDCCH (assuming 3 symbols) and the first valid SRS resource for transmission is 19 symbols. If the same UE supports PDCCH monitoring capability 3-5b/3-2/the new Rel. 16 PDCCH capability, etc, and does not support FG 2-58, the gap is much shorter since the PDCCH monitoring occasion could essentially be anywhere in the slot. - FG proposed under Proposal 7 does not indicate incapability; it states that the UE can handle partial cancellation due to the reception of PDSCH/CSI-RS/SFI. The conclusion the tries to clarify that partial cancellation is not supported for the UEs not reporting the proposed FG including the Rel. 15 UEs. |

FL proposal 5:

- Add a Rel-15 FG for supporting offset between the end of PDCCH triggering A-SRS and the SRS transmission for CB PUSCH and antenna switching for UEs supporting PDCCH capabilities besides FG 3-1.

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Moderator (NTT DOCOMO) | Companies inputs are necessary to make a decision. |
| Ericsson | We can see from a UE perspective that aligning the delays would be attractive, but it seems the burden is now moved to the network given the number of different signaling values for the component the network has to take into account. Not supportive. |
| Qualcomm | In response to Ericsson, it should be noted that if a UE support cap#2, only PDCCH under FG 3-1 and does not support 2-58, it will have 19 symbols gap. However, if the same UE supports any other PDCCH capability, given N2 = 5.5 in 30KHz, it will have about 8 symbols gap between PDCCH triggering A-SRS and the first SRS resource. It is the same UE and it supports cap#2, but supporting other PDCCH capabilities should not mean that the UE is able to perform some processing about 2x faster. |

FL proposal 6:

- Add a Rel. 15 FG for supporting partial cancellation of configured PUCCH/PUSCH/PRACH due to dynamic SFI, dynamically granted PDSCH and CSI-RS.

Companies are encouraged to check above FL proposals and to provide feedback if any in below. If you cannot accept the FL proposals, please put your company name after “Cannot accept the proposals” below and please provide your alternative proposal (in your comment) which could be acceptable to all in your consideration.

Cannot accept the proposals:

| Company | Comment |
| --- | --- |
| Moderator (NTT DOCOMO) | Companies inputs are necessary to make a decision. |
| Huawei, HiSi02 | Not needed as partial cancelation is not supported in R15 except for SRS, by specification. The following agreements also confirm this and it was extended to PRACH as well when go to spec. |
| Ericsson | Given Qualcomm’s comment there is in practice no fallback Rel-15 behavior and this nullifies the Rel-15 functionality. |
| Qualcomm | The text in the specification is quite ambiguous. However, given the agreement cited above, it should then mean that partial cancellation of UL configured transmission due to SFI or DL DCI is not supported in Rel. 15 (which carries over to Rel. 16 too.) If this is, in fact, a common understanding, then it should be fine. To response to Ericsson, the argument was that if this capability is added, then any UE reporting it would support the feature. It does not mean that the functionality is removed from Rel. 15. In any case, as mentioned, if the expected behavior is based on the above agreement, which must be, then, there should be no issue. |

- Conclusion

Agreements

- A new FG for indicating supported option for UL Tx switching for inter-band UL CA is added in the UE features list

  - Candidate values set is {option1, option2, [both option 1 and option 2]}

  - Type of the FG is “Per BC”

  - This FG is "Conditional mandatory with capability signalling".  Signaling of this FG is mandatory conditioned on the support of switching time capability for Tx switching between two uplink carriers in inter-band UL CA band combinations in RAN4 FG 7-1 (i.e. Tx switching period between two uplink carriers).

  - 6-6 and RAN4 FG 7-1 are prerequisite feature groups for the new FG

  - Note “[it is up to RAN2 how to report support of both option 1 and option 2]”

  - Add "FR1 only" in the column of FR1/FR2 differentiation

Agreements:

- A new FG 22-2 for indicating supported option for UL Tx switching for EN-DC is added in the UE features list

  - Candidate values set is {option1, option2, [both option 1 and option 2]}

  - Type of the FG is “Per BC”

  - This FG is "Conditionally mandatory with capability signalling".  Signaling of this FG is mandatory conditioned on the support of switching time capability for Tx switching between two uplink carriers in EN-DC in RAN4 FG 7-1 (i.e. Tx switching period between two uplink carriers).

  - EN-DC and RAN4 FG 7-1 are prerequisite feature groups for the new FG

  - Add "FR1 only" in the column of FR1/FR2 differentiation

- Add a note depending on the outcome from [101-e-LS-TxSwitching-01] to both FG 22-1 and FG 22-2 - e.g. ["It has been agreed in RAN1 that UE can report support of one of the three candidates {option1, option2, both option1 and option2}.  It is up to RAN2 to design the corresponding UE capability signalling."]

Agreements:

- New FGs for simultaneous use of CBG-based transmission for PUSCH(s) with UE processing time capability 2 are introduced

  - 22-3a: CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3b: CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3c: CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3d: CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2

- New FGs for simultaneous use of CBG-based transmission for PDSCH(s) with UE processing time capability 2 are introduced

  - 22-3e: CBG based transmission for DL with 1 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3f: CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3g: CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

  - 22-3h: CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2

- Introduce one bit Rel-16 RRC parameter to confirm Rel-16 configuration

Agreements:

- New FGs for simultaneous use of CBG-based transmission for PUSCH(s) with UE processing time capability 1 are introduced

  - 22-4a: CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4b: CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4c: CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4d: CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1

- New FGs for simultaneous use of CBG-based transmission for PDSCH(s) with UE processing time capability 1 are introduced

  - 22-4e: CBG based transmission for DL with 1 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4f: CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4g: CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

  - 22-4h: CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1

- Introduce one bit Rel-16 RRC parameter to confirm Rel-16 configuration

Proposed agreements:

- Detailed design of FG22-3a to 3h is as below.

| 22-3a | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 2 | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22-3b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3e | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 2 | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3f | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3g | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-3h | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 2 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |

Proposed agreements:

- Detailed design of FG22-4a to 4h is as below.

| 22-4a | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 1 | CBG based transmission for UL with 1 unicast PUSCH per slot per CC with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 22-4b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4e | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 1 | CBG based transmission for DL with 1 unicast PDSCH per slot per CC with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4f | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for DL with up to 2 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4g | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for DL with up to 7 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 22-4h | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 | CBG based transmission for DL with up to 4 unicast PDSCHs per slot per CC for different TBs with UE processing time Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |

References

[1]	R1-2003336	Remaining issues on Rel-16 NR UE features	ZTE

[2]	R1-2003762	Rel-16 UE feature - Others	Intel Corporation

[3]	R1-2003902	UE features for other aspects	Samsung

[4]	R1-2004062	Discussion on the support of SRS transmission in all symbols of a slot	OPPO

[5]	R1-2004732	Discussion on UE features for Others	Qualcomm Incorporated

[6]	R1-2004682	General discussion on NR Rel-16 UE features	Ericsson

[7]	R1-2004628	Other aspects of Rel-16 NR UE features	Huawei, HiSilicon