3GPP TSG RAN WG1 Meeting #101-e	R1-2004846

E-Meeting, May 25 – June 5, 2020

Agenda Item:	6.2.3.1.1

Source:	Moderator (Huawei)

Title:	Feature summary on 101-e-LTE-LTE_DL_MIMO_EE-02

Document for:	Discussion and Decision

Introduction

This documents provides the proposals and summary of discussions of the following second phase email discussion for identified issues of LTE downlink MIMO efficiency enhancement [10].

[101-e-LTE-LTE_DL_MIMO_EE-02] Indexing of additional SRS symbols by 5/29 – Yubo (Huawei)

- Issues 1 in R1-2004706

Discussion

### Issue 1: Indexing of additional SRS symbols.

- As explained in [4], the parameter l in nSRS=lR may be misleading as the range of the parameter l is not defined. The interpretation that l is OFDM symbol index within the subframe should be avoided.

- Proposal: Endorse the following text proposal to TS 36.211

Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=l'R where l' is the index of the OFDM symbol number carrying additional SRS transmission number within the subframe, where l'=0 corresponds to the first OFDM symbol of SRS transmission not counting guard symbol(s) , and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum;

Please enter your comments on the TP:

| Companies | Comments |
| --- | --- |
| Qualcomm | Agree with the proposal with minor change to avoid multiple ‘where’ and also link with the defined parameter ‘starting OFDM symbol l0’ as “nSRS=l'R where l' is the index of the OFDM symbol number carrying additional SRS transmission number within the subframe with l'=0 corresponding to the starting OFDM symbol l0 not counting guard symbol(s) , and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum;” |
| Ericsson | Agree with the proposal. |
| LGE | Both Intel’s and Qualcomm’s TP are fine for us. But for more clarity, we can refer the potential guard symbol is not included in additional SRS transmission number. This is important since the n_SRS should be increased by actual transmission of additional SRS. So, we propose as follows with green part: “nSRS=l'R where l' is the index of the OFDM symbol number carrying additional SRS transmission number not counting guard symbol(s) within the subframe with l'=0 corresponding to the starting OFDM symbol l0 not counting guard symbol(s) , and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum;” |
| ZTE | Agree with above proposal from LGE. The guard symbol(s) should be not counted when n_SRS is increased. |
| Huawei, HiSilicon | We are fine the proposed change from LGE. |
| Nokia, Nokia Shnaghai Bell | We agree that this needs to be clarified in the specification. The TP from LGE seems OK. |
| Lenovo, Motorola Mobility | We prefer LGE’s text proposal. |

Summary

The propose TP with modification from LGE is acceptable, therefore it’s proposed to be endorsed.

- References

- R1-1913596, “RAN1 agreements for DL MIMO efficiency enhancements for LTE”, Huawei, Reno, USA, November 2019.

- R1-2003466	Maintenance of additional SRS symbols	ZTE

- R1-2003541	Corrections on additional SRS symbols	Huawei, HiSilicon

- R1-2003723	Corrections to additional SRS	Intel Corporation

- R1-2003784	Additional SRS symbols	Qualcomm Incorporated

- R1-2003818	Additional SRS symbols	Lenovo, Motorola Mobility

- R1-2003925	Text proposals on additional SRS symbols	LG Electronics

- R1-2003937	Remaining details of additional SRS symbols	Nokia, Nokia Shanghai Bell

- R1-2004431	Maintenance on additional SRS symbols	Ericsson

- R1-2004706, Feature summary on LTE DL MIMO efficiency enhancement, Moderator (Huawei), E-meeting, May 2020.