3GPP TSG-RAN WG1 Meeting #101-e											R1-2004873

e-Meeting, May 25 – June 05, 2020

Agenda Item:	7.2.2.2.2

Source:			Moderator (Huawei)

Title:			Summary of [101-e-NR-unlic-NRU-InitAccessProc-06]

Document for:	Discussion

Introduction

This document captures the discussion of the following email thread:

[101-e-NR-unlic-NRU-InitAccessProc-06] Email approval for a reply LS to R1-2003273 by 5/28. To be managed under 7.2.2.2.2 – Jiayin (Huawei)

Discussion

In RAN1#99, the following agreement was reached.

Agreement:

A UE shall not average CSI-RS measurements for channel estimation across different transmission bursts from the UE's perspective.

FFS: Potential issues due to AGC

It is captured in section 5.2.1.1 Reporting settings of TS38.214 v16.1.0 as following.

For operation with shared spectrum channel access, the UE should not average CSI-RS measurements for channel estimation from occasions of an NZP CSI-RS (defined in [4, TS 38.211]) located in different DL transmissions burst (defined in [X, TS 37.213]).

In RAN4#94bis-e, RAN4 sent an LS [1] to RAN1 stating the following observations.

- It is unclear whether the UE is able to distinguish whether the CSI-RS measurement are from different transmission bursts.

- The different transmit power of CSI-RS has impact on CSI-RS based L1-RSRP, RLM, BFD and CBD.

- The different transmit power of CSI-RS may lead to AGC problem

- The UE has to average CSI-RSs across different bursts to meet requirements and the UE will assume the power does not change over L1-period

Finally, RAN4 ask confirmation from RAN1 whether UE can assume that CSI-RS (for L1-RSRP, RLM, BFD and CBD) is transmitted with the same transmit power across different occasions during the measurement period (L1-period), and whether this also applies to SSB-based RRM measurements.

In the appendix, the views from companies are summarized.

- 8 companies (all companies providing feedback) supported CSI-RS for RLM have same transmit power across different occasions during the measurement period.

- 7 companies supported CSI-RS for BFD and CBD have same transmit power across different occasions during the measurement period.

- 5 companies supported CSI-RS for L1-RSRP have same transmit power across different occasions during the measurement period.

- 4 companies supported the constant transmit power assumption also applicable to SSB-based RRM measurement.

- 2 companies further clarified that the RAN1 agreement was only for CSI measurement.

- 1 company stated that there is no RAN1 impact.

According to section 4.1 in TS38.214, the UE assumption of same transmit power of SSB or CSI-RS can be guaranteed, if SS-RSRP or CSI-RSRP is used for L1-RSRP.

4.1 Power allocation for downlink

…

For the purpose of CSI-RSRP, CSI-RSRQ and CSI-SINR measurements, the UE may assume downlink EPRE of a port of CSI-RS resource configuration is constant across the configured downlink bandwidth and constant across all configured OFDM symbols.

…

On the other hand, L1-RSRP is one of CSI-related quantities in CSI reporting. It should be clarified whether L1-RSRP should be regarded as one of “CSI-RS measurements for channel estimation”.

Following two alternatives could be considered:

Alt 1: UE can assume same transmit power of CSI-RS for L1-RSRP. Add clarification in section 5.2.1.1 in TS38.214 stating that it is possible to average L1-RSRP measurement on NZP CSI-RS located in different DL transmissions burst.

Alt 2: UE cannot assume same transmit power of CSI-RS across different occasions during the measurement period for L1-RSRP measurement. Some updates on section 4.1 in TS38.214 is required.

Therefore, please provide responses in the tables below to the following questions:

Q1: Do you agree that CSI-RS for RLM, BFD and CBD is transmitted with the same transmit power across different occasions during the measurement period?

| Company | View |
| --- | --- |
| Huawei, HiSilicon | Yes |
| MediaTek | Yes |
| LG Electronics | Yes |
| Nokia, NSB | Yes |
| Samsung | Yes |
| Ericsson | I have some fundamental comments about how this discussion is framed, and in the reply LS to RAN4 we may need to clear up some confusion. In the same section of 38.214 as quoted above, there are two paragraphs that specify how the UE determines the CSI-RS EPRE (see highlighted text below). It is calculated by an RRC configured offset (powerControlOffsetSS) to an RRC configured value of the the SS/PBCH block power (ss-PBCH-BlockPower). This means that once the CSI-RS resource is configured, whatever type it is the power is fixed unless it is reconfigured at some point (rare), so of course it is constant across measurement periods. Please note that this applies to CSI-RS of all types. There is no need to differentiate between CSI-RS for RLM, BFD, CBD, L1-RSRP, etc. In some sense, it is unfortunate that we made the following agreement: Agreement: A UE shall not average CSI-RS measurements for channel estimation across different transmission bursts from the UE's perspective. FFS: Potential issues due to AGC because it seems to have caused some confusion in RAN4. I would propose that we be bold and remove the agreement from the RAN1 spec, since it is not needed in light of the above power and power offset parameters. Extract from 38.214 Section 4.1 The downlink SS/PBCH SSS EPRE can be derived from the SS/PBCH downlink transmit power given by the parameter ss-PBCH-BlockPower provided by higher layers. The downlink SSS transmit power is defined as the linear average over the power contributions (in [W]) of all resource elements that carry the SSS within the operating system bandwidth. The downlink CSI-RS EPRE can be derived from the SS/PBCH block downlink transmit power given by the parameter ss-PBCH-BlockPower and CSI-RS power offset given by the parameter powerControlOffsetSS provided by higher layers. The downlink reference-signal transmit power is defined as the linear average over the power contributions (in [W]) of the resource elements that carry the configured CSI-RS within the operating system bandwidth. |
| ZTE, Sanechips | Yes. The gNB implementation can achieve this purpose without any specification impacts |
| Qualcomm | Yes and Ericsson’s observation makes sense. We should make a general conclusion that NZP-CSI-RS power is constant as in Rel.15. |
| OPPO | yes |

Q2: Do you agree that CSI-RS for L1-RSRP is transmitted with the same transmit power across different occasions during the measurement period?

| Company | View |
| --- | --- |
| Huawei, HiSilicon | Yes. Clarification on L1-RSRP is not one of “CSI-RS measurements for channel estimation” should be added in 5.2.1.1 in TS38.214. |
| MediaTek | Yes. Clarification in 5.2.1.1 in TS38.214 is okay. |
| LG Electronics | Yes. Do we need to specify only for CSI-RS for L1-RSRP, or also for CSI-RS for RLM/RRM/CBD/BFD? |
| Nokia, NSB | Yes. See Q1 for CSI-RS for RLM/CBD/BFD |
| Samsung | Yes. |
| Ericsson | Please see comment for Q1 – no need to differentiate between CSI-RS types |
| ZTE, Sanechips | Yes. The gNB implementation can achieve this purpose without any specification impacts |
| Qualcomm | Agree with Ericsson. |
| OPPO | Agree no differentiation between CSI-RS types |

Q3: Do you agree that SSB (for SSB-based RRM measurement) is transmitted with the same transmit power across different occasions during the measurement period?

| Company | View |
| --- | --- |
| Huawei, HiSilicon | Yes |
| MediaTek | Yes |
| LG Electronics | Yes, also for SSB for RLM/BFD/CBD. |
| Nokia, NSB | Yes, and for SSB for RLM/BFD/CBD as well. |
| Samsung | Yes |
| Ericsson | Please see comment for Q1 – SS/PBCH power is RRC configured. Clearly it is constant. |
| ZTE, Sanechips | Yes. The gNB implementation can achieve this purpose without any specification impacts |
| Qualcomm | Yes. |
| OPPO | yes |

1st round Discussion Outcome

Based on the feedback on three questions, companies seems to have consensus to confirm with RAN4’s questions.

Proposal #1:

UE can assume that NZP CSI-RS or SS/PBCH block (for L1-RSRP, RLM, BFD, CBD and RRM) is transmitted with the same transmit power across different occasions during the measurement period, as in Rel-15.

- Send Reply LS [xxxx] to RAN4 inform the above agreement.

Furthermore, two companies are ok to clarify in section 5.2.1.1 of TS38.214 that L1-RSRP measurement on NZP-CSI-RS across bursts can be averaged. Three companies mentioned UE should assume constant transmit power of NZP CSI-RS for all use cases same as Rel-15. One company propose to remove the previous agreement on not averaging measurement on NZP CSI-RS resource across bursts from the specification in order to avoid confusion from RAN4. One company thought there is no RAN1 impact.

Proposal#2:

Consider one of following alternatives to reflect the agreement in the specification:

Alt 1: Clarify that measurement on NZP-CSI-RS (for L1-RSRP, [RLM, BFD, CBD and RRM]) across bursts can be averaged in 5.2.1.1 of 38.214

Alt 2: Remove following text in 5.2.1.1

For operation with shared spectrum channel access, the UE should not average CSI-RS measurements for channel estimation from occasions of an NZP CSI-RS (defined in [4, TS 38.211]) located in different DL transmissions burst (defined in [X, TS 37.213]).

Alt 3: No need to change

Therefore, please provide your views on 2 proposals:

| Company | View |
| --- | --- |
| Ericsson | Agree to Proposal #1. I suggest that in the LS we also include the following paragraph to clear up some confusion in RAN4. Additionally, RAN1 respectfully informs RAN4 that in 38.214 Section 5.1.4 it is specified how the UE determines the SS/PBCH power and the CSI-RS energy per resource element (EPRE). SS/PBCH power is configured by the RRC parameter ss-PBCH-BlockPower hence it is fixed, unless reconfigured. The CSI-RS EPRE is derived from the SS/PBCH block transmit power via a power offset given by the RRC parameter powerControlOffsetSS. Since the bandwidth of a CSI-RS resource is also fixed unless re-configured, the CSI-RS power is fixed. It is not expected that these parameters are re-configured very often, if at all. Regarding Proposal #2, we support Alt-2 based on the above Rel-15/16 specification. |

Summary

According to feedback on the two proposals, all companies provided feedback support proposal #1. One company suggest to remove 2nd paragraph on how UE determine transmit power of NZP CSI-RS in the draft reply LS v1 which might cause additional confusion.

As for proposal #2, 3 companies support alt 1 (with modification), 1 company support Alt 2 and four companies support Alt 3. After further discussion, companies think the current text in 5.2.1.1 is confusing (e.g. whether channel estimation include L1-RSRP) and clarification may be necessary.

Based on the observation, following two proposals are suggested

Proposal #1:

UE can assume that NZP CSI-RS or SS/PBCH block (for L1-RSRP, RLM, BFD, CBD and RRM) is transmitted with the same transmit power across different occasions during the measurement period, as in Rel-15.

- Send Reply LS [xxxx] to RAN4 inform the above agreement.

Proposal#2:

Clarify that measurement on NZP-CSI-RS (for L1-RSRP, [RLM, BFD, CBD and RRM]) across bursts can be averaged e.g., in 5.2.1.1 of 38.214, if necessary

A potential TP for proposal #2 can be

--------------------------------------------TP start -------------------------------------------------------------------------

- 5.2.1.1	Reporting settings

** No change text omitted **

For operation with shared spectrum channel access, the UE should not average CSI-RS measurements for channel estimation from occasions of an NZP CSI-RS (defined in [4, TS 38.211]) located in different DL transmissions burst (defined in [X, TS 37.213]). UE can assume same transmit power on the NZP CSI-RS occasions located in different DL transmission burst for L1-RSRP measurement and the measurement across bursts can be averaged.

--------------------------------------------TP end -------------------------------------------------------------------------

References

- R1-2003273, “LS on transmit power of CSI-RS across different occasions”, RAN4, RAN4#94bis-e, 3GPP TSG-RAN WG1 Meeting #101-e, May 25th – June 5th, 2020

Appendix

The summary of companies’ view in the tdocs.

Table 1: Related Contribution Summary

| Company | Discussion Tdoc | Related Proposal and Rationale |
| --- | --- | --- |
| ZTE, Sanechips | R1-2003448 R1-2003837 (draft LS) | Proposal 6: CSI-RS (for L1-RSRP, RLM, BFD and CBD) should be transmitted with the same power across different occasions during the measurement period. The gNB implementation can achieve this purpose without any specification impacts. |
| VIVO | R1-2003354 | Observation 1: CSI-RS resources can be separately configured for different purposes. Proposal 1: UE can assume EPRE of the CSI-RS resources for RRM, RLM, BFD, L1-RSRP, CBD constant once configured. It is up to network implementation to guarantee the EPRE constant across different occasions. - Send LS to RAN4 to inform above decision. Observation 2: For SSB based RRM in unlicensed spectrum, the UE can still assume that SSBs transmitted with the same block index on the same center frequency location are quasi co-located with respect to average gain. Proposal 2: Send LS to RAN4 to inform above understandings. |
| LG | R1-2004008 | Proposal: UE shall assume that CSI-RS or SSB (for RRM, RLM, BFD, and CBD) is transmitted with the same transmit power across different DL transmission bursts during the measurement period. - FFS: Whether to have RAN1 specification impact - Send a reply LS to RAN4 to inform this decision. |
| OPPO | R1-2004093 | Observation 1: the pointed RAN1 agreement by RAN4 seems to be only applicable for CSI measurement for CSI reporting. Observation 2: In NR Rel.15, UE assumes the same transmit power of CSI-RS across different occasion. Proposal 1: For RLM, BFD and CBD in NRU, UE may use the same assumption on the CSI-RS transmit power as Rel.15. Proposal 2: For RRM in NRU, the UE may assume the same transmit power of CSI-RS or SSB as in Rel.15. |
| Nokia, NSB | R1-2004513 (draft LS) | RAN1 agrees that a UE may average CSI-RS for L1-RSRP, RLM, BFD and CBD across different bursts if it validates CSI-RS within the bursts, but shall not average CSI-RS across different bursts for purpose of CSI estimation (as per above RAN1 agreement). |
| Huawei, HiSilicon | R1-2003510 R1-2004624 (draft LS) | Proposal 9: Confirm RAN4’s question that a UE can assume that • CSI-RS (for L1-RSRP, RLM, BFD and CBD) is transmitted with the same transmit power across different occasions during the measurement period. • SSB (for SSB-based RRM measurements) is transmitted with same transmit power across different occasions during the measurement period. |
| Samsung | R1-2003858 | Proposal 2: UE can assume that CSI-RS is transmitted with the same transmit power across different occasions during the measurement period. |
| Qualcomm | R1-2004441 | Proposal 3: UE expects CSI-RS for RLM to have fixed transmit power over all COTs. |