3GPP TSG RAN WG1 #101-e			R1-2004946

e-Meeting, May 25th – June 5th, 2020

Title:	[Draft] LS to RAN2 on resource allocation Mode-2 agreements related to QoS requirements

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	Intel Corporation [RAN1]

To:	RAN2

Cc:

Contact Person:

#### Name:	Sergey Panteleev

Tel. Number:

E-mail Address:	sergey.panteleev@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 made the following agreement which is assumed to be captured in MAC specification:

Agreement 1:

- In Step 2, a UE shall select resources so that HARQ retransmission resources can be reserved by a prior SCI, except that - In case no resource can be found for reservation (e.g., based on the identified candidate set after Step 1) for a retransmission of a TB, the re-transmission can be transmitted on a resource that is not reserved - After the resource selection is performed, HARQ retransmission on a resource not reserved by a prior SCI is allowed due to transmission dropping caused by prioritization, pre-emption and congestion control

RAN1 would like to highlight that Agreement 1 is not intended to conflict with the corresponding QoS requirements. If the requirements cannot be met, RAN1 assumes it is up to RAN2 to address accordingly.

Additionally, RAN1 made the following agreement to be captured in L1 specification:

Agreement 2:

- The UE shall indicate min(Nselected, N) first-in-time resources when setting the values of frequency resource assignment and time resource assignment in SCI format 0_1, where - Nselected is the number of resources selected by MAC within 32 slots (including the current one) - N is the maximum number of resources that can be signalled in one SCI

RAN1 would like to highlight that Agreement 2 is not intended to conflict with the corresponding QoS requirements. If RAN2 sees any issues, RAN2 can inform RAN1 accordingly.

2. Actions:

To RAN2

RAN1 respectfully requests RAN2 to take the above into account in the related work.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN WG1#102 		24 August – 4 September 2020       			E-Meeting