3GPP TSG RAN WG1 #101			                                   					       												                  R1-2005038

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.11.7

Source:	Moderator (AT&T)

Title:	Summary of email discussion/approval [101-e-NR- UE_pow_sav-UEFeatures-02]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [101-e-NR- UE_pow_sav-UEFeatures-02] during RAN1 #101-e. According to the Chairman’s Notes:

[101-e-NR- UE_pow_sav-UEFeatures-02] Email discussion/approval till 5/29 – Ralf (AT&T) - Discuss whether updates to the agreed FG 19-1 are needed and if so, agree the needed update(s)

The following was discussed and agreed during RAN1 #101-e within the scope of [101-e-NR- UE_pow_sav-UEFeatures-02].

# Summary of email discussion/approval [101-e-NR- UE_pow_sav-UEFeatures-02]

The following table represents the latest version of the NR UE feature list for UE power savings [1].

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 19.UE Power Saving | 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A |  | Yes |  | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | (1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1 minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset (2) minimumSchedulingOffset K2 configuration for PUSCH |  | N/A |  | Yes | N/A | Dynamic adaptation of the minimum value of K0min/K2min for cross-slot scheduling is not supported | Per UE | No | No | N/A | FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-3 | Maximum MIMO Layer Adaptation | - Support of maximum number of MIMO layer configuration per DL BWP |  | See Note |  | Yes | N/A |  | [Per UE ] | No | [Yes] | N/A | This capability is indicated only if UE supports the network configuration of maxMIMO-Layers according to maxLayersMIMO-Indication FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
|  | 19-4a | UE assistance information | Support of reporting preferred minimum K0/K2 via UE assistance information - 15kHz/30kHz SCS: {1, 2, 4, 6} slots - 60kHz/120kHz SCS: {2, 4, 8, 12} slots |  | 19-2 |  | Yes | N/A |  | Per UE | No | No | N/A | The minimum applicable value of K0 (K2) for an active DL (UL) BWP for the carrier where PDSCH(PUSCH) is transmitted FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |

Several companies suggested updating the agreed FG 19-1 (see summary in [2]):

Alt. 1:

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before the start of drx_onDurationTimer - Indication of UE whether or not to start drx_OnDuration timer for the next long DRX cycle by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured periodic CSI report apart from L1-RSRP when impacted by DCI format 2_6 that drx_OnDurationTimer does not start for the next long DRX cycle - Configured periodic L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start for the next long DRX cycle |  | Yes | N/A |  | Per UE | No | Yes |  | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the beginning of the slot where the UE would start the drx_onDurationTimer is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The candidate value set for 15kHz SCS: {1,3} slots - The candidate value set for 30kHz SCS: {1,6} slots - The candidate value set for 60kHz SCS: {1,12} slots - The candidate value set for 120kHz SCS: {2,24} slots UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2:

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration - Indication of UE whether or not to start drx_OnDurationTimer for the next long DRX by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured P-CSI / L1-SINR report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON - Configured periodic L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON | N/A | Yes | N/A |  | Per UE | No | Yes | N/A | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 3:

- Component (1) should be the (capability for) “detect format 2-6 during the PS-offset and before the minimum gap”. The current text “Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration” seems to only mean the capability for PS-offset.

Alt. 4:

- Add ‘reported’ before the minimum time gap for component (1)

- Capture the following agreement as new component: if the UE does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle, the UE shall start the drx-onDurationTimer for the next DRX cycle

Alt. 5:

- Update component (5) as follows: Configured periodic L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON

Companies are invited to provide their views on whether updates to the agreed FG 19-1 are needed in the following table. In case updates to the agreed FG 19-1 are supported, please provide all details.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | The alternatives are not completely mutually exclusive, and some elements can be combined in a final version. In our understanding the main mutually exclusive options are Alt.1 and Alt.2, where we have a preference for Alt.1. In any case we do not see the need for explicitly mentioned “long DRX” cycle in the component description and we would prefer to remove the word “long” there. The proposal for component 1 in Alt.3 is reasonable and we can support this kind of formulation. Alt. 4 should be clear enough from the context, and we don’t see the need for the extra component as that is clearly specification text and not a FG description. Alt.5 is already included in both Alt. 1 and Alt. 2, so no need to consider it separately if either Alt. 1 or Alt. 2 is chosen as baseline. |
| OPPO | We are supportive for more clear description for components. Alt 1 can be a starting point. We can incorporate Alt 3 into the Alt 1. |
| CATT | No need to update |
| Ericsson | OK with most component updates of Alt 1 (which has some overlap with Alt 2 and Alt 5), including the update in the note regarding the minimum time-gap values. - No need to add “long” in “long DRX cycle” in components 2,4,5 (spec is clear on this already) No strong view on Alt 3 though it should be already clear from specification and component 2. Alt 4 : New component is not needed - UE behavior is clear from the description and specification. |
| Samsung | We are OK to update. Alt 1 can be the starting point. New component in Alt 4 is not needed. |
| ZTE | We are supportive of ALT1. New component in ALT4 is not needed. |
| Panasonic | We are ok to alt 1. |
| MediaTek | We are OK to Alt 1. |
| Intel | We are Ok with Alt 1 |
| Huawei, HiSilicon | For Alt.1, - We are OK to replace ‘DRX ON’ with ‘drx_onDurationTimer’, since the latter one is the formal terminology. - We are supportive to add ‘periodic’ to (5) to make it aligned with the agreement. - We are OK to capture the candidate values for minimum gap in NOTE column. - However, we don’t agree to add ‘long’ before the DRX cycle. It is still under discussion in RAN2. For Alt.2, see our views for Alt.1. For Alt.3, we think it is reasonable. For Alt.4, - We are supportive to add ‘reported’ to make it clear. Note that ‘PS_offset’ is already clarified to be ‘configured’. - We are supportive to capture ‘if the UE does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle, the UE shall start the drx-onDurationTimer for the next DRX cycle’ as a new component. Because it is a part of the feature. For Alt.5, see our views for Alt.1. |
| Apple | We are fine with Alt 1 as a starting point for the discussion. |
| vivo | We are fine with Alt 1 as a starting point for the discussion. For alt 3,we would like to clarify if 19-1 is FR1/FR2 differential, whether capability of minimum time gap is reported per FR? |

# Conclusion

After further discussion by email on the RAN1 email reflector, the email discussion/approval was closed with the following agreement:

Agreements

| 19-1 | DRX Adaptation | - Configured PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI and reported minimum time gap before the start of drx_onDurationTimer DRX ON duration - Indication of UE whether or not to start drx_OnDuration timer at the DRX ON for the next DRX cycle by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active time - Configured periodic P-CSI/L1-SINR report apart from L1-RSRP when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON for the next DRX cycle - Configured periodic L1-RSRP report when impacted by DCI format 2_6 that drx_OnDurationTimer does not start at the DRX ON for the next DRX cycle |  | Yes | N/A |  | Per UE | No | Yes |  | The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the beginning of the slot where the UE would start the drx_onDurationTimer start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The candidate value set for 15kHz SCS: {1,3} slots - The candidate value set for 30kHz SCS: {1,6} slots - The candidate value set for 60kHz SCS: {1,12} slots - The candidate value set for 120kHz SCS: {2,24} slots - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to 3 ms UE is not required to monitor PDCCH for detection of DCI format 2_6 during the minimum time gap | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

- R1-2003195, Summary of Email Approval [100e-b-NR-UEFeatures-Remaining] — UE Power Saving Aspects, Moderator (AT&T)

- R1-2004286, Summary of UE features for UE power savings, Moderator (AT&T)