3GPP TSG RAN WG1 Meeting #101                                          	                    R1-2005100

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	              Test proposal from email discussion thread #02 for AI 7.2.4.3 sidelink synchronization mechanism

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

- Introduction

[101-e--NR-5G_V2X_NRSL-SYNC-02]

Email discussion regarding

- Issue 8: Start symbol of S-SSB

- Issue 17: In-device coexistence between LTE-V2X and NR-V2X

- Issue 5: Sidelink timing definition

Till 5/28. If consensus can be reached, any follow-up TP by 6/3 – Teng (CATT)

- TP 1

----------------------------------------------------Start of draft TP for 38.211----------------------------------------------------

8.4.3.1	Time-frequency structure of an S-SS/PSBCH block

In the time domain, an S-SS/PSBCH block consists of NsymbS-SSB OFDM symbols, numbered in increasing order from 0 to NsymbS-SSB-1 within the S-SS/PSBCH block, where S-PSS, S-SSS, and PSBCH with associated DM-RS are mapped to symbols as given by Table 8.4.3.1-1. The number of OFDM symbols in an S-SS/PSBCH block NsymbS-SSB=13 for normal cyclic prefix and NsymbS-SSB=11 for extended cyclic prefix. The first OFDM symbol in an S-SS/PSBCH block is the first OFDM symbol in the slot.

--------------------------------------------------< Unchanged parts are omitted >-------------------------------------------------

-----------------------------------------------------End of draft TP for 38.211-----------------------------------------------------

- TP 2

----------------------------------------------------Start of draft TP for 38.213----------------------------------------------------

16.2.x   Operation for in-device coexistence

If a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access occur to a UE, when

- the first channel/signal using E-UTRA radio access and the second channel/signal using NR radio access are time-division multiplexed, and

- the frame indexes of the first channel/signal using E-UTRA radio access and the frame indexes of the second channel/signal using NR radio access are known to the UE,

the UE transmits or receives the channel/signal so that the subframe boundary of the second channel/signal using NR radio access is aligned with the subframe boundary of the first channel/signal using E-UTRA radio access. It is up to UE implementation how to achieve subframe boundary alignment.

--------------------------------------------------< Unchanged parts are omitted >-------------------------------------------------

-----------------------------------------------------End of draft TP for 38.213-----------------------------------------------------

- Appendix: Corresponding Agreements of Email Thread#02

Agreements:

The first OFDM symbol in an S-SS/PSBCH block is mapped to the first OFDM symbol in the slot.

----------------------------------------------------Start of draft TP for 38.211---------------------------------------------------- 8.4.3.1 Time-frequency structure of an S-SS/PSBCH block In the time domain, an S-SS/PSBCH block consists of NsymbS-SSBNsymbS-SSB OFDM symbols, numbered in increasing order from 0 to NsymbS-SSBNsymbS-SSB - 1 within the S-SS/PSBCH block, where S-PSS, S-SSS, and PSBCH with associated DM-RS are mapped to symbols as given by Table 8.4.3.1-1. The number of OFDM symbols in an S-SS/PSBCH block NsymbS-SSBNsymbS-SSB = 13 for normal cyclic prefix and NsymbS-SSBNsymbS-SSB= 11 for extended cyclic prefix. The first OFDM symbol in an S-SS/PSBCH block is the first OFDM symbol in the slot. --------------------------------------------------< Unchanged parts are omitted >------------------------------------------------- -----------------------------------------------------End of draft TP for 38.211-----------------------------------------------------

Agreements:

- Adopt the following 38.213 TP

---

16.2.X   Operation for in-device coexistence

If a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access occur to a UE, when

- the first channel/signal using E-UTRA radio access and the second channel/signal using NR radio access are time-division multiplexed, and

- the frame indexes of the first channel/signal using E-UTRA radio access and the frame indexes of the second channel/signal using NR radio access are known to the UE,

the UE transmits or receives the channel/signal so that the subframe boundary of the second channel/signal using NR radio access is aligned with the subframe boundary of the first channel/signal using E-UTRA radio access. It is up to UE implementation how to achieve subframe boundary alignment.

--------------------------------------------------< Unchanged parts are omitted >-------------------------------------------------