3GPP TSG-RAN WG1 Meeting #101-e	R1-2005159

# e-Meeting, May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0092 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Cross-carrier Scheduling with Different Numerologies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-06-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections on Cross-carrier Scheduling with Different Numerologies |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sections 5.2.1.5.1 and 5.2.1.5.1a based on email discussion [100b-e-NR- LTE_NR_DC_CA-X-CC A-CSI-RS-01], as summarized in R1-2002841, the following changes have been made: - SCS-linked scaling to the beam switching timing d was missing in three separate instances (5.2.1.5.1a) - Supplement the description of d value for the case that the scheduling offset is equal to or greater than the threshold (5.2.1.5.1a). - Moving the paragraph for the case where te UE is not configured with minimumSchedulingOffset from 5.2.1.5.1 to 5.2.1.5.1a as it was unintentionally introduced to a wrong place against the intention agreed in RAN1#100e. In sections 5.2.1.5.1 based on email discussion [101-e-NR-LTE_NR_DC_CA-X-CC_A-CSI-RS], as summarized in R1-2004640, the following changes have been made: Introduced the the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier. Added a clause conditioning the default QCL assumption behaviour to the RRC configuration for cross-carrier A-CSI-RS triggering with the same SCS In sections 5.2.1.5.1a based on email discussion [101-e-NR-LTE_NR_DC_CA-X-CC_A-CSI-RS], as summarized in R1-2004640, the following changes have been made: Added a clause conditioning the default QCL assumption behaviour to the RRC configuration for cross-carrier A-CSI-RS triggering with different SCSs. As agreed earlier, for cross-carrier A-CSI-RS triggering with different SCS, RRC parameter [enableDefaultBeamForCCS] should be used to enable the default QCL assumption. In sections 5.2.1.5.1a based on email discussion [101-e-NR-LTE_NR_DC_CA-X-CC_A-CSI-RS], as summarized in R1-2004640, the following changes have been made: CORESET ID controlResourseSetID [minimumSchedulingOffset] minimumSchedulingOffsetK0 In sections 5.2.1.5.1a based on email discussion [101-e-NR-LTE_NR_DC_CA-X-CC_A-CSI-RS], as summarized in R1-2004640, the following changes have been made: Corrected the indentation of three bullets defining the default QCL assumption determination In sections 5.2.1.5.1a based on email discussion [101-e-NR-LTE_NR_DC_CA-X-CC_A-CSI-RS], as summarized in R1-2004640, the following changes have been made: Introduced of the Rel-16 RRC parameter name to provide the triggering offset. In sections 5.1.5 and 5.5 based on email discussion [100b-e-NR- LTE_NR_DC_CA-X-CC scheduling], as summarized in R1-2002842, the following changes have been made: - Specification terminology alignment when referring to a particular carrier’s SCS and when referring to the “slot of the PDSCH reception” - Adding the condition when the beam switching timing d is set to zero In sections 6.2.1 based on email discussion [100b-e-NR- LTE_NR_DC_CA-Unaligned_CA-01], as summarized in R1-2002780, the following changes have been made: - Extending the CA slot offset to apply to SRS triggered for positioning. In sections 5.1.2.1, 5.2.1.5.1a, 6.1.2.1, 6.2.1, replaced “CA-slot-offset” by “ca-SlotOffset”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect operation of cross-carrier scheduling with different numerologies |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1, 5.1.5, 5.2.1.5.1, 5.2.1.5.1a, 5.5, 6.1.2.1, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

#### 5.1.2.1	Resource allocation in time domain

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

Given the parameter values of the indexed row:

-	The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and  and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively,which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211].

<omitted text>

### 5.1.5	Antenna ports quasi co-location

<omitted text>

If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enableDefaultBeamForCCS]:

-	The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero;

-	For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell.

<omitted text>

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

<omitted text>

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}.

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

<omitted text>

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset.or aperiodicTriggeringOffsetExt-r16 The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. If the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

<omitted text>

##### 5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

<omitted text>

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH  in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

-	if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD',

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH  in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else,

-	if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else if the UE is configured with [enableDefaultBeamForCCS], when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH  in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

Table 5.2.1.5.1a-1: Additional beam switching timing delay d

| µPDCCH | d [PDCCH symbols] |
| --- | --- |
| 0 | 8 |
| 1 | 8 |
| 2 | 14 |

Aperiodic CSI-RS timing:

-	When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states.. The CSI-RS triggering offset has the values of {0, 1,…,31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16,

-	 μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

-	 Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and  μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] subclause 4.5

-	If the µPDCCH < µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than the first symbol of the CSI-RS carrier’s slot that starts at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

-	If the µPDCCH > µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

Table 5.2.1.5.1a: Ncsirs as a function of the subcarrier spacing of the triggering PDCCH

| µPDCCH | Ncsirs [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | [14] |

<omitted text>

## 5.5	UE PDSCH reception preparation time with cross carrier scheduling with different subcarrier spacings for PDCCH and PDSCH

This clause applies only if the PDCCH carrying the scheduling DCI is received on one carrier with one OFDM subcarrier spacing (µPDCCH), and the PDSCH scheduled to be received by the DCI is on another carrier with another OFDM subcarrier spacing (µPDCCH).

If the µPDCCH < µPDSCH, the UE is expected to receive the scheduled PDSCH, if the first symbol in the PDSCH allocation, including the DM-RS, as defined by the slot offset K0 and the start and length indicator SLIV of the scheduling DCI starts no earlier than the first symbol of the slot of the PDSCH reception starting at least Npdsch PDCCH symbols after the end of the PDCCH scheduling the PDSCH, not taking into account the effect of receive timing difference between the scheduling cell and the scheduled cell.

If the µPDCCH > µPDSCH, the UE is expected to receive the scheduled PDSCH, if the first symbol in the PDSCH allocation, including the DM-RS, as defined by the slot offset K0 and the start and length indicator SLIV of the scheduling DCI starts no earlier than Npdsch PDCCH symbols after the end of the PDCCH scheduling the PDSCH, not taking into account the effect of receive timing difference between the scheduling cell and the scheduled cell.

Table 5.5-1: Npdsch as a function of the subcarrier spacing of the scheduling PDCCH

| µPDCCH | Npdsch [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | 14 |

<omitted text>

#### 6.1.2.1	Resource allocation in time domain

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, and the number of repetitions (if numberofrepetitions is present in the resource allocation table) to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a CSI request field on a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where  are the corresponding list entries of the higher layer parameter

-	reportSlotOffsetListForDCI-Format0-2, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListForDCI-Format0-2 is configured;

-	reportSlotOffsetListForDCI-Format0-1, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListForDCI-Format0-1 is configured;

-	reportSlotOffsetList, otherwise;

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks =, otherwise, and where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively,

-	Nslot, offset, PDCCHCAand μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, PUSCHCA and μoffset,PUSCH are the Nslot, offsetCA and the,respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the PUSCH, as defined in clause 4.5 of [4, TS 38.211], and

-	for PUSCH scheduled by DCI format 0_1, if PUSCHRepTypeIndicator-ForDCIFormat0_1 is set to ‘pusch-RepTypeB’, the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. For PUSCH scheduled by DCI format 0_2, if PUSCHRepTypeIndicator-ForDCIFormat0_2 is set to ‘pusch-RepTypeB’, the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. Otherwise, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

<omitted text>

### 6.2.1	UE sounding procedure

<omitted text>

If the UE has an active semi-persistent SRS resource configuration and has not received a deactivation command, the semi-persistent SRS configuration is considered to be active in the UL BWP which is active, otherwise it is considered suspended.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2. Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 + 14. The minimal time interval in units of OFDM symbols is counted based on the minimum subcarrier spacing between the PDCCH and the aperiodic SRS.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and except when SRS is configured with the higher layer parameter [SRS-for-positioning], the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell,  Ks =, otherwise, and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	Nslot, offset, PDCCHCAand μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter [SRS-for-positioning], the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell,  Ks =, otherwise, and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

Nslot, offsetCA.

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	if the UE is configured with the higher layer parameter spatialRelationInfo containing the ID of a reference 'ssb-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo contains the ID of a reference 'csi-RS-Index', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo contains the ID of a reference 'srs', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS. When the SRS is configured by the higher layer parameter [SRS-for-positioning] and if the higher layer parameter spatialRelationInfo contains the ID of a reference ‘DL-PRS-ResourceId’, the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.xx of [10, TS 38.321], for an SRS resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ. [The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise.] When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

<omitted text>