3GPP TSG-RAN WG1 Meeting #101-e	R1-200xxxx

# e-Meeting, May 25th – June 5th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0040 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on 5G V2X sidelink features after RAN1#100bis-e and RAN1#101-e |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-06-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections based on text-proposals/agreements made at RAN1#100bis-e and RAN1#101-e including alignment across specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction based on email thread [100b-e-NR-5G_V2X_NRSL-PHYstructure-04] (as endorsed text proposal in R1-2003034); - Correction based on email thread [100b-e-NR-5G_V2X_NRSL-PHY-Procedure-03] (as endorsed text proposal in R1-2003009); - Correction based on email thread [100b-e-NR-5G_V2X_NRSL-Mode-1-04] (as endorsed text proposal in R1-2003110); - Correction based on email thread [101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-03] (as endorsed text proposal in R1-2004933); - Correction based on email thread [101-e-NR-5G_V2X_NRSL-Mode-1-02] (as endorsed text proposal in R1-2005007); - Correction based on email thread [101-e-NR-5G_V2X_NRSL-Mode-1-03] (as endorsed text proposal in R1-2005008); - Correction based on email thread [101-e-NR-5G_V2X_NRSL-PHYstructure-01] (as endorsed text proposal in R1-2005018); - Correction based on email thread [101-e-NR-5G_V2X_NRSL-PHYstructure-03] (as endorsed text proposal in R1-2005020); - Correction based on email thread [101-e-NR-5G_V2X_NRSL-PHYstructure-04] (as endorsed text proposal in R1-2005021); - SCI format naming modification: A SCI consists of two stages: 1st-stage SCI and 2nd-stage SCI, to be represented as SCI 1-X and SCI 2-X, with X being A, B, C… meaning different types. SCI 1-A means 1st-stage SCI Ath type; SCI 2-A means 2nd-stage SCI Ath type; SCI 2-B means 2nd-stage SCI Bth type, and so on. - Add 2nd-stage SCI formats field value table of SCI format 1-A in 8.3.1.1. - Reference alignment and editorial changes - For DCI format 3-0: - Time and Frequency domain resource allocation for SL are defined in 8.1.2.1 and 8.1.2.2 of TS 38.214 respectively. - Time and frequency resources of SL HARQ report are defined in 16.5 of TS 38.213. - For SCI format 1-A: - Priority is defined from TS 23.287. - Time and frequency domain resource allocation for SL are defined in 8.1.2.1 and 8.1.2.2 of TS 38.214 respectively. - Update number of bits for DMRS pattern field of SCI format 1-A in 8.3.1.1 redfined, based on entries of higher layer parameter sl-PSSCH-DMRS-TimePatternList. - For “Reserved”, of which number of bits are provided by the higher layer parameter, remove the value range of the higher layer parameter and any particular value assigned to the reserved value. - Table index alignment. - For SCI format 2-B: - Zone ID is determined at 5.8.1.1 of TS38.331 - For rate matching of 2nd-stage SCI - Replace GSCI2 by GSCI2 to align the notation used in section 8.2.1 Data and control multiplexing. - Replace 2 as QmSCI2 in the equation of QSCI2', to align the usage of the modulation order of 2nd-stage SCI elsewhere in TS38.212. - For PSBCH - Typo correction on “parameter” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of 5G V2X sidelink. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.3.1.1.2, 7.3.1.4.1, 8.1, 8.2, 8.2.1, 8.3.1.1, 8.4.1.1, 8.4.1.2(new clause), 8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

< Unchanged parts are omitted >

[10]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)"

[11]	3GPP TS 36.212: "Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding"

[12]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services"

< Unchanged parts are omitted >

## 3.3	Abbreviations

< Unchanged parts are omitted >

RI	Rank indicator

RSRP	Reference signal received power

SCI	Sidelink control information

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SL-BCH	Sidelink broadcast channel

# 7	Downlink transport channels and control information

< Unchanged parts are omitted >

7.3.1	DCI formats

< Unchanged parts are omitted >

7.3.1.1.2	Format 0_1

< Unchanged parts are omitted >

-	SCell dormancy indication – 0 bit if higher layer parameter Scell-groups-for-dormancy-within-active-time is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter Scell-groups-for-dormancy-within-active-time, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter Scell-groups-for-dormancy-within-active-time, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group. The field is only present when this format is carried by PDCCH on the primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

-	Sidelink assignment index – 0, 1 or 2 bits:

-	1 bit if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static and, in addition,  the UE is configured with a SL configured grant type 1 or to monitor DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI;

-	2 bits if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic and, in addition, the UE is configured with a SL configured grant type 1 or to monitor DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI;

-	0 bit otherwise.

A UE does not expect that the bit width of a field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_1 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_1 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell.

< Unchanged parts are omitted >

##### 7.3.1.4.1	Format 3_0

DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell.

The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI:

-	Resource pool index –log2I  bits, where I is the number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling.

-	Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214]

-	HARQ process number – log2Nprocess bits as defined in clause 16.4 of [5, TS 38.213]

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213]

-	Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits as defined in clause 8.1.2.2 of [6, TS 38.214]

-	SCI format 1-A fields according to clause 8.3.1.1:

-	Frequency resource assignment.

-	Time resource assignment.

-	PSFCH-to-HARQ feedback timing indicator –log2Nfb_timing bits , where Nfb_timing is the number of entries in the higher layer parameter sl-PSFCH-ToPUCCH, as defined in clause 16.5 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in clause 16.5 of [5, TS 38.213].

-	Configuration index – 0 bit if the UE is not configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI; otherwise 3 bits as defined in clause 8.1.2 of [6, TS 38.214]. If the UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, this field is reserved for DCI format 3_0 with CRC scrambled by SL-RNTI.

-	Counter sidelink assignment index – 2 bits

-	2 bits as defined in clause 16.5.2 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic

-	 2 bits as defined in clause 16.5.1 of [5, TS 38.213] if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static

-	Padding bits, if required

If multiple transmit resource pools are provided in sl-TxPoolScheduling, zeros shall be appended to the DCI format 3_0 until the payload size is equal to the size of a DCI format 3_0 given by a configuration of the transmit resource pool resulting in the largest number of information bits for DCI format 3_0.

If the UE is configured to monitor DCI format 3_1 and the number of information bits in DCI format 3_0 is less than the payload of DCI format 3_1, zeros shall be appended to DCI format 3_0 until the payload size equals that of DCI format 3_1.

< Unchanged parts are omitted >

8	Sidelink transport channels and control information

< Unchanged parts are omitted >

8.1	Sidelink broadcast channel

The processing for SL-BCH transport channel follows the BCH according to clause 7.1, with the following changes:

-	Clause 7.1.1 for PBCH payload generation is replaced by Clause 8.1.1.

-	Clause 7.1.2 for scrambling is not performed.

-	In clause 7.1.5, the rate matching output sequence length E = 1386 when higher layer parameter cyclicPrefix is configured, otherwise, E = 1782.

< Unchanged parts are omitted >

8.2	Sidelink shared channel

The processing for SL-SCH transport channel follows the UL-SCH according to clause 6.2, with the following changes:

-	Rate matching of SL-SCH follows the rate matching according to clause 6.2.5 by setting ILBRM=0

-	Clause 6.2.7 is replaced by clause 8.2.1

8.2.1	Data and control multiplexing

Denote the coded bits for SL-SCH as g0SL-SCH, g1SL-SCH, g2SL-SCH, g3SL-SCH,⋯, gGSL-SCH-1SL-SCH.

Denote the coded bits for the 2nd-stage SCI, as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2-1SCI2.

Denote the multiplexed data and control coded bit sequence as g0,g1,⋯,gG-1, where G is the total number of coded bits for transmission.

Assuming that NL is the number of layers onto which the SL-SCH transport block is mapped, the multiplexed data and control coded bit sequence g0,g1,⋯,gG-1 is obtained as follows:

Denote QmSCI2 is modulation order of the 2nd-stage SCI.

8.3	Sidelink control information on PSCCH

< Unchanged parts are omitted >

8.3.1	1st-stage SCI formats

< Unchanged parts are omitted >

8.3.1.1	SCI format 1-A

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority – 3 bits as defined in clause 5.4.3.3 of [12, TS 38.287].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.2.2 of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-MaxNumPerReserve is configured to 3, as defined in clause 8.1.2.1 of [6, TS 38.214].

-	Resource reservation period – log2(NreservPeriod) log2Nrsv_period bits as defined in clause 8.1.4 of [6, TS 38.214], where Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; 0 bit otherwise.

-	DMRS pattern – log2Npattern bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], where Npattern is the number of DMRS patterns configured by higher layer parameter sl-PSSCH-DMRS-TimePatternList; 0 bit if sl-PSSCH-DMRS-TimePatternList is not configured.

-	2nd-stage SCI format – 2 bits as defined in Table 8.3.1.1-1.

-	Beta_offset indicator – 2 bits as provided by higher layer parameter sl-BetaOffsets2ndSCI and Table 8.3.1.1-2.

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-3.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-	Additional MCS table indicator – as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl- Additional-MCS-Table; 0 bit otherwise.

-    PSFCH overhead indication – 1 bit as defined clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

-	Reserved – a number of  bits as determined by higher layer parameter sl-NumReservedBits.

Table 8.3.1.1-1: 2nd-stage SCI formats

| Value of 2nd-stage SCI format field | 2nd-stage SCI format |
| --- | --- |
| 00 | SCI format 2-A |
| 01 | SCI format 2-B |
| 10 | Reserved |
| 11 | Reserved |

Table 8.3.1.1-2: Mapping of Beta_offset indicator values to indexes in Table 9.3-2 of [5, TS38.213]

| Value of Beta_offset indicator | Beta_offset index in Table 9.3-2 of [5, TS38.213] |
| --- | --- |
| 00 | 1st index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 01 | 2nd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 10 | 3rd index provided by higher layer parameter sl-BetaOffsets2ndSCI |
| 11 | 4th index provided by higher layer parameter sl-BetaOffsets2ndSCI |

Table 8.3.1.1-3: Number of DMRS port(s)

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

< Unchanged parts are omitted >

8.4	Sidelink control information on PSSCH

< Unchanged parts are omitted >

8.4.1	2nd-stage SCI formats

< Unchanged parts are omitted >

8.4.1.1	SCI format 2-A

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number – log2Nprocess bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause 16.4 of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1.

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214].

Table 8.4.1.1-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast |
| 10 | Unicast |
| 11 | Reserved |

8.4.1.2	SCI format 2-B

SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-B:

-	HARQ process number – log2Nprocess bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause 16.4 of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Zone ID – 12 bits as defined in clause 5.8.1.1 of [9, TS 38.331].

-	Communication range requirement – 4 bits as defined in [9, TS 38.331]

< Unchanged parts are omitted >

8.4.4	Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols generated for 2nd-stage SCI transmission prior to duplication for the 2nd layer if present, denoted as QSCI2', is determined as follows:

QSCI2''=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)r=0CSL-SCH-1KrQmSCI2∙R, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the 2nd-stage SCI bits

-	LSCI2 is the number of CRC bits for the 2nd-stage SCI, which is 24 bits.

-	βoffsetSCI2 is indicated in the corresponding 1st-stage SCI.

CSL-SCH

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers;

-	MscDMRS(l) is the number of subcarriers in OFDM symbol l that carries DMRS, in the PSSCH transmission.

-	MscPT-RS(l) is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PSSCH transmission.

MscCSI-RS(l)l

-	MscSCI2(l) is the number of resource elements that can be used for transmission of the 2nd-stage SCI in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH-1NsymbolPSSCH and for NsymbolPSSCH=Nsymbsh-NsymbPSFCH, in PSSCH transmission, where NsymbshNsymbslot = sl-lengthSymbols - 2, where sl-lengthSymbols is the number of sidelink symbols within the slot provided by higher layers NsymbolPSSCHas defined in [6, TS 38.214]. If higher layer parameter sl-PSFCH-Period = 2 or 4,  NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise. If higher layer parameter sl-PSFCH-Period = 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) MscCSI-RS(l)

-	γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs.

Krr

-	R is the coding rate as indicated by "Modulation and coding scheme" field in SCI format 1-A.

-	α is configured by higher layer parameter sl-Scaling.

The input bit sequence to rate matching is d0, d1,d2,d3, ⋯,dN-1, where N is the number of coded bits.

Rate matching is performed according to Clause 5.4.1 by setting IBIL=1.

The output bit sequence after rate matching is denoted as g0SCI2, g1SCI2, g2SCI2, g3SCI2,⋯, gGSCI2-1SCI2, where GSCI2GSCI2=QSCI2'∙QmSCI2 and QmSCI2 is modulation order of the 2nd-stage SCI. A UE is not expected to have GSCI2>K.

< Unchanged parts are omitted >