3GPP TSG-RAN WG1 Meeting #101											R1-200xxxx

e-meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			Text proposal on the alignment of RAR terminology

Document for:	Discussion

Introduction

The following has been agreed by the first phase email discussion.

Conclusion:

- Separate the terminologies for RAR UL grant and fallbackRAR/successRAR UL grant in 38.213.

- Continue the discussion on the detailed TP based on TP#1c, to check if there are other places in 38.213 mentioning the RAR UL grant need to be modified.

TP for the above conclusion till 6/3

Apart from the original TP#1c in R1-2004836, two more places of “RAR UL grant” in section 4.2 and section 8 of 38.213 has been revised.

In addition, it seems there is no need to mention fallbackRAR in section 8.3 and 8.4, given the clarification in section 8.2A as follows:

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

…

TP for the terminology of RAR UL grant

Information for the cover page

Reasons for change

To align the terminologies for RAR UL grant and fallbackRAR/successRAR UL grant.

Summary of changes

Implement the above updates

Specs/Sections impacted

TS 38.213, Section 4.2, Section 8, and Section 11.1.

Text proposal

------------------------------------------Text proposal starts for TS 38.213---------------------------------------- 4.2 Transmission timing adjustments <Unchanged Text Omitted> For a timing advance command received on uplink slot and for a transmission other than a PUSCH scheduled by a RAR UL grant or a fallbackRAR UL grant as described in Clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in Clause 8.2A, the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot where , is a time duration in msec of symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, is a time duration in msec of symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214], is the maximum timing advance value in msec that can be provided by a TA command field of 12 bits, is the number of slots per subframe, and is the subframe duration of 1 msec. and are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers. For , the UE assumes [6, TS 38.214]. Slot and are determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG. is determined with respect to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and for all configured initial UL BWPs provided by initialUplinkBWP. The uplink slot is the last slot among uplink slot(s) overlapping with the slot(s) of PDSCH reception assuming TTA=0, where the PDSCH provides the timing advance command and TTA is defined in [4, TS 38.211]. <Unchanged Text Omitted> 8 Random access procedure <Unchanged Text Omitted> From the physical layer perspective, the Type-1 L1 random access procedure includes the transmission of random access preamble (Msg1) in a PRACH, random access response (RAR) message with a PDCCH/PDSCH (Msg2), and when applicable, the transmission of a PUSCH scheduled by a RAR UL grant, and PDSCH for contention resolution. From the physical layer perspective, the Type-2 L1 random access procedure includes the transmission of random access preamble in a PRACH and of a PUSCH (MsgA) and the reception of a RAR message with a PDCCH/PDSCH (MsgB), and when applicable, the transmission of a PUSCH scheduled by a fallbackRARRAR UL grant, and PDSCH for contention resolution. <Unchanged Text Omitted> 11.1 Slot configuration <Unchanged Text Omitted> If a UE is not configured to monitor PDCCH for DCI format 2_0, for a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, or a RAR UL grant, fallbackRAR UL grant, or successRAR <Unchanged Text Omitted> - 11.1.1 UE procedure for determining slot format <Unchanged Text Omitted> For a set of symbols of a slot, a UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols in the slot as downlink and to detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, DCI format 2_3, or a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot. <Unchanged Text Omitted> For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - if one or more symbols from the set of symbols are symbols in a CORESET configured to the UE for PDCCH monitoring, the UE receives PDCCH in the CORESET only if an SFI-index field value in DCI format 2_0 indicates that the one or more symbols are downlink symbols - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format indicating to the UE to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives PDSCH or CSI-RS in the set of symbols of the slot - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format , or a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot the UE transmits the PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible, and the UE does not detect a DCI format indicating to the UE to receive PDSCH or CSI-RS, or the UE does not detect a DCI format, or a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot, the UE does not transmit or receive in the set of symbols of the slot - if the UE is configured by higher layers to receive PDSCH or CSI-RS in the set of symbols of the slot, the UE receives the PDSCH or the CSI-RS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink - if the UE is configured by higher layers to receive DL PRS in the set of symbols of the slot, the UE receives the DL PRS in the set of symbols of the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as downlink or flexible. - if the UE is configured by higher layers to transmit PUCCH, or PUSCH, or PRACH in the set of symbols of the slot, the UE transmits the PUCCH, or the PUSCH, or the PRACH in the slot only if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as uplink - if the UE is configured by higher layers to transmit SRS in the set of symbols of the slot, the UE transmits the SRS only in a subset of symbols from the set of symbols of the slot indicated as uplink symbols by an SFI-index field value in DCI format 2_0 - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format, or a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot <Unchanged Text Omitted> For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, or a RAR UL grant, fallbackRAR UL grant, or successRAR - the UE receives PDCCH as described in Clause 10.1 <Unchanged Text Omitted> ------------------------------------------Text proposal ends for TS 38.213----------------------------------------

Any comments?

| Company | Comment |
| --- | --- |
| CATT | For 38.213, we need consider below sections should be changed. - 8.2ARandom access response - Type-2 random access procedure an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or We need have common understanding on that fallbackRAR and RAR UL grant are equivalent in section 8.2, 8.3 &8.4 - 7.1.1 UE behaviour If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3, - 9.2.1 PUCCH Resource Sets The UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3. For 38.214, only 1 place need be considered for changing. - 6.1.2.1 Resource allocation in time domain The minimum scheduling restriction is not applied when PUSCH transmission is scheduled by RAR UL grant for RACH procedure, or when PUSCH is scheduled with TC-RNTI. |

Appendix

Companies’ views for the issue in the first phase email discussion.

| Company | Alt.1 or Alt.2? And which TP is preferred? | Comments |
| --- | --- | --- |
| Samsung | Alt.1 TP#1c | It is true that the structure of RAR and fallback RAR is the same, however, since the concept of fallback RAR and successful RAR is new to 2step RACH, which is not multiplexed with RAR (in 4step RACH); we think it’s better the keep them separately. And TP#1c is preferred. |
| CATT | Alt.2 TP#1b | Because Alt.2 TP#1b can resolve misalignment of terminology for RAR between TS 38.213 and TS 38.214 and spec change for Alt. TP#1b is less than that for Alt.1. In addition, we suggest adding the word “success RAR” to section 11.1 in 38.213 in order to make spec more complete in Alt.2 TP#1b. Regarding TP#1b in 7.1.1.1, 38.213, we add “type-2 random access procedure” to judgment condition in order to apply PUSCH power control methodology related to TP in 7.1.1.1 for fallback case of 2-step RACH. |
| Ericsson | Alt.2 | No strong view, but since RAR UL grant is the same in msg2 and MsgB for fallback for scheduling msg3 which may be more reasonable to not separate them. Success RAR should be treated separately which only schedules PUCCH for MsgB ACK feedback. So, we slightly prefer alt2. |
| Intel | Alt. 1 TP#1c | In our view, it would be good to follow 214 for RAR terminology to make the spec clear. In this case, “RAR UL grant” can be updated to “RAR UL grant, fallbackRAR UL grant, or successRAR” |
| Spreadtrum | Alt. 1 TP#1c | Shared view as Samsung. In 38.321, it is described that Random Access Response (i.e. MAC RAR or fallbackRAR), it is better to keep them seperately. |
| OPPO | TP#1b | 1b is more concise. All these RARs including legacy RAR, fallback RAR, successful RAR are RAR. |
| NTT DOCOMO | Alt. 2 TP#1d | In 38.321, it is described that “if the uplink grant was received in a Random Access Response (i.e. in a MAC RAR or a fallback RAR)”. In our understanding, this means “RAR UL grant” includes both RAR UL grant for Type-1 random access procedure and fallback RAR UL grant for Type-2 random access procedure. On the other hand, success RAR does not have UL grant, and it is not included. We think that it would be good to have this clarification for the terminology of “RAR UL grant” in the spec. Thus, we prefer Alt. 2 TP#1d. |
| Nokia | Alt 1, TP#1c | It is more clear to have explicit mentioning of the two new response messages for the cases/situations where they are needed. This would leave no room for misunderstanding. |
| Apple | Alt1, TP#1c | Separating the UL grant and fallback RAR UL grant is clearer and no confusion, and it’s aligned with the spec 38.214 and 38.321. |
| Qualcomm | Alt 2 | RAN2 has decided to use the same RAR grant format for msg2 and fallback scenario of 2-step RACH. There is no need to introduce extra terminology to serve the same purpose. |
| Moderator (ZTE) |  | Alt.1 is supported by 5 companies, and all supporting companies prefer TP#1c; Alt.2 is supported by 5 companies, 2 companies prefer TP#1b, 1 company prefers TP#1d, and 2 companies did not mention the preference on the TPs. To my understanding, this is an editorial issue to make the terminology consistent among different RAN1 specs, and thus adopting either alternative will not have fundamental changes on the specification. So maybe we can collect views on the other way, i.e. is there any strong objection to either of the alternatives? If not, my preference is to adopt Alt.1, as it seems easier to converge on TP#1c. |
| CATT |  | In principal, we are fine with these two alternatives. But we need completed proposal on changing all of places on The terminology “RAR UL grant” for both alternatives. If we go to Alt1 with TP#1c, only section 11.1 in 38.213 is changed. Regarding Alt.1, TP#1c is incomplete proposal because total 28 places in the whole spec 38213 address ‘RAR UL grant’. Some places on ‘‘RAR UL grant’ in 38.213 need also be modified because RAR UL grant can’t include fallback RAR based on Alt.1 For example: - For Section 8.3 and section 8.4, fallback RAR procedure on 2s RACH will use these 2 sections, whether we need add fallback RAR to the corresponding places with RAR UL grant section 8.3 and section 8.4 or not? - Regarding 2sRACH general description in section 8 as below From the physical layer perspective, the Type-2 L1 random access procedure includes the transmission of random access preamble in a PRACH and of a PUSCH (MsgA) and the reception of a RAR message with a PDCCH/PDSCH (MsgB), and when applicable, the transmission of a PUSCH scheduled by a RAR UL grant, and PDSCH for contention resolution From the physical layer perspective, the Type-2 L1 random access procedure includes the transmission of random access preamble in a PRACH and of a PUSCH (MsgA) and the reception of a RAR message with a PDCCH/PDSCH (MsgB), and when applicable, the transmission of a PUSCH scheduled by a RAR UL grant, and PDSCH for contention resolution. Whether we need change a RAR UL grant to fallback RAR? - For 4.2 Transmission timing adjustments in 38.213, For a timing advance command received on uplink slot and for a transmission other than a PUSCH scheduled by a RAR UL grant as described in Clause 8.2A or 8.3, or a PUCCH with HARQ-ACK information in response to a successRAR as described in Clause 8.2A, - 7.1.1 UE behavior, If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in Clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH transmission scheduled by a RAR UL grant as described in Clause 8.3, …. For Alt.1, if a complete proposal is offered and both spec 38.213 and 38.214 can be considered, we agree with it. Regarding TP#1b in Alt2, we offer to complete proposal on RAR UL grant for both 38.213 and 38.214. |
| Intel |  | We support Alt. 1 and TP#1c. To CATT, we also think different alternatives would work, as it is for the RAR terminology alignment. But looking at TP#1b, it seems the updates are not trivial as this also tries to remove the all the fallbackRAR UL grant in 214. In our view, this makes the spec at least in 214 less readable. In addition, we noticed that some texts in 4.2 and 7.1.1 are also updated, which needs more careful assessment and more discussion. For the text in Section 8 as you mentioned, we are open to discuss whether we also need to include fallbackRAR UL grant. Again, we’d like to make the spec more clear and less confusion for the support of 2-step RACH. We do not think it is a good practice to simply remove fallbackRAR in all places. |
| Samsung |  | Agree with moderator’s proposal. And in response to comments from CATT, yes, if there is other place to check the need of adding fallbackRAR, we can do it. The critia is easy, if that part is for 4step RACH only, we don’t need to add; otherwise, we can add. We can figure out TP in next step (week), I think. |