# Revised TP#2 according to the comments from email discussion

---------------------------------- < Start of TP#2 for Clause 4.1 of 38.213 > ----------------------------------------

4	Synchronization procedures

4.1	Cell search

< Unchanged parts are omitted >

For operation with shared spectrum channel access, a UE assumes that transmission of SS/PBCH blocks in a half frame is within a discovery burst transmission window that starts from the first symbol of the first slot in a half-frame. The UE can be provided per serving cell by DiscoveryBurst-WindowLength-r16 a duration of the discovery burst transmission window. If DiscoveryBurst-WindowLength-r16 is not provided, the UE assumes that the duration of the discovery burst transmission window is a half frame. For a serving cell, the UE assumes that a periodicity of the discovery burst transmission window is same as a periodicity of half frames for receptions of SS/PBCH blocks in the serving cell. The UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the discovery burst transmission window and have candidate SS/PBCH blocks indices corresponding to SS/PBCH block indices provided by ssb-PositionsInBurst. If MSB k, k≥1, of ssb-PositionsInBurst is set to 1, the UE assumes that one SS/PBCH block among SS/PBCH block(s) within the discovery burst transmission window with candidate SS/PBCH block indices corresponding to SS/PBCH block index equal to k-1 may be transmitted; if MSB k is set to 0, the UE assumes that the SS/PBCH block(s) are not transmitted.

< Unchanged parts are omitted >

-------------------------------------------------- < End of TP#2> -----------------------------------------------------

# Revised TP#3 according to the comments from email discussion (merged with TP#13)

==================== Start of TP#3 for Clause 5.1.4 of TS 38.214 =======================

*** Unchanged text is omitted ***

5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of the candidate SS/PBCH blocks corresponding to SS/PBCH block indices provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213].

================================== End of TP#3 ============================

# Revised TP#4 according to the comments from email discussion

===================== Start of TP#4 for Clause 8.1 of TS 38.213 =======================

8.1	Random access preamble

------------------------------------------------------ Unchanged parts omitted ------------------------------------------------------

For unpaired spectrum,

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last SS/PBCH block reception symbol, where  is provided in Table 8.1-2.

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index  provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in Clause 4.1.

-	If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last downlink symbol and at least  symbols after a last SS/PBCH block symbol, where  is provided in Table 8.1-2, and if ChannelAccessType-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213]

-	the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in Clause 4.1.

-------------------------------------------------------- End of TP#4 ------------------------------------------------------------------

# Revised TP#5 according to the comments from email discussion

----------------------------------------------- Start of TP#5 for Clause 11.1.1 in TS 38.213 -------------------------------------

11.1.1	UE procedure for determining slot format

------------------------------------------------------ Unchanged parts omitted ------------------------------------------------------

For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indices indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in Clause 4.1, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

-------------------------------------------------------- End of TP#5 ------------------------------------------------------------------

# Revised TP#6 according to the comments from email discussion

----------------------------------------------- Start of TP#6 for Clause 9.2.3 in TS 38.213 -------------------------------------

9.2.6	PUCCH repetition procedure

------------------------------------------------------ Unchanged parts omitted ------------------------------------------------------

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, as described in Clause 4.1.

-------------------------------------------------------- End of TP#6 ------------------------------------------------------------------

# Revised TP#7 according to the comments from email discussion

===========================Start of TP#7 for Clause 5 in TS 38.213===========================

5	Radio Link Monitoring

================================ Unchanged Texts Omitted ==============================

For operation with shared spectrum channel access, when a UE is provided a SS/PBCH block index by ssb-Index, the UE is expected to perform radio link monitoring using SS/PBCH block(s) in the discovery burst transmission window as described in Clause 4.1, where the SS/PBCH block(s) have candidate SS/PBCH block index(es) corresponding to SS/PBCH block index provided by ssb-Index.

================================ Unchanged Texts Omitted =============================

================================= End of TP#7 for TS 38.213 =============================

# Revised TP#12 according to the comments from email discussion

==================== Start of TP#12-A for Clause 4.1 of TS 38.213 =======================

*** Unchanged text is omitted ***

The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to Lmax-1, where Lmax is determined according to SS/PBCH block patterns for Cases A through E. Lmax is a maximum number of SS/PBCH block indices in a cell, and

-	for operation without shared spectrum channel access, Lmax=Lmax

-	for operation with shared spectrum channel access, Lmax=8 for Lmax=10 and 15 kHz SCS of SS/PBCH blocks and for Lmax=20 and 30 kHz SCS of SS/PBCH blocks. The maximum number of transmitted SS/PBCH blocks within a discovery burst transmission window is 8.

For Lmax=4, a UE determines the 2 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211].

*** Unchanged text is omitted ***

================================== End of TP#12 ============================

==================== Start of TP#12-B for Clause 4.1 of TS 38.213 =======================

*** Unchanged text is omitted ***

The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to Lmax-1, where Lmax is determined according to SS/PBCH block patterns for Cases A through E. Lmax is a maximum number of SS/PBCH block indices in a cell, and the maximum number of transmitted SS/PBCH blocks within a half frame is Lmax.

-	For operation without shared spectrum channel access, Lmax=Lmax

-	For operation with shared spectrum channel access, Lmax=8 for Lmax=10 and 15 kHz SCS of SS/PBCH blocks and for Lmax=20 and 30 kHz SCS of SS/PBCH blocks

For Lmax=4, a UE determines the 2 LSB bits of a candidate SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH as described in [4, TS 38.211].

*** Unchanged text is omitted ***

================================== End of TP#12 ============================