3GPP TSG RAN WG1 Meeting #101-e                    	R1-200xxxx

e-Meeting, May 25 – June 5, 2020

Agenda Item:	7.2.2.2.2

Source:	Charter Communications

Title:	Draft [101-e-NR-unlic-NRU-InitAccessProc-02]

Document for:	Discussion and Decision

# Introduction

Four email discussions have been sanctioned in RAN1#101-e on initial access procedures for NR-U based on contributions submitted to this meeting [1]-[13]. This second discussion that aims to converge by 5/29 has the following scope:

[101-e-NR-unlic-NRU-InitAccessProc-02] Email discussion/approval on the following issues from R1-2003306 until 5/29; if necessary endorse associated TPs by 6/4 – Amitav (Charter)

- (#3.2) For the minimum value N of the MsgA PRACH-PUSCH gap in NR-U, select one of the alternatives in RAN1 #101-e:

  - Alt 1: N=2 (same as licensed operation)

  - Alt 2: N can be smaller than 2 for identified cases, and N=2 for the rest

    - For example, N = [0 or 1] when MsgA PUSCH has the same SCS and bandwith as MsgA PRACH

    - Note: it can be further discussed whether existing CP extension scheme can be applicable for the identified cases

These issues have been selected based on the preparatory discussion summarized in [14].

# Company views

It is proposed to first converge on either Alt. 1 or Alt. 2 before discussing TPs.

| Company | Views |
| --- | --- |
| Samsung | Support Alt. 1 Alt.2 can be beneficial over Alt.1 only when all the following conditions are satisfied: - Single beam operation or PRACH format B4 with particular RACH configuration (not more than 50%) - Same SCS and bandwidth of PRACH and PUSCH - LBT for PRACH succeeds but LBT for PUSCH fails in Alt.1 From our perspective, the chance of all the above conditions being satisfied is very low, so Alt.2 is just an enhancement for a corner case, which is not essential at this stage. Also note that Alt.2 introduces bias of SSBs in the case of multiple ROs in a slot, and leads to different operation on licensed and unlicensed spectrum (an implementation change comparing to licensed spectrum), with the only benefit of single LBT procedure. More detailed reasoning can be found in our contribution R1-2003861. |
| ZTE | We slightly prefer Alt2. Alt2 does provide some benefits to NRU of single LBT although the configuration choice is limited. Actually, from my understanding, limited configuration doesn’t mean limited use cases, gNB could always use such preamble configurations for NRU if applicable. Besides, for Format B4, there is only one RO in a slot, so there is no such bias of SSBs. In addition, with the condition that SCS and bandwidth of PRACH and PUSCH are the same, the UE implementation complexity could be further reduced. |
| Nokia, NSB | We prefer Alt2 and support ZTE’s views. In particular we believe that the use of CP extension may be beneficial on top of Alt1. |

# References

R1-2003371	Remaining issues on initial access procedure for NR-U	vivo

R1-2003451	Remaining issues on the initial access procedure for NR-U	ZTE, Sanechips

R1-2003513	Maintenance on the initial access procedures	Huawei, HiSilicon

R1-2003657	Remaining issues on initial access procedure for NR-U operation	MediaTek Inc.

R1-2003729	Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

R1-2003844	Enhancements to initial access procedures	Ericsson

R1-2003861	Initial access procedures for NR-U	Samsung

R1-2003973	Remaining issues on initial access procedure for NR-U	ETRI

R1-2004001	Remaining issues on initial access procedure	Spreadtrum Communications

R1-2004014	Remaining issues of initial access and mobility for NR-U	LG Electronics

R1-2004086	Discussion on the remaining issues of enhancements to initial access procedure	OPPO

R1-2004444	TP for Initial access and mobility procedures for NR-U	Qualcomm Incorporated

R1-2004526	On Enhancements to Initial Access Procedure for NR-U	Nokia, Nokia Shanghai Bell

R1-2003306     Feature lead summary on for initial access procedures enhancements	Charter Communications