3GPP TSG RAN WG1 #101-e			R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Title:	[DRAFT] Reply LS on NR-U SSB monitoring capabilities

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN1

To:	RAN4

Cc:	-

Contact Person:

Name:	Michel ROBERT

E-mail Address:	michel.robert@nokia.com

1. Overall Description:

RAN1 would like to thank RAN4 for their LS [1] related to NR-U SSB monitoring capabilities.

Related to the four questions asked by RAN4, RAN1 feedback is as follows.

[Question 1] Provide feedback whether monitoring within a given discovery burst transmission window all candidate SS/PBCH block indexes corresponding to the same SS/PBCH block index is mandatory for UEs.

[RAN1 answer] During RAN1 discussion, we did not reach consensus on how to set N1 and N2 values. However, it is RAN1 understanding that RAN4 may choose not to define different RLM/RRM performance requirements corresponding to different N1/N2 capabilities. Hence, assuming a single RLM/RRM performance requirement, the introduction of N1/N2 UE capabilities signalling is not necessary. It is RAN1 understanding that how many candidate SS/PBCH block indexes corresponding to the same SS/PBCH block index the UE should monitor in a given discovery burst transmission window can be left as UE implementation, as long as the single RLM/RRM performance requirement is met.

As a consequence, RAN1 has agreed that from RAN1 perspective, N1 and N2 should not be defined as UE capabilities signalling.

[Question 2] Provide feedback on the values of N1 and N2, considering the impact on the network performance if UEs are not monitoring all candidate positions.

[RAN1 answer]

See answer to question 1.

[Question 3] Provide feedback on whether differentiation is needed for UEs operating in FBE and LBE modes.

[RAN1 answer] See answer to question 2.

[Question 4] Provide feedback for the case when Q is not provided to the UE

[RAN1 answer] For both RRM and RLM/BFD/CBD measurements, Q is always provided to the UE. More details of the indication of Q can be found in R1-2003044 [2].

2. Actions:

To RAN4.

ACTION: RAN1 respectfully ask RAN4 to take the above answers into account.

3. References

[1] R1-2003274/R4-2005418, “LS on NR-U SSB monitoring capabilities”, Nokia, RAN4

[2] R1-2003044, “LS on Signalling of Q Parameter for NR-U”, Charter Communications, RAN1

4. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #102				24th – 28th August 2020		e-Meeting

TSG-WG1 Meeting #102bis			12th – 16th October 2020		e-Meeting