3GPP TSG RAN WG1 #101		                         R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.2.2.5

Source: 	Moderator (LG Electronics)

Title: 	Summary on maintenance of wide-band operation for NR-U

Document for:	Discussion and decision

- Introduction

This is the summary document for 7.2.2.2.5 on remaining issues of wide-band operation for NR-U, based on the contributions listed in reference section. The identified topics are categorized into three issues, as follows:

- Issue A: RB set and intra-cell guard bands

- Issue B: CORESET and search space configuration

- Issue C: DL reception or UL transmission on RB set(s)

Further details for the above issues and preliminary views on the priority for each sub-issue are provided in Sections 2 to 4. The priority for each specific issue is summarized in Section 5. Text proposals corresponding to sub-issues are collected in Appendix A.

- Issue A: RB set and intra-cell guard bands

Issue A1: No guard band configuration

One open issue is 1) how to configure a DL cell without intra-cell guard bands and 2) how to define RB set for the DL cell.

For 1) how to configure a DL cell without intra-cell guard bands, several companies (vivo [1] and Sharp [10]) suggested to configure the size of all intra-cell guard bands as zero, same as for a UL cell without intra-cell guard bands, while one company (LG Electronics [7]) suggested to be left up to RAN2 decision.

For 2) how to define RB set for a DL cell without intra-cell guard bands, one company (LG Electronics [7]) suggested not to define RB set for the DL cell.

One company (MediaTek [4]) proposed text proposals for PDCCH monitoring and CSI-RS reception behaviours based on RB set indicator, which may need to add the description considering RB set indicator has one bit field for a DL cell without intra-cell guard bands.

In addition, one company (Ericsson [5]) addressed an issue on how to configure a DL/UL cell with a single RB set (i.e., 20 MHz cell). The issue can be resolved by using default RB set configuration (defined in 38.101-1) and defining only RB set index 0 for UL carrier configured with interlaced PUSCH/PUCCH.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High for DL cell without intra-cell guard bands | Clarification for 20 MHz carrier can be discussed if current specification is not clear. |
| MediaTek | High for the some identified issues | High: PDCCH monitoring and CSI-RS reception behaviours based on RB set indicator, spec has to be corrected to align with RAN1 agreement for a DL cell without intra-cell guard bands. Low: How to configure a DL cell without intra-cell guard bands could be left up to RAN2 decision. High: How to define a single RB set for a 20 MHz UL cell should be discussed in this meeting |
| Huawei, HiSilicon | Low | It was already clear that single RB set for DL if no intra cell guard configured. For uplink, defining a single RB set for a 20 MHz UL cell could be clarified. |
| OPPO | High | Spec for DL RB set configuration is needed in the case of zero guard band, because otherwise the DCI 0_0 in CSS design will not be complete |
| Lenovo, Motorola Mobility | High | We think RB set configuration for resource allocation is important in case of no intra-cell guard band configured. |
| Sharp | High for DL cell without intra-cell guard bands | For 1), the issue impacts on PUSCH resource allocation in DCI format 0_0 in CSS. That’s why it’s essential. For 2), currently it is not agreed how to indicate a DL carrier without intra-cell guard bands. The last meeting agreement is only for UL carrier. For 3), RAN4 default guard bands doesn’t include intra-cell guard bands for 20 MHz carrier. No signalling is necessary for that. |

Issue A2: BWP configuration for UL cell without intra-cell guard bands

To figure out FFS point (FFS: Whether BWP can be configured to be partially overlapping with a RB set) for the agreement made in RAN1#100bis-e,

- Alt 1: UL BWP can be configured to include parts of a RB set, with some restriction such as at least 10 RBs in [at least one or each] RB set.

  - Supported by vivo [1], ZTE [2], Ericsson [5], Samsung [6], LG Electronics [7], Sharp [10]

- Alt 2: The UE does not expect that the BWP is configured to include parts of a RB set, same as for a carrier with intra-cell guard bands.

  - Supported by Huawei [3], Nokia [9]

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High |  |
| Huawei, HiSilicon | High |  |
| OPPO | High |  |
| Lenovo, Motorola Mobility | High |  |
| Sharp | High |  |

Issue A3: Details on RRC parameters for intra-cell guard bands

Several companies (Ericsson [5] and Nokia [9]) suggested to add intraCellGuardBandUL-r16 or intraCellGuardBandDL-r16 to ServingCellConfig IE, in addition to ServingCellConfigCommon IE which is captured in current TS 38.331 specification.

One company (LG Electronics [7]) addressed an issue to resolve FFS values for RRC parameters regarding RB set configuration. As captured below from running CR for TS 38.331, it is observed that there are still FFS points for RRC parameters related to intra-cell guard band configuration.

IntraCellGuardBand-r16 ::=         SEQUENCE (SIZE (1..ffsValue)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell

GuardBand-r16       ::= SEQUENCE {

startCRB-r16         INTEGER (0..ffsValue), --FFS upper range 275

nrofCRBs-r16         INTEGER (0..ffsValue)

}

For IntraCellGuardBand-r16,

- The number of entries of GuardBand-r16 can be up to 4 considering 100 MHz is the maximum carrier bandwidth for FR1.

For GuardBand-r16,

- The value range of startCRB-r16 is from 0 to 2474 (=2199+275), considering that

  - The value range of offsetToCarrier RRC parameter indicating offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on the carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) is defined from 0 to 2199 (=275*8-1), and

  - The value range of carrierBandwidth RRC parameter indicating the width of a carrier in number of PRBs is defined from 1 to 275

- The value range of nrofCRBs-r16 is from 0 to [10], considering the agreement made in RAN1#100bis-e and the table below captured from RAN4 agreement made in RAN4#93.

| 15 kHz |  | 30 kHz |  | 60 kHz |  |
| --- | --- | --- | --- | --- | --- |
| Usable RBs | Guard RBs | Usable RBs | Guard RBs | Usable RBs | Guard RBs |
| 104, 105 | 6, 7 | 49, 50, 51 | 5, 6, 7 | [22, 23, 24] | [3, 4, 5] |

Therefore, above RRC parameters can be updated as follows:

IntraCellGuardBand-r16 ::=         SEQUENCE (SIZE (1..4)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell

GuardBand-r16       ::= SEQUENCE {

startCRB-r16         INTEGER (0..2474), --FFS upper range 275

nrofCRBs-r16         INTEGER (0..[10])

}

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High for value ranges of RRC parameters | Detailed value ranges can be changed based on discussion. It would be preferable that RAN1 determine value ranges for those RRC parameters and recommend to RAN2 what RAN1 decide. Whether to add intraCellGuardBandUL-r16 or intraCellGuardBandDL-r16 to ServingCellConfig IE can be decided by RAN2. |
| MediaTek | High for all identified issues | High: Inform RAN2 to add intraCellGuardBandUL-r16/ intraCellGuardBandDL-r16 to ServingCellConfig IE High: Decide the detailed value ranges and inform RAN2 the RAN1 decision |
| OPPO | High | This can be discussed together with Issue A4 |
| Lenovo, Motorola Mobility | High |  |
| Sharp | High |  |

Issue A4: UE capability for intra-cell guard bands

One company (OPPO [8]) suggested to define NEW UE capability to support guard band size smaller than default guard band size. In other words, a UE without this capability only supports guard band size not less than default guard band size defined in RAN4 specification.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | High | Need to resolve this issue since it’s related to UE capability signalling. |
| MediaTek | High | Capability signalling may not be needed. Instead, specify that UE doesn't expect size of intra-cell guard band is smaller than the default guard band size defined in RAN4 when intra-cell guard band is configured. |
| Huawei, HiSilicon | Low | The proposal is not clear. To our understanding, RAN4 will define minimum requirement. How will UE meet inband leakage if configured intra cell guard is smaller than the minimum requirement? Or is it talking about the capability to configure non-zero guard? |
| OPPO | High | Agree with FL Answer to HW’s comment: our concern is exactly your first question, i.e. how will UE meet in-band leakage if the configured guard band is smaller than the minimum requirement. |
| Sharp | Low | Can be discussed in UE feature AI. |

Issue A5: Corrections for TS 38.214

Several companies proposed to capture the previous RAN1 or RAN2 agreements.

- LG Electronics [7]: To capture RAN1 agreements for a UL cell without intra-cell guard bands.

- OPPO [8]: To capture RAN1 agreement that the configured RB set shall not be partially overlapped with a LBT subband in unlicensed spectrum.

- Nokia [9]: To remove brackets sub-clause 7 in TS38.214

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | For LG’s TP, it seems better to handle the TP once all of related remaining issues settle down. For OPPO’s TP, the understanding is that each RB set corresponds to a channel defined in TS 37.213, which may need to be clarified. For Nokia’s TP, it seems editorial and can be provided to the editor directly. |
| MediaTek | High for Nokia’s TP Low for other TPs |  |
| Huawei, HiSilicon | Low | It should depends on the conclusion of other topic. |
| OPPO | Low | We are fine to down-prioritize our proposed correction for other more urgent topics. Answer to FL’s comment: thank you, but we fail to find that the current spec captures the clarification we raised. |
| Lenovo, Motorola Mobility | Low | Agree with FL proposal |
| Sharp | High for LG, Nokia | For LG’s proposal. to capture the agreement at the last RAN1 meeting should be made at this meeting. For OPPO’s proposal, we think it should be discussed. However, we also think that it should be discussed once the RB-set configuration is finalized. For Nokia’s proposal, to capture the agreement at the last RAN2 meeting should be made at this meeting. |

- Issue B: CORESET and search space configuration

Issue B1: Dropping rule for search space configured with freqMonitorLocations-r16

| Company | Views |
| --- | --- |
| Huawei [3] | Proposal 2: UE can drop PDCCH BD per monitoring location in a search space. The corresponding text proposal is in TP#1 in the appendix. |
| Panasonic [12] | Proposal 1: If DCI format 2_0 indicates one or more RB sets are unavailable, starting from a first slot that is at least P symbols after the last symbol of GC-PDCCH carrying DCI format 2_0, PDCCH candidates and the corresponding non-overlapping CCEs in the indicated unavailable RB sets are not allocated. |

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | TP proposed by Huawei [3] seems optimization. TP proposed by Panasonic [12] is understood that PDCCH overbooking rule can be impacted by dynamic information from RB set indicator, which is not aligned with the understanding that PDCCH overbooking rule is semi-statically pre-determined based on search space set configuration. |
| MediaTek | Low |  |
| Huawei, HiSilicon | High | If not specified, the many PDCCH candidate will be wasted especially when search spaces with 4 or 5 monitoring locations. |
| OPPO | Low | Agree with FL |
| Lenovo, Motorola Mobility | Low | Agree with FL proposal |
| Sharp | Low | Same view as LG |

Issue B2: Clarifications for CORESET configuration

| Company | Views |
| --- | --- |
| Samsung [6] | Proposal 2: A UE does not expect a CORESET to be associated with a search space set configured with freqMonitorLocations-r16 and another search space set not configured with freqMonitorLocations-r16 Proposal 3: When a configured RB set contains different size of RBs than RB set 0, select one of the alternatives to resolve potential CORESET resource bitmap misalignment in the configured RB set: - Alternative 1: UE ignores the PDCCH candidates which are not fully overlapped with the RB set. - Alternative 2: UE does not expect a CORESET configuration which has CORESET resource not confined within any of the configured RB set in freqMonitorLocations-r16 |

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | Proposal 2 is not necessary. According to the agreement made in RAN1#100-e meeting, if a CORESET is configured with at least one search space with freqMonitorLocations-r16, the CORESET f-domain resource is determined based on rb-Offset-r16 (or default value 0 if not provided) regardless of whether the associated search space is configured with freqMonitorLocations-r16 or not. For Proposal 3, a clarification can be needed. The understanding is that gNB has to (or UE is expected to) set ‘0’ for RBs outside of the first RB set if the corresponding CORESET would be associated with a search space with freqMonitorLocations-r16. |
| MediaTek | High (for proposal 3 in [6]) |  |
| Huawei, HiSilicon | Low | The agreement mentioned by LG already implied that a CORESET can be associated with SS set with freqMonitorLocations-r16 and SS set without freqMonitorLocations-r16. |
| OPPO | High | Proposal 3 should be clarified. |
| Lenovo, Motorola Mobility | Low | Agree with FL proposal |
| Sharp | Low | Same view as LG |

Issue B3: Corrections for TS 38.213

One company (Huawei [3]) suggested a text proposal to capture the agreement that the number of PDCCH candidates per aggregation level configured by nrofCandidates or nrofCandidates-SFI within a SearchSpace IE applies to each of RB sets configured by freqMonitorLocations-r16.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | It was clarified by spec editor in the previous meeting that the corresponding agreement is already captured in current specification. |
| MediaTek | Low |  |
| Huawei, HiSilicon | High | After further check part of 213, the description of ms,nCI and Ms,nCIL for PDCCH candidate calculation did not reflect the agreement. |
| Lenovo, Motorola Mobility | Low | Agree with FL proposal |
| Sharp | Low |  |

- Issue C: DL reception or UL transmission on RB set(s)

Issue C1: RB set indicator in DCI format 2_0

One company (Qualcomm [11]) suggested the followings regarding RB set indicator in DCI format 2_0 and provided the corresponding text proposal for TS 38.213,

- All zero state of RB set indicator in DCI format 2_0 is considered as unknown.

- If RB set indicator is not configured, UE considers all RB sets are available for DL reception when DCI format 2_0 is detected.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | It would be preferable to discuss under DL signal/channel agenda item (7.2.2.1.2), if deemed necessary. |
| MediaTek | Low |  |
| OPPO | Low | Agree with FL |
| Lenovo, Motorola Mobility | Low | Agree with FL proposal |
| Sharp | Low | Same view as LG. |

Issue C2: CSI-RS configuration

One company (OPPO [8]) proposed that the configured CSI-RS resource for tracking in unlicensed band shall be restricted within a RB set.

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | Up to gNB configuration |
| MediaTek | Low |  |
| Huawei, HiSilicon | Low |  |
| OPPO | Low |  |
| Lenovo, Motorola Mobility | Low |  |
| Sharp | Low |  |

Issue C3: PUSCH transmission

| Company | Views |
| --- | --- |
| Huawei [3] | Proposal 4: The intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets will result zero frequency resource allocation. The text should be reworded as TP#2. Proposal 5: For resource allocation type 0, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBG(s) and union of RB set(s) overlapping with the indicated RBG(s) and intra cell guard bands between the adjacent RB sets overlapping with the indicated RBG(s), if any. The corresponding text proposal is in TP#2 in the appendix. |

Comments for priority

| Company | Priority (High or Low) | Comments |
| --- | --- | --- |
| LG Electronics | Low | It would be preferable to discuss under UL signal/channel agenda item (7.2.2.1.3), if deemed necessary. |
| MediaTek | Low |  |
| Huawei, HiSilicon | High | The description in the current spec read problematic. Fine to discuss in UL. |
| OPPO | Low |  |
| Lenovo, Motorola Mobility | Low | Agree with FL proposal. |
| Sharp | Low | Same view as LG. |

- Summary on the priority for the remaining issues

| Issue | HIGH priority | LOW priority |
| --- | --- | --- |
| Issue A: RB set and intra-cell guard bands |  |  |
| Issue A1: No guard band configuration | LG Electronics (for DL cell without intra-cell guard bands), MediaTek (PDCCH monitoring and CSI-RS reception behaviours based on RB set indicator, define a single RB set for a 20 MHz UL cell), OPPO, Lenovo, Motorola Mobility, Sharp (for DL cell without intra-cell guard bands) | MediaTek (how to configure a DL cell without intra-cell guard bands), Huawei, HiSilicon, |
| Issue A2: BWP configuration for UL cell without intra-cell guard bands | LG Electronics, Huawei, HiSilicon, OPPO, Lenovo, Motorola Mobility, Sharp |  |
| Issue A3: Details on RRC parameters for intra-cell guard bands | LG Electronics (for value ranges of RRC parameters), MediaTek (for all identified issues), OPPO (discuss together with A4), Lenovo, Motorola Mobility, Sharp |  |
| Issue A4: UE capability for intra-cell guard bands | LG Electronics, MediaTek (for discussion on the identified issue), OPPO | Huawei, HiSilicon, Sharp |
| Issue A5: Corrections for TS 38.214 | MediaTek (Nokia’s TP), Sharp (LG’s TP and Nokia’s TP) | LG Electronics, MediaTek (others), Huawei, HiSilicon, OPPO, Lenovo, Motorola Mobility |
| Issue B: CORESET and search space configuration |  |  |
| Issue B1: Dropping rule for search space configured with freqMonitorLocations-r16 | Huawei, HiSilicon | LG Electronics, MediaTek, OPPO, Lenovo, Motorola Mobility, Sharp |
| Issue B2: Clarifications for CORESET configuration | High (for proposal 3 in [6]), OPPO (proposal 3) | LG Electronics, Huawei, HiSilicon, Lenovo, Motorola Mobility, Sharp |
| Issue B3: Corrections for TS 38.213 | Huawei, HiSilicon | LG Electronics, MediaTek, Lenovo, Motorola Mobility, Sharp |
| Issue C: DL reception or UL transmission on RB set(s) |  |  |
| Issue C1: RB set indicator in DCI format 2_0 |  | LG Electronics, MediaTek, OPPO, Lenovo, Motorola Mobility, Sharp |
| Issue C2: CSI-RS configuration |  | LG Electronics, MediaTek, Huawei, HiSilicon, OPPO, Lenovo, Motorola Mobility, Sharp |
| Issue C3: PUSCH transmission | Huawei, HiSilicon (can be discussed in UL) | LG Electronics, MediaTek, OPPO, Lenovo, Motorola Mobility, Sharp |

- Reference

- R1-2003374	Remaining issues on wideband operation in NR-U	vivo

- R1-2003454	Remaining issues on the wideband operation for NR-U	ZTE, Sanechips

- R1-2003516	Maintenance on the wideband operation procedures	Huawei, HiSilicon

- R1-2003659	Remaining issues on wideband operation for NR-U	MediaTek Inc.

- R1-2003847	Wideband operation	Ericsson

- R1-2003864	Wide-band operation for NR-U	Samsung

- R1-2004017	Remaining issues of wide-band operation for NR-U	LG Electronics

- R1-2004089	Discussion on the remaining issues of wide-band operations	OPPO

- R1-2004256	Remaining issues on Wideband operation in NR-U	Nokia, Nokia Shanghai Bell

- R1-2004324	Remaining issues on wideband operation for NR-U	Sharp

- R1-2004447	TP for Wideband operation for NR-U operation	Qualcomm Incorporated

- R1-2004511	Remaining issues on Rel-16 NR-U wideband operations	Panasonic

Appendix A: Text proposals corresponding to sub-issues

Issue A1

From MediaTek [4],

================================Text Proposal 1 Starts================================== ### 10 UE procedure for receiving control information <Unchanged parts are omitted> If a UE is provided availableRB-SetPerCell-r16 for a serving cell, and - if intraCellGuardBandDL-r16 for the serving cell indicates no intra-cell guard-bands are configured, the UE is not required to monitor PDCCH candidates on the serving cell that are indicated as unavailable for receptions by DCI format 2_0 as described in Clause 11.1.1. - if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets on the serving cell that are indicated as unavailable for receptions by DCI format 2_0 as described in Clause 11.1.1. <Unchanged parts are omitted> =============================== Text Proposal 1 Ends=================================== ================================Text Proposal 2 Starts================================== ### 11.1.1 UE procedure for determining slot format <Unchanged parts are omitted> If intraCellGuardBandDL-r16 for a serving cell indicates intra-cell guard-bands are configured and a UE is configured by higher layers to receive a CSI-RS or detects a DCI format 0_1 indicating to the UE to receive a CSI-RS in one or more RB sets and a set of symbols of a slot in the serving cell, and the UE detects a DCI format 2_0 with bitmap indicating that any RB set from the one or more RB sets is not available for reception, the UE cancels the CSI-RS reception in the set of symbols of the slot in the serving cell. If intraCellGuardBandDL-r16 for a serving cell indicates no intra-cell guard-bands are configured and a UE is configured by higher layers to receive a CSI-RS or detects a DCI format 0_1 indicating to the UE to receive a CSI-RS in a set of symbols of a slot in the serving cell, and the UE detects a DCI format 2_0 with one bit indicating that the serving cell is not available for reception, the UE cancels the CSI-RS reception in the set of symbols of the slot in the serving cell. <Unchanged parts are omitted> =============================== Text Proposal 2 Ends===================================

From Ericsson [5],

------------------------------------------ Text Proposal (TP#1) for 38.214, Section 7 ----------------------------------- *** Unchanged text omitted *** 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstart,μ+GB s,xsize,μ. The RB set s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. For Ngrid,ULsize,μ≤106,51 for μ=0,1, respectively, the UE determines only a single UL RB set and no guard bands. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. For Ngrid,DLsize,μ≤106,51 for μ=0,1, respectively, the UE determines only a single UL RB set and no guard bands. For a carrier with intra-cell guard band(s) or for an UL carrier with Ngrid,ULsize,μ≤106,51 for μ=0,1, respectively, and useInterlacePUCCH-PUCCH is configured in any of BWP-UplinkCommon and BWP-UplinkDedicated, the UE expects N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1 for a BWP i configured by BWP-Downlink or BWP-Uplink. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] *** Unchanged text omitted *** -------------------------------------------------------- End Text Proposal --------------------------------------------------------

From Nokia [9],

# 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands This sub-clause applies to carrier operating with shared spectrum channel access. For operation with shared spectrum channel access, wWhen the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. When UE is provided nrofCRBs-r16=0, carrier does not include intra-cell guard bands. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstart,μ+GB s,xsize,μ. The RB set s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB set according to the default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. When UE is provided nrofCRBs-r16=0 in intraCellGuardBandUL-r16, UE expects RB set to include 10 or 11 RBs for each interlace index defined in Clause 4.4.4.6 in [4, TS 38.211]. When UE is provided nrofCRBs-r16=0 in intraCellGuardBandDL-r16, there are no RB sets present on the carrier. For a carrier with intra-cell guard band(s), the UE expects N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1for a BWP i configured by BWP-Downlink or BWP-Uplink. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] <unchanged text omitted>

Issue A2

From Sharp [10],

Text proposal#1 --------- beginning of text proposal for TS 38.214 ##### 6.1.2.2.3 Uplink resource allocation type 2 In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to NRB-set,ULBWP contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, the UE shall determine the resource allocation in frequency domain within the active uplink BWP as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. For DCI 0_0 monitored in a common search space, the UE shall determine the resource allocation in frequency domain within the active uplink BWP as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP. The uplink RB set is the one that intersects with the downlink RB set of the active downlink BWP in which the UE detects the DCI 0_0. If there is no intersection, the uplink RB set is RB set 0 in the active uplink BWP. -------- Unchanged contents are omitted --------- end of text proposal

Text proposal#2 --------- beginning of text proposal for TS 38.214 # 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstart,μ+GB s,xsize,μ. The RB set s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB set according to the default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB set according to the default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. For a carrier with intra-cell guard band(s), the UE expects N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1for a BWP i configured by BWP-Downlink or BWP-Uplink. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. For a carrier without intra-cell guard band(s), RB sets that overlaps at least partially with the BWP i are numbered in increasing order from 0 to NRB-set,xBWP-1. The UE shall adjust N BWP,istart,μ to RB s0,xstart,μ when N BWP,istart,μ is larger than RB s0,xstart,μ. The UE shall adjust N BWP,isize,μ to RB s1,xend,μ-RB s0,xstart,μ+1 when N BWP,isize,μ is smaller than RB s1,xend,μ-RB s0,xstart,μ+1. [ -------- Unchanged contents are omitted --------- end of text proposal

Issue A5

From LG Electronics [7],

# 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstart,μ+GB s,xsize,μ. The RB set s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. For a carrier with intra-cell guard band(s), the UE expects N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1for a BWP i configured by BWP-Downlink or BWP-Uplink. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on an UL carrier configured with any of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, the UE is indicated that no intra-cell guard-bands are configured for the UL carrier. For UL carrier with no intra-cell guard-bands, the UE expects that the number of common resource blocks in an interlace contained within a RB set s is no less than 10 and no more than 11.

From OPPO [8],

----------------------------------------TP1: Start of 38.214 section 7 -------------------------------------- 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xendsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xendstart,μ+GB s,xsize,μ1. The RB set s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. For a carrier with intra-cell guard band(s), the UE expects N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-10≤s0≤s1≤NRB-set-1for a BWP i configured by BWP-Downlink or BWP-Uplink. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 For a carrier with intra-carrier guard band(s), the UE does not expect to receive a RB set configuration by intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 partially overlapping with a channel on which a channel access procedure is performed in shared spectrum [TS 37.213]. where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. [The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] ----------------------------------------End of 38.214 section 7 --------------------------------------

From Nokia [9],

# 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsize,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. UE determines RB 0,xstart,μ=Ngrid,xstart,μ, RBNRB-set-1,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1, and the remaining start and end CRBs as RB s,xend,μ=GB s,xstart,μ-1 and RB s+1,xstart,μ=GB s,xstart,μ+GB s,xsize,μ. The RB set s consists of RBs,xsize,μ resource blocks where RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB set according to the [default intra-cell GB pattern from [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ]. For a carrier with intra-cell guard band(s), the UE expects N BWP,istart,μ=RB s0,xstart,μ, and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1 where 0≤s0≤s1≤NRB-set,x-1for a BWP i configured by BWP-Downlink or BWP-Uplink. Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1 where NRB-set,xBWP is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier. <omitted text>

Issue B1

From Huawei [3],

## TP#1: TS38.213 v16.1.0 ### 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** For all search space sets within a slot , denote by a set of CSS sets with cardinality of and by a set of USS sets with cardinality of . The location of USS sets , , in is according to an ascending order of the search space set index. Denote by , , the number of counted PDCCH candidates for monitoring for CSS set and by , , the number of counted PDCCH candidates for monitoring for USS set . Denote by MSussj L, 0≤j<Juss, , the number of counted PDCCH candidates for each monitoring location for USS set Sussj, if freqMonitorLocations-r16 is configured. For the CSS sets, a UE monitors PDCCH candidates requiring a total of non-overlapping CCEs in a slot. The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided value 0 for first CORESETs, and is provided value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. Denote by the set of non-overlapping CCEs for search space set and by the cardinality of where the non-overlapping CCEs for search space set are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , . Set Set Set while AND allocate PDCCH candidates for monitoring to USS set ; ; ; end while if freqMonitorLocations-r16 is configured, there are KMLmonitoring locations in frequency domain in the search space set Sussj. Denote by VCCESussj the set of non-overlapping CCEs in each monitoring location of search space set Sussj and by CVCCESussj the cardinality of VCCESussj where the non-overlapping CCEs in each monitoring location of search space set Sussj are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets Sussj, 0≤k≤j . Set ; Set ; Set ; While LMSussj L≤MPDCCHuss AND CVCCESussj≤CPDCCHuss Set l=0; While l+1∙LMSussj L≤MPDCCHuss AND l+1∙CVCCESussj≤CPDCCHuss AND l<KML k=k+1; end while allocate k∙LMSussj L PDCCH candidates for monitoring to USS set MPDCCHuss=MPDCCHuss-k∙LMSussj L; CPDCCHuss=CPDCCHuss-k∙CVCCESussj; j=j+1; end while *** Unchanged text is omitted ***

From Panasonic [12],

… The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided CORESETPoolIndex with value 0 for first CORESETs, and is provided CORESETPoolIndex with value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring. If freqMonitorLocations-r16 indicates to a UE to monitor the search space set over multiple RB sets and if DCI format 2_0 indicates one or more RB sets are unavailable, PDCCH candidates and the corresponding non-overlapping CCEs are not allocated to the RB sets that are indicated as unavailable for DCI format 2_0, starting from a first slot that is at least P symbols after the last symbol of GC-PDCCH carrying DCI format 2_0. …

Issue B2

From Samsung [6],

38.213 TP for proposal 2 10.1 UE procedure for determining physical downlink control channel assignment ----omitted---- For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset-r16 is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset-r16. - if a CORESET is associated with search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set k in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position RBs0+k,DLstart,μ [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RBs0+k,DLstart,μ+NRBoffset and k is indicated by freqMonitoringLocations-r16 if provided for a search space set; otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. A UE does not expect a CORESET to be associated with a search space set configured with freqMonitorLocations-r16 and another search space set not configured with freqMonitorLocations-r16. ----omitted----

38.213 TP for proposal 3 Alt 1 10. UE procedure for receiving control information ----omitted---- If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in Clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. If at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList-r16, the UE is not required to monitor the PDCCH candidate. If a UE is provided availableRB-SetPerCell-r16, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets that are indicated as unavailable for receptions by DCI format 2_0 as described in Clause 11.1.1. If a UE is provided freqMonitorLocation-r16, the UE is not required to monitor PDCCH candidates which are not confined within an RB set defined in Clause 7 TS 38.214. ----omitted----

38.213 TP for proposal 3 Alt 2 ## 10.1 UE procedure for determining physical downlink control channel assignment ## *** Unchanged text is omitted *** For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. - if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-Offset-r16 is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-Offset-r16. - if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in each RB set k in the DL BWP bandwidth of NRBBWP PRBs with starting common RB position RBs0+k,DLstart,μ [6, TS 38.214], where the first common RB of the first group of 6 PRBs has common RB index RBs0+k,DLstart,μ+NRBoffset and k is indicated by freqMonitoringLocations-r16 if provided for a search space set; otherwise, k=0. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-Offset-r16 or NRBoffset=0 if rb-Offset-r16 is not provided. A UE does not expect the CORESET is not confined in any RB set.

Issue B3

From Huawei [3],

## TP#1: TS38.213 v16.1.0 ### 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** - a number of PDCCH candidates per CCE aggregation level by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively; or a number of PDCCH candidates Ms(L) per CCE aggregation level L per RB set if the UE is provided freqMonitorLocation-r16 for the search space set. *** Unchanged text is omitted *** For a search space set s associated with CORESET p, the CCE indexes for aggregation level L corresponding to PDCCH candidate ms,nCI of the search space set and per RB set if any in slot ns,fμ for an active DL BWP of a serving cell corresponding to carrier indicator field value nCIare given by where for any CSS, Yp,ns,fμ=0; for a USS, Yp,ns,fμ=Ap⋅Yp,ns,fμ-1modD, Yp,-1=nRNTI≠0, Ap=39827 for pmod3=0, Ap=39829 for pmod3=1, Ap=39839 for pmod3=2, and D=65537; i=0,⋯,L-1; NCCE,p is the number of CCEs, numbered from 0 to NCCE,p-1, in CORESET p and, if any, per RB set; nCI is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, nCI=0; ms,nCI=0,⋯,Ms,nCIL-1, where Ms,nCIL is the number of PDCCH candidates the UE is configured to monitor for aggregation level L of a search space set s and per RB set if any for a serving cell corresponding to nCI; for any CSS, Ms,maxL=Ms,0L; for a USS, Ms,maxL is the maximum of Ms,nCIL over all configured nCI values for a CCE aggregation level L of search space set s ; the RNTI value used for nRNTI is the C-RNTI. A UE does not expect to be provided freqMonitorLocations-r16 for a search space set s in a serving cell if intraCellGuardBandDL-r16 indicates that no intra-cell guard-bands are configured for the serving cell. *** Unchanged text is omitted *** For all search space sets within a slot , denote by a set of CSS sets with cardinality of and by a set of USS sets with cardinality of . The location of USS sets , , in is according to an ascending order of the search space set index. Denote by , , the number of counted PDCCH candidates for monitoring for CSS set and by , , the number of counted PDCCH candidates for monitoring for USS set . Denote by MSussj L, 0≤j<Juss, , the number of counted PDCCH candidates for each monitoring location for USS set Sussj, if freqMonitorLocations-r16 is configured. *** Unchanged text is omitted ***

Issue C1

From Qualcomm [11],

============TP for 38.213 Section 11.1.1==================================== --Unchanged part omitted------------------------ For each serving cell in the set of serving cells, the UE can be provided: - an identity of the serving cell by servingCellId - a location of a SFI-index field in DCI format 2_0 by positionInDCI - a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes - one or more slot formats indicated by a respective slotFormats for the slot format combination, and - a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId - for unpaired spectrum operation, a reference SCS configuration by subcarrierSpacing and, when a supplementary UL carrier is configured for the serving cell, a reference SCS configuration by subcarrierSpacing2 for the supplementary UL carrier - for paired spectrum operation, a reference SCS configuration for a DL BWP by subcarrierSpacing and a reference SCS configuration for an UL BWP by subcarrierSpacing2 - a location of an available RB set indicator field in DCI format 2_0 that is - one bit, if intraCellGuardBandDL-r16 for the serving cell indicates no intra-cell guard-bands are configured, where a value of '1' indicates that the serving cell is available for receptions, a value of '0' indicates that the serving cell is not available for receptions, by availableRB-SetPerCell-r16, and the serving cell remains available or unavailable for reception until the end of the indicated channel occupancy duration - a bitmap having a one-to-one mapping with the RB sets [6, TS 38.214] of the serving cell, if intraCellGuardBandDL-r16 for the serving cell indicates intra-cell guard-bands are configured, where the bitmap includes NRB,set,DL bits and NRB,set,DL is the number of RB sets in the serving cell, a value of '1' indicates that an RB set is available for receptions, a value of '0' indicates that an RB set is not available for receptions, by availableRB-SetPerCell-r16, and an RB set remains available or unavailable for receptions until the end of the indicated channel occupancy duration. When all bits in the bitmap are ‘0’, in the availableRB-SetPerCell-r16, the availability for all RB sets for reception are considered as unknown, till another DCI format 2_0 is received. - When availableRB-SetPerCell-r16 is not configured for a serving cell configured with CO-DurationPerCell-r16 or slotFormatCombinations, the UE considered all RB sets available for reception when the DCI format 2_0 is detected - a location of a channel occupancy duration field in DCI format 2_0, by CO-DurationPerCell-r16, that indicates a remaining channel occupancy duration for the serving cell starting from a first symbol of a slot where the UE detects the DCI format 2_0 by providing a value from CO-DurationList-r16. The channel occupancy duration field includes maxlog2COdurationListSize,1 bits, where COdurationListSize is the number of values provided by CO-DurationList-r16. If CO-DurationPerCell-r16 is not provided, the remaining channel occupancy duration for the serving cell is a number of slots, starting from the slot where the UE detects the DCI format 2_0, that the SFI-index field value provides corresponding slot formats - a location of a search space set group switching field in DCI format 2_0, by SearchSpaceSwitchTrigger-r16, that indicates a group from two groups of search space sets for PDCCH monitoring for scheduling on the serving cell as described in Clause 10.4. --Unchanged part omitted------------------------ ===============================================================

Issue C2

From OPPO [8],

----------------------------------------TP2: Start of 38.214 section 5.1.6.1.1 -------------------------------------- 5.1.6.1.1 CSI-RS for tracking <Unchanged text is omitted> Each CSI-RS resource, defined in Subclause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions: - the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of - , , or for frequency range 1 and frequency range 2, - , , , , , or for frequency range 2. - a single port CSI-RS resource with density given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping. - the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks within a RB set, or is equal to NBWP,isize resource blocks. <Unchanged text is omitted> ----------------------------------------End of 38.214 section 5.1.6.1.1 --------------------------------------

Issue C3

From Huawei [3],

## TP#2: TS38.214v16.0.0 ##### 6.1.2.2.1 Uplink resource allocation type 0 In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured in pusch-Config and the size of the bandwidth part as defined in Table 6.1.2.2.1-1. For operation with shared spectrum channel access mechansim, The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated RBGs and the union of RB sets overlapping with the indicated RBGs and intra-cell guard bands defined in Clause 7 between the adjacent RB sets overlapping with the indicated RBGs, if any. *** Unchanged text is omitted *** ##### 6.1.2.2.3 Uplink resource allocation type 2 In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and a set of up to N RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. The UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the union of indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. *** Unchanged text is omitted ***

Appendix B: Previous agreements

Agreement: (RAN1#92bis)

- At least for band where absence of Wi-Fi cannot be guaranteed (e.g. by regulation), LBT can be performed in units of 20 MHz.

  - FFS: details on how to perform LBT for as single carrier with bandwidth greater than 20 MHz, i.e., integer multiples of 20 MHz.

Agreement: (RAN1#94bis)

- NR-U should support that a serving cell can be configured with bandwidth larger than 20 MHz.

  - For DL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

    - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on one or more BWPs

    - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on single BWP

    - Option 2: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on a single BWP if CCA is successful at gNB for the whole BWP

    - Option 3: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB

  - Note: CCA is declared to be successful or not in multiples of 20 MHz.

  - FFS for UL operation including some or all of above options can be applied

- Note: Capture the following in TR only after further discussion for down-selecting from the options in RAN1#95.

Agreement: (RAN1#95)

- For wideband operation for both DL and UL,

  - Bandwidth larger than 20 MHz can be supported with multiple serving cells.

  - NR-U should support that a serving cell can be configured with bandwidth larger than 20 MHz.

- For DL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

  - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on one or more BWPs

  - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PDSCH on single BWP

  - Option 2: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on a single BWP if CCA is successful at gNB for the whole BWP

  - Option 3: Multiple BWPs can be configured, single BWP activated, gNB transmits PDSCH on parts or whole of single BWP where CCA is successful at gNB

- For UL operation, the following options for BWP-based operation within a carrier with bandwidth larger than 20 MHz can be considered.

  - Option 1a: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on one or more BWPs

  - Option 1b: Multiple BWPs configured, multiple BWPs activated, transmission of PUSCH on single BWP

  - Option 2: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on a single BWP if CCA is successful at UE for the whole BWP

  - Option 3: Multiple BWPs can be configured, single BWP activated, UE transmits PUSCH on parts or whole of single BWP where CCA is successful at UE

- It is noted that CCA is declared to be successful or not in multiples of 20 MHz.

- Detailed design and potential selection from the above options can be further discussed when specifications are developed considering protocol and RF aspects.

Agreement: (RAN1#AH1901)

- For wideband operation in DL with a single serving cell operation within a carrier with bandwidth larger than 20 MHz

  - Multiple BWPs can be configured, single BWP activated, gNB may transmit PDSCH on parts or whole of single active BWP where CCA is successful at gNB (i.e., option 2 and 3 from previous agreement)

    - FFS: Restrictions on supportable gaps and combinations of gaps between discontiguous blocks where

      - each block spans contiguous (one or) multiple successful LBT sub-bands

      - each gap spans one or multiple contiguous unsuccessful LBT sub-bands

    - FFS: Transmission bandwidth adaptation delay, potentially different delay for e.g., different number of supported gaps, different transmission bandwidths and different positions of the LBT sub-bands where transmissions occur

    - FFS: Limit on the occupied LBT sub-bands due to regulation and coexistence considerations (not intended to imply that regulation and coexistence considerations will not be addressed)

    - FFS: Whether/how to indicate gNB’s transmitted LBT sub-bands

    - FFS: Enhancements to PDCCH/PDSCH configuration/transmission for the parts of BWP where gNB does not transmit due to CCA failure

- Send LS to RAN4 to inform above decision with the description that RAN1 requires RAN4’s feedback on the first three FFS parts in addition to what was requested in earlier LSs.

Agreement: (RAN1#AH1901)

Operation with multiple active BWPs for a carrier on unlicensed bands is not supported for DL or UL at least in Rel-16 NR-U WI.

- Inform RAN2 of this decision

Agreement: (RAN4#90bis)

- It is feasible to operate single carrier wideband operation when when LBT is successful in all LBT sub-bands

  - FFS whether guardbands are needed in between LBT sub-bands or not

- Mode 2 (Single wideband carrier when LBT is successful in a subset of the LBT sub-bands which are contiguous) is feasible at least if PRBs within the guardband of two contiguous LBT sub-bands are not scheduled by gNB.

  - FFS filter adaptation time if PRBs within the guardband of two contiguous LBT sub-bands are scheduled by gNB.

  - is feasible at least for WiFi-like requirements for in-carrier leakage (e.g. 20dbr).

  - FFS what regional regulatory requirements apply in LBT sub-bands where LBT fails.

    - RAN4 will investigate the feasibility whether regional regulatory requirements are met or not for in-carrier leakage.

- Mode 3 (Single wideband carrier when LBT is successful in a subset of the LBT sub-bands which are non-contiguous)

  - is feasible at least if PRBs within the guardband of two contiguous LBT sub-bands are not scheduled by gNB.

  - is feasible at least for WiFi-like requirements for in-carrier leakage (e.g. 20dbr).

  - FFS what regional regulatory requirements apply in LBT sub-bands where LBT fails.

    - RAN4 will investigate the feasibility whether regional regulatory requirements are met or not for in-carrier leakage.

  - FFS what level of in-carrier leakage and blocking requirements can be met at the BS and UE

  - FFS how to specify this in RAN4

  - FFS filter adaptation time if PRBs within the guardband of two contiguous LBT sub-bands are scheduled by gNB.

Agreement: (RAN1#96bis)

For UL transmissions in a serving cell with carrier bandwidth greater than LBT bandwidth, for the case where UE performs CCA before UL transmission, support at least Alt. 1 among the following alternatives

- Alt. 1: UE transmits the PUSCH only if CCA is successful at UE in all LBT bandwidths of the scheduled PUSCH.

- Alt. 2: UE transmits the PUSCH in all or a subset of LBT bandwidths of the scheduled PUSCH for which CCA is successful at the UE.

  - Decision on whether this alternative is supported will depend on feedback from RAN4

  - FFS on restrictions to the subset of LBT bandwidths, e.g., only contiguous LBT bandwidths allowed, based on feedback from RAN4

- Necessity of guard bands within the scheduled PUSCH should be determined by RAN4

- FFS: Whether this applies also to configured grant PUSCH

- FFS: Whether this applies also to PUCCH

Agreement: (RAN1#96bis)

- Support a mechanism for a UE to detect gNB is transmitting across

- Multiple carriers

- Multiple LBT bandwidths in a carrier.

- The following mechanisms are to be considered:

- Option 1: Explicit indication via PDCCH

  - FFS: The type of PDCCH (e.g., group common PDCCH or UE-specific PDCCH)

  - FFS: Signaling details of the indication

- Option 2: Explicit indication via selection of a PDCCH DM-RS sequence from a set of PDCCH DM-RS sequences

  - FFS: Details of the indication

- Option 3: Via UE implementation, i.e., implicit method based on NR-based signal such as DM-RS and/or corresponding PDCCH detection

  - FFS: Which signals/channels or combination of signals/channels could be used by the UE

- Note: Above options are not mutually exclusive

Agreement: (RAN1#97)

When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

- FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

- FFS: Whether and how to support the mechanism at the beginning of DL transmission burst

- FFS: Whether and how to handle the case when GC-PDCCH is not configured or not received by the UE

Conclusion: (RAN1#97)

A UE can receive a PDSCH scheduled within an LBT bandwidth or over multiple LBT bandwidths as per Rel-15 and current agreements in Rel-16.

Conclusion: (RAN1#98)

The following are unchanged from Rel-15 for PDCCH.

- The maximum number of monitored PDCCH candidates per slot and per serving cell.

- The maximum number of non-overlapped CCEs per slot and per serving cell.

- CCE-to-REG mapping rule and hashing function.

Agreement: (RAN1#98)

For CORESET configuration in a serving cell with carrier bandwidth greater than LBT bandwidth,

- For the case where a CORESET is confined within a LBT bandwidth, the search space set configuration associated with the CORESET can have multiple monitoring locations in the frequency domain (per LBT bandwidth)

  - Send an LS to RAN2 informing them of this agreement and providing clarifications on the above if necessary

- Note: For scenarios in which gNB transmits PDCCH/PDSCH on a single BWP if CCA is successful at gNB for the whole BWP, CORESET(s) need not all be confined within an LBT bandwidth, and no specification impact is foreseen

Agreement: (RAN1#98bis)

For a search space set configuration associated with multiple monitoring locations in the frequency domain (as per the previous agreement defining such a search space set associated with a CORESET confined within an LBT bandwidth):

- PRBs allocated by frequencyDomainResources in the CORESET configuration are confined within one of LBT bandwidths within the BWP corresponding to the CORESET.

- Within the search space set configuration associated with the CORESET, each of the one or more monitoring locations in the frequency domain corresponds to (and is confined within) an LBT bandwidth and has a frequency domain resource allocation pattern that is replicated from the pattern configured in the CORESET.

- CORESET parameters other than frequency domain resource allocation pattern are identical for each of the one or more monitoring locations in the frequency domain.

- Include this and the prior agreement on this issue in an LS to RAN2

Agreement: (RAN1#98bis)

The intra-carrier guard bands on a carrier can be semi-statically adjusted with an RB level granularity. The RAN4 minimum guard band requirements are used as the guard bands when no semi-static adjustment is applied.

- The guard bands adjustments do not affect the already agreed restrictions on PUCCH resource allocation.

- FFS: Whether and how to handle the case where the intra-carrier guard bands are part of a resource allocation

Agreement: (RAN1#98bis)

- For UL transmissions in a serving cell with carrier bandwidth greater than the LBT bandwidth, for the case where UE performs CCA before UL transmission, UE transmits on the UL only if CCA is successful at UE in all LBT bandwidths that overlap with the resource allocation for the UL transmission

- The UE is not expected to receive resource allocations in discontiguous LBT bandwidths within a wideband carrier

  - This does not preclude such resource allocation in discontiguous LBT bandwidths being supported by specifications managed by RAN1 in Rel-16.

Agreement: (RAN1#99)

UE determines the number of RB sets (corresponding to LBT bandwidths) and the available PRBs in each RB set, both for DL and UL, based on,

- If configured, the RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 (if UL is configured) configure the lists of intra-carrier guard-bands per cell, e.g.,

  - If intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 are given as {GB1low, GB1high}, …, {GBN-1low, GBN-1high} where GB is given by CRB index, the UE determines

    - The number of RB sets for a cell: N

    - The available PRBs in each RB set: [RBstart, GB1low-1] for RB set #1, [GB1high+1, GB2low-1] for RB set #2,…, [GBN-1high+1, RBend] for RB set#N, where RBstart and RBend corresponds to starting and ending RB index of cell, respectively.

    - Note: {GB1low, GB1high}, …, {GBN-1low, GBN-1high} may be provided separately for DL and UL

- If intraCellGuardBandDL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- If intraCellGuardBandUL-r16 is not configured, then {GB1low, GB1high}, …, {GBN-1low, GBN-1high} is derived from the RAN4 specifications

  - Note: This supersedes a previous agreement

- Note: This addresses the FFS in sections 5.1.2.2 and 6.1.2.2 in 38.214.

Agreement: (RAN1#99)

For the frequency domain resource allocation that is provided with frequencyDomainResources in CORESET configuration,

- Introduce a new RRC parameter rb-Offset (with the value range of 0,1,…,5) in ControlResoureSet IE.

  - If rb-Offset is not configured, rb-Offset is 0

- The bits of the 45-bit bitmap frequencyDomainResources have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the BWP with the starting PRB position as {the first PRB index in the BWP + rb-Offset} for a CORESET.

- FFS: For multi-cluster CORESET configuration, rb-Offset also applies to the RB offset between the starting PRB index of the first 6 PRB group and the first PRB index in each RB set. Full 6 PRB groups are counted till the end of the RB set. The bits in frequencyDomainResources sequentially maps to the 6 RB groups in all RB sets in the BWP.

- Note: Cluster above implies a group of resource blocks that are not contiguous in frequency

Conclusion: (RAN1#99)

For a legacy CORESET configuration, the UE can expect to process PDCCH as per Rel-15 behaviour

Agreement: (RAN1#99)

For a search space set configuration with multiple monitoring locations in the frequency domain,

- Within the SearchSpace IE, the agreed RRC parameter freqMonitorLocations-r16 provides a bitmap (where the first bit in the bitmap corresponds to the first RB set in the BWP, and the second bit corresponds to the second RB set, and so on). For a RB set indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is aligned with {the first PRB of the RB set + rb-Offset provided by the associated CORESET configuration}.

- The frequency domain resource allocation pattern for each monitoring location is determined based on the first A bits in frequencyDomainResources provided by the associated CORESET configuration, where A = floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6).

Agreement: (RAN1#99)

- The RRC parameters intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 include a mechanism to indicate that no intra-carrier guard-bands are configured

  - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP

- For a carrier with intra-carrier guard bands, the UE does not expect that the dedicated BWP is configured to include parts of a RB set.

Agreement: (RAN1#99)

If a UE is configured with a CSI-RS spanning over multiple LBT bandwidths,

- The UE assumes that the CSI-RS is not transmitted if the UE is monitoring DCI format 2_0 carrying an LBT BW indication and detects the DCI format 2_0 indicating any of corresponding LBT bandwidths is not available for DL reception.

Agreement: (RAN1#99)

For CSI-RS for tracking in unlicensed spectrum,

- Text proposal for section 5.1.6.1.1 in TS 38.214:

  - The bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,i size resource blocks, or is equal to NBWP,i size resource blocks.

Agreement: (RAN1#100-e)

- If CORESET p is not configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart, where the first common RB of the first group of 6 consecutive RBs has common RB index 6∙NBWPstart6, i.e., same as in Rel-15.

- If CORESET p is not configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset, where the UE assumes the default value rb-Offset = 0.

- If CORESET p is configured with rb-offset, and is not associated with any search space set configured with freqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- If CORESET p is configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- Note: A bits in above bullets is defined as floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6), as per previous agreement.

- TS 38.213 editor to implement this agreement

Agreement: (RAN1#100bis-e)

For a DL cell without intra-cell guard bands

- The bit-width of available RB-set indicator (if configured) in DCI format 2_0 is equal to 1

- UE does not expect to be configured with search space with freqMonitorLocations-r16

Agreement: (RAN1#100bis-e)

To support UL bandwidth part wider than 20 MHz with no intra-cell guard band, UE can be configured with zero GBs by setting GB width to 0 when configuring intraCellGuardBandUL-r16 (e.g., such gNB creates 4 RB-sets in 80MHz UL carrier).

- Inform RAN2 of this agreement

Agreement: (RAN1#100bis-e)

For an UL carrier without intra-cell guard bands when the parameter useInterlacePUCCH-PUCCH is configured in any of BWP-UplinkCommon and BWP-UplinkDedicated:

- The UL carrier can be configured with NRB-set,UL≥1non-overlapping RB set(s)

- For each RB set except for RB set 0, the starting CRB index is given by startCRB-r16

  - For RB set 0, the starting CRB index is given by Ngrid,ULstart,μ

- The UE expects nrofCRBs-r16 set to 0 for all GBs between two adjacent RB sets within the UL carrier.

- The UE expects N RBs contained in each interlace of each RB set, wherein 10 <= N <= 11.

  - For 30 kHz SCS, the number of RBs within any RB set is between 50 and 55, and for 15 kHz SCS, the number of RBs within any RB set is between 100 and 110

- Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP (from RAN1#99 agreement)

- Note: It’s up to gNB’s configuration to fulfill RAN4 requirement with  e.g., on maximum transmission bandwidth configuration, spectral emission mask, and so on.

- Note: In order to reuse existing PUCCH/PUSCH resource allocation mechanisms, this proposal applies to all supported carrier bandwidths except 10 MHz

- FFS: Whether BWP can be configured to be partially overlapping with a RB set

Agreement: (RAN1#100bis-e)

The number of PDCCH candidates per aggregation level configured by nrofCandidates or nrofCandidates-SFI within a SearchSpace IE applies to each of RB sets configured by freqMonitorLocations-r16.

- nrofCandidates-SFI is 1 for a search space configured with freqMonitorLocations-r16